## ENER 29  Renewable energy in gross energy inland consumption

Key message

The share of renewable energy sources in gross inland energy consumption (GEIC) increased in the EU-27 from 4.4% in 1990 to 7.8% in 2007. The strongest increase came from wind (more than one hundred thirty-fold) and solar energy (eightfold). In absolute terms, biomass accounted for 79.2 % of the increase and wind for 13.1 %. Because the gross inland energy consumption of the EU-27 increased by 8.7% between 1990 and 2007, some of the environmental benefits (e.g. reductions in GHG emissions and air pollution) brought about by an increased share of renewable energy sources were offset.

In 2007, the share of renewable energy in total gross inland consumption in EU-15 was 8%, hence a significant effort will be needed to meet the indicative target of 12 % share of renewables by 2010.

Rationale

Renewable energies are generally considered more environmentally benign than fossil fuels with regard to emissions of greenhouse gases as well as other air pollutants such as SO2 and NOx. The share of energy consumption from renewable energy thus provides a broad indication of progress towards reducing the environmental impact of energy consumption, although its overall impact has to be seen within the context of growth in energy use, the total fuel mix, potential impacts on biodiversity and the extent to which pollution abatement equipment is fitted.

This indicator is closely related to indicator ENER28, “renewable energy in final energy consumption”

Key policy question: how fast the share of renewable energy in total gross energy inland consumption is increasing in Europe?

Key assessment: renewable energy in GEIC

- The contribution of renewable energy sources to gross energy inland consumption (GEIC) increased in the EU-27 from 4.4 % in 1990 to 7.8 % in 2007. For the EU-15, in 2007, the share of renewables in total gross inland consumption in these countries accounted for 8%, falling substantially short of the indicative target set in the White Paper on renewable energy (COM(97) 599 final) of 12 % by 2010 (see Figures 1,2,3).

- Between 1990 and 2007 the average annual growth rate of total renewable energy consumption in EU-27 was 3.9 %. Wind and solar PV showed very high growth rates of respectively 33.4% and 43.5 % followed by solar thermal with 11.4%. Between 2006 and 2007, the annual growth rate of renewables was more than double the average: 8.6 % (see Figure 3).

- The non-EU EEA countries showed an increase of 19.1 % in total renewable energy consumption between 1990 and 2007 compared with 93.0% in EU-27 over the same period. The significant lower penetration rate of renewables in these countries can be explained by the much higher increase in gross energy inland consumption (59.1% compared to 8.7% in EU-27), which led to a decrease in the share of renewable energy in GIEC from 25.7 % in 1990 to 19.2 % in 2007. Almost 2/3 of the renewable energy consumption was hydropower, followed by biomass and waste. Together they account for approximately 85% of the renewable energy consumption in non-EU EEA countries.

- In 2007, globally, the share of renewables in total energy consumption was 12.6 % (compared with the EEA region 8.7%). Combustible renewables and waste accounted for 78 % of the total renewable energy, followed by hydro (17 %). The high share in Africa (77.6 %) is mostly due to the use of wood for example for burning in cooking stoves (combustible renewables are 97 % of the total renewables). In Russia and the Middle East, the share of hydropower is the largest (see Table 1).

Specific policy question: how fast the share of renewable energy from biomass and waste is increasing in Europe?

Specific assessment: renewable energy from biomass and waste in GEIC

- In 2007, renewable energy from biomass and waste accounted for the largest share in total renewable energy (69.8 %) and the largest in GEIC (5.4%). This category represented 79.2 % of the absolute growth in renewables during the period 1990-2007. Between 1990 and 2007, the share of renewable energy from biomass and waste increased by 120.7 % in the EU-27, at an average annual growth of 4.8 %. Biomass and waste can be used to produce electricity and heat and biofuels for transport. It is also seen as one of the main areas for future growth in renewable energy (see Figures 1,2,3).

- Between 2006 and 2007 the total amount of biomass and waste increased by 9.7 %, more than double the average annual growth rate for this category.

Specific policy question: how fast the share of renewable energy from hydro is increasing in Europe?

Specific assessment: renewable energy from hydro in GEIC

- Hydropower consumption increased by 6.2 % over the period 1990-2007 at an average annual growth rate of 0.4 %. Hydropower production has fluctuated substantially in recent years as a result of changing rainfall patterns. In 2007, the share of hydropower in total renewable energy was 18.9 % and 1.5 % of GEIC in the EU-27. Energy consumption from hydropower is not expected to increase significantly in the future due to environmental concerns and a lack of suitable sites, particularly within the EU-15. For example, the Water Framework Directive (2000/60/EC) places a strong emphasis on achieving a good environmental status of European rivers, hence future construction of new hydro-power plants (particularly small-scale hydro) will need to take these constraints into account (link with the workshop report on methodology to assess the small-scale hydro potential ) (see Figures 1 and 3).

- Between 2006 and 2007 the total hydropower consumed increase by 0.3%.

Specific policy question: how fast the share of renewable energy from wind is increasing in Europe?

Specific assessment: renewable energy from wind in GEIC

- Between 1990 and 2007, wind energy in the EU-27 grew by a factor of 134 at an annual growth rate of 33.4 %. This development was largely due to strong growth in Spain and Germany, which together accounted for 64.5 % of all the EU-27 wind production in 2007. Wind power is a fast-growing energy source worldwide, and this trend is expected to continue throughout the EU-27, particularly as offshore developments take off. At present however, wind energy accounts for only 0.5 % of EU-27 GEIC and 6.4 % of renewable energy consumption in the EU-27 (see Figures 1 and 3).

- From 2006 to 2007, consumption from wind energy increased by 26.7 %, slightly less than average over the 17 years period but more than in the preceding years (20 % in 2004-2005 and 17 % in 2005-2006).

Specific policy question: how fast the share of renewable energy from solar is increasing in Europe?

Specific assessment: renewable energy from solar in GEIC

- Between 1990 and 2007, renewable energy from solar (both PV and solar thermal) in the EU-27 grew by a factor of 8.4 at an average annual growth rate of 13.3 %. Solar thermal energy developments in Austria, Germany and Greece benefited greatly from proactive government policy coupled with subsidy schemes and communication strategies that emphasised the benefits of solar thermal. In 2006, Spain passed a law making solar panels compulsory in new and renovated buildings (this resulted in a 65.1 % annual increase in 2007, compared to 27.7 % in 2006). In most Member States solar energy comes from solar thermal energy, rather than electricity generated using photovoltaic (PV) cells. At present the use of PV cells is limited due to relatively high production and installation costs, but represent a medium- to long-term opportunity as costs are beginning to fall (JRC, 2004). Solar energy accounted for 0.9 % of total renewable energy consumption and only 0.1 % of GEIC in 2007 (see Figures 1 and 3).

- Between 2006 and 2007 solar energy grew by 28.3 %, a much slower pace than preceding years

Specific policy question: how fast the share of renewable energy from geothermal is increasing in Europe?

Specific assessment: renewable energy from geothermal in GEIC

- Between 1990 and 2007, the consumption of geothermal energy (both electricity and heat) increased by 80.9 % in the EU-27, at an average annual growth of 3.5 %. The use of geothermal schemes depends on the quality (temperature and density) of the heat available. Geothermal energy contributed only 4.1 % to renewable energy consumption and 0.3 % of GEIC in the EU-27 in 2007, with Italy accounting for around 89 % of the total amount of consumed geothermal energy. There is still significant potential to exploit geothermal heat, particularly in the form of heat pump technology for space heating (IEA, 2004) (see Figures 1 and 3).

- Between 2006 and 2007 geothermal energy grew by 3.5 %, the same as the annual average over 1990 to 2007.

Fig. 1: Contribution of renewable energy sources to primary energy consumption in the EU-27

Data source: Eurostat

Fig. 2: Total primary energy consumption by energy source in 2007, EU-27

Data source: Eurostat

Fig 3: 	Annual average growth rates in renewable energy consumption (%), EU-27

Data source: Eurostat

Table 1: Share of renewable energy in total gross energy inland consumption (in %)

|  | 1990 | 1995 | 2000 | 2005 | 2006 | 2007 |
| --- | --- | --- | --- | --- | --- | --- |
| EEA | 5.6 | 6.4 | 7.0 | 7.7 | 8.1 | 8.7 |
| EU-27 | 4.4 | 5.1 | 5.8 | 6.7 | 7.1 | 7.8 |
| World | 12.8 | 13.3 | 13.2 | 12.8 | 12.9 | 12.6 |
| Africa | 49.8 | 56.6 | 64.5 | 74.5 | 76.3 | 77.6 |
| Middle East | 1.0 | 0.8 | 0.6 | 0.8 | 0.8 | 0.7 |
| United States | 5.2 | 5.3 | 4.8 | 4.7 | 5.0 | 5.0 |
| China | 24.2 | 20.9 | 20.9 | 15.0 | 14.1 | 12.3 |
| India | 16.0 | 13.8 | 13.9 | 9.6 | 9.0 | 8.8 |
| Russia | 3.0 | 3.8 | 3.4 | 3.4 | 3.4 | 3.3 |
| Belgium | 1.3 | 1.3 | 1.3 | 2.4 | 2.9 | 3.1 |
| Bulgaria | 0.6 | 1.6 | 4.2 | 5.6 | 5.5 | 4.7 |
| Czech Republic | 0.2 | 1.4 | 1.5 | 4.0 | 4.3 | 4.7 |
| Denmark | 6.7 | 7.6 | 10.9 | 16.4 | 15.6 | 17.3 |
| Germany | 1.6 | 1.9 | 2.8 | 5.1 | 6.0 | 8.3 |
| Estonia | 4.5 | 8.7 | 10.2 | 10.6 | 9.8 | 10.0 |
| Ireland | 1.6 | 1.4 | 1.6 | 2.4 | 2.7 | 2.9 |
| Greece | 4.9 | 5.3 | 5.0 | 5.2 | 5.7 | 5.0 |
| Spain | 7.0 | 5.4 | 5.7 | 6.0 | 6.5 | 7.0 |
| France | 7.2 | 7.7 | 7.0 | 6.3 | 6.6 | 7.0 |
| Italy | 4.2 | 4.8 | 5.2 | 6.5 | 7.0 | 6.9 |
| Cyprus | 0.4 | 2.1 | 1.8 | 1.9 | 1.9 | 2.5 |
| Latvia | 13.2 | 27.2 | 31.8 | 33.0 | 31.0 | 29.7 |
| Lithuania | 2.0 | 5.7 | 9.2 | 8.8 | 9.3 | 8.9 |
| Luxembourg | 1.3 | 1.4 | 1.5 | 1.6 | 1.7 | 2.5 |
| Hungary | 1.8 | 2.4 | 2.1 | 4.4 | 4.8 | 5.3 |
| Malta | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Netherlands | 1.4 | 1.5 | 2.4 | 3.4 | 3.6 | 3.6 |
| Austria | 20.0 | 21.8 | 22.9 | 21.1 | 22.3 | 23.8 |
| Poland | 1.6 | 3.9 | 4.2 | 4.8 | 5.1 | 5.1 |
| Portugal | 18.7 | 16.2 | 15.3 | 13.2 | 17.1 | 17.7 |
| Romania | 4.1 | 5.9 | 10.9 | 12.6 | 11.7 | 11.9 |
| Slovenia | 4.6 | 9.3 | 12.3 | 10.6 | 10.5 | 10.0 |
| Slovakia | 1.6 | 2.8 | 2.8 | 4.3 | 4.6 | 5.5 |
| Finland | 19.0 | 21.1 | 23.8 | 23.1 | 22.7 | 22.6 |
| Sweden | 24.9 | 25.9 | 31.4 | 29.6 | 29.4 | 30.9 |
| United Kingdom | 0.5 | 0.9 | 1.1 | 1.7 | 1.9 | 2.1 |
| Turkey | 18.4 | 17.3 | 13.1 | 11.9 | 11.1 | 9.5 |
| Iceland | 64.8 | 67.5 | 71.3 | 72.9 | 74.9 | 74.9 |
| Norway | 53.1 | 48.8 | 50.9 | 40.3 | 46.5 | 46.7 |
| Switzerland | 13.7 | 16.4 | 16.5 | 16.7 | 16.3 | 18.8 |

Data source: Eurostat, IEA
Note: No data for Liechtenstein is available from Eurostat. No data available for Iceland for 2007. Data for World, Middle East, Africa, United States, China, India and Russia are extracted from IEA database.

Policy target:

The White Paper on renewable energy (COM(97) 599 final) sets an indicative target of 12 % of renewables in total GEIC in the EU-15 by 2010. The contribution of renewable energy sources to gross energy inland consumption (GEIC) in EU-15 was 8 % in 2007, falling significantly short of the 12% indicative target (see Figure 4 and Table 2). As can be seen in Figure 4 already five countries reached their target (Sweden, Austria, Finland, Portugal and Denmark). The other ten are still significantly below the target. Especially the UK, Luxembourg, Ireland, Belgium, The Netherlands and Greece are very much behind schedule and it is estimated they will not reach the target in time.

Fig 4: 	Share of RE in GEIC, compared to target in COM(97) 599 final (%, in 2007)

Data source: Eurostat

Indicator rationale

Environmental context

The share of electricity consumption from renewable energy sources provides a broad indication of progress towards reducing the environmental impact of electricity consumption on the environment as renewable electricity is generally considered to have lower life-cycle environmental impact per unit of electricity produced than fossil-fuelled power plants (link with the EEA LCA analysis). Increasing the share of renewables in electricity consumption will help the EU to reduce the GHG emissions from power generation but the overall impact will depend on which generation sources are being replaced in the energy system.

Emissions of air pollutants are also generally lower for renewable electricity production than for electricity produced from fossil fuels. The exception to this is the incineration of Municipal and Solid Waste (MSW), which due to high costs of separation, usually involves the combustion of some mixed wastes including materials contaminated with heavy metals. Emissions to the atmosphere from MSW incineration are subject to stringent regulations including tight controls on emissions of cadmium, mercury, and other such substances.

The exploitation of renewable energy sources may have negative impacts on landscapes, habitats and ecosystems, although many impacts can be minimised through careful site selection. Hydropower schemes in particular can have adverse impacts including flooding, disruption of ecosystems and hydrology, and socio-economic impacts if resettlement is required (for large hydro). Some solar photovoltaic schemes require relatively large quantities of heavy metals in their construction and geothermal energy can release pollutant gases carried by hot fluids if not properly controlled. Wind turbines can have visual impacts on the areas in which they are sited. Some types of biomass and biofuel crops have considerable land, water and agricultural input requirements such as fertilisers and pesticides.

Policy context

- Council adopted on 6 April 2009 the climate-energy legislative package containing measures to fight climate change and promote renewable energy. This package is designed to achieve the EU's overall environmental target of a 20 % reduction in greenhouse gases and a 20 % share of renewable energy in the EU's total energy consumption by 2020.The climate action and renewable energy (CARE) package includes the following main policy documents

Directive 2009/29/ec of the European parliament and of the Council amending directive 2003/87/ec so as to improve and extend the greenhouse gas emission allowance trading scheme of the community

Directive 2009/31/ec of the European parliament and of the Council on the geological storage of carbon dioxide

Directive 2009/28/ec of the European parliament and of the Council on the promotion of the use of energy from renewable sources

Community guidelines on state aid for environmental protection (2008/c 82/01)

Directive 2008/101/ec of the European parliament and of the Council amending directive 2003/87/ec so as to include aviation activities in the scheme for greenhouse gas Emission allowance trading within the community

Regulation (ec) no 443/2009 of the European parliament and of the Council setting emission performance standards for new passenger cars as part of the community’s integrated approach to reduce CO2 emissions from light-duty vehicles

- Directive on GHG emissions of fuels and biofuels; COM(2007) 18 final/2

Sets targets for the GHG emissions from different fuel types (e.g. by improving refinery technologies) and allows the blending of up to 10 % of biofuels into diesel and petrol.

- Strategic Energy Technology Plan (SET-plan); COM(2007) 723

Focuses on increasing the competitiveness of new renewable energy technologies. It aims at identifying technologies that require better coordination of Member State policies or the development of public-private partnerships with the industry.

- White Paper: Energy for the future - renewable sources of energy; COM(97) 599 final

Sets a target for the EU-15 countries for a share of 12 % RE in GEIC.

- Directive on  Waste; Directive 2006/12/EC

it requires all EU Member States to take the necessary measures to ensure that waste is treated and disposed of correctly, sets targets for re-use and recycling, and requires Member States to draw up binding national programmes for waste prevention.

References

COD/2008/0013 - Proposal for a Directive of the European Parliament and of the Council amending Directive 2003/87/EC so as to improve and extend the greenhouse gas emission allowance trading system of the Community

COD/2008/0014 - Proposal for a Decision of the European Parliament and of the Council on the effort of Member States to reduce their greenhouse gas emissions to meet the Community’s greenhouse gas emission reduction commitments up to 2020

COM(2007) 18 final/2 - Directive of the European Parliament and of the Council amending Directive 98/70/EC as regards the specification of petrol, diesel and gas-oil and introducing a mechanism to monitor and reduce greenhouse gas emissions from the use of road transport fuels and amending Council Directive 1999/32/EC, as regards the specification of fuel used by inland waterway vessels and repealing Directive 93/12/EEC

COM(2007) 723 - A European Strategic Energy Technology plan (set-plan)

COM(97) 599 final - Energy for the future: Renewable sources of energy. White Paper for a Community strategy and action plan.

Directive 2001/77/EC - Directive of the European Parliament and of the Council of 27 September 2001 on the promotion of electricity produced from renewable energy sources in the internal electricity market.

Directive 2003/30/EC - Directive 2003/30/EC on the promotion of the use biofuels or other renewable fuels for transport, Brussels, 2003

Directive 2006/12/EC - Directive 2006/12/EC of the European Parliament and of the Council of 5 April 2006 on waste, Brussels, 2006

Directive 2009/28/EC  - Directive 2009/28/EC of the European Parliament and of the Council of 23 April 2009 on the promotion of the use of energy from renewable sources and amending and subsequently repealing Directives 2001/77/EC and 2003/30/EC, Brussels, 2009

EU (2009) Climate action and renewable energy package (CARE Package)

http://ec.europa.eu/environment/climat/climate_action.htm

IEA (2004) - International Energy Agency Geothermal Energy Annual Report 2004.

JRC (2004) - PV Status Report 2004: Research, Solar Cell Production and Market Implementation of Photovoltaics, European Commission Joint Research Centre. http://streference.jrc.cec.eu.int/pdf/PV %20Status %20Report %202004.pdf.

Meta data

Technical information

- Data source:
Eurostat (historic data), http://ec.europa.eu/eurostat/   
Renewable energy consumption is one of the European Environment Agency’s core-set indicators. More information can be found at http://themes.eea.eu.int/IMS/CSI.
IEA Data Services, http://data.iea.org/IEASTORE/DEFAULT.ASP

- Description of data / Indicator definition:
The share of renewable energy consumption is the ratio between gross inland consumption of energy from renewable sources (TOE) and total gross inland energy consumption (TOE) calculated for a calendar year, expressed as a percentage. Both renewable energy and total energy consumption are measured in thousand tonnes of oil equivalent (ktoe).
Renewable energy sources are defined as renewable non-fossil sources: wind, solar, geothermal, wave, tidal, hydropower, biomass, landfill gas, sewage treatment plant gas and biogases.
Projections are for 2020 from the POLES (IPTS) Baseline and GHG Reduction Scenario and from the WEO 2007 (IEA) Reference and Alternative Policy Scenario.

- Geographical coverage:
The Agency had 32 member countries at the time of writing of this fact sheet. These are the 27 European Union Member States and Turkey, plus Iceland, Norway, Liechtenstein and Switzerland.
No energy data available for Liechtenstein, no 2007 data available for Iceland. 
Data for World, United States, China, India, Russia, the Middle East and Africa.

- Temporal coverage: 1990-2007, projections 2020.

- Methodology and frequency of data collection:
Data collected annually.
Eurostat definitions for energy statistics http://circa.europa.eu/irc/dsis/coded/info/data/coded/en/Theme9.htm
Eurostat metadata for energy statistics http://epp.eurostat.ec.europa.eu/cache/ITY_SDDS/EN/nrg_base.htm

- Methodology of data manipulation:
Renewable energy consumption is the ratio between the gross inland consumption of energy from renewable sources and the total gross inland energy consumption calculated for a calendar year.
The coding (used in the Eurostat New Cronos database) and specific components of the indicator are:
Numerator: solar energy 5530 gross inland consumption 100900 + biomass and waste 5540 gross inland consumption 100900 + geothermal energy 5550 gross inland consumption 100900 + hydropower 5510 gross inland consumption 100900 + wind energy 5520 gross inland consumption 100900.
Denominator: (total) gross inland consumption (of energy) 100900
Average annual rate of growth calculated using: [(last year/base year) ^ (1/number of years) –1]*100
Solar-thermal: 100900 Gross inland consumption (5530 solar energy) – 100100 Primary production (5534 Photovoltaic power)
IEA data:
Report ‘Energy balances Non-OECD countries’ and ‘Energy balances OECD countries’, table ‘Energy balances (ktoe)’, products ‘Hydro’, ‘Geothermal’, ‘Solar/Wind/Other’, ‘Combustible renewables and waste’ and ‘Total’, flow ‘Total Primary Energy Supply’.
Eurostat data set and IEA data set show small inconsistencies of < 1%.
Projections:
POLES: Gross inland consumption Other (EJ)/Gross inland consumption Total (EJ)
WEO 2007: Primary energy demand Biomass and waste (Mtoe)/Total primary energy demand (Mtoe)
Primary energy demand Hydro (Mtoe)/Total primary energy demand (Mtoe)
Primary energy demand Other renewables (Mtoe)/Total primary energy demand (Mtoe)

Qualitative information

- Strengths and weaknesses (at data level)
Data have been traditionally compiled by Eurostat through the annual Joint Questionnaires, shared by Eurostat and the International Energy Agency, following a well established and harmonised methodology. Methodological information on the annual Joint Questionnaires and data compilation can be found in Eurostat's web page for metadata on energy statistics. http://epp.eurostat.ec.europa.eu/cache/ITY_SDDS/EN/nrg_quant_sm1.htm See also information related to the Energy Statistics Regulation http://www.europarl.europa.eu/oeil/file.jsp?id=5431232

- Reliability, accuracy, robustness, uncertainty (at data level):
Biomass and wastes, as defined by Eurostat, cover organic, non-fossil material of biological origin, which may be used for heat production or electricity generation. They comprise wood and wood waste, biogas, municipal solid waste (MSW) and biofuels. MSW comprises biodegradable and non-biodegradable wastes produced by different sectors. Non-biodegradable municipal and solid wastes are not considered to be renewable, but current data availability does not allow the non-biodegradable content of wastes to be identified separately, except for industry.
The indicator measures the relative consumption of energy from renewable sources in total energy consumption for a particular country. The share of renewable energy could increase even if the actual energy consumption from renewable sources falls. Similarly, the share could fall despite an increase in energy consumption from renewable sources. CO2 emissions depend not on the share of renewables but on the total amount of energy consumed from fossil sources. Therefore, from an environmental point of view, attaining the 2010 target for the share of renewable energy does not necessarily imply that CO2 emissions from energy consumption will fall.

- Overall scoring (historical data)

Relevance: 1

Accuracy: 2

Comparability over time: 1

Comparability over space: 1

Iceland, Norway, Switzerland, Turkey