## ENER27 Electricity production by fuel

Key message

Fossil fuels and nuclear energy continue to dominate the fuel mix for electricity production in EU-27. In 2007, the share in total gross electricity production of the electricity generated from fossil fuels was 55.4 %, and of the electricity generated from nuclear was 27.9 %. By comparison, the electricity generated from renewable sources was 15.7% (in 2007). The total electricity production increased significantly by 35.0 % since 1990, thus offsetting some of the emissions reductions achieved due to fuel switching from solid fuels to natural gas.

Rationale

The trend in electricity production by fuel provides a broad indication of the environmental impacts associated with electricity production. The type and the extent of the related environmental pressures depends upon the type and the amount of fuels used for electricity generation as well as the use of abatement technologies.

Key policy question: is the electricity production becoming less carbon intensive in Europe?

Key assessment: electricity production by fuel

- Electricity production from fossil fuels continues to dominate total gross electricity production in EU-27, with an almost constant share of 55 % all the way from 1990 to 2007. Natural gas was the fuel of choice for new power plants between 1990-2007, choice mainly driven by economic considerations (more advantageous gas prices compared to coal) and environmental concerns. This fuel switching led to a decrease in greenhouse gas and air pollution emissions from public power generation but increased the dependency on imported fuels (see ENER12). However, with an increase in natural gas prices relative to coal since 1999 (IEA, 2005) as well as a decrease in hydro electricity production since 2002 due to decreased rainfall, the use of coal in power generation has increased in recent years. Although the share of coal in electricity production has remained more or less constant since 1999, its use has increased in absolute terms due to the overall increase in electricity demand. Consequently, emissions from public power generation have begun to rise again (see ENER 01, 06, 07, 08 and 09)

- In 2007, the average carbon intensity of the electricity production in EU-27 was 392 grCO2 /kWh.

Specific policy question: what are the trends concerning the electricity production from renewable sources in Europe?

Specific assessment: electricity production from renewable sources

- The total electricity produced from renewable sources increased by 67.8 % between 1990 and 2007, at an annual average growth rate of 3.1 %. The absolute growth of electricity production from renewable sources between 1990 and 2007 was 212 TWh (see Figure 1). However, 1 percentage point of this increase was observed from 2006-2007 alone, which could be largely attributed to increased renewable electricity production in Germany (+26.5 %) and Spain (+13.5 %). Substantial growth will be required to meet the indicative EU-27 target of a 20 % share of renewable electricity in final electricity consumption by 2020 (see also ENER 30 and ENER 29).

Specific policy question: what are the trends concerning the electricity production from natural gas in Europe?

Specific assessment: electricity production from natural gas

- The total electricity produced from natural gas increased by 252.1 % between 1990 and 2007, at an annual average

growth rate of 7.7 %. The absolute growth of electricity production from natural gas between 1990 and 2007 was 544 TWh. The primary motive for the switch to gas was economic, with low gas prices for much of the 1990s compared to coal and stricter environmental legislation. Because of this, significant investments were made in the transportation infrastructure for the delivery of gas from within and outside the EU-27 (see Figure 2). This rapid increased in gas demand also contributed to the increase in fossil fuels imports (see ENER12).

Specific policy question: what are the trends concerning the electricity production from solid fuels in Europe?

Specific assessment: electricity production from solid fuels

- The total electricity produced from solid fuels increased by 6.5 % between 1990 and 2007, at an annual average rate of 0.4 %. The absolute growth of electricity production from solid fuels between 1990 and 2007 was 61 TWh (see Figure 2) The electricity production from coal and lignite picked up in recent years due to narrowing of the price differential between solid fuels and natural gas and security of supply reasons (see also ENER 12).

Specific policy question: what are the trends concerning the electricity production from nuclear power plants in Europe?

Specific assessment: electricity production from nuclear

- The total electricity produced from nuclear increased by 17.7 % between 1990 and 2007, at an annual average rate of 1.0 %. The absolute growth of electricity production from nuclear between 1990 and 2007 was 140 TWh. The share of electricity production from nuclear in gross electricity production declined in recent years due to the fact that few additional nuclear plants have been built. However, in recent years an increased interest towards building new nuclear power plants (or extending existing ones) can be observed in countries like the UK, the Baltic States, Poland, Sweden, and Finland or extending the life times of existing NPP’s (for instance in the Netherlands) due to concerns over security of supply, high volatility of energy commodity prices and climate change (see also ENER 13).

Fig. 1: Gross electricity production by fuel, EU-27

Data Source: Eurostat (Historic data)

Note Data shown are for gross electricity production and include electricity production from both public plants and auto-producers. Renewables include electricity produced from hydro (excluding pumping), biomass, municipal waste, geothermal, wind and solar PV. The share of renewables presented in the chart is that for production and hence does not correspond to the share, for consumption, as required by Directive 2001/77/EC. The difference between both shares is accounted for by the net balance between imports and exports of electricity. The EU-27 value for 1990 includes (former) West Germany only and since 1991 it refers to Germany. ‘Other fuels’ include electricity produced from power plants not accounted for elsewhere, such as those fuelled by certain types of industrial wastes. It also includes the electricity generated as a result of pumping in hydro-power stations.

Fig. 2: Share of electricity production by fuel type, 1990-2007 (%), EU-27

Data source: Eurostat

Note: Electricity produced from pumping in hydro power plants is not considered a renewable source of energy and it is not shown in the chart.

Table 1: Share of electricity production by fuel type in 2007

|  | Coal and lignite (%) | Oil (%) | Natural and derived gas (%) | Nuclear (%) | Renewables (%) | Other fuels (%) | Total gross electricity production (TWh) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EEA | 27.6 | 3.2 | 22.8 | 25.6 | 19.8 | 1.0 | 3,764.8 |
| EU-27 | 29 | 3 | 23 | 28 | 16 | 1 | 3,357.9 |
| World | 40.8 | 5.6 | 21.5 | 13.7 | 18.4 | 0.0 | 19,854.9 |
| Africa | 43.2 | 11.0 | 27.5 | 1.8 | 16.4 | 0.0 | 618.1 |
| Middle East | 5.2 | 34.9 | 56.7 | 0.0 | 3.2 | 0.0 | 713.6 |
| China | 80.3 | 1.0 | 1.9 | 1.9 | 15.0 | 0.0 | 3,318.2 |
| India | 68.2 | 4.1 | 8.5 | 2.1 | 17.1 | 0.0 | 803.4 |
| Russia | 16.0 | 1.7 | 48.7 | 15.8 | 17.9 | 0.0 | 1,015.3 |
| United States | 48.6 | 1.8 | 21.1 | 19.2 | 9.2 | 0.0 | 4,348.9 |
| Belgium | 7.3 | 0.9 | 30.7 | 54.3 | 4.5 | 2.3 | 88.8 |
| Bulgaria | 51.7 | 1.3 | 5.6 | 33.8 | 6.7 | 0.8 | 43.3 |
| Czech Republic | 61.0 | 0.1 | 4.8 | 29.7 | 3.9 | 0.5 | 88.2 |
| Denmark | 50.8 | 2.8 | 17.7 | 0.0 | 28.3 | 0.4 | 39.2 |
| Germany | 47.3 | 1.8 | 13.3 | 22.2 | 14.8 | 0.7 | 634.0 |
| Estonia | 93.7 | 0.3 | 4.8 | 0.0 | 1.2 | 0.0 | 12.2 |
| Ireland | 27.2 | 7.0 | 54.8 | 0.0 | 9.8 | 1.2 | 28.2 |
| Greece | 54.6 | 15.2 | 21.7 | 0.0 | 7.2 | 1.3 | 63.5 |
| Spain | 24.1 | 6.1 | 31.0 | 18.2 | 19.6 | 0.9 | 302.8 |
| France | 4.3 | 1.1 | 4.5 | 77.2 | 12.0 | 1.0 | 569.8 |
| Italy | 14.1 | 11.3 | 56.8 | 0.0 | 15.7 | 2.2 | 313.8 |
| Cyprus | 0.0 | 100.0 | 0.0 | 0.0 | 0.0 | 0.0 | 4.9 |
| Latvia | 0.0 | 0.4 | 40.3 | 0.0 | 59.3 | 0.0 | 4.8 |
| Lithuania | 0.1 | 2.9 | 17.2 | 70.2 | 4.1 | 5.5 | 14.0 |
| Luxembourg | 0.0 | 0.0 | 72.7 | 0.0 | 7.4 | 19.8 | 4.0 |
| Hungary | 18.5 | 1.3 | 38.4 | 36.7 | 5.1 | 0.0 | 40.0 |
| Malta | 0.0 | 100.0 | 0.0 | 0.0 | 0.0 | 0.0 | 2.3 |
| Netherlands | 24.1 | 2.2 | 60.6 | 4.1 | 8.9 | 0.1 | 103.2 |
| Austria | 9.9 | 2.0 | 17.7 | 0.0 | 66.0 | 4.4 | 63.4 |
| Poland | 91.4 | 1.4 | 3.2 | 0.0 | 3.4 | 0.6 | 159.3 |
| Portugal | 26.3 | 10.3 | 27.8 | 0.0 | 34.9 | 0.7 | 47.2 |
| Romania | 40.7 | 1.8 | 19.1 | 12.5 | 26.0 | 0.0 | 61.7 |
| Slovenia | 36.5 | 0.2 | 3.0 | 37.9 | 22.4 | 0.0 | 15.0 |
| Slovakia | 17.1 | 2.5 | 7.3 | 54.7 | 17.7 | 0.8 | 28.1 |
| Finland | 26.3 | 0.6 | 13.7 | 28.8 | 30.1 | 0.5 | 81.2 |
| Sweden | 0.4 | 0.7 | 1.0 | 45.0 | 52.5 | 0.3 | 148.8 |
| United Kingdom | 34.5 | 1.2 | 41.9 | 15.9 | 5.1 | 1.3 | 396.1 |
| Turkey | 27.3 | 3.4 | 50.2 | 0.0 | 19.0 | 0.1 | 191.6 |
| Iceland | 0.0 | 0.0 | 0.0 | 0.0 | 100.0 | 0.0 | 9.9 |
| Norway | 0.0 | 0.0 | 0.6 | 0.0 | 98.4 | 0.9 | 137.5 |
| Switzerland | 0.0 | 0.3 | 1.1 | 41.1 | 55.0 | 2.5 | 67.9 |

Data Source: EEA. Eurostat. IEA

Notes: The share of renewables above refers to production and therefore it does not necessarily match exactly the share. for consumption. as required by Directive 2001/77/EC. The difference between both shares is accounted for by the net balance between imports and exports of electricity. ‘Other fuels’ include electricity produced from power plants not accounted for elsewhere. such as those fuelled by certain types of industrial wastes. It also includes the electricity generated as a result of pumping in hydro-power stations. For Iceland no data for 2007 were available from Eurostat, the 2006 data were used as an estimate.

Indicator rationale

Environmental context

The indicator shows the evolution of the shares of electricity production from different fuels in total gross electricity production and aims to indicate to what extent there has been a decarbonisation of the electricity production in Europe. Electricity production from fossil fuels (such as crude oil, oil products, hard coal, lignite and natural and derived gas) can provide a proxy indicator of resource depletion, CO2 and other greenhouse gas emissions and air pollution levels (e.g. SO2 and NOX). The degree of environmental impact depends on the relative share of different fossil fuels and the extent to which pollution abatement measures are used. Natural gas, for instance, has approximately 40 % less carbon content than coal per unit of energy content, and 25 % less carbon content than oil, and contains only marginal quantities of sulphur.

The level of nuclear electricity production provides an indication of the trends in the amount of nuclear waste generated and of the risks associated with radioactive leaks and accidents. Increasing consumption of nuclear energy at the expense of fossil fuels would on the other hand contribute to reductions in CO2 emissions.

Renewable electricity production is a measure of the contribution from technologies that are, in general, more environmentally benign, as they produce less net CO2 and of other pollutants. Renewable electricity can, however, have impacts on landscapes and ecosystems (for example, potential flooding and changed water levels from large hydro power) and the incineration of municipal waste (which is generally made up of both renewable and non-renewable material) may also generate local air pollution (EEA study on  LCA analysis)

The efficiency with which electricity is produced also determines the scale of the environmental impacts of electricity production and consumption (see ENER19), as it determines the amount of input fuel required to generate a given quantity of electricity.

The impact also depends upon the total amount of electricity demanded and hence the level of electricity production required (see ENER18 for more details on electricity consumption). Thus another way of reducing energy-related pressures on the environment includes using less electricity on the demand-side, through improved efficiency, conservation or a combination of the two.

Policy context

- Council adopted on 6 April 2009 the climate-energy legislative package containing measures to fight climate change and promote renewable energy. This package is designed to achieve the EU's overall environmental target of a 20 % reduction in greenhouse gases and a 20 % share of renewable energy in the EU's total energy consumption by 2020.The climate action and renewable energy (CARE) package includes the following main policy documents

Directive 2009/29/ec of the European parliament and of the Council amending directive 2003/87/ec so as to improve and extend the greenhouse gas emission allowance trading scheme of the community

Directive 2009/31/ec of the European parliament and of the Council on the geological storage of carbon dioxide

Directive 2009/28/ec of the European parliament and of the Council on the promotion of the use of energy from renewable sources

Community guidelines on state aid for environmental protection (2008/c 82/01)

Directive 2008/101/ec of the European parliament and of the Council amending directive 2003/87/ec so as to include aviation activities in the scheme for greenhouse gas Emission allowance trading within the community

Regulation (ec) no 443/2009 of the European parliament and of the Council setting emission performance standards for new passenger cars as part of the community’s integrated approach to reduce CO2 emissions from light-duty vehicles

- Second Strategic Energy Review; COM(2008) 781 final- active web link to the policy document

Strategic review on short, medium and long term targets on EU energy security.

References

COM(2008) 781 final  Second Strategic Energy Review

Directive 2009/28/EC  - Directive 2009/28/EC of the European Parliament and of the Council of 23 April 2009 on the promotion of the use of energy from renewable sources and amending and subsequently repealing Directives 2001/77/EC and 2003/30/EC, Brussels, 2009

EEA (2008) Annual European Community greenhouse gas inventory 1990 – 2006 and inventory report 2008

EU (2009) Climate action and renewable energy package (CARE Package); http://ec.europa.eu/environment/climat/climate_action.htm

IEA (2005): Electricity information 2004 – IEA statistics.

UN (1998): Kyoto Protocol to the United Nations Framework Convention on Climate Change; adopted at COP3 in Kyoto. Japan. on 11 December 1997

Treaty of Accession to the European Union. Annex II. Part 12. page 588. which amends Directive 2001/77/EC in order to set targets for new Member States on the contribution of renewable energy to electricity generation.

Meta data

Technical information

- Data source:
Electricity production by fuel and total gross electricity generation: Eurostat (historical data): http://ec.europa.eu/eurostat/

Energy balances: Electricty output in GWh: IEA http://data.iea.org/IEASTORE/DEFAULT.ASP

- Description of data/Indicator definition: 
Total gross electricity generation covers gross electricity generation in all types of power plants. The gross electricity generation at the plant level is defined as the electricity measured at the outlet of the main transformers. i.e. the consumption of electricity in the plant auxiliaries and in transformers is included. 
Electricity production by fuel is the gross electricity generation from plants utilising the following fuels: coal and lignite. oil. nuclear. natural and derived gas. renewables (wind. hydro. biomass and waste. solar PV and geothermal) and other fuels. The latter include electricity produced from power plants not accounted for elsewhere. such as those fuelled by certain types of industrial wastes which are not classed as renewable. Other fuels also include the electricity produced as a result of pumping in hydro power stations.
The share of each fuel in electricity production is taken as the ratio of electricity production from the relevant category against total gross electricity generation. It should be noted that the share of renewable electricity in this indicator. based on production. is not directly comparable with the share required under Directive 2001/77/EC which is based upon the share of renewables in electricity consumption. The difference between both shares is accounted for by the net balance between imports and exports of electricity and by how much domestic electricity generation is increased or reduced as a result.
Units: Electricity generation is measured in either GWh or TWh (1000 GWh)

Projections are for 2020 from the POLES (IPTS) Baseline and GHG Reduction Scenario and from the WEO 2007 (IEA) Reference and

Alternative Policy Scenario.

- Geographical coverage:
The Agency had 32 member countries at the time of writing of this fact sheet. These are the 27 European Union Member States and Turkey. plus the EFTA4 countries (Iceland. Switzerland and Norway). In addition, data are reported Africa, the Middle East, China, India, Russia, the United States and the whole World.

- Temporal coverage: 1990-2007. projections 2020

- Methodology and frequency of data collection: Data collected annually.
Eurostat definitions for energy statistics http://circa.europa.eu/irc/dsis/coded/info/data/coded/en/Theme9.htm
Eurostat metadata for energy statistics http://epp.eurostat.ec.europa.eu/cache/ITY_SDDS/EN/nrg_base.htm

- Methodology of data manipulation:
Average annual rate of growth calculated using: [(last year/base year) ^ (1/number of years) –1]*100
Share of electricity production by fuel calculated as ratio of electricity production by fuel type to total gross electricity generation.
The coding (used in the Eurostat database) and specific components of the indicator are:
	• Coal and lignite: gross electricity generation coal fired power stations 107006 + gross electricity generation lignite fired power 	stations 107007
	• Oil: gross electricity generation oil fired power stations 107008
	• Natural and derived gas: gross electricity generation natural fired power stations 107009 + gross electricity generation derived-	gas fired power stations 107010
	• Nuclear: gross electricity generation nuclear power plants 107003
	• Renewables: gross electricity generation biomass fired power stations 107011 + gross electricity generation geothermal power 	plants 107002 + primary production 100100 hydro power 5510 + gross production from photovoltaic systems 107023 + gross 	electricity generation wind turbines 107005
	• Other fuels: gross electricity generation other power stations 107012 + pumping in hydro power stations (calculated as gross 	electricity generation hydro power plants 107001 Electrical energy 6000 - primary production 100100 hydro power 5510)
It should be noted that in the Eurostat database ‘Other fuels – 107012’ also includes ‘gross production from photovoltaic systems - 107023’ and although almost negligible in overall terms it has been subtracted from 107012 in the calculation of the indicator.
For the denominator. where required: total gross electricity generation 107000

The coding (used in the IEA database) and specific components of the indicator are:

• Coal and lignite: electricity output in GWh Coal and coal products

• Oil: electricity output in GWh Crude. NGL and Feedstocks + electricity output in GWh Petroleum products

• Natural and derived gas: electricity output in GWh Natural Gas

• Nuclear: electricity output in GWh Nuclear

• Renewables: electricity output in GWh Hydro +  electricity output in GWh Geothermal +  electricity output in GWh Solar/Wind/Other +

electricity output in GWh Combustable Renewables and Waste +  electricity output in GWh Heat Production from non-specified

comb.fuels + electricity output in GWh Heat

• Other fuels: n/a

POLES: Net electricity generation (TWh, EU-27)

WEO 2007: Total generation (TWh)

Qualitative information

- Strengths and weaknesses (at data level)
Data has been traditionally compiled by Eurostat through the annual Joint Questionnaires. shared by Eurostat and the International Energy Agency. following a well established and harmonised methodology. Methodological information on the annual Joint Questionnaires and data compilation can be found in Eurostat's web page for metadata on energy statistics. http://epp.eurostat.ec.europa.eu/cache/ITY_SDDS/EN/nrg_quant_sm1.htm See also information related to the Energy Statistics Regulation http://www.europarl.europa.eu/oeil/file.jsp?id=5431232

- Reliability. accuracy. robustness. uncertainty (at data level): 
Indicator uncertainty (historic data):
Biomass and wastes. as defined by Eurostat. cover organic. non-fossil material of biological origin. which may be used for heat production or electricity generation. They comprise wood and wood waste. biogas. municipal solid waste (MSW) and biofuels. MSW comprises biodegradable and non-biodegradable wastes produced by different sectors. Non-biodegradable municipal and solid wastes are not considered to be renewable. but current data availability does not allow the non-biodegradable content of wastes to be identified separately. except for that from industry.
Also. electricity data (unlike that for overall energy consumption) for 1990 refers to the western part of Germany only.
Electricity consumption within the national territory includes imports of electricity from neighbouring countries. It also excludes the electricity produced nationally but exported abroad. In some countries the contribution of electricity trade to total electricity consumption and the changes observed from year to year need to be looked at carefully when analysing trends in electricity production by fuel. Impacts on the (national) environment are also affected since emissions are accounted where the electricity is produced whereas consumption is accounted where the electricity is consumed.

- Overall scoring – historic data (1 = no major problems. 3 = major reservations):

Relevance: 1

Accuracy: 1

Comparability over time: 1

Comparability over space: 1

- Forthcoming EEA indicator. The main sources of data are Eurostat’s energy balances and the EU greenhouse gas inventory to the UNFCCC. Autoproducers are not considered in this methodology because CO2 from autoproducers are reported in the respective sector so the associated electricity production had to be excluded from the calculation. Import/exports of electricity also are excluded in this methodology because CO2 emissions from stationary sources are based on the territorial definition used for UNFCCC reporting.