## ENER30 Renewable Electricity

Key message

In 2007, the share of renewable electricity in gross electricity consumption in the EU-27 was 15.6 % compared to 11.9% in 1990. The substantial increase in the total amount of renewable electricity generation (up by 68% since 1990) was partially off-set by the increase in electricity consumption. Between 2006 and 2007, the electricity production from wind increased by 26.7% and that from photovoltaics increased by 51.4%. Despite good progress, only three countries have already met the indicative national target, meaning that much more needs to be done to achieve the overall target of 21% by 2010.

Rationale

Electricity generated from renewable energy sources generally tends to have lower environmental impact (e.g. greenhouse gas (GHG) and air pollution emissions) over the entire life-cycle of the production chain than electricity generated from fossil fuels. An increased share of renewable electricity will thus help the EU to meet its targets on GHG and air pollution emissions and generally reduce the impact of electricity production on the environment and human health. In addition, electricity consumption is likely to increase in the future due to changing life-styles and expected penetration of electric vehicles. A higher share of renewable energy will thus help to diminish the environmental pressures stemming from the electricity production in the future.

This indicator is closely linked with ENER 27, ENER 28 and ENER 29.

Key policy question: how fast the share of renewable electricity in total gross electricity consumption is increasing in Europe?

Key assessment: renewable electricity

- In recent years, renewables increased their contribution to meeting the electricity consumption in EU-27. The share in gross electricity consumption in 2007 was 15.6 % compared to 11.9 % in 1990 (see Table 1). Renewable electricity production grew by 68 % between 1990 and 2007 (see also ENER 27) - faster than the growth in overall electricity consumption (a 32.8 % increase over the same period, see also ENER18). On average, the share of large hydro in gross electricity consumption has declined since 2001, mainly as a result of lower rainfall. Nevertheless, hydropower still dominates renewable electricity production in most Member States with an approximate share of 59.0 % across the EU-27 in 2007, compared to 19.4 % for biomass and waste, 19.8 % from wind, and the remainder from geothermal (1.1 %), and solar (0.7 %). The highest growth rates in renewable electricity production in 2006-2007 were observed for photovoltaics (51.4 %), wind (26.7 %) and biomass (13.3 %) (see Figure 1).

- Some difference can be observed between relative growth rate developments of PV and wind technologies. The relative growth rate of PV in 2005-2006 was 71.4 % but in 2006-2007 it decreased to 51.4%. However, the absolute growth in 2006-2007 (1.3 TWh) was larger than in 2005-2006 (1.0 TWh). For wind, the relative growth rate increased in both periods (2005-2006 was 16.8% while for the period 2006-2007 the relative growth rate was 26.7%). Similar with developments in the PV sector, the absolute growth in wind was larger in 2006-2007 (21.9 TWh) than in 2005-2006 (11.8 TWh).

- There are significant differences in the share of renewables between the EU-27 Member States. These reflect differences in the availability of natural resources in each country, as well as the policies chosen to support the development of renewable energy. Amongst the EU-27, Austria (59.8 %), Sweden (52.1 %) and Latvia (36.4 %) had the greatest shares of renewable electricity in gross electricity consumption in 2007 if large hydropower is included. Denmark showed the largest share of renewable electricity (29 %) when large hydropower is excluded. The non-EU EEA countries Iceland and Norway both have about a 100 % share of renewable electricity.

Fig. 1: Average annual growth rates of renewable energy in electricity consumption (EU-27) for 1990-2007 and 2006-2007

Data source: Eurostat

Table 1 Share of renewable electricity in gross electricity consumption (%) 1990-2007 and 2010 indicative targets

|  | 1990 | 1995 | 2000 | 2005 | 2006 | 2007 | 2010 targets |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EEA | 17.3 | 18.5 | 19.0 | 18.5 | 18.7 | 19.8 | - |
| EU-27 | 11.9 | 13.0 | 13.8 | 14.0 | 14.6 | 15.6 | 21.0 |
| World | 19.9 | 20.5 | 19.1 | 18.6 | 18.7 | 18.4 | - |
| Africa | 18.5 | 17.1 | 17.7 | 16.9 | 17.0 | 16.4 | - |
| Middle East | 5.0 | 3.3 | 1.7 | 3.3 | 3.5 | 3.2 | - |
| China | 19.5 | 18.7 | 16.2 | 15.8 | 15.2 | 15.0 | - |
| India | 24.8 | 17.5 | 13.8 | 15.8 | 16.5 | 17.1 | - |
| Russia | 15.3 | 20.7 | 19.1 | 18.6 | 17.9 | 17.9 | - |
| United States | 12.3 | 11.7 | 9.2 | 9.4 | 10.1 | 9.2 | - |
| Belgium | 1.1 | 1.2 | 1.5 | 2.8 | 3.9 | 4.2 | 6.0 |
| Bulgaria | 4.1 | 4.2 | 7.4 | 11.8 | 11.2 | 7.5 | 11.0 |
| Czech Republic | 1.9 | 3.9 | 3.6 | 4.5 | 4.9 | 4.7 | 8.0 |
| Denmark | 2.6 | 5.9 | 16.7 | 28.3 | 26.0 | 29.0 | 29.0 |
| Germany | 3.8 | 5.0 | 6.5 | 10.5 | 12.0 | 15.1 | 12.5 |
| Estonia | 0.0 | 0.1 | 0.3 | 1.1 | 1.4 | 1.5 | 5.1 |
| Ireland | 4.8 | 4.1 | 4.9 | 6.8 | 8.5 | 9.3 | 13.2 |
| Greece | 5.0 | 8.4 | 7.7 | 10.0 | 12.1 | 6.8 | 20.1 |
| Spain | 17.2 | 14.3 | 15.7 | 15.0 | 17.7 | 20.0 | 29.4 |
| France | 14.8 | 17.8 | 15.1 | 11.3 | 12.5 | 13.3 | 21.0 |
| Italy | 13.9 | 14.9 | 16.0 | 14.1 | 14.5 | 13.7 | 25.0 |
| Cyprus | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 6.0 |
| Latvia | 43.9 | 47.1 | 47.7 | 48.4 | 37.7 | 36.4 | 49.3 |
| Lithuania | 2.5 | 3.3 | 3.4 | 3.9 | 3.6 | 4.6 | 7.0 |
| Luxembourg | 2.1 | 2.2 | 2.9 | 3.2 | 3.4 | 3.7 | 5.7 |
| Hungary | 0.5 | 0.7 | 0.7 | 4.6 | 3.7 | 4.6 | 3.6 |
| Malta | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 5.0 |
| Netherlands | 1.4 | 2.1 | 3.9 | 7.5 | 7.9 | 7.6 | 9.0 |
| Austria | 65.4 | 70.6 | 72.4 | 57.4 | 56.6 | 59.8 | 78.1 |
| Poland | 1.4 | 1.6 | 1.7 | 2.9 | 2.9 | 3.5 | 7.5 |
| Portugal | 34.5 | 27.5 | 29.4 | 16.0 | 29.4 | 30.1 | 39.0 |
| Romania | 23.0 | 28.0 | 28.8 | 35.8 | 31.4 | 26.9 | 33.0 |
| Slovenia | 25.8 | 29.5 | 31.7 | 24.2 | 24.4 | 22.1 | 33.6 |
| Slovakia | 6.4 | 17.9 | 16.9 | 16.7 | 16.6 | 16.6 | 31.0 |
| Finland | 24.4 | 27.0 | 28.5 | 26.9 | 24.0 | 26.0 | 31.5 |
| Sweden | 51.4 | 48.2 | 55.4 | 54.3 | 48.2 | 52.1 | 60.0 |
| United Kingdom | 1.7 | 2.0 | 2.7 | 4.3 | 4.6 | 5.1 | 10.0 |
| Turkey | 40.9 | 41.9 | 24.3 | 24.7 | 25.5 | 19.2 | - |
| Iceland | 99.9 | 99.8 | 99.9 | 99.9 | 100.0 | 100.0 | - |
| Norway | 114.6 | 104.6 | 112.2 | 108.4 | 98.4 | 106.1 | - |
| Switzerland | 56.5 | 64.8 | 63.4 | 50.1 | 49.5 | 56.7 | - |

Data source: Eurostat, IEA, EIA

Note: The renewable electricity share in Norway is above 100% in some years because a part of the (renewable) electricity generated domestically is exported to other countries. The share of renewable electricity in Germany in 1990 refers to West Germany only. National indicative targets for the share of renewables electricity in 2010 are taken from Directive 2001/77/EC. Notes to their 2010 indicative targets are made by Italy, Luxemburg, Austria, Portugal, Finland and Sweden in the directive; Austria and Sweden note that reaching the target is dependent upon climatic factors affecting hydropower production, with Sweden considering 52% a more realistic figure if long-range models on hydrologic and climatic conditions were applied. No energy data for Liechtenstein available from Eurostat. No 2007 data available for Iceland, 2006 data were used.

The data for World, Africa, Middle East, China, India, Russia and the United States are extracted from the EIA-database from the US DOE. These figures slightly differ from the Eurostat data.

Policy target

According to the renewable electricity directive (2001/77/EC), the overall EU-27 target is a share of 21.0 % of renewable electricity in gross electricity consumption by 2010. In 2007, the share was 15.6 %. Within the EU-27, only Germany, Denmark and Hungary have already reached their targets (see Figure2) and according to the latest progress report (EU 2009), EU-27 is likely to miss the 2010 target.

Fig. 2: Renewable electricity as a percentage of gross electricity consumption, 2007

Data source: Eurostat

Note: The renewable electricity directive (2001/77/EC) defines renewable electricity as the share of electricity produced from renewable energy sources in total electricity consumption. The latter includes imports and exports of electricity. The electricity generated from pumping in hydropower plants is included in total electricity consumption but it is not included as a renewable source of energy. Large hydropower plants have a capacity of more than 10 MW.

Indicator rationale

Environmental context

The share of electricity consumption from renewable energy sources provides a broad indication of progress towards reducing the environmental impact of electricity consumption on the environment as renewable electricity is generally considered to have lower life-cycle environmental impact per unit of electricity produced than fossil-fuelled power plants (link with the EEA LCA analysis). Increasing the share of renewables in electricity consumption will help the EU to reduce the GHG emissions from power generation but the overall impact will depend on which generation sources are being replaced in the energy system.

Emissions of air pollutants are also generally lower for renewable electricity production than for electricity produced from fossil fuels. The exception to this is the incineration of Municipal and Solid Waste (MSW), which due to high costs of separation, usually involves the combustion of some mixed wastes including materials contaminated with heavy metals. Emissions to the atmosphere from MSW incineration are subject to stringent regulations including tight controls on emissions of cadmium, mercury, and other such substances.

The exploitation of renewable energy sources may have negative impacts on landscapes, habitats and ecosystems, although many impacts can be minimised through careful site selection. Hydropower schemes in particular can have adverse impacts including flooding, disruption of ecosystems and hydrology, and socio-economic impacts if resettlement is required (for large hydro). Some solar photovoltaic schemes require relatively large quantities of heavy metals in their construction and geothermal energy can release pollutant gases carried by hot fluids if not properly controlled. Wind turbines can have visual impacts on the areas in which they are sited. Some types of biomass and biofuel crops have considerable land, water and agricultural input requirements such as fertilisers and pesticides.

Policy context

- Council adopted on 6 April 2009 the climate-energy legislative package containing measures to fight climate change and promote renewable energy. This package is designed to achieve the EU's overall environmental target of a 20 % reduction in greenhouse gases and a 20 % share of renewable energy in the EU's total energy consumption by 2020.The climate action and renewable energy (CARE) package includes the following main policy documents

Directive 2009/29/ec of the European parliament and of the Council amending directive 2003/87/ec so as to improve and extend the greenhouse gas emission allowance trading scheme of the community

Directive 2009/31/ec of the European parliament and of the Council on the geological storage of carbon dioxide

Directive 2009/28/ec of the European parliament and of the Council on the promotion of the use of energy from renewable sources

Community guidelines on state aid for environmental protection (2008/c 82/01)

Directive 2008/101/ec of the European parliament and of the Council amending directive 2003/87/ec so as to include aviation activities in the scheme for greenhouse gas Emission allowance trading within the community

Regulation (ec) no 443/2009 of the European parliament and of the Council setting emission performance standards for new passenger cars as part of the community’s integrated approach to reduce CO2 emissions from light-duty vehicles

- The Renewable Electricity Directive  ( 2001/77/EC)

Sets an indicative target of 21% of renewable electricity in gross electricity consumption. Fulfilling this target will also help meeting the new, mandatory target of 20% renewables in final energy consumption in 2020 set by the Directive 2009/28/EC (see also ENER 28).

- The European Strategic Energy Technology Plan; COM(2007) 723

It focuses on bringing new renewable energy technologies to market competitiveness.

References

COM(2007) 723 - A European Strategic Energy Technology plan (set-plan)

COM(2008) 16 final - Directive of the European Parliament and of the Council amending Directive 2003/87/EC so as to improve and extend the greenhouse gasemission allowance trading system of the Community

Directive 2001/77/EC - Directive 2001/77/EC of the European Parliament and of the Council of 27 September 2001 on the promotion of electricity produced from renewable energy sources in the internal electricity market.

Directive 2009/28/EC  - Directive 2009/28/EC of the European Parliament and of the Council of 23 April 2009 on the promotion of the use of energy from renewable sources and amending and subsequently repealing Directives 2001/77/EC and 2003/30/EC, Brussels, 2009

EEA (2008) Greenhouse gas data viewer

http://dataservice.eea.europa.eu/PivotApp/pivot.aspx?pivotid=455

Meta data

Technical information

- Data source:
Eurostat (historical data), http://ec.europa.eu/eurostat/
Renewable electricity consumption is one of the European Environment Agency’s core-set indicators. More information can be found at http://themes.eea.eu.int/IMS/CSI
IEA Data Services, http://data.iea.org/IEASTORE/DEFAULT.ASP
EIA (Energy Information Administration of the U.S. Department of Energy), http://www.eia.doe.gov/

- Description of data / Indicator definition:
The share of renewable electricity is the ratio between the electricity produced from renewable energy sources and gross national electricity consumption calculated for a calendar year, expressed as a percentage. It measures the contribution of electricity produced from renewable energy sources to the national gross electricity consumption. As well as being one of the EEA's core set indicators, it is also one of the structural indicators used to underpin the European Commission's analysis in its annual Spring report to the European Council. The methodologies are identical for both indicators.
Renewable energy sources are defined as renewable non-fossil energy sources: wind, solar, geothermal, wave, tidal, hydropower, biomass, landfill gas, sewage treatment plant gas and biogases. Electricity produced from renewable energy sources comprises the electricity generation from hydro plants (excluding that produced as a result of pumping storage systems), wind, solar, geothermal and electricity from biomass/wastes. Electricity from biomass/wastes comprises electricity generated from wood/wood wastes and the burning other of solid wastes of a renewable nature (straw, black liquor), municipal solid waste incineration, biogas (incl. landfill, sewage, farm gas) and liquid biofuels. Gross national electricity consumption comprises total gross national electricity generation from all fuels (including autoproduction), plus electricity imports, minus exports.
The data sets of Eurostat, IEA and EIA differ <1%.
Projections are for 2020 from the POLES (IPTS) Baseline and GHG Reduction Scenario and from the WEO 2007 (IEA) Reference and Alternative Policy Scenario.

- Geographical coverage:
The Agency had 32 member countries at the time of writing of this fact sheet. These are the 27 European Union Member States and Turkey, plus Iceland, Norway, Liechtenstein and Switzerland. 
No energy data available for Liechtenstein, no 2007 data for Iceland
Data for World, United States, China, India, Russia, Africa and the Middle East.

- Temporal coverage: 1990-2007, projections 2020.

- Methodology and frequency of data collection: Data collected annually.
Eurostat definitions for energy statistics http://circa.europa.eu/irc/dsis/coded/info/data/coded/en/Theme9.htm
Eurostat metadata for energy statistics http://epp.eurostat.ec.europa.eu/cache/ITY_SDDS/EN/nrg_base.htm

- Methodology of data manipulation:
Share of electricity produced from renewable energy sources as a percentage of total national electricity consumption. The coding (used in the Eurostat New Cronos database) and specific components of the indicator are:
Numerator: hydro 5510 primary production 100100 + wind 5520 primary production 100100 + photovoltaic (PV) 5534 primary production 100100 + 6000 electrical energy 107002 gross electricity generation - geothermal power plants + 6000 electrical energy 107011 gross electricity generation -biomass-fired powerstations.
Denominator: 6000 electrical energy 107000 total gross electricity generation + 6000 electrical energy 100900 gross inland consumption (i.e. net balance of imports minus exports).
Average annual rate of growth calculated using: [(last year/base year) ^ (1/number of years) –1]*100

IEA data set:
- Report ‘Electricity Information’, table ‘OECD, Electricity and Heat Generation’, balance ‘Gross Electricity Production (GWh)’, plant ‘Total plants’, products ‘Hydro’, ‘Pumped Hydro Production’, ‘Geothermal’, ‘Solar’, ‘Tide, Wave and Ocean’, ‘Wind’, ‘Municipal Waste (renew)’, ‘Municipal Waste (non-renew)’, ‘Wood/Woodwaste/Other solid waste’, ‘Landfill Gas’, ‘Sewage Sludge Gas’, ‘Other Biogas’, ’Liquid Biofuels’, ‘Non-specified comb. renew and waste’, ‘Non-specified comb. fuels for Heat’, ‘Other Sources’ and ‘Total Sources’.
[Eurostat 100100 Primary production (5510 Hydro power) equals IEA Hydro -/- Pumped Hydro Production (<1% difference)]
- Report ‘Energy Balances of OECD-countries’, table ‘Energy Balances’, product ‘Electricity’, flow ‘Import’, ‘Export’
- Share renewables electricity in total electricity consumption(%) calculated by (Sum Renewables) / (Gross Electricity Generation Total Sources + Imports -/- Exports) in TWh.

EIA data set:
International Electricity Generation, 6.3  World Total Net Electricity Generation, 2.6  World Net Hydroelectric Power Generation, 2.8  World Net Geothermal, Solar, Wind, and Wood and Waste Electric Power Generation.
Eurostat 100100 Primary production (5510 Hydro power) equals EIA 2.6 Net Hydroelectric Power Generation (<1% difference)

Projections:
POLES: Net electricity generation Hydro (TWh)/Net electricity generation Total (TWh)
Net electricity generation Renewables (TWh)/Net electricity generation Total (TWh)
WEO 2007: Electricity generation Hydro (TWh)/Electricity generation Total (TWh)
Electricity generation Biomass and waste (TWh)/Electricity generation Total (TWh)
Electricity generation Wind (TWh)/Electricity generation Total (TWh)
Electricity generation Geothermal (TWh)/Electricity generation Total (TWh)
Electricity generation Solar (TWh)/Electricity generation Total (TWh)
Electricity generation Tide and wave (TWh)/Electricity generation Total (TWh)

Qualitative information

- Strengths and weaknesses (at data level)
Data gaps for breakdown of large hydropower. No projection data for Croatia, Iceland and Liechtenstein.
Data have traditionally been compiled by Eurostat through the annual Joint Questionnaires, shared by Eurostat and the International Energy Agency, following a well established and harmonised methodology. Methodological information on the annual Joint Questionnaires and data compilation can be found on Eurostat's website in the section on metadata on energy statistics: http://epp.eurostat.ec.europa.eu/cache/ITY_SDDS/EN/nrg_quant_sm1.htm See also information related to the Energy Statistics Regulation http://www.europarl.europa.eu/oeil/file.jsp?id=5431232

- Reliability, accuracy, robustness, uncertainty (at data level):
Indicator uncertainty (historic data)
The renewables electricity directive (2001/77/EC) defines the share of renewable electricity as the percentage of electricity produced from renewable energy sources in gross electricity consumption. The numerator includes all electricity generated from renewable sources, most of which is for domestic use. The denominator contains all electricity consumed in a country, thus including imports and excluding exports of electricity. Therefore, the share of renewable electricity can be higher than 100 % in a country if all electricity is produced from renewable sources and some of the over-generated renewable electricity is exported to a neighbouring country.
Biomass and wastes, as defined by Eurostat, cover organic, non-fossil material of biological origin, which may be used for heat production or electricity generation. They comprise wood and wood waste, biogas, municipal solid waste (MSW) and biofuels. MSW comprises biodegradable and non-biodegradable wastes produced by different sectors. Non-biodegradable municipal and solid wastes are not considered to be renewable, but current data availability does not allow the non-biodegradable content of wastes to be identified separately, except for industry.
The electricity produced as a result from hydropower storage systems is not classified as a renewable source of energy in terms of electricity production, but is part of the gross electricity consumption in a country.
The share of renewable electricity could increase even if the actual electricity produced from renewable sources falls. Similarly, the share could fall despite an increase in electricity generation from renewable sources. Therefore, from an environmental point of view, attaining the 2010 target for the share of renewable electricity does not necessarily imply that carbon dioxide emissions from electricity generation will fall.
Electricity consumption within the national territory includes imports of electricity from neighbouring countries. It also excludes the electricity produced nationally but exported abroad. In some countries the contribution of electricity trade to total electricity consumption and the changes observed from year to year need to be looked at carefully when analysing trends in renewable electricity. Impacts on the (national) environment are also affected since emissions are accounted where the electricity is produced whereas consumption is accounted where the electricity is consumed.
It should also be noted that electricity consumption in 1990 for Germany refers only to the western part.

- Overall scoring – historic data (1 = no major problems, 3 = major reservations):

Relevance: 1

Accuracy: 1

Comparability over time: 1

Comparability over space: 1