# Slovenia

## Sources of Information

For policies and measures: Slovenia’s Second and Third National Communication to the Conference of the Parties of the United Nations Framework Convention on Climate Change, Republic of Slovenia, Ministry of the Environment, Spatial Planning and Energy, July 2004. Updates to policies and measures provided in the report submitted under the Monitoring Mechanism in June 2005.

For projections: Slovenia’s Second and Third National Communication to the Conference of the Parties of the United Nations Framework Convention on Climate Change. Updated projections provided in the Report submitted under the Monitoring Mechanism in June 2005.

## Quality and transparency of Reporting

In Slovenia’s Second and Third National Communication, Slovenia describes an approach to reducing its GHG emissions by 8 % in the 2008 - 2012 period, relative to the base year, in order to meet Slovenia's Kyoto Protocol target. The base year for CO2, CH4 and N2O is 1986 and the base year for F-gases is 1995.  In this analysis the projections are based on the 2005 Monitoring Mechanism from Slovenia ‘Poročilo Eu V Skladu S Sklepi 280/2004/EC In 2005/166/EC’.  The policies and measures assessment was based on the Second and Third National Communication.

According to recent estimates (2003 CRF submitted in April 2005, greenhouse gas emissions in the base year in Slovenia were approximately 20.2 million tons of CO2 equivalent, and an 8 % reduction would entail annual average emissions of app. 18.6 million tons of CO2 eq. during the first commitment period (2008-2012).  Based on the additional domestic measures projection excluding sinks, Slovenia plans to almost meet the required 8 % reduction in the first commitment period (20.3 million tons). The commitment is just going to be met when sinks of 1.68Mt are included in the GHG emission budget.

Beside domestic measures, the Kyoto Mechanisms that could be considered are: Emissions Trading, Joint Implementation and the Clean Development Mechanism.  Slovenia sees Emission Allowance Trading as the most important flexible mechanism, which will enable polluters to reduce emissions in the most cost effective way. Joint Implementation is interesting to Slovenia as a performer of projects in other countries in Annex 1. Projects based on JI from which Slovenia would obtain emission rights would also improve the technological penetration of the Slovenian economy.

Chapter 4 in Slovenia’s National Communication deals with policies and measures and Chapter 5 addresses projections and the total effect of policies and measures.  Policies and measures to reduce greenhouse gas emissions and projected emissions are discussed for the following sectors: energy, transport, industry, agriculture and forestry, waste, F-gases and cross-sectoral measures.  Summary tables on policies and measures are clearly presented. Projections for the with domestic measures and with additional domestic measures scenarios are provided, with a breakdown by gas and by sector.  Appendix C gives more detail on the assumptions and parameters that the projections are based on.

- Table 1: Information Provided on Policies and Measures

| - Information provided | - Level provided | - Comments |
| --- | --- | --- |
| - Policy names | - +++ | - PaMs discussed in individual sectors (energy, transport, industry, agriculture and forestry, waste, F-gases and cross-sectoral measures); clear summary table for PaMs |
| - Objective of policies | - +++ | - Type of instrument provided for every policy and measure |
| - Which greenhouse gas? | - CO2, CH4, N2O, F-gases |  |
| - Status of Implementation | - +++ | - Status of implementation given for policies and measures as implemented or adopted. |
| - Implementation body specified | - +++ | - Implementing entity given in all cases |
| - Quantitative assessment of implementation | - ++ | - Quantified for most PaMs |
| - Interaction with other P&Ms discussed | - ++ | - Some discussion on interactions and impacts of other PaMs |

- +, ++, +++ level of information available increases as the number of + signs increases

- Slovenia’s Second and Third National Communication provided a high standard of reporting on policies and measures.  All categories required by the directive, such as status and quantification of implementation, were covered for most or all policies and measures.  A policies and measures summary table clearly listed the details of each policy and measure in a straightforward layout, with policies grouped by sector.

- Table 2: Information provided on Projections

| - Category of Information | - Level of Information Provided | - Comments |
| --- | --- | --- |
| - Scenarios considered | - With measures and with additional measures | - With additional measures projection is based on a higher implementation intensity of adopted and implemented measures and the realisation of all planned measures. |
| - Expressed relative to inventory for previous years | - +++ | - Overall emission estimates in CO2 eq. given for 2005, 2010, 2015, 2020 and 2008-2012 average compared with revised base year (1986) and also 1990, 1995, 2000 and 2002 |
| - Starting year | - 1999 |  |
| - Projections | - 2005, 2010, 2015, 2020 average 2008-2012 | - Comprehensive split by GHG and sector; split by gas for “with measures scenario” not given |
| - Split of projections | - Yes | - Projections split by IPCC main sectors and by gas (“with additional measures”) |
| - Presentation of results | - +++ | - All results presented in a summary table with more detailed tables given for each sector projection |
| - Description of model (level of detail, approach and assumptions) | - +++ | - Good description of model approach, detail and key assumptions in model provided |
| - Discussion of uncertainty | - +++ | - Key uncertainty issues explained and quantified where relevant |
| - Details of parameters and assumptions | - +++ | - Key assumptions and parameters presented in a table for the relevant years (past and projected) |

- +, ++, +++ level of information available increases as the number of + signs increases

## Assessment of Policies and Measures

The projections of emissions have, with regard to the second and third Submission to the Conference of Parties to the United Nations Framework Convention on Climate Change undergone a change in the following sectors:

- Electricity And Heat Production (for the years 2008-2015 the consumption of coal in the Šoštanj Thermal Power Plant (TEŠ) has been increased in accordance with the long-term agreement on the supply on lignite to 38.5 PJ). This correction has been taken into account in the projection "with additional measures";

- Projections for Transport (road transport and other) have been calculated according to the COPERT methodology:

- Projections for Waste have been calculated according to improved methodology.

Average emissions of greenhouse gases without sinks in the period 2008–2012 according to the projection "with measures" amount to 21,172 Gg CO2 eq, according to the projection "with additional measures" to 20,255 Gg CO2 eq Their most important source is fuel combustion in sector Electricity And Heat Production, followed by Transport, Other areas as well as Industry and Construction. For gases, by far the greatest share is that of CO2.

- Table 3 gives an overview of effects of policies and measures.

- Table 3: Summary of the Effect of Policies and Measures by 2010 (MtCO2  eq.)

|  | With measures | With additional measures |
| --- | --- | --- |
| Energy | 1.2 | 0.5 |
| Industrial Processes | 0.00 | 0.3 |
| Solvents | 0.0 | 0.0 |
| Agriculture | 0.04 | 0.0 |
| Waste | 0.2 | 0.1 |
| Total (excl. sinks) | 1.4 | 0.9 |

The effect of policies in WAM projection is the difference between the WM and WAM projections.  The effect of policies in the WM projection is taken directly from the 2005 MM Submission.

Table 4: Detailed information on Policies and Measures – Summary (Assessment of the Measure Impact is made for 2010)

| Name of policy or measure | Objective and/or activity affected | GHG affected | Type of instrument | Status | Implementing entity or entities: | Emission reduction in 2010 [Gg CO2] |
| --- | --- | --- | --- | --- | --- | --- |
| Energy |  |  |  |  |  | 2697 |
| Promotion of Electricity Production from Renewable Sources and Combined Heat and Power Generation | Increasing the Share of RES and cogeneration of electrical energy and Heat | CO2 | Economic, Regulatory | Implemented | MG | 500 |
| Opening the Electricity Market | Changing the structure of electricity production, greater import possibilities | CO2 | Regulatory | Implemented | MG | 1350 |
| Opening the Natural Gas Market | Partial exchange of coal for nat. gas (thermal power plants, power cogeneration plants) and increased production of el. energy from nat. gas | CO2 | Regulatory | Implemented | MG |  |
| Construction of Large Hydro | Power Plants Increasing the share of RES in electricity pro¬duction | CO2 |  | Implemented | HSE, MG | 400 |
| Incentives for Carrying out the EEU Measures and for Investment in RES | Greater Energy Efficiency and Increased Share of RES | CO2 | Economic, Promotion | Implemented | MOP | NE |
| Education, Training and Public Awareness | Increasing the level of public awareness and releasing information to the public | CO2 | Promotion, Education, Information | Implemented | MOP | 100 |
| Energy Labelling of Household Appliances | Higher energy efficiency | CO2 | Information, Regulatory | Implemented | MOP |  |
| Regular Inspections of Small Boilers and Air-Conditioning Systems | Higher energy efficiency due to improved effectiveness of boilers and air-conditioning systems | CO2 | Regulatory | Implemented | MOP | Not estimated (NE) |
| Thermal Protection and Energy Labelling of Buildings | Increasing energy efficiency | CO2 | Regulatory, Information | Implemented | MOP | 310 |
| Cost Accounting for Heating According to Actual Consumption | Stimulating the public for efficient energy use | CO2 | Stimulation | Implemented | MOP | 19 |
| Third Party Financing ('Contracting') | Incentives for investments in efficient energy use | CO2 | Economic / Voluntary | Implemented | Public sector, Municipalities | 18 |
| Certification of Energy Source (“Green certificates”) | Stimulating of electricity production from renewable energy sources | CO2 | Regulatory | Implemented | MG, AGEN-RS | NE |

Table 4: Continued

| Name of policy or measure | Objective and/or activity affected | GHG affected | Type of instrument | Status | Implementing entity or entities: | Emission Reduction in 2010 [Gg CO2] |
| --- | --- | --- | --- | --- | --- | --- |
| Demand Side Management | Stimulating consumers to use energy efficiently | CO2 | Promotional | Planned | MG, MOP | NE |
| Introduction of Excise Tax on Fossil Fuels and Electricity | Increasing energy efficiency in households and the public sector | CO2 | Fiscal | Planned | MF | NE |
| Transport |  |  |  |  |  | 900 |
| Excise Duties on Fuels | Reducing fuel consumption and, consequently, decreasing CO2 emissions | CO2 | Fiscal | Implemented | MF | 150 |
| Control of Exhaust Composition and Engine Adjustment in Motor Vehicles | Reducing emission through technical improvement of vehicles | CO2 | Regulatory | Implemented | MOP | 185 |
| Informing Consumers on Fuel Consumption and CO2 Emissions of Motor Vehicles & Agreement between the European Commission and Car Manufacturers | Reducing emission through improved energy efficiency of vehicles | CO2 | Promotional, Information | Implemented | MOP | 250 |
| Promotion of Biofuel Consumption | Reducing GHG emission through the use of environment friendly fuel | CO2 | Fiscal | Implemented | MF, MOP | 100 |
| Promotion of Public Passenger Transport | Decreasing emission and air pollution in towns | CO2 | Promotional | Implemented/ Planned | MOP, MP, Municipalities | 100 |
| Increasing the Share of Railway in the Transport of Goods and Passengers | Reducing GHG emission through the transport of goods and passengers on the railway | CO2 | Promotional, Regulatory | Implemented/ Planned | Government, MP | 50 |
| Strategy of the Regional and Spatial Development | Reducing GHG emissions through shorter transport distances | CO2 | Regulatory | Adopted/ Planned | MOP, Municipalities | 65 |
| Industry |  |  |  |  |  | 460 |
| EEU in Industry | Increasing energy efficiency in industry | CO2 | Promotional, Economic | Implemented | MOP | 400 |

Table 4: Continued

| Name of policy or measure | Objective and/or activity affected | GHG affected | Type of instrument | Status | Implementing entity or entities: | Emission Reduction in 2010 [Gg CO2] |
| --- | --- | --- | --- | --- | --- | --- |
| Giving incentives for the Introduction of Systems of Environment Management According to ISO 14001 and Joining the EMAS System | Increasing energy efficiency and productivity in industry | CO2 | Promotional, Economic, Voluntary | Implemented | MOP, MG, GZS |  |
| Eco–Labelling of Products | Increasing energy efficiency | CO2 | Voluntary | Implemented | MOP, GZS | NE |
| IPPC Directive | Increasing energy efficiency and productivity in industry | CO2 | Regulatory | Implemented | MOP, MG, GZS | 60 |
| Agriculture and Forestry |  |  |  |  |  | 40 |
| Rural Areas Development Programme | Stimulating sustainable practices in agriculture | N2O, CH4, CO2 | Economic, Voluntary | Implemented | MKGP | 9 |
| Good Agricultural Practice in Manuring | N2O Emission Reduction Due to the lower Level of Manuring | N2O | Regulatory | Implemented | MKGP, MOP | 11 |
| Promotion of Biogas for Electricity and Heat Production | Reducing GHG emission through exploitation of animal and agricultural waste for producing energy | CH4 | Economic, Regulatory | Implemented / Planned | MOP | 20 |
| Sustainable Forest Management | Conserving biodiversity, productivity, regeneration ability, volume and vitality of forest | CO2 | Regulatory | Implemented | MKGP | 840 |
| Incentives for Cultivating Biodiesel Crops | Reducing consumption of fossil fuel in transport, conserving humus in the soil and soil fertility — reduced use of fertilizers | CO2, N2O | Regulatory, Economic | Planned | MKGP | NE |

Table 4: Continued

| Name of policy or measure | Objective and/or activity affected | GHG affected | Type of instrument | Status | Implementing entity or entities: | Emission Reduction in 2010 [Gg CO2] |
| --- | --- | --- | --- | --- | --- | --- |
| Waste |  |  |  |  |  | 256 |
| Rules on the Landfill of Waste and on the Management of Waste | Reducing emission through improved management and closure of solid waste disposal sites and improved waste management | CH4 | Regulatory | Implemented | MOP | NE |
| Waste Disposal Tax | Reducing the amount of waste | CH4 | Fiscal | Implemented | MOP, MF | NE |
| Separate Waste Collection and Packaging Waste Management | Reducing the amount of waste at source | CH4, CO2 | Regulatory | Implemented | Municipalities, MOP, GZS | 42 |
| Landfill Gas Extraction and Combustion, Energy Exploitation or Use of Landfill Gas | Reducing GHG emission through exploitation of methane | CH4, CO2 | Economic Regulatory | Implemented | MOP | 126 |
| Waste Incineration | Reducing the amount of waste and using waste for production of energy | CH4, CO2 | Regulatory | Planned | MOP | 88 |
| F–Gases |  |  |  |  |  | 200 |
| Directive on F–gases | F–gases Emission Reduction | HFC, PFC, SF6 | Regulatory | Planned | Government | 200 |
| Cross-Sectoral |  |  |  |  |  | NE |
| CO2 Tax | More Efficient Energy Use | CO2 | Fiscal | Implemented | MOP, MF | NE |
| Kyoto Mechanisms |  |  |  |  |  | NE |
| Emission Trading Scheme | Giving the polluters a chance for a more cost-effective way of reducing emissions | CO2 | Regulatory, Economic | Implemented | MOP | NE |
| Clean Development Mechanism (CDM) and Joint Implementation (JI) | GHG Emission Reduction | All GHG | Economic | Planned | Goverment | NE |
| Total |  |  |  |  |  | 4553 |

## Evaluation of Projections

The data in Tables 5-7 are based on information in Slovenia’s report under the Monitoring Mechanism submitted in June 2005.

Table 5: Summary of projections by gas in 2010 (MtCO2 equivalent)

|  | Base year | with measures | with additional measures |
| --- | --- | --- | --- |
| Carbon dioxide (excl. sinks) | 16,0 |  | 16,6 |
| Methane | 2,2 |  | 1,9 |
| Nitrous oxide | 1,7 |  | 1,6 |
| F-gases* | 0,3 |  | 0,2 |
| Total (excl. sinks) | 20,2 | 21,2 | 20,3 |
| % change relative to base year (excl. sinks) |  | 4,9% | 0,4% |

- *The base year for F-gases is 1995

- Table 5 presents the emission projection scenarios for the commitment period, the average over 2008-2012, split by greenhouse gas.  The base year for all gases is 1986, except for F-gases which have a base year of 1995.  Under the ‘with domestic measures’ projection excluding sinks, Slovenia’s emissions increase to 4.9% above the base year, but if additional measures are included, the emissions projections are merely 0.4% above the base year in the commitment period.  The Kyoto commitment of an 8% cut is just met if savings from sinks and additional measures are accounted for.

- Table 6: Summary of projections by sector in 2010 (MtCO2e)

|  | Base year (1986) | with measures | % change relative to 1986 | with additional measures | % change relative to 1986 |
| --- | --- | --- | --- | --- | --- |
| Energy | 15,658 | 16,884 | 4% | 16,343 | 4% |
| of which Transport | 2,008 | 4,558 | 127% | 4,558 | 127% |
| Industrial Processes | 1,306 | 1,415 | -11% | 1,164 | -11% |
| Solvents | 0,128 | 0,035 | -73% | 0,035 | -73% |
| Agriculture | 2,367 | 2,189 | -9% | 2,145 | -9% |
| Waste | 0,726 | 0,65 | -22% | 0,568 | -22% |
| Total (excl. sinks) | 20,2 | 21,2 | 5% | 20,3 | 0% |

Table 7: Assessment of the Target (2010)

|  | Excl. Sinks |  |  | Incl. Sinks |  |
| --- | --- | --- | --- | --- | --- |
|  | MtCO2 equiv. | % of base year level (six gas basket) | MtCO2 equiv. |  | % of base year level (six gas basket) |
| Base year emissions | 20.18 | 100% | 20.2 |  | 100% |
| Kyoto Commitment/burden sharing | 18.6 | -8% | 18.6 |  | -8% |
| With existing P&Ms projections | 21.17 | 105% | 21.2 |  | 105% |
| Gap (-ve means no gap) | 2.6 | 13% | 2.6 |  | 13% |
| With additional P&Ms projections | 20.255 | 100% | 18.6 |  | 92% |
| Remaining gap | 1.69 | 8% | 0.0 |  | 0% |
| Effect of flexible mechanisms | NA |  | NA |  |  |
| Remaining gap (if target includes flex mechs) |  |  |  |  |  |

## Description of Modelling ApproacH

Slovenia’s 2nd and 3rd National Communication and the report submitted under the Monitoring Mechanism give a comprehensive breakdown of assumptions and parameters for the projections model and a detailed explanation of the methodology behind the projections.

To prepare the IPPC Energy sector projections, Slovenia used a set of models whose main tool is a reference energy ecological model called REES-SLO, based in the MESAP (Modular Energy Systems Analysis and Planning) environment.  In addition to the REES-SLO the following models were used:

- PETSLO – market penetration of energy-saving final use technologies assessment model

- ELAM-SLO – simulation of an electrical load, curve model

- ELBIVIM – model calculating electricity production balance on a free market

The REES-SLO model was developed as a linear network model of processes and connections (reference energy system).  This enables consistent modelling of energy use based on needs of energy services and energy supply.

There are certain assumptions in the models that are subject to greater uncertainty; these are outlined in the National Communication.  The dynamics of transition to natural gas in electricity production depends on future market trends and social problems with reducing the domestic coal production.  This uncertainty in the future of coal mining means the fugitive emission projections are uncertain.  Transport emission projections are uncertain due to the potential increase in transit transport and possible influences of neighbouring countries.  Projections in the energy sector for the higher growth scenario shows the span of GHG emissions as between 16.0 and 17.2 Mt CO2 eq in 2010, which reflects the uncertainty of the performed projections.

Some of the main assumptions behind the projections are shown below. Assumptions for the energy sector projections are at three levels: general data, scenario data and a system of strategic instruments for GHG reduction.  A key assumption for the agricultural projection is the number of livestock.

The indicators for projections and the main assumptions are presented in the two tables below.

Indicators for projections to monitor and evaluate progress with policies and measures

| Eurostat sectors | Indicator | Numerator/denominator | 2005 | 2010 | 2015 | 2020 |
| --- | --- | --- | --- | --- | --- | --- |
| MACRO | CO2 intensity of GDP, t/Mio Euro | Total CO2 emissions, kt |  |  |  |  |
|  |  | GDP, Bio Euro (EC95) | 718,7 | 591,7 | 541,7 | 459,4 |
| TRANSPORT C0 | CO2 emissions from passenger cars, kt | CO2 emissions from passenger cars, kt |  |  |  |  |
|  | Number of kilometres by passenger cars, Mkm | Number of kilometres by passenger cars, Mkm | 157,8 | 152,4 | 148,6 | 146 |
| TRANSPORT D0 | CO2 emissions from freight transport (all modes), kt | CO2 emissions from freight transport (all modes), kt |  |  |  |  |
|  | Freight transport (all modes), Mtkm | Freight transport (all modes), Gtkm | 112,2 | 111,2 | 101,9 | 93,4 |
| INDUSTRY A1 | Energy related CO2 intensity of industry, t/Mio Euro | CO2 emissions from fossil fuel consumption industry, kt |  |  |  |  |
|  |  | Gross value-added total industry, Bio Euro (EC95) | 347,8 | 294,2 | 242,6 | 209,7 |
| HOUSEHOLDS A1 | Specific CO2 emissions of households, t/dwelling | CO2 emissions from fossil fuel consumption households, kt |  |  |  |  |
|  |  | Stock of permanently occupied dwellings, 1000 | 2,1 | 1,9 | 1,8 | 1,7 |
| SERVICES A0 | CO2 intensity of the services sector, t/Mio Euro | CO2 emissions from fossil fuel consumption services, kt |  |  |  |  |
|  |  | Gross value-added - services, Bio Euro (EC95) | 135,7 | 107,3 | 85,6 | 68,3 |
| TRANSFORMATION B0 | Specific CO2 emissions of public and autoproducer power plants, t/TJ | CO2 emissions from public and autoproducer thermal power stations, kt |  |  |  |  |
|  |  | All products –output by public and autoproducer thermal power stations, PJ | 103,9 | 89,9 | 78,1 | 71,3 |
| AGRICULTURE | Specific N2O emissions of fertiliser and manure use, kg/kg | N2O emissions from synthetic fertiliser and manure use, kt |  |  |  |  |
|  |  | Use of synthetic fertiliser and manure, Mt nitrogen | 16,51 | 16,48 | 16,47 | 16,47 |
| AGRICULTURE | Specific CH4 emissions of cattle production, kg/head | CH4 emissions from cattle, kt |  |  |  |  |
|  |  | Cattle population, mio heads | 74,8 | 74,3 | 73,1 | 73,1 |
| WASTE | Specific CH4 emissions from landfills, kt/kt | CH4 emissions from landfills, kt |  |  |  |  |
|  |  | Municipal solid waste going to landfills, Mt | 24,058 | - | - | - |

Mandatory parameters on projections

| Mandatory parameters on projections |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 2005 | 2010 | 2015 | 2020 |
| GENERAL ECONOMIC PARAMETERS |  |  |  |  |  |  |  |
| GDP |  |  | MEUR[95]/a | 21,412 | 25,618 | 28,700 | 31,837 |
| POPULATION |  |  | [1000 inh] | 1985 | 1980 | 1975 | 1964 |
| INTERNATIONAL PRICES | COAL |  | EUR[95] / t | 46.6 | 46.6 | 46.6 | 46.6 |
|  | OIL |  | EUR[95] / barrel | 22.8 | 20.8 | 23.8 | 27.7 |
|  | GAS |  | EUR[95] / m3 | 2.8 | 2.7 | 3.1 | 4.0 |
| ENERGY SECTOR |  |  |  |  |  |  |  |
| GROSS INLAND CONSUMPTION | OIL |  | [PJ] | 105.58 | 107.72 | 110.76 | 109.60 |
|  | GAS |  | [PJ] | 40.34 | 48.41 | 58.51 | 68.45 |
|  | COAL |  | [PJ] | 65.15 | 51.96 | 43.50 | 36.90 |
|  | RENEWEBLES |  | [PJ] | 32.86 | 35.31 | 39.59 | 40.86 |
|  | NUCLEAR |  | [PJ] | 63.38 | 63.97 | 58.51 | 58.51 |
| TOTAL ELECTRICITY PRODUCTION | THERMAL |  | [TWh] | 5.81 | 5.45 | 5.36 | 5.36 |
|  | HYDRO AND WIND |  | [TWh] | 3.94 | 4.51 | 4.95 | 5.16 |
|  | NUCLEAR |  | [TWh] | 5.42 | 5.92 | 6.56 | 8.01 |
| ENERGY DEMAND | ENERGY INDUSTRIES | OIL | [PJ] | 1.32 | 0.86 | 0.71 | 1.36 |
|  |  | GAS | [PJ] | 6.99 | 13.91 | 22.09 | 31.31 |
|  |  | COAL | [PJ] | 61.32 | 48.05 | 39.74 | 33.28 |
|  |  | RENEWEBLES | [PJ] | 16.62 | 18.52 | 22.05 | 23.11 |
|  |  | NUCLEAR | [PJ] | 63.38 | 63.97 | 58.51 | 58.51 |
|  | INDUSTRY | OIL | [PJ] | 7.54 | 7.57 | 6.98 | 6.64 |
|  |  | GAS | [PJ] | 26.05 | 26.38 | 26.53 | 26.25 |
|  |  | COAL | [PJ] | 3.25 | 3.40 | 3.41 | 3.40 |
|  |  | RENEWEBLES | [PJ] | 3.19 | 3.22 | 3.27 | 3.30 |

|  |  |  |  | 2005 | 2010 | 2015 | 2020 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ELECTRICITY | [PJ] | 22.45 | 22.27 | 22.17 | 22.51 |
|  | COMMERCIAL | OIL | [PJ] | 16.35 | 16.39 | 15.84 | 14.94 |
|  |  | GAS | [PJ] | 4.05 | 4.94 | 5.66 | 6.22 |
|  |  | COAL | [PJ] | 0.23 | 0.19 | 0.13 | 0.07 |
|  |  | RENEWEBLES | [PJ] | 1.91 | 2.42 | 2.78 | 3.05 |
|  |  | ELECTRICITY | [PJ] | 8.83 | 10.40 | 11.34 | 11.97 |
|  | RESIDENTIAL | OIL | [PJ] | 14.99 | 14.26 | 13.56 | 12.90 |
|  |  | GAS | [PJ] | 3.16 | 3.58 | 4.04 | 4.46 |
|  |  | COAL | [PJ] | 0.32 | 0.27 | 0.20 | 0.14 |
|  |  | RENEWEBLES | [PJ] | 10.29 | 10.31 | 10.21 | 10.08 |
|  |  | ELECTRICITY | [PJ] | 10.16 | 11.00 | 11.56 | 12.21 |
|  | TRANSPORT | OIL | [PJ] | 59.64 | 63.52 | 65.48 | 65.70 |
|  |  | COAL | [PJ] | 0.01 | 0.01 | 0.01 | 0.01 |
|  |  | ELECTRICITY | [PJ] | 0.73 | 0.86 | 1.00 | 1.08 |
| HEATING DEGREE DAYS |  |  | [°C day] | 2836 | 2836 | 2836 | 2836 |
| INDUSTRY SECTOR |  |  |  |  |  |  |  |
| SHARE OF INDUSTRIAL SECTOR IN GDP |  |  | [%] | 28.5 | 28.6 | 28.8 | 28.7 |
| TRANSPORT |  |  |  |  |  |  |  |
| PASSENGER PERSON KILOMETRES |  |  | [mio pkm] | 28,435 | 31,088 | 33,491 | 35,374 |
| FREIGHT TONNE KILOMETRES |  |  | [mio tkm] | 10,967 | 13,343 | 15,847 | 17,929 |
| BUILDINGS |  |  |  |  |  |  |  |
| FLOOR SPACE |  |  | [1000 m2] | 54,970 | 60,046 | 64,401 | 68,560 |
| NUMBER OF DWELLINGS |  |  | [] | 749,264 | 779,997 | 800,486 | 817,525 |

|  |  |  |  | 2005 | 2010 | 2015 | 2020 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AGRICULTURE SECTOR |  |  |  |  |  |  |  |
| CATTLE | DAIRY COWS |  | [] | 133,986 | 123,996 | 114,000 | 114,000 |
|  | SUCKLING COWS |  | [] | 66,777 | 86,384 | 86,384 | 86,384 |
|  | OTHER CATTLE |  | [] | 290,803 | 309,889 | 327,316 | 327,316 |
| PIGS |  |  | [] | 657,819 | 661,410 | 665,000 | 665,000 |
| POULTRY |  |  | [] | 5,111,769 | 5,331,385 | 5,551,000 | 5,551,000 |
| HORSES |  |  | [] | 15,000 | 15,000 | 15,000 | 15,000 |
| SHEEP |  |  | [] | 100,000 | 100,000 | 100,000 | 100,000 |
| GOAT |  |  | [] | 22,000 | 22,000 | 22,000 | 22,000 |
| USE OF MINERAL FERTILISERS |  |  | [t] | 34,267 | 33,662 | 33,100 | 33,100 |
| WASTE SECTOR |  |  |  |  |  |  |  |
| MUNICIPAL SOLID WASTE |  |  | [t] | 1217 | 1217 | 1217 | 1217 |
| FRACTION OF ORGANIC WASTE |  |  | [%] | 47 | 47 | 47 | 47 |
| TREATMENT OF WASTE | DISPOSED |  | [%] | 94 | 0 | 0 | 0 |
|  | INCINERATED |  | [%] | 0 | 63 | 50 | 50 |
|  | RECYCLED |  | [%] | 6 | 38 | 50 | 50 |

## Country Conclusions

The two main documents used for this summary were: Slovenia’s Second and Third National Communication, July 2004 for policies and measures and the 2005 report submitted under the Monitoring Mechanism (Decision 280/2004/EC).   The projections and the base year emissions (1986 and 1995 for F-gases) in the tables above are based on the 2005 Monitoring Mechanism Submission and the latest 2003 CRF submitted in April 2005.  The presentation in chapters dealing with policies and measures is in line with UNFCCC guidelines. The measures for the with domestic measures scenario and with additional domestic measures scenario are discussed for the following sectors: energy, transport, agriculture and forestry, waste, F-gases and cross sectoral.

The key policies and measures are clearly presented in tables, potential greenhouse gas savings are summarised by sector and for some individual policies and measures.  Projections are provided for the with domestic measures and with additional domestic measures scenarios.  Projections are broken down into 6 greenhouse gases and by IPPC sector (energy, industrial processes, solvent use, agriculture, waste and sinks). Key assumptions behind the projections are provided and a more detailed breakdown of parameters and assumptions behind both projection scenarios is given.

Slovenia’s Kyoto commitment is an 8% reduction in greenhouse gas emissions relative to the base year. The with additional domestic measures projection shows that with implemented, adopted and planned measures, Slovenia will increase its greenhouse gas emissions by 2010 by approximately 0.4% without LUCF.   When sinks are included in the with additional domestic measures projection, Slovenia’s emissions decrease by 8% relative to the base year, and Slovenia is therefore projected to just meet its Kyoto target in the first commitment period.

Total sinks are estimated at 1.68 Mt CO2 annually for 2008-2012.  It is the saving from total sinks and additional domestic measures that gives the emission reduction of 8% relative to the base year for the additional domestic measures projection.

For the main EEA Trends and Projections report, all projections for all member states are used without land use change and forestry, if considered without sinks, Slovenia will be about 8% short of its Kyoto target based on the ‘with additional measures’ projection.

Transport in construction, agriculture, forestry and railway transportation

Sinks are not taken into account in the total potencial of measures for sector Agriculture and Forestry.

SLOVENIA’S EU SUBMISSION IN ACCORDANCE WITH DECISIONS 280/2004/EC AND 166/2005/EC.  English Translation submitted to ETC ACC on 20/10/2005