# Finland

## Sources of Information

Finland’s report to the EU Commission of projected progress in accordance with article 3.2 of the Council Decision No280/2004/EC on the mechanism for monitoring Community greenhouse gas emissions and for implementing the Kyoto Protocol. Submitted in June 2005.

- Information on Kyoto Mechanisms based on the submission of comments on the EEA Draft Report by Finland on 23rd September 2005.

## Quality and Transparency of Reporting

The report submitted by Finland provides information on policies and measures, and a summary of its progress towards a commitment under the EU burden sharing agreement to maintain greenhouse gas emissions at 1990 levels.

The report gives only a brief summary of policies and measures being taken or planned for implementation, but explains that they are described in the national Climate and Energy Strategy. These are common and co-ordinated polices such as the EU ETS, the landfill directive and the promotion of electricity from renewable sources. These policies and measures are presented without possible greenhouse gas savings and therefore a comprehensive quantitative assessment by policy would be needed to carry out a full analysis of Finland’s progress.

Projections are presented as “with measures” and “with additional measures” and are presented by both gas and sector.

- Table 1: Information Provided on Policies and Measures

| - Information provided | - Level provided | - Comments |
| --- | --- | --- |
| - Policy names | - ++ | - Only common or co-ordinated policies. |
| - Objectives of policies | - + |  |
| - Which greenhouse gases? | - o | - No description of gas savings by policy |
| - Status of Implementation | - ++ | - Clearly stated |
| - Implementation body specified | - + | - Information provided directly from Finnish Government |
| - Quantitative assessment of implementation | - o | - No quantitative assessment of impacts of policies |
| - Interaction with other P&Ms discussed | - ++ | - CCPM interaction given |

- o, +, ++, +++ level of information available increases as the number of + signs increases

- Table 2: Information provided on Projections

| - Category of Information | - Level of Information Provided | - Comments |
| --- | --- | --- |
| - Scenarios considered | - +++ | - “with measures” and “with additional measures” |
| - Expressed relative to inventory for previous years | - 1990, 2000 |  |
| - Starting year | - 1990 | - To 2020 |
| - Split of projections | - ++ | - Split by sector and gas. Not clear what base year was |
| - Presentation of results | - ++ |  |
| - Description of model (level of detail, approach and assumptions) | - ++ |  |
| - Discussion of uncertainty | - + | - No discussion |
| - Details of parameters and assumptions | - +++ | - Excellent coverage of assumptions and parameters |

- o, +, ++, +++ level of information available increases as the number of + signs increases

- The 2005 Finland report contains projections for the “with measures” and “with additional measures” scenarios for carbon dioxide, methane, nitrous oxide and F-gases.  CO2 projections “with measures” and “with additional measures” only describe the projections excluding sinks. Base years are not made explicitly clear, but tables of projections data do indicate 1990 as the base year, which is correct.  Greenhouse gas savings by policy and measure (the effect of policies and measures) were not provided. This is because Finland has only summarised common policies and measures.   Projections of gases provided by Finland are for five-year intervals to the year 2020.  An excellent summary of parameters is provided.

## Assessment of Policies and Measures

- The effect of policies and measures is the difference between the ‘with measures’ and ‘with additional measures’ projections.

- Table 3: Summary of the Effect of Policies and Measures Included in the Projections (MtCO2)

|  | With measures | With additional measures |
| --- | --- | --- |
| Energy (total, excluding transport) |  | 7.4 |
| Transport (energy) |  | 0.7 |
| Industrial processes |  | 0.4 |
| Waste |  | 0.1 |
| Agriculture |  | 0.0 |
| Total (excl. sinks) |  | 8.7 |

The policies in Table 4 below are those included in the “with measures” projections of carbon dioxide savings. Policies and measures in the “with additional measures” projection were not presented.

- Table 4: Detailed Information on Polices and Measures

| Sector | Name | Objective | GHG affected | Type of instrument |  | Status | Implementing entity | Estimate of savings (MtCO2) |  | ### CCPM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | 2010 | 2020 |  |
| Policies and measures in the “with measures” projection |  |  |  |  |  |  |  |  |  |  |
| Energy | Taxation of energy products, Council Directive 2003/96/EC | Duty on certain fuels and products | CO2 |  | Fiscal | Implemented | Ministry of Finance |  |  | Yes |
| Cross-sectoral | Emissions Trading Directive 2003/87 | Trading of emissions allowances to reduce carbon emissions from energy intensive industries | CO2 |  | Regulatory | Implemented | Ministry of Trade and Industry |  |  | Yes |
| Households | Directive 2002/91/EC on the energy performance of buildings | Standards for thermal insulation, indoor climate and ventilation | CO2 |  | Regulatory | Implemented | Ministry of the Environment |  |  | Yes |
| Energy | Directive 2001/77/EC on the promotion of electricity produced from renewable energy sources in the internal electricity market |  | CO2 |  | Regulatory | Implemented | Ministry of Trade and Industry |  |  | Yes |
| Energy | The Electricity Market Act |  |  |  |  | Implemented | Ministry of Trade and Industry |  |  | Yes |
| Waste | Council Directive 1999/31/EC on the landfill of waste | Reduction in waste to landfills | CH4 |  | Regulatory | Implemented | Ministry of the Environment |  |  | Yes |
|  | Council Regulation (EC) 1783/2003 |  |  |  |  | Implemented | Ministry of Agriculture and Forestry |  |  | Yes |
| Transport | 2001/12 EC, 2001/13/EC, 2001/14/EC | Modal shift, particularly towards rail travel | CO2 |  |  | Implemented | Ministry of Transport and Communications |  |  | Yes |

## Evaluation of Projections

- The projection scenario represents a projection “with measures” and “with additional measures”. The projections were finalized quite recently, and the “with additional measures” projection was still a preliminary one as the National Climate and Energy Strategy was not finalized at that time.  The results of the projections “with additional measures” show that Finland will achieve its EU Burden Sharing commitment under the Kyoto Protocol, if Kyoto Mechanisms are included. Excluding Kyoto mechanisms, Finland misses its EU burden sharing commitment by only 0.9% or 0.6Mt CO2e and one should note that data for 2010 is not necessarily the same as the average for years 2008–2012 and that the 2010 value is to be taken as indicative for the ‘with additional measures projection’.  The 0.6Mt CO2e reduction in the target caused by the Kyoto mechanisms enables Finland to meet it target of 0% emissions change relative to the base year.

- Table 5: Summary of projections by gas in 2010 (MtCO2)

|  | Base year | with measures | with additional measures |
| --- | --- | --- | --- |
| Carbon dioxide (excl. LULUCF) | 56.3 | 67.9 | 59.8 |
| Methane | 6.4 | 4.3 | 4.2 |
| Nitrous oxide | 7.6 | 6.5 | 6.4 |
| F-gases* | 0.1 | 1.1 | 0.6 |
| Total (excl. LULUCF) | 70.5 | 79.7 | 71.1 |
| % change relative to base year (excl. LULUCF) |  | 13.2% | 0.9% |
| *base year for F-gases for Finland is 1990 |  |  |  |

Table 6:  Summary of projections (6 gas basket) by sector in 2010 (MtCO2eq)

|  | Base year | with measures | % change relative to 1990 | with additional measures | % change relative to 1990 |
| --- | --- | --- | --- | --- | --- |
| Energy (total, excluding transport) | 43.8 | 51.9 | 18% | 44.4 | 1% |
| Transport (energy) | 12.6 | 13.9 | 11% | 13.2 | 5% |
| Industrial processes | 3.0 | 5.0 | 64% | 4.6 | 49% |
| Waste | 4.0 | 2.3 | -41% | 2.3 | -43% |
| Agriculture | 7.0 | 6.6 | -6% | 6.6 | -6% |
| Total (excl. sinks) | 70.5 | 79.7 | 13% | 71.1 | 1% |

Table 7: Assessment of the Target

|  | Excl. sinks |  |
| --- | --- | --- |
|  | MtCO2 equiv. | % of base year level (six gas basket) |
| Base year emissions (from projections) | 70.5 | 100% |
| Kyoto Commitment/burden sharing | 70.5 | 0.0% |
| With existing P&Ms projections | 79.7 | 113.2% |
| Gap (-ve means no gap) | 9.3 | 13.2% |
| With additional P&Ms projections | 71.1 | 100.9% |
| Remaining gap | 0.6 | 0.9% |
| Effect of flexible mechanisms | 0.6 | 0.9% |
| Remaining gap (if target includes flex mechs) | 0.0 | 0.0% |

In the case of Finland the ‘with additional measures’ projection value for 2010 is for year 2010, not the average over 2008-2012.

## Description of Modelling ApproacH & indicator reporting

Finland provided description on the assumptions and methodology behind the projections model.  The calculations of With Measures –projection are based on average growth of an over 2 per cent in the gross domestic product over the whole period considered. At the end of the period examined, the economic growth will be somewhat slower than at its beginning.

Finland estimated that services will grow slightly more quickly in the whole period examined than the gross domestic product, which increases the proportion of services of the national economy.  The technological development is assumed to proceed as a trend both in energy and other sectors. No quick technological leaps or transfers are expected in the period examined.

As a result of demand assumptions the total consumption of electricity will grow by an average of a little more than 1 per cent per year for the period examined. The most remarkable change in the supply of electricity is the fifth nuclear power unit, which is assumed to be completed in 2009. Net imports of electricity are assumed to decrease due to decreasing opportunities of other Nordic countries to export electricity. The production of hydropower will remain at approximately the current level. The amount of wind power and other new modes of electricity production will increase but remain at a quite slow.

A sample of the key parameters provided by Finland is given below.

Modelling parameters additional to core indicators

|  | Parameters | Years |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No |  | 2005 | 2010 | 2015 | 2020 |
|  | General economic parameters |  |  |  |  |
| 1 | Gross domestic product | 126 337 | 145 902 | 162 025 | 178 983 |
| 2 | Population | 5 230 | 5 275 | 5 303 | 5 319 |
| 3 | International coal prices at given years in € per GJ | 1.64 | 1.71 | 1.75 | 1.79 |
| 4 | International oil prices at given years in € per GJ | 4.71 | 4.27 | 4.64 | 5 |
| 5 | International gas prices at given years in € per GJ | 3.61 | 3.44 | 3.69 | 3.95 |
| 6 | Total gross inland consumption, PJ | 1458 | 1549 | 1584 | 1616 |
| 13 | Total electricity production, TWh | 76.54 | 85.72 | 90.14 | 95.69 |
| 20 | Energy demand by fuel, PJ |  |  |  |  |
| 21 | energy industries | 607 | 674 | 701 | 735 |
| 22 | oil | 12 | 8 | 7 | 7 |
| 23 | gas | 88 | 123 | 128 | 133 |
| 24 | coal | 122 | 32 | 48 | 68 |
| 25 | renewables | 89 | 98 | 102 | 108 |
| 26 | nuclear | 235 | 377 | 377 | 377 |
| 27 | other | 62 | 37 | 39 | 42 |
| 56 | heating degree days | 4560 | 4560 | 4560 | 4560 |
| 57 | Production index of industrial sector (2000=100) | 112 | 129 | 142 | 156 |
| 60 | The rate of change of floor space for tertiary buildings and dwellings | 1.20% | 1.10% | 1.00% | 0.90% |
| 66 | Waste generation per head of population (t/head) | 0.47 | 0.49 | 0.5 | 0.51 |
| 67 | The organic fractions of municipal solid waste, Mt | 2.1 | 2.19 | 2.25 | 2.29 |
| 68 | Municipal solid waste, Mt | 2.48 | 2.58 | 2.64 | 2.7 |
| 69 | disposed to landfills | 1.31 | 1.23 | 0.96 | 0.9 |
| 70 | incinerated | 0.26 | 0.3 | 0.51 | 0.56 |
| 71 | composted | 0.3 | 0.4 | 0.49 | 0.52 |

A comprehensive list of indicators was provided by Finland and up-dated in the June submission.  The indicators are shown below in Table 8.  In future years, it would be useful if Finland also provided the numerator or denominator as well as the indicators.

Table 8) Indicators for projections to monitor and evaluate progress with policies and measures (2005/166/EC) Annex III

| No | Eurostat Sectors | Indicator | 2005 | 2010 | 2015 | 2020 | Numerator/denominator | 2005 | 2010 | 2015 | 2020 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Macro | CO2 intensity of GDP, t/Euro million | 516 | 449 | 409 | 388 | Total CO2 emissions, kt |  |  |  |  |
|  |  |  |  |  |  |  | GDP, bio Euro (EC95) |  |  |  |  |
| 2 | Transport C0 | CO2 emissions from passenger cars, kt | 6751 | 6513 | 6389 | 6240 |  |  |  |  |  |
|  |  | Number of kilometres by passenger cars, Mkm | 41415 | 42117 | 42952 | 43981 |  |  |  |  |  |
| 3 | Transport D0 | CO2 emissions from freight transport (all modes), kt | 4646 | 4868 | 4878 | 4854 |  |  |  |  |  |
|  |  | Freight transport (all modes), Mtkm |  |  |  |  |  |  |  |  |  |
| 4 | Industry A1 | Energy related CO2 intensity of industry, kt/Euro million | 0.54 | 0.54 | 0.51 | 0.47 | CO2 emissions from fuel consumption industry, mt |  |  |  |  |
|  |  |  |  |  |  |  | Gross value-added total industry, Bio Euro (EC 95) |  |  |  |  |
| 5 | Households A1 | Specific CO2 emissions of households, t/dwelling | 1.1 | 0.9 | 0.8 | 0.7 | CO2 emissions from fossil fuel consumption households, Mt | 1.1 | 0.9 | 0.8 | 0.7 |
|  |  |  |  |  |  |  | Stock of permanently occupied dwellings, 1000 |  |  |  |  |
| 6 | Services A0 | CO2 intensity of the services sector, t/Euro million | 13.8 | 9.7 | 6.5 | 4.2 | CO2 emissions from fossil fuel consumption services, kt | 1086 | 872 | 657 | 474 |
|  |  |  |  |  |  |  | gross value-added services, bio Euro (EC95) | 78.9 | 89.8 | 101.4 | 113.4 |
| 7 | Transformation B0 | Specific CO2 emissions of public and autoproducer power plants, t/TJ | 135.7 | 110 | 105.7 | 140.9 | CO2 emissions from public and autoproducer thermal power stations, kt |  |  |  |  |
|  |  |  |  |  |  |  | all products-output by public and autoproducer thermal power stations, PJ |  |  |  |  |
| 8 | Agriculture | Specific N2O emissions of fertilizer and manure use, kg/kg | 0.017 | 0.017 | 0.017 | 0.017 | N2O emissions from synthetic fertilizer and manure use, kt |  |  |  |  |
|  |  |  |  |  |  |  | use of synthetic fertiliser and manure, kt nitrogen |  |  |  |  |
| 9 | Agriculture | Specific CH4 emissions of cattle production, kg/head | 0.068 | 0.07 | 0.072 | 0.074 | CH4 emissions from cattle, kt |  |  |  |  |
|  |  |  |  |  |  |  | cattle populations, 1000 head |  |  |  |  |
| 10 | Waste | Specific CH4 emissions from landfills, kt/kt | 1.8 | 1.6 | 1.4 | 1.3 | CH4 emissions from landfills, kt |  |  |  |  |

## Country Conclusions

Projections for Finland are presented in the June 2005 Monitoring Mechanism Submission. Finnish emissions of the basket of six greenhouse gases in 2010 “with measures” are projected to be 13.2% above base year levels, due to increases in carbon dioxide and a minor increase in F-gases. The biggest contributors to this change are the energy sector and industrial processes which increase by 18% and 64% respectively compared to base years. Land use changes and forestry are not included in this projection, so the expected savings may differ slightly if this data was included.  Finland’s EU burden sharing target is to meet the 1990 levels exactly. When the “with additional measures” scenario is included, Finnish emissions are expected to exceed the base year levels by 0.9% or 0.6MtCO2e, putting very close to achieving the burden sharing commitment. One should remember that the emissions for year 2010 are indicative and may not be equal to the 2008–2012 average.  Finland is exactly on target to meet its 0% Kyoto commitment, if the reduction of 0.6MtCO2e from Kyoto Mechanisms is included.

The report splits projections into five broad sectors (energy, transport, industrial processes, agriculture, waste and other) and four gases (CO2, CH4, N2O and F-gases). Projections were presented clearly. A summary solely on the projections by gas would have benefited inclusions in the projections section. The report presented information on policies and measures, but there was no quantification of the effect of policies and measures.   More information could be provided on sector, objectives, type of policy/measure, greenhouse gas and implementing body. Finland provided a good data set for indicators for projections.  Finland submitted a comprehensive indicators list that will be used in the annex to the EEA Report.