# UK

## Sources of Information

- UK’s report to the EU Commission made under Decision 280/2004/EC of the European Parliament and of the Council of 11 February 2004 concerning a mechanism for monitoring Community greenhouse gas emissions and for implementing the Kyoto Protocol, June 2005.

- UK’s report to the EU Commission made under Decision 280/2004/EC of the European Parliament and of the Council of 11 February 2004 concerning a mechanism for monitoring Community greenhouse gas emissions and for implementing the Kyoto Protocol, March 2005

- In September 2004, the UK commenced a review of its Climate Change Programme published in November 2000.  The review is looking at how existing policies are performing and the range of policies and measures that might be put in place in future to put the UK back on track to achieving its domestic goal of reducing carbon dioxide emissions by 20 per cent below 1990 levels by 2010 and to make "real progress by 2020" towards the UK's longer-term goal of some 60 per cent reduction in carbon dioxide emissions below 1990 levels by 2050.  A revised Programme will be published before the end of the year.  The Programme will include disaggregated data on the performance of adopted UK policies and measures.

## Quality and Transparency of Reporting

The UK’s report under the monitoring mechanism, submitted in June 2005, provides a summary of the progress towards the UK’s commitment under the EU burden sharing agreement to reduce its greenhouse gas emissions by 12.5%.

The report gives a detailed description of national policies and measures to reduce greenhouse gases, covering the EU Emissions Trading scheme and six broad sectors – energy supply, business, transport, households, agriculture, forestry and land use and the public sector.  A table of savings by group of policies is presented, though a more comprehensive split of savings by individual policy would facilitate the analysis of results.

Projections are presented clearly for the ‘with measures’ projection broken down by sector and gas.  However, no ‘with additional measures’ projections are given in this submission because the UK projects it can comfortably meet its Kyoto target based on the ‘with measures’ projection alone.

- Table 1: Information Provided on Policies and Measures

| - Information provided | - Level provided | - Comments |
| --- | --- | --- |
| - Policy names | - ++ | - Good description of policies, policy tables could be more comprehensive and disaggregated |
| - Objectives of policies | - ++ | - Text explains policy objectives |
| - Which greenhouse gases? | - All | - Clear split of projections by GHG |
| - Status of Implementation | - ++ |  |
| - Implementation body specified | - ++ | - Information was provided |
| - Quantitative assessment of implementation | - ++ | - Clearly presented, more disaggregation of savings would help |
| - Interaction with other P&Ms discussed | - ++ | - Interaction of P&Ms discussed for specific policies |

- o, +, ++, +++ level of information available increases as the number of + signs increases

- Table 2: Information provided on Projections

| - Category of Information | - Level of Information Provided | - Comments |
| --- | --- | --- |
| - Scenarios considered | - With measures | - With additional measures not covered as WM meets Kyoto target |
| - Expressed relative to inventory for previous years | - 2002 | - Based on 2002 GHG inventory data |
| - Starting year | - 1990 | - From 1990 to 2020 |
| - Split of projections | - +++ | - Good split of projections by gas and sector |
| - Presentation of results | - +++ | - Clear presentation of projections tables |
| - Description of model (level of detail, approach and assumptions) | - +++ | - Clear description of model |
| - Discussion of uncertainty | - +++ | - Breakdown of uncertainty sources given |
| - Details of parameters and assumptions | - +++ | - Detailed description of indicators and parameters |

- o, +, ++, +++ level of information available increases as the number of + signs increases

The latest projections from the 2005 report contain a with measures  scenario covering all six greenhouse gases with a base year of 1990 for carbon dioxide, methane and nitrous oxide and 1995 for hydrofluorocarbons, perfluorocarbons and sulphur hexafloride.  This projection includes measures introduced since Kyoto.  The effect of policies and measures is presented for groups of policies, a greater breakdown of savings for key policies would improve the information assessment.  However, only the effect of policies and measures addressing carbon dioxide emissions is covered –  savings of the other five GHGs by policy are not provided.

Projections are for five year intervals to 2020 and are disaggregated both by end user and by source categories. There are 9 source category sectors: energy supply; business; industrial processes; transport; residential, public, agriculture, land use change and waste management.

## Assessment of Policies and Measures

- The greenhouse gas savings for the policies in the table below are included in the latest carbon dioxide projections for the ‘with domestic measures’ scenario.  The unallocated energy policy savings are cross sectoral.

- Table 3: Summary of the Effect of Domestic Policies and Measures Included in the Projections (MtCO2)

|  | With measures | With additional measures |
| --- | --- | --- |
| Households (energy) | 11.0 |  |
| Business (energy) | 19.0 |  |
| Public (energy) | 2.2 |  |
| Unallocated (energy) | 4.9 |  |
| Transport (energy) | 16.2 |  |
| Agriculture, Forestry and Land Use | 2.4 |  |
| Total | 55.7 | 0.0 |

- The policies in Table 4 below are included in the latest carbon dioxide projections.  The ‘with domestic measures’ projection includes the emission reductions from policies introduced after the Kyoto Protocol was negotiated and also since the Climate Change Programme was published in 2000.  Carbon savings were converted from MtC to MtCO2 using the conversion factor 44/12.

- Table 4: Detailed Information on Polices and Measures

| Sector | Name | Objective | GHG affected | Type of instrument | Status | Implementing entity | Estimate of savings (MtCO2) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 2010 | 2020 |
| Policies and measures in the “with measures” projection |  |  |  |  |  |  |  |  |
| Households | Energy Efficiency Commitment, Warm Front, Building Regulations 2002 & Community Energy | Increase domestic energy efficiency. Reduce fuel poverty. Implement CHP systems. | CO2 | Regulatory, Fiscal, | Implemented | Power companies, central government, Energy Saving Trust | 11.04 | - |
| Business | Climate Change Agreements, UK emissions trading scheme, Carbon Trust programmes and Building Regulations 2002 | Reduce carbon emissions from the business sector and encourage energy efficiency measures. | CO2 | Negotiated agreements, Economic, Fiscal, Information, Regulatory | Implemented | Central government, trade associations, Carbon Trust | 19.03 | - |
| Public | New targets for central government, schools and National Health Service | Reduce carbon emissions from fuel and electricity used in central government buildings. Promote energy efficiency in schools. NHS energy saving targets. | CO2 | Information, other. | Implemented | Central government, NHS, schools | 1.83 | - |
| Transport | Voluntary Agreements, the 10 Year Transport Plan, sustainable distribution, Off road programmes | Increased mobility with reduced environmental impact. | CO2 | Voluntary agreements, Fiscal. | Implemented | Car manufacturers, Central government | 16.21 | - |
| Agriculture, Forestry and Land Use | Afforestation | Increase tree cover to act as a carbon sink. | CO2 | Other | Implemented |  | 2.38 | - |
| Scottish Executive | Building Regulations, new central estate and National Health Service in Scotland targets | Reduced building energy consumption. NHS energy saving targets. | CO2 | Regulatory, information, other. | Implemented | Scottish Executive | 0.33 | - |
| Total |  |  |  |  |  |  | 50.82 | - |
| Unallocated measures | Policies include additional CCAs, Building Regulations 2005, minimum product standards | Improved energy performance in buildings. Market shift towards more energy efficient products. | CO2 | Negotiated agreement, regulatory, economic | Adopted/ Implemented | Central government | 4.913333 | - |
| Total savings including unallocated measures |  |  |  |  |  |  | 55.73 | - |

## Evaluation of Projections

- Table 5: Summary of projections by gas in 2010 (MtCO2)

|  | Base year | with measures | with additional measures |
| --- | --- | --- | --- |
| Carbon dioxide (incl. Land use, exc. sinks) | 591.4 | 511.9 | - |
| Carbon dioxide (excl. LULUCF) | 588.9 | 514.1 | - |
| Methane | 77.4 | 33.4 | - |
| Nitrous oxide | 67.8 | 40.3 | - |
| HFCs* | 15.4 | 9.9 | - |
| PFCs | 0.4 | 0.4 | - |
| SF6 | 1.5 | 1.5 | - |
| Total (inc land use, excl. sinks) | 753.9 | 597.3 | - |
| Total ( excl. LULUCF) | 751.3 | 599.5 | - |
| % change relative to base year (inc land use, excl. sinks) | - | -20.8% | - |
| % change relative to base year (excl LULUCF) | - | -20.2% | - |
| *base year for F-gases is 1995 |  |  |  |

Table 6:  Summary of projections (6 gas basket) by sector in 2010 (MtCO2eq)

|  | Base year | with measures | % change relative to 1990 | with additional measures | % change relative to 1990 |
| --- | --- | --- | --- | --- | --- |
| Energy (total, excluding transport) | 481.8 | 367.0 | -24% | - | - |
| Energy supply | 274.6 | 191.4 | -30% | - | - |
| Energy – business | 111.5 | 89.1 | -20% | - | - |
| Energy – residential | 79.6 | 75.5 | -5% | - | - |
| Energy - public | 16.1 | 11.0 | -32% | - | - |
| Transport (energy) | 125.4 | 143.4 | 14% | - | - |
| Industrial processes | 60.1 | 36.7 | -39% | - | - |
| Waste | 27.9 | 8.1 | -71% | - | - |
| Agriculture | 56.5 | 43.6 | -23% | - | - |
| Land Use Change (sinks) | 2.6 | -2.2 | -186% | - | - |
| Total (excl. sinks & land use) | 751.7 | 598.8 | -20% | - | - |
| Total (incl. land use, exc sinks) | 754.2 | 596.6 | -21% | - | - |

| Table 7: Assessment of the Target | Excl. sinks & land use |  | Excl. sinks, Inc Land use |  |
| --- | --- | --- | --- | --- |
|  | MtCO2 equiv. | % of base year level (six gas basket) | MtCO2 equiv. | % of 1990 level (six gas basket) |
| Base year emissions (from projections) | 751.7 | 100% | 753.9 | 100% |
| Kyoto Commitment/burden sharing | 657.7 | -12.5% | 659.6 | -12.5% |
| With existing P&Ms projections | 598.8 | 79.7% | 597.3 | 79.2% |
| Gap (-ve means no gap) | -58.9 | -7.8% | -62.3 | -8.3% |
| With additional P&Ms projections | 0.0 | 0.0% | 0.0 | 0.0% |
| Remaining gap | -58.9 | -7.8% | -62.3 | -8.3% |

## Description of Modelling ApproacH & indicator reporting

Modelling parameters additional to core indicators

|  | 2006 | 2011 | 2016 | 2021 |
| --- | --- | --- | --- | --- |
| Population (M) | 60.25 | 61.4 | 62.6 | 63.8 |
|  | 2005 | 2010 | 2015 | 2020 |
| GDP annual growth rate and base year | 3.25% (change on 2004) | 2.5% (change on 2009) | 2.5% (change on 2014) | 2.5% (change on 2019) |
| International fuel prices |  |  |  |  |
| Coal (Euro/t in 2004 prices) | 61.2 | 27.8 | 27.8 | 27.8 |
| Oil (Euro/bb in 2004 prices) | 23.8 | 18.4 | 20.2 | 21.9 |
| Gas (Euro/GJ in 2004 prices) | 3.55 | 2.89 | 3.10 | 3.32 |

Table 8) Indicators for projections to monitor and evaluate progress with policies and measures (2005/166/EC) Annex III

| No | Eurostat Sectors | Indicator | 2005 | 2010 | 2015 | 2020 | Numerator/denominator | 2005 | 2010 | 2015 | 2020 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Macro | CO2 intensity of GDP, t/Euro million |  |  |  |  | Total CO2 emissions, kt | 148,600 | 139,600 | 140,400 | 142,200 |
|  |  |  |  |  |  |  | GDP, bio Euro (EC95) | 1,223.118 | 1,387.221 | 1,569.514 | 1,775.761 |
| 2 | Transport C0 | CO2 emissions from passenger cars, kt | 79,770 | 84,743 | 81,537 | 81,934 |  |  |  |  |  |
|  |  | Number of kilometres by passenger cars, Mkm | 418,881 | 484,674 | 497,569 | 513,253 |  |  |  |  |  |
| 3 | Transport D0 | CO2 emissions from freight transport (all modes), kt | 48,571 | 51,374 | 53,764 | 55,718 |  |  |  |  |  |
|  |  | Freight transport (all modes), Mtkma | 178,627 | 183,519 | 194,630 | 201,250 |  |  |  |  |  |
| 4 | Industry A1 | Energy related CO2 intensity of industry, t/Euro million |  |  |  |  | CO2 emissions from fuel consumption industry, kt | 23,970 | 25,120 | 25,850 | 26,560 |
|  |  |  |  |  |  |  | Gross value-added total industry, Bio Euro (EC 95) | 284.5 | 310.3 | 338.4 | 369.1 |
| 5 | Households A1b | Specific CO2 emissions of households, t/dwelling |  |  |  |  | CO2 emissions from fossil fuel consumption households, kt |  |  |  |  |
|  |  |  |  |  |  |  | Stock of permanently occupied dwellings, 1000 |  |  |  |  |
| 6 | Services A0c | CO2 intensity of the services sector, t/Euro million |  |  |  |  | CO2 emissions from fossil fuel consumption services, kt |  |  |  |  |
|  |  |  |  |  |  |  | gross value-added services, bio Euro (EC95) |  |  |  |  |
| 7 | Transformation B0d | Specific CO2 emissions of public and autoproducer power plants, t/TJ |  |  |  |  | CO2 emissions from public and autoproducer thermal power stations, kt |  |  |  |  |
|  |  |  |  |  |  |  | all products-output by public and autoproducer thermal power stations, PJ |  |  |  |  |
| 8 | Agriculture | Specific N2O emissions of fertilizer and manure use, kg/kg |  |  |  |  | N2O emissions from synthetic fertilizer and manure use, kt | 88.7 | 82.7 | 81.9 | 81.2 |
|  |  |  |  |  |  |  | use of synthetic fertiliser and manure, kt nitrogene | 2,124 | 2,006 | 1,973 | 1,952 |
| 9 | Agriculture | Specific CH4 emissions of cattle production, kg/head |  |  |  |  | CH4 emissions from cattle, ktf | 710.2 | 618.9 | 613.9 | 612.9 |
|  |  |  |  |  |  |  | cattle populations, 1000 head | 10,849 | 8,963 | 8,849 | 8,818 |
| 10 | Waste | Specific CH4 emissions from landfills, kt/kt |  |  |  |  | CH4 emissions from landfills, kt | 333 | 271 | 227 | 194 |
|  |  |  |  |  |  |  | Municipal solid waste going to landfills, kt | 27,750 | 26,040 | 22,530 | 18,140 |

## Country Conclusions

The latest projections for the UK are presented in the March 2005 Monitoring Mechanism Submission.  UK emissions of the basket of six greenhouse gases are expected to be about 21% below base year levels in 2010, based on the most recent ‘with domestic measures’ projection, including land use.  Excluding land use change and forestry, UK emissions are expected to be slightly more than 20% below base year levels in 2010.  Compared to the EU burden sharing target of a 12.5% cut, the UK is comfortably on track to meet its Kyoto target, based on the ‘with domestic measures’ projection.  A ‘with additional measures scenario’ is not presented in the UK’s 2005 report because the UK meets its target ‘with measures’.

A comprehensive split of projections into 9 sectors and 6 greenhouse gases was provided.  Projections were presented clearly and the data totals were consistent with the breakdowns.  However, the quantification of effects of policies and measures could have been more comprehensive.  Savings from policies and measures were presented in groups, not individually, and so it was difficult to determine savings for key policies such as renewables and CHP initiatives.  Also, only savings relating to carbon dioxide were presented and policies addressing the other greenhouse gases were omitted from the summary table.