IRELAND

Overview	2

Sources of Information	3

Overview of CCPM implementation in MS	3

Quality and Transparency of Reporting	4

Assessment of Policies and Measures	5

Evaluation of Projections	13

Emissions projections by gas are not presented in the 2006 report.	13

Description of Modelling Approach	14

Overview of modelling approach	14

Sensitivity analysis	15

Details of the uncertainty assessment	15

Projection indicator reporting	15

Reporting of parameters on projections	15

Country Conclusions	22

Changes to projections	22

Indicators	28

| IRELAND Share in total EU-15 GHG emissions 2004 2 % Emissions base year (latest inventory) 55.8 Mt Emissions 2004 68.5 Mt Emissions base year (for projections) 55.8 Mt Projections 2010 with existing measures 72.3 Mt No projections with additional measures n.a. Kyoto target (absolute, based on latest inventory) 63.0 Mt Kyoto target (% from base year) + 13.0 % Change base year to 2004 + 22.7 % Change 2003–04 + 0.1 % Change base year to 2010 with existing measures + 29.6 % No projections with additional measures n.a. Distance to linear target path 2004 +6.5 (+13.6) percent points Use of Kyoto mechanisms 3.6 Mt Sinks (Articles 3.3. and 3.4) 2.1 Mt Emissions in 1990 (Article 3.7) n.a. |  | Note: ‘Target with Kyoto mechanisms’ is calculated by combining the agreed burden-sharing target with Kyoto units from JI and CDM and sinks according to Articles 3.3 and 3.4. | Past emissions: Ireland’s GHG emissions were 0.1 % above those of 2003 and 22.7 % above base-year levels in 2004. Increasing emissions from road transport in 2004 compared to the previous year were offset be decreases in fossil fuel combustion in public electricity and heat production and fugitive emissions from oil and natural gas. Between 1990 and 2004, fossil fuel combustion both in road transport and in electricity and heat production was by far the largest contributor to emission increases. Emission projections: Emissions in 2004 were below the level projected in the ‘with measures’ scenario for 2010. With existing domestic measures, Ireland exceeds the Kyoto target by 17 percentage points. In addition, the Irish government plans to purchase 3.6 Mt Kyoto units per year of the commitment period to comply with its Kyoto target. Additionally, Ireland intends to make use of carbon sinks according to Article 3.3 of 2.1 million tons per year. This will reduce the gap between projections and target to 7 percentage points. Again, no projections with additional measures have been provided. |
| --- | --- | --- | --- |

# Ireland

Please note that the projections relating to the NAP II are excluded from the EU Country Profiles as they were not available in time to be assessed for the 2006 Country Profiles.  Ireland is an exception to this as the projections in preparation for the NAP were sent to the EC in April 2006 with a request to use them as Ireland’s main greenhouse gas projection submission.

- Sources of Information

Projections:

Determining the Share of National Greenhouse Gas Emissions for Emissions Trading in Ireland 2008-2012, March 2006, ICF/BOC for the DEHLG.

Policies and measures:

A submission from Ireland on policies and measures was made under the Monitoring Mechanism in 2005.

- Overview of CCPM implementation in MS

| CCPM |  | Ireland |  |
| --- | --- | --- | --- |
| Taxation of energy products 2003/96/EC |  | B |  |
| Emissions trading 2003/87/EC |  | N |  |
| Promotion of electricity from RE sources 2001/77/EC |  | R |  |
| Promotion of cogeneration 2004/8/EC |  | N |  |
| Directives on energy labelling of appliances |  | N |  |
| Motor challenge, voluntary EC programme |  | N |  |
| Energy performance of buildings 2002/91/EC |  | N |  |
| Eco-management & audit scheme (EMAS) EC 761/2001 |  |  |  |
| Efficiency of hot water boilers 92/42/EEC |  | N |  |
| Transport modal shift to rail 2001/12/EC etc. |  | N |  |
| Promotion of biofuels for transport 2003/30/EC |  | N |  |
| Consumer information on cars 1999/94/EC |  | N |  |
| Agreement with car manufacturers ACEA etc. |  |  |  |
| Support under CAP (1782/2003) |  | N |  |
| Support under CAP - ammendment (1783/2003) |  | N |  |
| Landfill directive 1999/31/EC |  | N |  |
| N | New National PAM implemented after CCPM was adopted |  |  |
| R | Existing National PAM re-enforced by CCPM |  |  |
| B | National PAM already in force before CCPM was adopted |  |  |
|  | Not reported/not implemented |  |  |

Source: MS responses to the CCPMs questionnaire in 2005

- Quality and Transparency of Reporting

Table 1: Information Provided on Policies and Measures

| Information provided | Level provided | Comments |
| --- | --- | --- |
| Policy names | ++ | PAMs from 2005 MM – none provided with new projections |
| Objectives of policies | ++ |  |
| Which greenhouse gases? | CH4, CO2, N2O |  |
| Status of Implementation | +++ |  |
| Implementation body specified | O |  |
| Quantitative assessment of implementation | ++ |  |
| Interaction with other P&Ms discussed | + | CCPM linkage given |

o, +, ++, +++ level of information available increases as the number of + signs increases

The table above is based on the policies and measures submission to the Monitoring Mechanism in 2005.  The submission was not comprehensive and it was not transparent as to whether the savings from the policies and measures are incorporated in the projections.  The policies that were described were clearly presented in a tabular form, with the main information categories completed.

Ireland submitted a projections report in preparation for the NAP II ‘Determining the Share of National Greenhouse Gas Emissions for Emissions Trading in Ireland 2008-2012, March 2006, ICF/BOC for the DEHLG’.  This report did not provide an up-date of policies and measures in line with the revised projections.  The most complete overview of PAMs is still Ireland’s National Climate Change Strategy, and so the savings in Table 3 are based on the NCCS, with up-dates provided by Ireland in 2005.

Table 2: Information provided on Projections

| Category of Information |  | Level of Information Provided | Comments |
| --- | --- | --- | --- |
| Scenarios considered | + |  | No firm WAM projection – 3 modelled at different prices for effects of ETS |
| Expressed relative to base year | ++ |  | Clear projections base year |
| Starting year | 2004 |  |  |
| Split of projections | + |  | Not split by gas |
| Presentation of results | + |  | No PAMs presented with projections |
| Description of model (level of detail, approach and assumptions) | ++ |  |  |
| Sensitivity analysis (key inputs to model / high, central and low projections scenarios / robustness of model) | ++ |  | Sensitivity analysis for possible WAM projection based on carbon price |
| Discussion of uncertainty | ++ |  | Several issues raised |
| Details of parameters and assumptions | ++ |  | Detailed assumptions discussion – no time series given. |

o, +, ++, +++ level of information available increases as the number of + signs increases

There is a good level of clarity and split of projections for the sector breakdown in the 2006 submission.  However, the lack of a breakdown by gas is a significant omission and limits the analysis that can be completed on Ireland’s greenhouse gas projections.

- Assessment of Policies and Measures

Table 3: Summary of the Effect of Policies and Measures Included in the Projections (MtCO2)

|  | With measures | With additional measures |
| --- | --- | --- |
| Energy supply | 2.25 |  |
| Industry, commercial and services* | 2.175 |  |
| Built environment and residential | 0.9 |  |
| Transport | 2.12 |  |
| Waste | 0 |  |
| Agriculture | 2.41 |  |
| Total | 9.885 |  |

The policies and measures set out in the table above are taken from the National Climate Change Strategy (NCCS) and are now beginning to be out dated with respect to the latest projections, though the values above were up-dated in a communication from Ireland under the Monitoring Mechanism in 2005.   The table above has incorporated:  the decision not to convert the major coal fired plant to gas which reduced savings in the energy sector by 3.4 Mt. A review of the effectiveness of VRT and labelling measures in the transport sector indicate a reduction of savings of 0.55 Mt from that set out above.

Table 4: Detailed Information on Polices and Measures

Source: European Climate Change Programme (ECCP), Database on Policies and Measures in Europe

http://www.oeko.de/service/pam/index.php

| Member State | Sector | Name | Category | Type | GHG | Status | Absolute Reduction [kt (Gg) CO2 eq. p.a.] |  |  | Reduction in trading sector | Costs [EUR/t] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 2005 | 2010 | 2020 |  |  |
| Ireland | ENC | Total measures on energy efficiency in existing buildings |  |  |  |  |  | 400 |  |  |  |
| Ireland | ENS |  | energy plan |  |  |  |  | 2250 |  | high |  |
| Ireland | TRA | Public Transport & Traffic Management |  |  |  |  |  | 270 |  |  |  |
| Ireland | AGR | Extensification premia | livestock reduction | Economic | CH4 | implemented |  |  |  | no effect |  |
| Ireland | AGR | Special Beef Premium | livestock reduction | Economic | CH4 | implemented |  |  |  | no effect |  |
| Ireland | AGR | Suckler Cow Premium | livestock age | Economic | CH4 | implemented |  |  |  | no effect |  |
| Ireland | AGR | Rural Environmental Protection Scheme (REPS) Voluntary scheme designed to compensate and reward farmers for delivering environmental benefits | N management (organic and inorganic) | voluntary/ negotiated agreement | N2O | implemented |  |  |  | no effect |  |
| Ireland | AGR | Good Farming Practice | N management (organic and inorganic) | Regulatory | N2O | implemented |  |  |  | no effect |  |
| Ireland | CRS | Taxation | Taxation | Fiscal | CO2 | planned |  |  |  |  |  |
| Ireland | CRS | Emissions Trading | Emissions trading | Economic | CO2 | planned |  |  |  |  |  |
| Ireland | ENC | Building Regulation Standards | Buildings - energy efficiency |  | CO2 | implemented; planned |  | 250 |  | low |  |
| Ireland | ENC | Energy Efficiency | Energy efficiency - Companies | other | CO2 | planned |  |  |  | low |  |
| Ireland | ENC | New Building methods | Buildings - energy efficiency | education | CO2 | implemented |  |  |  | low |  |
| Ireland | ENC | Integrated Energy Planning for residential areas | Buildings - energy efficiency |  | CO2 |  |  | 250 |  |  |  |
| Ireland | ENC | Sustainable Energy Ireland - Home Energy Rating | Energy efficiency - buildings | planning | CO2 | planned |  |  |  | no effect |  |
| Ireland | ENC | Sustainable Energy Ireland - Programme for low income households | Energy efficiency - households | Economic | CO2 | planned |  |  |  |  |  |
| Ireland | ENC | Sustainable Energy Ireland - Public Sector Programme | Energy efficiency - public | Economic; Information | CO2 | implemented |  |  |  |  |  |
| Ireland | ENC | Energy Efficiency Rating of Buildings; House of Tomorrow | Energy efficiency - buildings | Information; Research | CO2 | planned |  |  |  | no effect |  |
| Ireland | ENC | Refurbishment of Commercial Buildings; Research & Demonstration for energy efficiency in Industry and Commercial | Energy efficiency - buildings; Energy efficieny - companies; Energy efficiency - commercial | voluntary/ negotiated agreement; Research | CO2 | planned |  |  |  | low |  |
| Ireland | ENC | Increased thermal energy requirements |  | Regulatory | CO2 | planned |  | 100 |  | low |  |
| Ireland | ENC | Energy Efficiency in industry |  | other | CO2 | implemented |  |  |  | moderate |  |
| Ireland | ENC | ?No regrets?/low cost energy efficiency gains |  |  | CO2 |  |  | 750 |  | moderate |  |
| Ireland | ENC | Up to £75 tonne CO2 efficiency measures |  |  | CO2 |  |  | 250 |  | moderate |  |
| Ireland | ENC | Commercial and Services |  |  | CO2 |  |  | 175 |  | low |  |
| Ireland | ENC | Sustainable Energy Ireland - Industry Agreements | Energy efficiency - industry | voluntary/ negotiated agreement | CO2 |  |  | 120 |  |  |  |
| Ireland | ENS | Combined Heat and Power (CHP) | CHP | Economic | CO2 | implemented |  | 250 |  | high |  |
| Ireland | ENS | National target for Renewables | Renewables | Economic | CO2 | implemented |  | 977 |  | complete |  |
| Ireland | ENS | Sustainable Energy Ireland - Body for energy efficiency | Renewables; Energy efficiency | Information; planning | CO2 | implemented |  | 100 |  |  |  |
| Ireland | ENS | Planning Guidelines for Wind Energy | Renewables | planning | CO2 | implemented |  |  |  | complete |  |
| Ireland | ENS | Gas supply and network expansion | Infrastructure | Economic | CO2 | implemented; adopted |  |  |  | moderate |  |
| Ireland | ENS | Peat Plant Modernisation for Electricity Generation | Energy efficiency | Regulatory | CO2 | implemented |  |  |  | complete |  |
| Ireland | ENS | Fugitive CH4 emission reduction | gas distribution and transmission network | other | CH4 | implemented |  | 73 |  | no effect |  |
| Ireland | ENS | Demand Side Management | Energy efficiency |  | CO2 | implemented |  | 150 |  | moderate |  |
| Ireland | IND | Process substitution for cement | Cement Production | Regulatory | CO2 | implemented | 150 | 500 |  | complete |  |
| Ireland | IND | Industrial Gases |  |  | N2O; HFCs; PFCs; SF6 |  |  | 500 |  | no effect |  |
| Ireland | LUC | Forest strategy, national development plan | afforestation | Economic | CO2 | implemented |  | 1.5 M tonnes CO2 over 2008-2012 = 0.3 Million tonnes CO2 per year |  | no effect |  |
| Ireland | TRA | Road network investment | improve road infrastructure | Economic | CO2 | implemented |  | 27 |  |  |  |
| Ireland | TRA | Economic and Social Infrastructure Operational Programme (ESIOP) | improve road infrastructure; change of modal split - public transport | voluntary/ negotiated agreement; Economic | CO2 | planned |  |  |  |  |  |
| Ireland | TRA | National Roads Priority | improve road infrastructure | Economic | CO2 | planned |  |  |  |  |  |
| Ireland | TRA | Fuel Economy labelling | vehicle efficiency | education; Regulatory | CO2 | implemented |  | 380 |  |  |  |
| Ireland | TRA | Public Transport | change of modal split - public transport | Fiscal; education; Regulatory | CO2 | implemented |  | 100; 270 | 100 |  |  |
| Ireland | TRA | Traffic Management; EU strategy for reduced CO2 emissions from new cars (ACEA agreement) | road-traffic management; vehicle efficiency | Regulatory; voluntary/ negotiated agreement | CO2 | planned; implemented |  | 200; 770 |  |  |  |
| Ireland | TRA | Fuel Measures (displace bunkering) | freight | Fiscal | CO2 | planned |  | 900 |  |  |  |
| Ireland | TRA | Road tax rebalancing | taxation | Fiscal | CO2 | implemented |  | 50; 100 |  |  |  |
| Ireland | TRA | Modal shift - freight | freight - logistics and combined transport | planning | CO2 | implemented |  | 50 |  |  |  |
| Ireland | TRA | Fuel Switching | alternative fuels and engine technologies | planning | CO2 | planned |  | 300 |  |  |  |
| Ireland | WAM | Waste Hirachy | waste management practices | Fiscal | CO2 | planned |  |  |  | no effect |  |

- Evaluation of Projections

Projections of greenhouse gas emissions are provided in the report ‘Determining the Share of National Greenhouse Gas Emissions for Emissions Trading in Ireland 2008-2012, March 2006, ICF/BOC for the DEHLG’, which is being used for Ireland’s Second National Allocation Plan for the EU ETS.

- Emissions projections by gas are not presented in the 2006 report.

Table 5: Summary of projections by gas in 2010 (MtCO2)

|  | Base year | with measures | with additional measures |
| --- | --- | --- | --- |
| Carbon dioxide (excl. sinks) | not available |  |  |
| Methane | not available |  |  |
| Nitrous oxide | not available |  |  |
| F-gases * | 0.2 | 0.7 |  |
| Total (excl. sinks) | 55.8 | 72.3 | 0.0 |
| % change relative to base year (excl. sinks) |  | 29.6% | -100.0% |
| *base year for F-gases is 1995 |  |  |  |

Table 6:  Summary of projections (6 gas basket) by sector in 2010 (MtCO2eq)

|  | Base year | with measures | % change relative to 1990 | with additional measures | % change relative to 1990 |
| --- | --- | --- | --- | --- | --- |
| Energy (total, excluding transport) | 29.0 | 39.1 | 35% |  | -100% |
| Energy supply | 11.8 | 18.7 | 59% |  | -100% |
| Energy - Industry, commercial, services | 9.8 | 13.5 | 38% |  | -100% |
| Energy – residential | 7.4 | 6.8 | -7% |  | -100% |
| Transport (energy) | 5.2 | 13.0 | 153% |  | -100% |
| Industrial processes | 0.2 | 0.7 | 233% |  | -100% |
| Waste | 1.5 | 1.8 | 25% |  | -100% |
| Agriculture | 20.0 | 17.6 | -12% |  | -100% |
| Total (excl. sinks) | 55.8 | 72.3 | 30% | 0.0 | -100% |

No information was given on a ‘by gas’ gas basis for the 2010 projections, thus it was not possible to provide a table showing the breakdown of sectors by gas.

Table 7: Assessment of the Target

|  | Excl. sinks |  |
| --- | --- | --- |
|  | MtCO2 equiv. | % of base year level (six gas basket) |
| Base year emissions (from projections) | 55.8 | 100% |
| Kyoto Commitment/burden sharing | 63.0 | 13.0% |
| With existing P&Ms projections | 72.3 | 129.6% |
| Gap (-ve means no gap) | 9.2 | 16.6% |
| With additional P&Ms projections | 72.3 | 129.6% |
| Remaining gap | 9.2 | 16.6% |
| Effect of flexible mechanisms | 3.6 | 6.5% |
| Remaining gap (if target includes flex mechs) | 5.6 | 10.1% |

Table above does not include LUCF.  Please refer to the LUCF section in the main report for further information.

- Description of Modelling Approach

- Overview of modelling approach

The projections report was generated as a preparation to the 2nd National Allocation Plan and so is heavily influenced by future permit prices inputted into the model.  The projections for the ETS sectors for growth levels we made by asking the companies themselves what their expected growth would be over the period to 2012.  The projections for the non-ETS sectors were based on existing models and some models had to rely on limited input data such as the transport models.

ICF-BOC used three analytical approaches to develop an assessment of the likely costs for Ireland to reduce GHG emissions.

They  used one of the energy industry’s most widely used power market models, ICF Consulting’s Integrated Planning Model (IPM), to assess the key implications for the Irish power sector operating under carbon constraints, with a particular focus on the level of GHG emission reductions achieved under different market-clearing prices for EU Allowances as well as the likely changes to wholesale power prices.

They have developed the marginal abatement cost curves (MACC) for the sectors within the EU Emissions Trading Scheme (EU ETS sector) and the sectors outside the EU ETS (non-ETS sector) using the best available international data on abatement costs for all known technological options with potential applicability in the Irish context.

They  have used Irish data to assess the elasticity of electricity and fuel demand.

Their aim was to help inform the amount of cost-effective abatement possible while trying to preserve economic efficiency.

ICF/BOC highlighted in 2004 and 2006 that Ireland needed a national model of transport emissions for the whole country.  The transport projections are based on patchy data and estimations are often needed to be made as sound data inputs are not available.  Ireland is working to improve its transport modelling in the longer term.

- Sensitivity analysis

A sensitivity analysis was conducted on a potential ‘With additional measures’ scenario, based on three different future carbon prices.  The “With Additional Measures” scenario represents the ICF-BOC view of what GHG emissions will likely be in 2008-2012, taking into account the EU ETS. The distance to target represents the difference between the projected emission under the “With Additional Measures” scenario and the GHG emissions target.

- Details of the uncertainty assessment

Key areas of uncertainty are the input parameters and the model for the transport projections.  Other uncertainty comes from future carbon price and fuel prices. To account for uncertainty in the price of EUAs so far in the future, for each year ICF/BOC  developed three scenarios, with the average price of EUAs of €15/t CO2e, €20/t CO2e and €30/t CO2e.

- Projection indicator reporting

All projection indicators were reported for 2010.  This was appreciated as MS were not obliged to report projection indicators this year.  Member States will be obliged to provide a full time series for the projection indicators in 2006 (covering 2005 to 2020).

- Reporting of parameters on projections

Only energy prices (for 2000) and GDP (for 2010) is provided for the projection parameters. The majority of the ‘mandatory parameters on projections’ are not reported, but this is likely to be because 2005 is not a mandatory reporting year.  The parameters will need to be provided in 2006.

Table 8) Indicators for projections to monitor and evaluate progress with policies and measures (2005/166/EC) Annex III

| No | Eurostat Sectors | Indicator | 2005 | 2010 | 2015 | 2020 | Numerator/denominator | 2005 | 2010 | 2015 | 2020 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Macro | CO2 intensity of GDP, t/Euro million |  | 509.3 |  |  | Total CO2 emissions, kt |  | 71268 |  |  |  |
|  |  |  |  |  |  |  | GDP, bio Euro (EC95) |  | 139.9 |  |  |  |
| 2 | Transport C0 | CO2 emissions from passenger cars, kt |  | 6251 |  |  |  |  |  |  |  |  |
|  |  | Number of kilometres by passenger cars, Mkm |  | 57355 |  |  |  |  |  |  |  |  |
| 3 | Transport D0 | CO2 emissions from freight transport (all modes), kt |  | NA |  |  |  |  |  |  |  |  |
|  |  | Freight transport (all modes), Mtkm |  | NA |  |  |  |  |  |  |  |  |
| 4 | Industry A1 | Energy related CO2 intensity of industry, t/Euro million |  | 110.1 |  |  | CO2 emissions from fuel consumption industry, kt |  | 6473 |  |  |  |
|  |  |  |  |  |  |  | Gross value-added total industry, Bio Euro (EC 95) |  | 58.77 |  |  |  |
| 5 | Households A1 | Specific CO2 emissions of households, t/dwelling |  | 4.976 |  |  | CO2 emissions from fossil fuel consumption households, kt |  | 7161 |  |  |  |
|  |  |  |  |  |  |  | Stock of permanently occupied dwellings, 1000 |  | 1439 |  |  |  |
| 6 | Services A0 | CO2 intensity of the services sector, t/Euro million |  | 101.6 |  |  | CO2 emissions from fossil fuel consumption services, kt |  | 4265 |  |  |  |
|  |  |  |  |  |  |  | gross value-added services, bio Euro (EC95) |  | 41.98 |  |  |  |
| 7 | Transformation B0 | Specific CO2 emissions of public and autoproducer power plants, t/TJ |  | 158.4 |  |  | CO2 emissions from public and autoproducer thermal power stations, kt |  | 18502 |  |  |  |
|  |  |  |  |  |  |  | all products-output by public and autoproducer thermal power stations, PJ |  | 116.8 |  |  |  |
| 8 | Agriculture | Specific N2O emissions of fertilizer and manure use, kg/kg |  | 0.03 |  |  | N2O emissions from synthetic fertilizer and manure use, kt |  | 21.42 |  |  |  |
|  |  |  |  |  |  |  | use of synthetic fertiliser and manure, kt nitrogen |  | 717.8 |  |  |  |
| 9 | Agriculture | Specific CH4 emissions of cattle production, kg/head |  | 0.089 |  |  | CH4 emissions from cattle, kt |  | 481.3 |  |  |  |
|  |  |  |  |  |  |  | cattle populations, 1000 head |  | 5398 |  |  |  |
| 10 | Waste | Specific CH4 emissions from landfills, kt/kt |  | 0.014 |  |  | CH4 emissions from landfills, kt |  | 30.09 |  |  |  |
|  |  |  |  |  |  |  | Municipal solid waste to landfill, kt |  | 2100 |  |  |  |

Table 9) List of Parameters on Projections (Annex IV Implementing Provisions)

| 1. Mandatory Parameters on Projections | 2000 | 2010 | 2015 | 2020 |
| --- | --- | --- | --- | --- |
| Assumptions for general economic parameters |  |  |  |  |
| — GDP, bio Euro (EC95) |  | 139.9 |  |  |
| — Population (value at given years or annual growth rate and base year) |  |  |  |  |
| — International coal prices at given years in euro per tonne or GJ (Gigajoule) |  |  |  |  |
| — International oil prices at given years in euro per barrel or GJ |  |  |  |  |
| — International gas prices at given years ($/107 Kcal2) EU15 | 157.9 |  |  |  |
| US | 112.3 |  |  |  |
| IRL | 157.3 |  |  |  |
| Assumptions for the energy sector |  |  |  |  |
| — Total gross inland consumption (PJ) (split by oil,gas,coal,renewables,nuclear,other) |  |  |  |  |
| — Total electricity production by fuel type (oil, gas, coal, renewables, nuclear, other) |  |  |  |  |
| — Energy demand by sector split by fuel (delivered) |  |  |  |  |
| — Assumptions on weather parameters, especially heating or cooling degree days |  |  |  |  |
| Assumptions for the industry sector |  |  |  |  |
| For Member States using macroeconomic models: |  |  |  |  |
| — The share of the industrial sector in GDP and growth rate |  |  |  |  |
| For Member States using other models: |  |  |  |  |
| — The production index for industrial sector |  |  |  |  |
| Assumptions for the transport sector |  |  |  |  |
| For Member States using macroeconomic models: |  |  |  |  |
| — The growth of transport relative to GDP |  |  |  |  |
| For Member States using other models: |  |  |  |  |
| — The growth of passenger person kilometres |  |  |  |  |
| — The growth of freight tonne kilometres |  |  |  |  |
| Assumptions for buildings (in residential and commercial or tertiary sector) |  |  |  |  |
| For Member States using macroeconomic models: |  |  |  |  |
| — The level of private consumption (excluding private transport) |  |  |  |  |
| — The share of the tertiary sector in GDP and the growth rate |  |  |  |  |
| For Member States using other models: |  |  |  |  |
| — The rate of change of floor space for tertiary buildings and dwellings |  |  |  |  |
| — The number of dwellings and number of employees in the tertiary sector |  |  |  |  |
| Assumptions in the agriculture sector |  |  |  |  |
| For Member States using macroeconomic models: |  |  |  |  |
| — The share of the agriculture sector in GDP and relative growth |  |  |  |  |
| For Member States using other models: |  |  |  |  |
| — Livestock numbers by animal type (for enteric fermn cows, sheep, for manure, pigs, poultry) |  |  |  |  |
| — The area of crops by crop type |  |  |  |  |
| — Emissions factors by type of livestock for enteric fermentn and manure management (t) |  |  |  |  |
| Assumptions in the waste sector |  |  |  |  |
| — Waste generation per head of population or tonnes of municipal solid waste |  |  |  |  |
| — The organic fractions of municipal solid waste |  |  |  |  |
| — Municipal solid waste disposed to landfills, incinerated or composted (in tonnes or %) |  |  |  |  |
| Assumptions in the forestry sector |  |  |  |  |
| — Forest definitions |  |  |  |  |
| Areas of: |  |  |  |  |
| — managed forests |  |  |  |  |
| — unmanaged forests |  |  |  |  |

| 2. Recommended parameters on projections | 2005 | 2010 | 2015 | 2020 |
| --- | --- | --- | --- | --- |
| Assumptions for general economic parameters |  |  |  |  |
| — GDP growth rates split by industrial sectors in relation to 2000 |  |  |  |  |
| — Comparison projected data with official forecasts |  |  |  |  |
| Assumptions for the energy sector |  |  |  |  |
| — National coal, oil and gas energy prices per sector (including taxes) |  |  |  |  |
| — National electricity prices per sector as above (may be model output) |  |  |  |  |
| — Total production of district heating by fuel type |  |  |  |  |
| Assumptions for the industry sector |  |  |  |  |
| — Assumptions fluorinated gases: |  |  |  |  |
| — Aluminium production and emissions factors |  |  |  |  |
| — Magnesium production and emissions factors |  |  |  |  |
| — Foam production and emissions factors |  |  |  |  |
| — Stock of refrigerant and leakage rates |  |  |  |  |
| For Member States using macroeconomic models: |  |  |  |  |
| — Share of GDP for different sectors and growth rates |  |  |  |  |
| — Rate of improvement of energy intensity (1990 = 100) |  |  |  |  |
| For Member States using other models: |  |  |  |  |
| — Index of production for different sectors |  |  |  |  |
| — Rate of improvement or index of energy efficiency |  |  |  |  |
| Assumptions for buildings (in residential and commercial / tertiary sector) |  |  |  |  |
| For Member States using macroeconomic models: |  |  |  |  |
| — Share of tertiary and household sectors in GDP |  |  |  |  |
| — Rate of improvement of energy intensity |  |  |  |  |
| For Member States using other models: |  |  |  |  |
| — Number of households |  |  |  |  |
| — Number of new buildings |  |  |  |  |
| — Rate of improvement of energy efficiency (1990 = 100) |  |  |  |  |
| Assumptions for the transport sector |  |  |  |  |
| For Member States using econometric models: |  |  |  |  |
| — Growth of transport relative to GDP — split by passenger and freight |  |  |  |  |
| — Improvements in energy efficiency — split by vehicle type |  |  |  |  |
| — Improvements in energy efficiency — split by vehicle type, whole fleet/new cars |  |  |  |  |
| — Rate of change of modal split (passenger and freight) |  |  |  |  |
| — The growth of passenger road kilometres |  |  |  |  |
| — The growth of passenger rail kilometres |  |  |  |  |
| — The growth of passenger aviation kilometres |  |  |  |  |
| — The growth of freight tonne kilometres on the road |  |  |  |  |
| — The growth of freight tonne kilometres by rail |  |  |  |  |
| — The growth of freight tonne kilometres by navigation |  |  |  |  |
| Assumptions for the agriculture sector |  |  |  |  |
| For Member States using econometric models: |  |  |  |  |
| — Agricultural trade (import/export) |  |  |  |  |
| — Domestic consumption (e.g. milk/beef consumption) |  |  |  |  |
| For Member States using other models: |  |  |  |  |
| — Development of area of crops, grassland, arable, set-aside, conversion to forests etc |  |  |  |  |
| — Macroeconomic assumptions behind projections of agricultural activity |  |  |  |  |
| — Description of livestock (e.g. by nutrient balance, output/animal production, milk prodn) |  |  |  |  |
| — Development of farming types (e.g. intensive conventional, organic farming) |  |  |  |  |
| — Distribution of housing/grazing systems and housing/grazing period |  |  |  |  |
| — Parameters of fertiliser regime: |  |  |  |  |
| — Details of fertiliser use (type of fertiliser, timing of application, inorganic/organic ratio); |  |  |  |  |
| — Volatilisation rate of ammonia, following spreading of manure on the soil; |  |  |  |  |
| — Efficiency of manure use. |  |  |  |  |
| — Parameters of manure management system: |  |  |  |  |
| — Distribution of storage facilities (e.g. with or without cover): |  |  |  |  |
| — Nitrogen excretion rate of manures |  |  |  |  |
| — Methods of application of manure |  |  |  |  |
| — Extent of introduction of control measures (storage systems, manure appn), BAT |  |  |  |  |
| — Parameters related to nitrous oxide emissions from agricultural soils |  |  |  |  |
| — Amount of manure treatment |  |  |  |  |

- Country Conclusions

Ireland’s limitation target of 13% corresponds to a quantitative limitation target of 63 Mt CO2 equivalent per annum (excluding sinks) for the period 2008 –2012.  This is 3 Mt CO2 eq higher than last year’s target estimate and the changes can be explained by the changes in the base year detailed in the projections report preparing for 2nd National Allocation Plan submitted in 2006.

Ireland’s ‘with measures’ projection estimates that 2010 emissions will be 72.3 Mt CO2e per annum (excluding sinks).  This represents a nearly a 17% exceedence of its burden sharing commitment (but is closer than the 2005 projections which had over a 20% exceedance).  This year’s with measures projection shows a 30% increase on 1990 emissions.

The use of Kyoto mechanisms, through international emissions trading, will bring the gap to the target down to 5.6 Mt CO2 eq., but Ireland will need to include ‘with additional measures’ in it’s projections as this will further reduce the exceedence towards the 13% commitment.

These projections are based on a document assessing greenhouse gas emissions for emissions trading. These correspond to a with measures projection, the implemented policies are described in a 2005 submission under the Monitoring Mechanism. Ireland should aim to provide a full up-date on policies and measures savings which are consistent with projections for the 2007 Monitoring Mechanism submission, as this has not been made available to the Commission in recent years.  It is also vital to provide a split of projections by gas in next year’s submission.

- Changes to projections

Over the past ten years Ireland has seen a steep increase in transport emissions.  The ‘with measures projection’ for transport submitted in 2006 shows a continued increase in emissions to 2010, but at a slower rate than recent trends indicate.  Ireland’s transport data has significant gaps which they are working to resolve and hope to improve the data and projections in the next two to four years.

In 2003, Ireland’s waste emissions were already well above the projected value for the 2010 ‘with measures scenario’.  Thus the actual increase in waste emissions is likely to be significantly larger than the 25% predicted in the 2006 projections report for Ireland.  Ireland has sought a derogation to delay their targets under the Landfill directive for diverting waste from landfill as they would not be able to meet them in the short to medium term

.

IRELAND - Priority indicators (see Chapters A5.2 and A5.4 for more information on indicators)

IRELAND – Reported indicators

Priority and additional priority indicators

Supplementary indicators

Data on flexible mechanisms from Member State response to EEA ETC ACC questionnaire under the EC mechanism for monitoring Community greenhouse gas emissions and for implementing the Kyoto Protocol (Council Decision 280/2004/EC).