SPAIN

Overview	2

Overview of CCPM implementation in MS	4

Sources of Information	5

Quality and Transparency of Reporting	5

Assessment of Policies and Measures	7

Evaluation of Projections	17

Description of Modelling Approach	21

Projection indicator reporting	22

Reporting of parameters on projections	22

Country Conclusions	22

Indicators	23

| SPAIN Share in total EU-15 GHG emissions 2004 10 % Emissions base year (latest inventory) 289.4 Mt Emissions 2004 427.9 Mt Emissions base year (for projections) (CO2) 288.4 Mt Projections 2010 with existing measures (CO2) 436.3 Mt No projections with additional measures n.a. Kyoto target (absolute, based on latest inventory) 332.8 Mt Kyoto target (% from base year) + 15.0 % Change base year to 2004 + 47.9 % Change 2003–04 + 4.8 % Change base year to 2010 with existing measures + 51.3 % No projections with additional measures n.a. Distance to linear target path 2004 +31.2 (+37.4) percent points Use of Kyoto mechanisms 20.0 Mt Sinks (Articles 3.3 and 3.4) 5.5 Mt Emissions in 1990 (Article 3.7) n.a. |  | Note: ‘Target with Kyoto mechanisms’ is calculated by combining the agreed burden-sharing target with Kyoto units from JI and CDM and sinks according to Articles 3.3 and 3.4. | Past emissions: Spain’s GHG emissions were 4.8 % above those of 2002 and 47.9 % above base-year levels in 2004. Increases between 2003 and 2004 were mainly due to increasing emissions from manufacturing industries, road transportation, fossil fuel combustion from public power and heat production and iron and steel production. Between 1990 and 2004, fuel consumption in electricity and heat production and road transport are the largest contributors to emission increases. Also fuel combustion in industry and households and services and emissions from landfills and from agriculture increased substantially. Emission projections: Emissions in 2004 were below the level projected in the with measures scenario for 2010. Spain will not achieve the Kyoto target with domestic measures. Spain is currently planning to use Kyoto mechanisms to purchase an average of 20.0 million tonnes of CO2 equivalent reductions per year during the commitment period and to make use of carbon sinks according to Articles 3.3 and 3.4 of 5.5 million tonnes. |
| --- | --- | --- | --- |

# Spain

- Please note that the projections relating to the NAP II are excluded from all the EU Country Profiles as they were not available in time to be assessed for the 2006 Country Profiles.

- Overview of CCPM implementation in MS

- Source: MS responses to the CCPMs questionnaire in 2005 and 4th National Communication

| CCPM | ES |
| --- | --- |
| Taxation of energy products 2003/96/EC |  |
| Emissions trading 2003/87/EC | N |
| Promotion of electricity from RE sources 2001/77/EC | R |
| Promotion of cogeneration 2004/8/EC |  |
| Directives on energy labelling of appliances | N |
| Motor challenge, voluntary EC programme |  |
| Energy performance of buildings 2002/91/EC | R |
| Eco-management & audit scheme (EMAS) EC 761/2001 | N |
| Efficiency of hot water boilers 92/42/EEC | N |
| Transport modal shift to rail 2001/12/EC etc. | N |
| Promotion of biofuels for transport 2003/30/EC | N |
| Consumer information on cars 1999/94/EC | N |
| Agreement with car manufacturers ACEA etc. | 1 |
| Support under CAP (1782/2003) | N |
| Support under CAP - ammendment (1783/2003) |  |
| Landfill directive 1999/31/EC | R |
| CODE |  |
| New National PAM implemented after CCPM was adopted | N |
| Existing National PAM re-enforced by CCPM | R |
| National PAM already in force before CCPM was adopted | B |
| Not reported |  |

There is no reporting on this voluntary agreement since it is not a national legislative measure.

- Sources of Information

- Spain’s 4th National Communication under the United Nations Framework Convention and its Kyoto Protocol and questionnaires on the use of flexible mechanisms and Article 3.3/3.4 of the Kyoto Protocol (submission under Decision 280/2004/EC).

- Quality and Transparency of Reporting

- The quality of reporting and information available has increased considerably through the submission of Spain’s 4th National Communication. Projections are available for all sectors and gases for the first time and more PAM have been quantified.

- Table 1: Information Provided on Policies and Measures

| - Information provided | - Level provided | - Comments |
| --- | --- | --- |
| - Policy names | - +++ |  |
| - Objectives of policies | - +++ |  |
| - Which greenhouse gases? | - All |  |
| Status of Implementation | - +++ |  |
| - Implementation body specified | - +++ |  |
| - Quantitative assessment of implementation | - ++ | - Not all PAM quantified, many estimates only on aggregate level for period of several years, different periods chosen for different PAMs |
| - Interaction with other P&Ms discussed | - ++ | - Relationship to CCPM included in several cases. |

- o, +, ++, +++ level of information available increases as the number of + signs increases

- Table 2: Information provided on Projections

| - Category of Information | - Level of Information Provided | - Comments |
| --- | --- | --- |
| - Scenarios considered | - ++ | - No ‘with add. measures’ scenario provided |
| - Expressed relative to base year | - +++ |  |
| - Starting year | - 2000 |  |
| - Split of projections | - +++ |  |
| - Presentation of results | - +++ | - Tables and graphs by gas and sector |
| - Description of model (level of detail, approach and assumptions) | - ++ | - Detailed description of PAM included in scenario, only limited information on assumptions and parameters provided |
| Sensitivity analysis (key inputs to model / high, central and low projections scenarios / robustness of model) | - - | - Not reported |
| - Discussion of uncertainty | - - | - Not discussed |
| - Details of parameters and assumptions | - - | - Not reported |

- o, +, ++, +++ level of information available increases as the number of + signs increases

- Assessment of Policies and Measures

- Table 3: Summary of the Effect of Policies and Measures Included in the 2010 Projections (MtCO2)2

|  | With measures |
| --- | --- |
| Energy (total, excluding transport) | 27.5 |
| Public Electricity Generation | 10.7 |
| Commercial, Institutional and Residential Combustion Installations | 2.7 |
| Industrial Combustion Installations | 13.7 |
| Extraction and Distribution of fossil Fuels | 0.3 |
| Transport (energy) | 3.8 |
| Road transport | 2.2 |
| Other transport modes | 1.6 |
| Industrial processes | 4.2 |
| Solvents and other Product use | 7.3 |
| Waste | 7.6 |
| Agriculture | 7.1 |
| Other Sources & Sinks | 1.4 |
| Total (excluding sinks) | 58.8 |

Source: Fourth National Communiation

- 2 The projections included in the Fourth National Communication of Spain do not take into account the most recently approved plan, the Renewable Energy Plan 2005-2010.

- Table 4: Detailed Information on Polices and Measures

- Source: European Climate Change Programme (ECCP), Database on Policies and Measures in Europe

- http://www.oeko.de/service/pam/index.php

| Member State | Sector | Name | Category | Type | GHG | Status | Absolute Reduction [kt (Gg) CO2 eq. p.a.] |  |  | Reduction in trading sector | Costs [EUR/t] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 2005 | 2010 | 2020 |  |  |
| Spain | AGR |  |  |  |  |  | 1040 | 1690 |  |  |  |
| Spain |  |  |  |  |  |  |  | 790 |  |  |  |
| Spain | AGR | Agricultural geographic information system | Monitoring | Information; Research | CH4; N2O | implemented |  |  |  | no effect |  |
| Spain | AGR | Interaction between agriculture and environment | improved feeding | Research | CH4; N2O | implemented |  |  |  | no effect |  |
| Spain | AGR | Agro-environmental requirements by the Common Agricultural Policy | Burning of agricultural residues | Regulatory; Economic | CH4; N2O | implemented | 60 |  |  | no effect |  |
| Spain | AGR | Feeding of intensive livestock | improved feeding | Research | CH4 | implemented |  |  |  | no effect |  |
| Spain | AGR | Production of energy biomass | biomass | Economic | CO2 | implemented |  |  |  | high |  |
| Spain | AGR | Agricultural use of compost from sludges of sewage treatment plants and from municipal waste | fertilizers | Regulatory; Economic; Information | CO2; CH4; N2O | implemented | 1300 |  |  | no effect |  |
| Spain | AGR | Action programmes in areas vulnerable to nitrate contamination | fertilizers | Regulatory; Information | CO2; N2O | implemented |  |  |  | no effect |  |
| Spain | AGR | Codes of good agricultural practices | fertilizers | education; voluntary/ negotiated agreement | CO2; N2O | implemented |  |  |  | no effect |  |
| Spain | AGR | Agro-environmental requirements by the Common Agricultural Policy | abandonment of land management | Regulatory; Economic | CO2; N2O | implemented |  |  |  | no effect |  |
| Spain | AGR | Increase of woody biomass | biomass | Regulatory; Economic | CO2 | implemented |  |  |  | no effect |  |
| Spain | AGR | Strategy for energy saving and efficiency in Spain 2004 - 2012 (E4): Plan of Action 2005 - 2007: agricultural sector | energy efficiency | Regulatory; Economic; Information; education | CO2 | implemented | 20 |  |  | moderate |  |
| Spain | CRS | Law 62/2003, about fiscal, administrative and social measures | Vehicles | Economic | CO2 | implemented |  |  |  | no effect |  |
| Spain | CRS | Law 53/2002, about fiscal, administrative and social measures | biofuels | Economic | CO2 | implemented |  |  |  | no effect |  |
| Spain | CRS | Law 51/2002, about reforms of law 39/1988, regulating local financial authorities | taxation | Fiscal | CO2 | implemented |  |  |  | moderate |  |
| Spain | CRS | Network of cities for the climate | Network | Information; education | CO2 | implemented |  |  |  | moderate |  |
| Spain | CRS | National Allocation Plan 2005-2007 | EU ETS | Economic | CO2 | implemented | 1860 |  |  | complete |  |
| Spain | CRS | Carbon Fund | FlexMex | Economic; voluntary/ negotiated agreement | CO2 | implemented |  | 9800 |  |  |  |
| Spain | ENC | Technical building code | Buildings - energy efficiency | Regulatory | CO2 | implemented |  |  |  | no effect |  |
| Spain | ENC | Regulations for thermal installations in buildings | Buildings - heat supply | Regulatory | CO2 | implemented |  |  |  | no effect |  |
| Spain | ENC | Energy certification of buildings | Buildings - energy efficiency | Regulatory | CO2 | planned |  |  |  | no effect |  |
| Spain | ENC | Strategy for energy saving and efficiency in Spain 2004 - 2012 (E4): Plan of Action 2005 - 2007: building sector | Buildings - energy efficiency | Regulatory; Economic | CO2 | implemented | 92 |  |  | moderate |  |
| Spain | ENC | Strategy for energy saving and efficiency in Spain 2004 - 2012 (E4): Plan of Action 2005 - 2007: public services sector | Energy efficiency | Economic | CO2 | implemented | 61 |  |  | high |  |
| Spain | ENC | Strategy for energy saving and efficiency in Spain 2004 - 2012 (E4): Plan of Action 2005 - 2007: residential and office equipment sector | Energy efficiency | Economic; Information; education | CO2 | implemented | 90 |  |  | high |  |
| Spain | ENS | Renewable Energy Plan: extension of the use of thermoelectric solar energy | Renewables | Economic | CO2 | implemented |  | 480 |  | complete |  |
| Spain | ENS | Renewable Energy Plan: extension of the use of wind power | Renewables | Economic | CO2 | implemented |  | 9600 |  | complete |  |
| Spain | ENS | Renewable Energy Plan: extension of the use of biomass | Renewables | Economic | CO2 | implemented |  | 7300 |  | moderate |  |
| Spain | ENS | Renewable Energy Plan: extension of the use of small hydro power | Renewables | Economic | CO2 | implemented |  | 470 |  | complete |  |
| Spain | ENS | Renewable Energy Plan: extension of the use of biogas | Renewables | Economic | CO2 | implemented |  | 1800 |  | high |  |
| Spain | ENS | Renewable Energy Plan: extension of the use of biofuels | Renewables | Fiscal | CO2 | implemented |  | 5900 |  | no effect |  |
| Spain | ENS | Renewable Energy Plan: extension of hydro power | Renewables | Economic | CO2 | implemented |  | 260 |  | complete |  |
| Spain | ENS | Renewable Energy Plan: extension of the use of photovoltaics | Renewables | Economic | CO2 | implemented |  | 210 |  | complete |  |
| Spain | ENS | Renewable Energy Plan: extension of the use of solar thermal energy | Renewables | Economic | CO2 | implemented |  | 1000 |  | no effect |  |
| Spain | ENS | Planning of the electricity and gas sectors. Development of the transport grid 2002-2011 | Infrastructure | planning; Regulatory | CO2 | implemented |  |  |  | high |  |
| Spain | ENS | Strategy for energy saving and efficiency in Spain 2004 - 2012 (E4): Plan of Action 2005 - 2007: energy sector | Energy efficiency | Economic; voluntary/ negotiated agreement | CO2 | implemented | 937 |  |  | high |  |
| Spain | IND | Strategy for energy saving and efficiency in Spain 2004 - 2012 (E4): Plan of Action 2005 - 2007: industry sector | Energy efficiency | voluntary/ negotiated agreement; Economic | CO2 | implemented | 89 |  |  | high |  |
| Spain | IND | Law 16/2002 about integrated prevention and control of pollution | IPPC | Regulatory | CO2 | implemented |  |  |  | moderate |  |
| Spain | IND | Programme for the Promotion of Technical Research |  | Research; Economic | CO2 | implemented |  |  |  |  |  |
| Spain | IND | Programme "Ingenio 2010" |  | Research | CO2 | implemented |  |  |  | low |  |
| Spain | LUC | Spanish Forestry Plan 2003-2032 | forest management | Regulatory | CO2 | implemented |  |  |  | no effect |  |
| Spain | LUC | Hydrological forest restoration | forest management | Regulatory | CO2 | implemented |  |  |  | no effect |  |
| Spain | LUC | Forest management and forest treatment | forest management | Regulatory | CO2 | implemented |  |  |  | no effect |  |
| Spain | LUC | Fight against forest fires | fire protection | Regulatory | CO2 | implemented |  |  |  | no effect |  |
| Spain | LUC | Forest health | Forest health | Regulatory; Information | CO2 | implemented |  |  |  | no effect |  |
| Spain | LUC | National Forest Inventory | forest inventory | Information | CO2 | implemented |  |  |  | no effect |  |
| Spain | LUC | National Soil Erosion Inventory | Soil erosion | Information | CO2 | implemented |  |  |  | no effect |  |
| Spain | LUC | National Forestry Map | Forest cartography | Information | CO2 | implemented |  |  |  | no effect |  |
| Spain | LUC | Biomass expansion factors | Studies | Research | CO2 | implemented |  |  |  | no effect |  |
| Spain | LUC | Carbon fluxes | Modelling | Research | CO2 | implemented |  |  |  | no effect |  |
| Spain | TRA | Strategic Plan of Infrastructure and Transport |  | Regulatory | CO2 | implemented |  |  | 30000 | no effect |  |
| Spain | TRA | Law 48/2003 on the economical operation mode and on harbour services of public interest | taxation | Fiscal | CO2 | implemented |  |  |  | no effect |  |
| Spain | TRA | Strategy for energy saving and efficiency in Spain 2004 - 2012 (E4): Plan of Action 2005 - 2007: transport sector | Energy efficiency | Economic; education | CO2 | implemented | 1407 |  |  | no effect |  |
| Spain | WAM | National Plan on Urban Waste | waste prevention or recycling | Regulatory; Economic; Information | CO2; CH4; N2O | implemented |  |  |  | no effect |  |
| Spain | WAM | National Prevention Programme | waste prevention or recycling | Regulatory; Economic; Information | CO2; CH4; N2O | implemented |  |  |  | no effect |  |
| Spain | WAM | National programmes on recovery and recycling of packing waste and used packages | waste prevention or recycling | Regulatory; Economic; Information | CO2; CH4; N2O | implemented |  |  |  | no effect |  |
| Spain | WAM | National Compost Programme | waste composting | Regulatory; Economic; Information | CO2; CH4; N2O | implemented |  |  |  | no effect |  |
| Spain | WAM | National energetic utilisation programme | waste incineration | Regulatory; Economic; Information | CO2; CH4; N2O | implemented |  |  |  | high |  |
| Spain | WAM | National disposal programme | waste disposal | Regulatory; Economic; Information | CO2; CH4; N2O | implemented |  |  |  | no effect |  |
| Spain | WAM | National Plan on Special Wastes | Special wastes | Regulatory; Economic; Information | HFCs | implemented |  |  |  | no effect |  |

- Evaluation of Projections

- Table 5: Summary of projections by gas in 2010 (MtCO2)

|  | Base year | no measures | with measures |
| --- | --- | --- | --- |
| Carbon dioxide (excl. sinks) | 224.5 | 397.2 | 370.3 |
| Methane | 32.0 | 49.1 | 35.4 |
| Nitrous oxide | 26.5 | 33.7 | 25.7 |
| HFCs* | 4.6 | 14.3 | 4.5 |
| PFCs | 0.8 | 0.7 | 0.3 |
| SF6 | 0.1 | 0.2 | 0.2 |
| Total (excl. sinks) | 288.4 | 495.1 | 436.3 |
| % change relative to base year (excl. sinks) |  | 71.7% | 51.3% |

*base year for F-gases is 1995

Table 6: Summary of projections (6 gas basket) by sector in 2010 (MtCO2eq)

|  | 1990 | with measures | % change relative to 1990 |
| --- | --- | --- | --- |
| Energy (total, excluding transport) | 142.2 | 205.3 | 44% |
| Public Electricity Generation | 78.2 | 109.2 | 40% |
| Commercial, Institutional and Residential Combustion Installations | 18.8 | 30.4 | 62% |
| Industrial Combustion Installations | 42.9 | 63.1 | 47% |
| Extraction and Distribution of fossil Fuels | 2.3 | 2.6 | 12% |
| Transport (energy) | 70.2 | 145.7 | 108% |
| Road transport | 52.3 | 118.8 | 127% |
| Other transport modes | 17.9 | 27.0 | 50% |
| Industrial processes | 24.3 | 31.4 | 29% |
| Solvents and other Product use | 0.4 | 3.4 | 702% |
| Waste | 9.8 | 9.6 | -2% |
| Agriculture | 34.0 | 36.5 | 7% |
| Other Sources & Sinks | 5.2 | 4.5 | -14% |
| Total (excl. sinks) | 286.2 | 436.3 | 52% |

|  | Carbon Dioxide |  |  | Methane |  |  | Nitrous Oxide |  |  | F-gases |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1990 | With measures | With additional measures | 1990 | With measures | With additional measures | 1990 | With measures | With additional measures | 1995 | With measures | With additional measures |
| Energy (exc transport) | 136.1 | 198.9 | -- | 3.3 | 3.2 | -- | 2.8 | 3.1 | -- | 0.0 | 0.0 | -- |
| Public Electricity Generation | 77.2 | 108.5 |  | 0.0 | 0.1 |  | 0.9 | 0.6 |  | 0.0 | 0.0 |  |
| Commercial, Institutional and Residential Combustion Installations | 17.3 | 29.1 |  | 0.8 | 0.5 |  | 0.7 | 0.9 |  | 0.0 | 0.0 |  |
| Industrial Combustion Installations | 41.6 | 61.3 |  | 0.1 | 0.1 |  | 1.3 | 1.7 |  | 0.0 | 0.0 |  |
| Extraction and Distribution of fossil Fuels | 0.0 | 0.0 |  | 2.3 | 2.6 |  | 0.0 | 0.0 |  | 0.0 | 0.0 |  |
| Transport (energy) | 69.1 | 142.2 |  | 0.3 | 0.2 |  | 0.9 | 3.3 |  | 0.0 | 0.0 |  |
| Road transport | 51.4 | 115.6 |  | 0.2 | 0.1 |  | 0.7 | 3.0 |  | 0.0 | 0.0 |  |
| Other transport modes | 17.7 | 26.6 |  | 0.0 | 0.1 |  | 0.2 | 0.3 |  | 0.0 | 0.0 |  |
| Industrial processes | 18.1 | 27.6 |  | 0.1 | 0.1 |  | 2.9 | 1.6 |  | 5.4 | 2.1 |  |
| Solvents and other Product use | 0.0 | 0.0 |  | 0.0 | 0.0 |  | 0.4 | 0.5 |  | 0.1 | 2.9 |  |
| Waste | 1.2 | 1.6 |  | 7.5 | 6.5 |  | 1.2 | 1.5 |  | 0.0 | 0.0 |  |
| Agriculture | 0.0 | 0.0 |  | 19.2 | 23.5 |  | 14.9 | 13.0 |  | 0.0 | 0.0 |  |
| Other Sources & Sinks | 0.0 | 0.0 |  | 1.7 | 1.8 |  | 3.5 | 2.6 |  | 0.0 | 0.0 |  |
| Total (excl. sinks) | 224.5 | 370.3 | -- | 32.0 | 35.4 | -- | 26.5 | 25.7 | -- | 3.3 | 5.0 | -- |

- Table 7: Assessment of the Target

|  | Excl. sinks |  |
| --- | --- | --- |
|  | MtCO2 equiv. | % of base year level (six gas basket) |
| Base year emissions (from projections) | 288.4 | 100% |
| Kyoto Commitment/burden sharing | 331.7 | 15.0% |
| With existing P&Ms projections | 436.3 | 151.3% |
| Gap (-ve means no gap) | 104.6 | 36.3% |
| Effect of flexible mechanisms | 20.0 | 6.9% |
| Remaining gap (if target includes flex mechs) | 84.6 | 29.3% |

Table above does not include LUCF.  Please refer to the LUCF section in the main report for further information.

- Description of Modelling Approach

An integrated model calculating projections for pollutants under the Geneva Convention as well as emissions of greenhouse gases is used by Spain. Base year for the calculations is 2000 and emissions are projected until 2010 on a year by year basis. The model is based on work undertaken by the European Environment Agency and the US Environmental Protection Agency. Methods used for projecting emissions depend on the sources and sinks but overall boundary parameters have been introduced to ensure consistency between the different projections.

No information is provided in the 4th National Communication on sensitivity analysis’ and uncertainty assessments undertaken.

- Projection indicator reporting

Indicators for the projections were not reported in the 4th National Communication. Spain should report these in the next submission to the Monitoring Mechanism due in 2007.

- Reporting of parameters on projections

Parameters used for the projections were not reported in the 4th National Communication. Spain should report these in the next submission to the Monitoring Mechanism due in 2007.

Table 8) Indicators for projections to monitor and evaluate progress with policies and measures (2005/166/EC) Annex III

Indicators for projections were not reported in the 4th National Communication

Table 9) List of Parameters on Projections (Annex IV Implementing Provisions)

Parameters used in the projections were not reported in the 4th National Communication

- Country Conclusions

The information available for Spain has increased significantly with the submission of its fourth National Communication under the UNFCCC. Many policies and measures have been quantified but the reduction effects are provided for specific years in some cases and periods of varying starting points and duration in other. These differences make comparisons between different policies and measures or with other Member States more difficult. Quantification for most measures addressing non-CO2 greenhouse gases have not been quantified.

The 4th National Communication includes only very few additional policies and measures. This is worrying in the light of the projections showing that Spain will not reach its Kyoto target with existing measures but needs to take decisive steps immediately.

Spain has provided a full set of projections for all years between 2000 and 2020 and all greenhouse gases for the first time for the ‘no measures’ and the ‘with measures’ scenarios. No projections for a ‘with additional measures’ scenario have been provided which is consistent with the very limited additional measures reported. No information on parameters and assumptions used for the projections and uncertainty assessments have been included in the national communication.

In the past, only projections for CO2 from combustion were available. With the new data a much better picture is available of the expected development of emission in the next 15 years. While emissions from non-CO2 gases in the with measures scenario for 2010 remain more or less at base year levels the release of carbon dioxide is expected to rise by around 65% in the same period. As CO2 is responsible for around 80% of Spain’s GHG emissions the Kyoto target under the EC burden sharing agreement of +15% will not be met.

Compared to the projections included in the 3rd National Communication for CO2 only a faster emission growth is predicted until 2010. The transport sector emissions show the biggest change, previously CO2 emissions were projected to rise by 48% relative to 1990, and now transport greenhouse gas emissions are predicted to rise by 108% relative to the base year. For energy generation, emissions increased by 40% relative to the base year in the latest ‘with measures’ projections compared to a 9% rise reported in the 3rd National Communication.

The intended use of Kyoto Mechanisms reduces the gap of approximately 105 Mt CO2 eq per year during the commitment period to 85 Mt CO2 eq per year. The potential for using sinks has been estimated to around 6 Mt CO2 eq per year by Spain. Even if the full potential will be used the effect will only reduce the gap by less than 10%. Additional efforts by Spain either domestically or through the use of the flexible mechanisms are therefore clearly needed.

SPAIN - Priority indicators (see Chapters A5.2 and A5.4 for more information on indicators)