DENMARK

Overview	2

Sources of Information	3

Overview of CCPM implementation in MS	3

Quality and Transparency of Reporting	3

Assessment of Policies and Measures	5

Evaluation of Projections	18

Description of Modelling Approach	23

Overview of modelling approach	23

Details of the uncertainty assessment	24

Projection indicator reporting	24

Reporting of parameters on projections	24

Country Conclusions	28

Indicators	29

| DENMARK Share in total EU-15 GHG emissions 2004 2 % Emissions base year (latest inventory) 69.3 Mt Emissions 2004 68.1 Mt Emissions base year (for projections) 69.6 Mt Projections 2010 with existing measures 72.5 Mt No projections with additional measures n.a. Kyoto target (absolute, based on latest inventory) 54.8 Mt Kyoto target (% from base year) – 21.0 % Change base year to 2004 - 1.8 % Change 2003–04 - 8.1 % Change base year to 2010 with existing measures + 4.2 % No projections with additional measures n.a. Distance to linear target path 2004 +7.9 (+12.9) percent points Use of Kyoto mechanisms 4.5 Mt Sinks (Articles 3.3. and 3.4) 0.5 Mt Emissions in 1990 (Article 3.7) n.a. |  | Note: ‘Target with Kyoto mechanisms’ is calculated by combining the agreed burden-sharing target with Kyoto units from JI and CDM and sinks according to Articles 3.3 and 3.4. | Past emissions: Denmark’s GHG emissions were 8.1 % below those of 2003 and 1.8 % below base-year levels in 2004. The main factor for decreasing emissions with regard to 2003 was a decrease of fossil fuel combustion in electricity and heat production due to higher hydropower production. Between 1990 and 2004, large emission decreases from agricultural soils and from households counterbalanced increasing road transport emissions. Emission projections: ‘With existing measures’ projections were 6 percentage points above the 2004 emissions. Nevertheless, Denmark will not achieve the Kyoto target based on these projections. Projections with additional measures were not provided. Denmark intends to close the gap between GHG projections and the Kyoto target through Kyoto mechanisms. However, with the budgets allocated so far, only 4.5 million tonnes of Kyoto units can be purchased per year. Additionally, Denmark intends to make use of carbon sinks according to Articles 3.3 and 3.4 of about 0.5 million tonnes. |
| --- | --- | --- | --- |

# Denmark

Please note that the projections relating to the NAP II are excluded from all the EU Country Profiles as they were not available in time to be assessed for the 2006 Country Profiles.

- Sources of Information

Denmark’s 4th National Communication to the United Nations Framework Convention on Climate Change and Report on Demonstrable Progress under the Kyoto Protocol, both submitted December 2005.

European Climate Change Programme (ECCP), Database on Policies and Measures in Europe

http://www.oeko.de/service/pam/index.php

- Overview of CCPM implementation in MS

Source: MS Responses to the CCPMs Questionnaire, 2005

- Quality and Transparency of Reporting

Denmark’s 4th National Communication presents a host of information on Denmark’s position with regard to climate change. Of particular relevance here, Denmark presents in detail policies and measures, and a summary of its progress towards its EU burden sharing commitment to reduce greenhouse gas emissions to 21% below 1990 levels.

The 4th National Communication provides good data on the “with measures” scenario policies and measures that have been implemented. Some policies and measures are presented with expected greenhouse gas savings to 2010.

Projections of emissions by sector were presented in the National Communication, and detailed projections tables by gas, further divided by sector are presented. Projections are presented for the “with measures” scenario. A “with additional measures” projection was not presented, as per last year.

Table 1: Information Provided on Policies and Measures

| Information provided | Level provided | Comments |
| --- | --- | --- |
| Policy names | +++ |  |
| Objectives of policies | +++ | Split by sector, including domestic and business. |
| Which greenhouse gases? | +++ |  |
| Status of Implementation | +++ | All implemented; many well established |
| Implementation body specified | +++ |  |
| Quantitative assessment of implementation | + | Few quantified. |
| Interaction with other P&Ms discussed | + | Some discussion. |

o, +, ++, +++ level of information available increases as the number of + signs increases

Table 2: Information provided on Projections

| Category of Information | Level of Information Provided | Comments |
| --- | --- | --- |
| Scenarios considered | + | Only with measures. Various scenarios used to project parameters for model input. |
| Expressed relative to base year | +++ |  |
| Starting year | 1990, 1995 (F gas) | Outstanding issue on electricity imports in base year. |
| Split of projections | +++ | Split by gas and sector. Each gas split by sector also. |
| Presentation of results | +++ | Clear. |
| Description of model (level of detail, approach and assumptions) | +++ |  |
| Sensitivity analysis (key inputs to model / high, central and low projections scenarios / robustness of model) | ++ |  |
| Discussion of uncertainty | + | Some discussion |
| Details of parameters and assumptions | +++ |  |

o, +, ++, +++ level of information available increases as the number of + signs increases

- Assessment of Policies and Measures

The quantification of the effect of polices and measures presented below is for 2010. Savings reported this year show no change compared to last year. The savings by sector given in Table 3 below will be used for analysis in the main report for the EEA’s Greenhouse gas Trends and Projections 2006.

In addition to the savings presented below, the Danish Energy Authority estimates that approx. 5.0 million tonnes CO2 annually will be offset, by increased electricity
exports based on the calculation assumptions of the climate strategy.

Table 3: Summary of the Effect of Policies and Measures Included in the Projections (MtCO2)

This table summarises the emission saving attributed to each type of policy.

Table 4: Detailed Information on Polices and Measures

Source: European Climate Change Programme (ECCP), Database on Policies and Measures in Europe

http://www.oeko.de/service/pam/index.php

| Member State | Sector | Name | Category | Type | GHG | Status | Absolute Reduction [kt (Gg) CO2 eq. p.a.] |  |  | Reduction in trading sector | Costs [EUR/t] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 2005 | 2010 | 2020 |  |  |
| Denmark | AGR | Action Plan for the Aquatic Environment I and II and Action Plan for Sustainable Agriculture | reduction of N losses | Regulatory; Information; Economic | N2O; CH4; CO2 | implemented |  | 2050 |  | No effect |  |
| Denmark | AGR | Ban on burning of straw on fields | Burning of agricultural residues | Regulatory | N2O; CO2 | implemented |  |  |  | no effect |  |
| Denmark | AGR | Ammonia action plan and the statuary order on manure | N management (organic and inorganic) | Regulatory | N2O | implemented |  | 30 |  | no effect |  |
| Denmark | AGR | Action Plan for the Aquatic Environment III | N management (organic and inorganic) | Economic; Regulatory | CH4; N2O | implemented | 200 | 200 |  | no effect |  |
| Denmark | AGR | Planting of windbreaks | C sequestration | Economic | CO2 | implemented |  | Max: 140 |  | no effect |  |
| Denmark | CRS | High technology foundation | Research programme | Research |  | planned |  |  |  |  |  |
| Denmark | CRS | Investment in research and development | Research programme | Research |  | planned |  |  |  |  |  |
| Denmark | CRS | EU Emission Trading Scheme | Emissions trading | Economic | CO2 | implemented |  |  |  |  |  |
| Denmark | CRS | JI Projects | Joint Implementation | voluntary/ negotiated agreement | CH4; CO2; HFCs; N2O; PFCs; SF6 |  |  |  |  |  |  |
| Denmark | CRS | Contracts for demanded CO2 credits | JI/CDM | voluntary/ negotiated agreement | CH4; CO2; HFCs; N2O; PFCs; SF6 | planned |  |  |  |  |  |
| Denmark | CRS | CDM Projects | CDM | voluntary/ negotiated agreement | CH4; CO2; HFCs; N2O; PFCs; SF6 | planned |  |  |  |  |  |
| Denmark | CRS | Taxes on CO2, mineral oils, gas, coal and electricity | Taxation | Economic | CO2 | implemented |  |  |  | moderate |  |
| Denmark | CRS | CO2 quota sheme | Emissions trading | Economic | CO2 | expired |  |  |  | complete |  |
| Denmark | ENC | Taxes on households and the public sectors energy consumption | Energy efficiency - Public | Fiscal | CO2 | implemented |  |  |  | moderate |  |
| Denmark | ENC | Energy labelling of small and large buildings (incl. public sector and business) | Buildings - energy efficiency | Information | CO2 | implemented |  |  |  |  |  |
| Denmark | ENC | Grants for energy savings in pensioners homes | Buildings - heat supply | Economic | CO2 | expired |  |  |  | low |  |
| Denmark | ENC | Electricity Saving Trust - conversion from electric heating | Buildings - heat supply | Economic | CO2 | implemented |  |  |  | complete |  |
| Denmark | ENC | Electricity Saving Trust - efficient appliances | Appliances | Information | CO2 | implemented |  |  |  | complete |  |
| Denmark | ENC | Energy labelling of electric appliances | Appliances | Information | CO2 | implemented |  |  |  | complete |  |
| Denmark | ENC | Grid, gas and district heating companies energy-saving activities | Energy efficiency - Industry | Information | CO2 | implemented |  |  |  | moderate |  |
| Denmark | ENC | Promoting particularly energy-efficient refrigerators and freezers | Appliances | Information | CO2 |  |  |  |  |  |  |
| Denmark | ENC | Phasing-out traditional double glazing - voluntary agreement | Energy efficiency | voluntary/ negotiated agreement | CO2 |  |  |  |  |  |  |
| Denmark | ENC | Tightening building regulations | Energy efficiency | Regulatory | CO2 |  |  |  |  |  |  |
| Denmark | ENC | Efficiency requirements for boilers and circulation pumps | Energy efficiency | Regulatory | CO2 |  |  |  |  |  |  |
| Denmark | ENC | Reduction of standby consumption | Energy efficiency | Information | CO2 |  |  |  |  |  |  |
| Denmark | ENC | Enhancement of research and development activities | Energy efficiency | Research | CO2 |  |  |  |  |  |  |
| Denmark | ENC | Circular on energy-efficiency in state institutions | Energy efficiency - Public | Regulatory | CO2 | implemented |  |  |  | high |  |
| Denmark | ENC | Agreements on energy efficiency with business | Energy efficiency - Industry | voluntary/ negotiated agreement | CO2 | implemented |  |  |  |  |  |
| Denmark | ENS | Biomass Agreement | Renewables | Economic; Research | CO2 | implemented |  | 1100 |  | moderate | 43,5825 |
| Denmark | ENS | Grant for electricity production from wind turbines | renewables; Renewables | Economic | CO2 | expired |  | 3400 |  | complete | 36,8775 |
| Denmark | ENS | Prioritisation of electricity from CHP plants | CHP | Economic | CO2 | expired |  | 2100 |  | high | 13,41 |
| Denmark | ENS | Grant to promote connection to coal-fired CHP | CHP |  | CO2 | expired |  | 100 |  |  | 113,985 |
| Denmark | ENS | Order to build offshore wind farms | Renewables | Economic | CO2 | expired |  | 900 |  | complete | 33,525 |
| Denmark | ENS | Scrapping scheme for old and unfortunately built wind turbines | Renewables | Economic | CO2 | implemented |  |  |  | complete |  |
| Denmark | ENS | Renewable energy grants and tax system | Renewables | Economic; Research | CO2 | expired |  | 100 |  | moderate | 201,15 |
| Denmark | ENS | Energy research | Energy plan | Research | CO2 | implemented |  |  |  |  |  |
| Denmark | ENS | Support of environmental friendly electricity | Renewables | Regulatory | CO2 |  |  |  |  | high |  |
| Denmark | ENS | Strategy for hydrogen technologies | Renewables | Research | CO2 | planned |  |  |  | low |  |
| Denmark | ENS | Strategy for liquid biofuels | Renewables | Research | CO2 | planned |  |  |  | low |  |
| Denmark | ENS | Strategy for wave energy | Renewables | Research | CO2 | planned |  |  |  | high |  |
| Denmark |  | Tender for off-shore wind turbines | Renewables | Regulatory; Economic | CO2 | implemented |  |  |  | high |  |
| Denmark | ENS | Biogas plant | renewables | Economic | CO2; CH4; N2O | implemented |  | 500 |  |  |  |
| Denmark | IND | Tax on HFCs, PFCs and SF6 | F-Gases | Fiscal | HFCs; PFCs; SF6 | implemented |  | 400 |  | no effect | 27 |
| Denmark | IND | Regulation on the use of industrial gases (HFCs, PFCs and SF&) | F-Gases | Regulatory | HFCs; PFCs; SF6 | implemented |  | 400 |  | no effect | 27 |
| Denmark | IND | Enterprise sheme on HFCs | F-Gases | Economic | HFCs | adopted |  |  |  |  |  |
| Denmark | LUC | Grant scheme for private afforestation on agricultural land | afforestation | Economic | CO2 | implemented |  | 262 |  | no effect | DKK 303 with outside effects, DKK 237 with side-effects (recreational value, groundwater protection) with discount rate 3% |
| Denmark | LUC | Public afforestation | afforestation | Regulatory; voluntary/ negotiated agreement | CO2 | implemented |  | 262 |  | no effect |  |
| Denmark | TRA | 1. Green owner tax on motor vehicles | taxation | Fiscal | CO2 | implemented |  | 540 |  |  |  |
| Denmark | TRA | 2. Information campaign on new cars' fuel consumption | vehicle efficiency | Information | CO2 | expired |  | 600 |  |  |  |
| Denmark | TRA | 3. Low-energy driving techniques | awareness rising | Information | CO2 | implemented |  | 600 |  |  |  |
| Denmark | TRA | 4. Action for compliance with current speed limits | road transport | Information; Economic | CO2 | implemented |  |  |  |  |  |
| Denmark | TRA | 5. Establishment of intermodal installations | change of modal split | Economic | CO2 | adopted |  |  |  |  |  |
| Denmark | TRA | 8. Promotion of environment-friendly freight transport | freight - logistics and combined transport | Economic; other; Information | CO2 | implemented |  |  |  |  |  |
| Denmark | TRA | 9. Promotion of company plans for road safety and environment, together with transport plans | change of modal split | Regulatory; Economic; other | CO2 | expired |  |  |  |  |  |
| Denmark | TRA | 10. Reduced travelling time for public transport | change of modal split - public transport | Regulatory | CO2 | implemented |  |  |  |  |  |
| Denmark | TRA | 11. Spatial planning | traffic reduction | Regulatory | CO2 | implemented |  |  |  |  |  |
| Denmark | TRA | 12. (3.1) Increases to fuel taxes | taxation; change of modal split | Fiscal | CO2 | implemented; adopted |  | 1200 |  |  | 755 DKK/t |
| Denmark |  | Reduction in registration tax for energy-efficient private cars | taxation | Fiscal | CO2 | implemented |  |  |  |  |  |
| Denmark | TRA | Development of green technologies | alternative fuels and engine technologies | Research | CO2 | implemented |  |  |  |  |  |
| Denmark | WAM | Obligation to send combustibel waste for incineration (in practice a ban on landfilling) | reduced disposal and stabilization of biodegradable waste on landfills | Regulatory | CH4 | implemented |  | 600 |  | high |  |
| Denmark | WAM | Waste tax | waste prevention or recycling | Fiscal | CH4 | implemented |  |  |  | no effect |  |
| Denmark | WAM | Weight-based taxes | waste prevention or recycling | Fiscal | CH4; CO2 | implemented |  |  |  | no effect |  |
| Denmark | WAM | Grant programme for cleaner products | waste prevention or recycling | Economic | CH4 | expired |  | 200 |  | no effect |  |
| Denmark | WAM | Increased recycling of waste plastic packaging | waste prevention or recycling | Regulatory | CO2 | adopted |  | 5 |  | no effect |  |
| Denmark | WAM | Implementation of the EU landfill directive | reduced disposal and stabilization of biodegradable waste on landfills | Regulatory | CH4 | adopted |  |  |  |  |  |
| Denmark | WAM | Support for (construction of facilities for) gas recovery at landfill sites | methane recovery | Economic | CH4 | expired |  |  |  |  |  |
| Denmark | WAM | Subsidy programme ? Enterprise Scheme (special scheme for businesses) | waste prevention or recycling | Economic | CH4 | implemented |  |  |  |  |  |

- Evaluation of Projections

There has been no change this year in the Denmark’s projections as reported in their 4th National Communication. Projections closely follow those contained in Denmark’s report to the EU Commission under Council Decision no. 280/2004/EC, submitted in June 2005.

The pie chart below shows that 80% of Denmark’s emissions stem from the energy and transport sectors. The projection scenario represents a “with measures” projection for Denmark.  A “with additional measures” projection was not submitted. The projections “with measures” demonstrate that Denmark will miss its target of 21% below 1990 emissions, with emissions projected to rise 4.2% above 1990 levels in 2010. This represents a gap of 17.5 MtCO2-eq. If flexible mechanisms are taken into consideration however, Denmark will only miss its target by 13 MtCO2-eq and the gap as a percentage of the base year will be reduced to 19%.

Table 5: Summary of projections by gas in 2010 (MtCO2)

Table 6:  Summary of projections (6 gas basket) by sector in 2010 (MtCO2eq)

| Table 6 Expanded: Summary of projections by sector and by gas in 2010 (MtCO2eq) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Carbon Dioxide |  |  | Methane |  |  | Nitrous Oxide |  |  | F-gases |  |  |
|  | Base year | With measures | With additional measures | Base year | With measures | With additional measures | Base year | With measures | With additional measures | Base year | With measures | With additional measures |
| Energy (exc transport) | 26.4 | 29.5 |  | 0.1 | 0.4 |  | 0.3 | 0.3 |  |  |  |  |
| Transport (energy) | 10.6 | 14 |  | 0.1 | 0.1 |  | 0.1 | 0.5 |  |  |  |  |
| Business | 8.2 | 8.9 |  | 0 | 0.1 |  | 1.1 | 0.1 |  | 0.3 | 0.8 |  |
| Domestic | 5 | 4.1 |  | 0.1 | 0.1 |  | 0.1 | 0.1 |  |  |  |  |
| Waste |  |  |  | 1.5 | 1.3 |  | 0.1 | 0.1 |  |  |  |  |
| Agriculture | 2.7 | 2.5 |  | 3.9 | 3.6 |  | 9 | 5.9 |  |  |  |  |
| Total (excl. sinks) | 52.9 | 59 | 0 | 5.7 | 5.6 | 0 | 10.7 | 7 | 0 | 0.3 | 0.8 | 0 |

Table 7: Assessment of the Target

Table above does not include LUCF.  Please refer to the LUCF section in the main report for further information.

In the Council decision (2002/358/EC) on the approval by the EU of the Kyoto Protocol the various commitments of the Member States are expressed as percentage changes from the base-year. In 2006 the respective emission levels will be expressed in terms of tonnes of CO2-equivalent. In this context, the Council of Environment Ministers and the Commission have, in a joint statement, agreed to take into account inter alia the assumptions in Denmark’s statement to the Council Conclusions of 16–17 June 1998 relating to base-year emissions.

Specifically Denmark have restated in the 4th National Communication that they would like to see their commitment in 2008-2012 to be corrected for imports of electricity. Denmark reports this difference as 5Mt CO2 eq. annually in the Kyoto period.

- Description of Modelling Approach

- Overview of modelling approach

An excellent description of the modelling approach is provided in the 4th National Communication. A detailed methodology, including details of assumptions and parameters used is presented for each sector. For instance, a variety of growth assumptions are modelled to 2030.

Models used for overall projections and to project key inputs include ADAM/EMMA macro model, Danish Energy Agency’s RAMSES model and, for transport, Danish assumptions from 2003 are used. Where relevant model inputs have been modified to account for local conditions e.g. the Nordic power market.

Sensitivity analysis

In their 4th National Communication, Denmark presents sensitivity analyses carried out to assess the effects of various factors on selected policies. These factors include changes in electricity price, discount rate and demand elasticity. A variety of other scenarios built for sensitivity analyses is detailed including those assessing the impact of high to low fuel prices and prices of CO2 allowances on energy consumption.

- Details of the uncertainty assessment

Uncertainty in projections is discussed in Denmark’s 4th National Communication. No uncertainty ranges are presented however. Uncertainty ranges were however presented for the greenhouse gas inventories.

- Projection indicator reporting

Denmark did not report on indicators with detail or quantification in the 4th national communication. Denmark should report these in the next submission to the Monitoring Mechanism. Some description is however provided on Denmark’s procedure for regular reporting of 14 key indicators.

- Reporting of parameters on projections

A small number of parameters for projections (mandatory and recommended parameters on projections under Annex IV of the Implementing Provisions) were reported in the 4th National Communication.

Table 8) Indicators for projections to monitor and evaluate progress with policies and measures (2005/166/EC) Annex III

Indicators for projections were not detailed in the 4th National Communication

Table 9) List of Parameters on Projections (Annex IV Implementing Provisions)

- Country Conclusions

Denmark’s most recent projections are contained within the 4th National Communication.  There has been no change this year in the Denmark’s projections as compared to Denmark’s report to the EU Commission under Council Decision no. 280/2004/EC, as submitted in June 2005.

In the 4th National Communication however, Denmark state that the 2005 projections will be updated early in 2006 to take account of the adoption of additional energy savings initiatives in 2005 as well as updated projections in the offshore sector and new IEA projections of energy prices. The projections reported here do not take in to account these new projections.

It is projected that Denmark’s emissions will increase by 4% by 2010 for the ‘with measures’ scenario. Comparing this projection with Denmark’s burden sharing commitment of 21% below the base year, the gap between the “with measures” projection and the Kyoto target, represents a 25% (17.5 MtCO2-eq per year 2008-2012) increase on the base year.  When the assumptions in Denmark’s statement to the Council Conclusions of 16–17 June 1998 relating to base-year emissions are taken into account as agreed by the Council of Environment Ministers and the Commission in a joint statement, the gap between the ‘with measures’ projection on the Kyoto target falls to 18% (12.5 MtCO2-eq per year 2008-2012).

If flexible mechanisms are included in the projection, the gap to target reduces to 19% of the base year (13 MtCO2-eq), or 11% including the joint statement on the base year emissions. A ‘with additional measures’ scenario is not included in these projections.

The “with measures” projection was split by carbon dioxide, methane, nitrous oxide, HFCs, PFCs and SF6. These projections (by gas) were comprehensively presented by the sector of origin.  Tables summarising the total quantified effects of policies and measures was also helpfully provided in the report. An excellent description of the modelling and methodology was provided. The projections tables allow a comprehensive analysis of Denmark’s progress in relation to the Kyoto commitments.

DENMARK - Priority indicators (see Chapters A5.2 and A5.4 for more information on indicators)

DENMARK – Reported Indicators

Priority indicators and additional priority indicators