AUSTRIA

Overview	2

Sources of Information	3

Overview of CCPM implementation in MS	4

Quality and Transparency of Reporting	4

Assessment of Policies and Measures	6

Evaluation of Projections	16

Description of Modelling Approach	21

Country Conclusions	22

Indicators	23

| AUSTRIA Share in total EU-15 GHG emissions 2004 2 % Emissions base year (latest inventory) 78.9 Mt Emissions 2004 91.3 Mt Emissions base year (for projections) 78.4 Mt Projections 2010 with existing measures 89.9 Mt Projections 2010 with additional measures 81.0 Mt Kyoto target (absolute, based on latest inventory) 68.7 Mt Kyoto target (% from base year) – 13 % Change base year to 2004 + 15.7 % Change 2003–04 - 1.3 % Change base year to 2010 with existing measures + 14.8 % Change base year to 2010 with additional measures + 3.3 % Distance to linear target path 2004 +17.9 (+24.8) percent points Use of Kyoto mechanisms 7.0 Mt Sinks (Articles 3.3. and 3.4) 0.7 Mt Emissions in 1990 (Article 3.7) n.a. |  | Note: ‘Target with Kyoto mechanisms’ is calculated by combining the agreed burden-sharing target with Kyoto units from JI and CDM and sinks according to Articles 3.3 and 3.4. |  |  | Past emissions: In 2004 Austria’s GHG emissions were 1.3 % below those of 2003 and 15.7 % above base-year levels. Main factors for decreasing emissions with regard to the previous year were decreasing public heat and power production and fuel combustion in households due to warmer temperatures in winter and increasing biomass use. Looking at the change 1990–2004, road transport is by far the largest contributor to emission increases, followed by iron and steel production, public heat and power production and the refinery. One reason for the large increase in transport-related GHG emissions is low road fuel prices in Austria which encourages fuel tourism. Emission projections: Emissions in 2004 were 1 index points above the level projected in the ‘with measures’ scenario for 2010. This is mainly the result of sharp increases in road transport-related emissions. Austria will not achieve the Kyoto target with additional domestic measures, therefore 7.0 million tonnes of Kyoto units per year of the commitment period will be used as well as sinks according to Article 3.3 (0.7 million tonnes). |
| --- | --- | --- | --- | --- | --- |

# Austria

Please note that the projections relating to the NAP II are excluded from all the EU Country Profiles as they were not available in time to be assessed for the 2006 Country Profiles.

Information on policies and measures and on projections is included in the Third National Communication and in the Austrian Climate Strategy. The Third National Communication provides two sets of projections: (1) projections based on model calculations; (2) projections based on the draft Austrian Climate Strategy. The Austrian Climate Strategy was adopted after the Third National Communication and includes projections which slightly differ from the projections included in the Third National Communication. Therefore, this document provides quantitative data on projections from the Austrian Climate Strategy. Tables 5a, 6a and 7a contain updated projections as reported to the European Commission in 2006.

- Sources of Information

Third National Communication to the UNFCCC:

Federal Ministry of Agriculture, Forestry, Environment and Water Management: Austria’s Third National Climate Report of the Austrian Federal Government – Austria’s Third National Communication in Compliance with the Obligations under the Framework Convention on Climate Change (Federal Law Gazette No. 414/1994), November 2001, Vienna.

Austrian Climate Strategy:

Bundesministerium für Land- und Forstwirtschaft, Umwelt und Wasserwirtschaft: Strategie Österreichs zur Erreichung des Kyoto-Ziels, Klimastrategie 2002/2012, vom Ministerrat angenommen am 18. Juni 2002, Wien.

National Allocation Plan (Austria):

Bundesministerium für Land- und Forstwirtschaft, Umwelt und Wasserwirtschaft: Nationaler Zuteilungsplan für Österreich gemäß § 11 EZG, 31. März 2004, mit Ergänzungen vom 7. April 2004 (http://www.eu-emissionshandel.at/)

Austria´s Emission Projection on Greenhouse Gases 2003-2020 – Interim Report (unpublished):

Umweltbundesamt, Vienna, June 15th 2005, according to the EU Monitoring Mechanism (Council Decision 280/2004/EC).

Austrian greenhouse gas emissions projections (2006); letter of the Austrian Federal Ministry of Agriculture, Forestry, Environment and Water Management to the European Commission.

- Overview of CCPM implementation in MS

Source: MS Responses to the CCPMs Questionnaire, 2005

| CCPM | Austria |
| --- | --- |
| Taxation of energy products 2003/96/EC | N |
| Emissions trading 2003/87/EC | N |
| Promotion of electricity from RE sources 2001/77/EC | N |
| Promotion of cogeneration 2004/8/EC |  |
| Directives on energy labelling of appliances | N |
| Motor challenge, voluntary EC programme |  |
| Energy performance of buildings 2002/91/EC | N |
| Eco-management & audit scheme (EMAS) EC 761/2001 | N |
| Efficiency of hot water boilers 92/42/EEC | N |
| Transport modal shift to rail 2001/12/EC etc. | N |
| Promotion of biofuels for transport 2003/30/EC | N |
| Consumer information on cars 1999/94/EC | N |
| Agreement with car manufacturers ACEA etc. | N |
| Support under CAP (1782/2003) | N |
| Support under CAP - ammendment (1783/2003) |  |
| Landfill directive 1999/31/EC | B |
| CODE |  |
| New National PAM implemented after CCPM was adopted | N |
| Existing National PAM re-enforced by CCPM | R |
| National PAM already in force before CCPM was adopted | B |
| Not reported |  |

- Quality and Transparency of Reporting

This chapter refers to the 3rd National Communication only.

The Third National Communication draws on the draft Austrian Climate Strategy 2010 and on model calculations. It provides a list of policies and measures to reduce greenhouse gas emissions in a range of sectors. The impact of the measures has been quantified in most cases. The with measures projection includes measures implemented and adopted by 2001; the with additional measures projection includes measures planned.

The table on policies and measures follows the UNFCCC guidelines.

Table 1: Information Provided on Policies and Measures

| Information provided | Level provided | Comments |
| --- | --- | --- |
| Policy names | +++ |  |
| Objectives of policies | +++ |  |
| Which GHGs? | CO2, CH4, N2O, HFC, SF6 |  |
| Status of Implementation | +++ |  |
| Implementation body specified | +++ |  |
| Quantitative assessment of implementation | +++ | Estimated mitigation effect for 2010; a few measures are not quantified |
| Interaction with other P&Ms discussed | ++ | In some cases |

+, ++, +++ level of information available increases as the number of + signs increases

Note: Information on ‘Type of instrument’ follows largely the UNFCCC Guidelines but introduces the term ‘promotive’, which in many cases seems to correspond with ‘economic’.

Two approaches are taken to develop projections of greenhouse gas emissions and to calculate the total effect of policies and measures. On the one hand, the emission projections are based on expert judgement (Austrian Climate Strategy). On the other hand, projections for the period 2000–2020 have been developed based on the model calculations (Third National Communication). Both approaches include with measures and with additional measures scenarios and show similar results.

Table 2: Information provided on Projections

| Category of Information | Level of Information Provided | Comments |
| --- | --- | --- |
| Scenarios considered | With measures With additional measures | Austrian Climate Strategy (expert judgment): Both scenarios are given for sectors partly corresponding with IPCC sectors Third National Communication (model calculations): Both scenarios are given for IPCC sectors and gases |
| Expressed relative to inventory for previous years | Yes |  |
| Starting year | 1999 |  |
| Split of projections | +++ | Austrian Climate Strategy (expert judgment): Projections split by sectors partly corresponding with IPCC sectors Third National Communication (model calculations): Projections split by IPCC sectors and gases |
| Presentation of results | +++ | Results presented in both tabular and graphical form |
| Description of model (level of detail, approach and assumptions) | ++ | Basic description of the models and further references provided |
| Discussion of uncertainty | No | A few results of sensitivity analysis provided |
| Details of parameters and assumptions | ++ | Summary table for key parameters (input and output parameter are marked) |

+, ++, +++ level of information available increases as the number of + signs increases

- Assessment of Policies and Measures

This chapter refers to the Austrian Climate Strategy only.

Table 3 gives an overview of effects of additional policies and measures from the Austrian Climate Strategy. The effects of existing policies and measures are included in the scenario ‘Trend 2010’.

Table 3: Summary of the Effect of Policies and Measures by 2010 (MtCO2)

|  | Reduction potential (additional measures) |
| --- | --- |
| CO2 | -10,9 |
| CH4 | -1,6 |
| N2O | -0,2 |
| F-gases | -1,2 |
| Energy demand (small combustion) | 4,0 |
| Energy supply | 2,1 |
| Waste | 1,1 |
| Transport | 3,7 |
| Industry (energy and process) | 1,25 |
| Agriculture | 0,4 |
| HFCs, PFCs, SF6 | 1,2 |
| Other (incl. solvent use) | 0,1 |
| Total | 13,85 |

Source: Austrian Climate Strategy

Table 4 gives details of the policies and measures.  The measures implemented or adopted by 2001 are included in the with measures  projections.  The measures planned are included in the with additional measures projections.

Table 4: Detailed Information on Polices and Measures

Source: European Climate Change Programme (ECCP), Database on Policies and Measures in Europe

http://www.oeko.de/service/pam/index.php

| Member State | Sector | Name | Category | Type | GHG | Status | Absolute Reduction [kt (Gg) CO2 eq. p.a.] |  |  | Reduction in trading sector | Costs [EUR/t] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 2005 | 2010 | 2020 |  |  |
| Austria | AGR | Extension of ecological farming | organic farming; N fertilization | Economic | CH4; N2O | implemented |  |  |  | no effect |  |
| Austria | CRS | Energy related taxes and earmarking for climate change related measures | Taxation | Fiscal | CO2 | adopted |  | 300 |  | high |  |
| Austria | CRS | GHG Emissions Trading Scheme | Emissions trading | Economic | CO2 | implemented |  |  |  | complete |  |
| Austria | ENC | Agreement between Federation and Länder on energy-saving | Buildings - energy efficiency | Regulatory | CO2 | implemented |  |  |  | moderate |  |
| Austria | ENC | Minimum thermal standards for buildings | Buildings - energy efficiency | Regulatory | CO2 | implemented; planned |  | 250 |  | low |  |
| Austria | ENC | Housing support schemes | Buildings - energy efficiency | Information; Economic | CO2; HFCs | implemented; planned |  | 750; 1500 |  | moderate |  |
| Austria | ENC | Consumption - related heating costs calculation | Buildings - heat supply | Regulatory | CO2 | implemented |  | 100 |  | no effect |  |
| Austria | ENC | Energy efficiency in federal public buildings | Energy efficiency - Public | voluntary/ negotiated agreement | CO2 | implemented |  | 50 |  | no effect |  |
| Austria | ENC | ´Contracting´for public buildings | Energy efficiency - Companies | Economic; voluntary/ negotiated agreement | CO2 | adopted |  | 500 |  | low |  |
| Austria | ENC | Replacement of old heating systems | Buildings - heat supply | Economic; Information |  | implemented; planned |  | 500; 1500 |  | moderate |  |
| Austria | ENC | Harmonised energy codes for buildings | Buildings - energy efficiency | voluntary/ negotiated agreement | CO2 | planned |  |  |  | low |  |
| Austria | ENC | Regular inspection of heating systems | Buildings - heat supply | Regulatory |  | implemented; planned |  | 100; 200 |  | no effect |  |
| Austria | ENC | Building of Tomorrow | Buildings - energy efficiency | voluntary/ negotiated agreement; Economic | CO2 | implemented |  |  |  | low |  |
| Austria | ENC | Electric Appliances and Others | Appliances | Regulatory | CO2 | implemented |  |  |  | complete |  |
| Austria | ENC | Promotion of Energy Efficient Electronic Products, The Group for Energy Efficient Appliances (GEEA) | Appliances | Information | CO2 | implemented |  |  |  | complete |  |
| Austria | ENC | Promotion of energy efficiency and renewable energy in industry |  | voluntary/ negotiated agreement | CO2 | expired |  | 200 |  | high |  |
| Austria | ENC | Energy concepts for energy intensive branches |  | Information | CO2 | implemented |  |  |  | high |  |
| Austria | ENC | Energy efficiency programme in industry |  | Information | CO2 | planned |  | 500 |  | moderate |  |
| Austria | ENC | Promotion of energy efficiency and renewable energy in industry |  | voluntary/ negotiated agreement | CO2 | planned |  | 300 |  | high |  |
| Austria | ENS | Voluntary agreements | Energy efficiency - industry | voluntary/ negotiated agreement | CO2 | planned |  |  |  | complete |  |
| Austria | ENS | Preferential market access for ?green electricity? | Renewables | Economic; Regulatory | CO2; CH4 | implemented |  | 500 |  | high |  |
| Austria | ENS | Public support for renewable energy projects and district heating | Renewables; CHP | Economic | CO2; CH4 | implemented |  | 700 |  | moderate |  |
| Austria | ENS | Energy tax rebates for CHP | CHP | Fiscal | CO2 | implemented |  |  |  | high |  |
| Austria | ENS | Preferential framework conditions for CHP | CHP | Economic | CO2 | implemented; adopted; planned |  | 500 |  | high |  |
| Austria | ENS | Further development of targets for renewable energy sources and implementation of the EU directive on renewables | Renewables | Regulatory | CO2; CH4 | planned |  | 350 |  | complete |  |
| Austria | ENS | Stepped-up public support for renewable energy and district heating projects | Renewables; CHP | other | CO2 | planned |  | 1750 |  |  |  |
| Austria | ENS | Green electricity purchasing for public buildings | Renewables | Economic | CO2 | adopted |  |  |  | complete |  |
| Austria | ENS | Demand-Side Management (DSM) | Energy efficiency | Information | CO2 | implemented |  |  |  | moderate |  |
| Austria | IND | Implementation of the IPPC Directive | IPPC | Regulatory | CO2 | implemented |  |  |  | moderate |  |
| Austria | IND | Voluntary Agreements and flexible instruments |  | voluntary/ negotiated agreement | CO2 | planned |  | 1500 |  | high |  |
| Austria | TRA | Reduction of emissions from passenger cars via labelling | vehicle efficiency | voluntary/ negotiated agreement; Information | CO2 | implemented |  | 500 |  |  |  |
| Austria | TRA | Fuel consumption levy | vehicle efficiency | Fiscal | CO2 | implemented |  |  |  |  |  |
| Austria |  | Road tolls | road pricing | Fiscal | CO2 | planned; implemented |  |  |  |  |  |
| Austria | TRA | Vehicle tax adaptaption 2000 | vehicle efficiency | Fiscal | CO2 | implemented |  |  |  |  |  |
| Austria | TRA | Rail infrastructure and public transport investments | change of modal split - public transport | Economic; Information | CO2 | implemented; planned |  | 300; 100; 200 |  |  |  |
| Austria | TRA | Improvement of fuel quality and promotion of "bio diesel" | biofuels; fuel quality | Regulatory; Fiscal | CO2 | implemented; planned |  | 100; 50 |  |  |  |
| Austria | TRA | Model projects and programmes for environmentally sound mobility | traffic reduction; change of modal split | Information; education; Research | CO2 | implemented |  |  |  |  |  |
| Austria | TRA | Model projects and programmes for environmentally sound logistics | freight - logistics and combined transport | Information; education; Research | CO2 | implemented |  |  |  |  |  |
| Austria | TRA | Mileage based toll for lorries | road pricing | Fiscal | CO2 | adopted |  | 300 |  |  |  |
| Austria | TRA | Promotion of energy efficient and alternative motor concepts | vehicle efficiency; alternative fuels and engine technologies | Information; Research | CO2 | planned |  | 100 |  |  |  |
| Austria | TRA | Public awareness raising measures | traffic reduction | Information; education | CO2 | adopted; planned |  | 300; 100; 200 |  |  |  |
| Austria | TRA | Improvement of transport logistics | freight - logistics and combined transport | Information | CO2 | planned; implemented |  | 700; 500; 200 |  |  |  |
| Austria | TRA | Further internalisation of costs | road pricing | Economic; Fiscal | CO2 | planned |  |  |  |  |  |
| Austria | TRA | Promotion of walking and cycling | change of modal split - bicycles and walking | Information; education | CO2 | implemented; planned |  | 300; 100; 200 |  |  |  |
| Austria | TRA | Improvement of spatial planning | traffic reduction | Regulatory; planning | CO2 | planned |  | 300 |  |  |  |
| Austria | TRA | Traffic management and speed limitation | road-traffic management | Information; Regulatory; Fiscal | CO2 | implemented; planned |  | 300 |  |  |  |
| Austria | WAM | Waste Management Act 1990 | waste management practices | Regulatory | CH4 | implemented |  |  |  | no effect |  |
| Austria | WAM | Landfill Regulation 1996 | reduced disposal and stabilization of biodegradable waste on landfills | Regulatory | CH4 | adopted; implemented |  | 900 |  | no effect |  |
| Austria | WAM | Landfill Charge Act 1989 | reduced disposal and stabilization of biodegradable waste on landfills | Regulatory; Fiscal; Economic | CH4 | implemented |  |  |  | no effect |  |
| Austria | WAM | Expansion of waste treatment capacities other than landfilling | reduced disposal and stabilization of biodegradable waste on landfills | Regulatory | CH4; CO2 | adopted |  | 300 |  | no effect |  |
| Austria | WAM | Promotion of waste recovery | waste prevention or recycling | voluntary/ negotiated agreement; education; Information; Research | CH4; CO2 | implemented; adopted |  | CL |  | no effect |  |
| Austria | WAM | Efficient energy recovery from waste | waste incineration | Economic | CH4; CO2 | adopted |  | CL |  | no effect |  |
| Austria | WAM | Other programmes to launch waste prevention and recovery | waste prevention or recycling | voluntary/ negotiated agreement; Information; Research; education | CH4; CO2 | implemented; adopted |  | CL |  | no effect |  |
| Austria | WAM | Define technical state of art for mechanical-biological treatment of waste | MBT | Regulatory | CH4 | planned |  | CL |  | no effect |  |

- Evaluation of Projections

The text relates to the Austrian Climate Strategy only!

Table 5 shows the projections by greenhouse gas for 2010, Table 6 summarises the projections by sector. Transport related emissions are projected to increase by 62% (with measures) and 32% (with additional measures). The largest decreases are projected for energy demand from small combustion with –28% in the with additional measures scenario. The emissions from agriculture and waste reduce in both, the trend 2010 scenario and the scenario with additional measures.

Table 5: Summary of projections by gas in 2010 (MtCO2)

|  | Base year | with measures | with additional measures |
| --- | --- | --- | --- |
| CO2 | 62.3 | 70.3 | 59.4 |
| CH4 | 11.3 | 8.8 | 7.2 |
| N2O | 2.3 | 2.3 | 2.1 |
| F-gases | 1.7 | 3.0 | 1.8 |
| Total | 77.6 | 84.4 | 70.5 |
| % change relative to base year |  | 8.8% | -9.1% |

Source: Austrian Climate Strategy

Table 5a: Update of projections by gas in 2010 (MtCO2)

|  | Base year | with measures | with additional measures |
| --- | --- | --- | --- |
| Carbon dioxide (excl. sinks) | 61.3 | 76.6 | 67.7 |
| Methane | 9.8 | 7.2 | 7.2 |
| Nitrous oxide | 5.7 | 4.6 | 4.6 |
| HFCs* | 0.0 | 0.7 | 0.7 |
| PFCs | 1.1 | 0.1 | 0.1 |
| SF6 | 0.5 | 0.7 | 0.7 |
| Total (excl. sinks) | 78.4 | 89.9 | 81.0 |
| % change relative to base year (excl. sinks) |  | 14.8% | 3.3% |

Source: Austrian greenhouse gas emissions projections (2006)

Table 6: Summary of projections by sector in 2010 (MtCO2)

|  | Base year | With measures (trend 2010) | % change relative to 1990 | with additional measures | % change relative 1990 (additional measures) |
| --- | --- | --- | --- | --- | --- |
| Energy demand (small combustion) | 14.60 | 14.50 | -0.7% | 10.50 | -28.1% |
| Energy supply | 14.44 | 14.50 | 0.4% | 12.40 | -14.1% |
| Waste | 6.26 | 4.80 | -23.3% | 3.70 | -40.9% |
| Transport | 12.32 | 20.00 | 62.3% | 16.30 | 32.3% |
| Industry (energy and process) | 21.71 | 22.00 | 1.3% | 20.75 | -4.4% |
| of which energy* | 13,12 | 13,29 | 1,3% | 12,54 | -4,4% |
| of which process* | 8,59 | 8,71 | 1,3% | 8,21 | -4,4% |
| Agriculture | 5.60 | 4.80 | -14.3% | 4.40 | -21.4% |
| HFCs, PFCs, SF6 | 1.74 | 3.00 | 72.4% | 1.80 | 3.4% |
| Other (incl. solvent use) | 0.97 | 0.80 | -17.5% | 0.70 | -27.8% |
| Total | 77.64 | 84.40 | 8.7% | 70.55 | -9.1% |

Source: Austrian Climate Strategy

*Industry emissions were not split into energy and process related emissions in the Austrian Climate Strategy, therefore, the percentage change as reported in the latest GHG inventory for the year 2002 has been applied.

Table 6a: Update of projections by sector in 2010 (MtCO2)

|  | Base year | with measures | % change relative to 1990 | with additional measures | % change relative to 1990 |
| --- | --- | --- | --- | --- | --- |
| Energy (total, excluding transport) | 42.3 | 47.2 | 12% | 43.8 | 4% |
| Energy supply |  |  |  |  |  |
| Energy – industry, construction | 26.8 | 32.3 | 20% | 29.2 | 9% |
| Energy – other (commercial, residential, agric) | 15.5 | 14.9 | -4% | 14.7 | -5% |
| Transport (energy) | 12.6 | 21.6 | 71% | 16.1 | 27% |
| Industrial processes | 10.5 | 11.1 | 6% | 11.0 | 6% |
| Waste | 4.5 | 2.8 | -38% | 2.8 | -38% |
| Agriculture | 8.5 | 7.3 | -14% | 7.2 | -14% |
| Total (excl. sinks) | 78.4 | 89.9 | 15% | 81.0 | 3% |

Source: Austrian greenhouse gas emissions projections (2006)

| Table 6a Expanded: Summary of projections by sector and by gas in 2010 (MtCO2eq) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Carbon Dioxide |  |  | Methane |  |  | Nitrous Oxide |  |  | F-gases |  |  |
|  | Base year | With measures | With additional measures | Base year | With measures | With additional measures | Base year | With measures | With additional measures | Base year | With measures | With additional measures |
| Energy (exc transport) | 41.122213 | 46.152745 | 42.836454 | 0.6857065 | 0.5742436 | 0.5594021 | 0.5011845 | 0.4507521 | 0.4429562 |  |  |  |
| Transport (energy) | 12.404873 | 21.426256 | 15.927832 | 0.0611079 | 0.0118554 | 0.0108166 | 0.1708106 | 0.1463587 | 0.1230434 |  |  |  |
| Industrial processes | 7.7148333 | 9.0410886 | 8.9624903 | 0.0074842 | 0.0075011 | 0.0075011 | 1.14452 | 0.6066638 | 0.6066638 | 1.585 | 1.473 | 1.473 |
| Waste | 0.0207016 | 0.0112708 | 0.0112708 | 4.4413374 | 2.4914737 | 2.4914737 | 0.0410627 | 0.2790349 | 0.2790349 |  |  |  |
| Agriculture | 0 | 0 | 0 | 4.6020503 | 4.1068574 | 4.0867533 | 3.8541781 | 3.1540677 | 3.1494243 |  |  |  |
| Total (excl. sinks) | 61.262621 | 76.631361 | 67.738047 | 9.7976863 | 7.1919312 | 7.1559469 | 5.7117559 | 4.6368773 | 4.6011226 | 1.585 | 1.473 | 1.473 |

The with measures projections show that the currently implemented or adopted measures of Austria do not reduce greenhouse gas emissions by 2010. Instead, greenhouse gas emissions are projected to be 8.7% above the base year level by 2010 in the with measures scenario.

Even in the with additional measures scenario Austria is expected to exceed its commitment of a 13% reduction in greenhouse gas emissions under the EU burden sharing agreement. Additional measures will reduce greenhouse gas emissions to 9.1% below 1990 levels by 2010. The remaining GHG reduction requirement of around 3 Mt CO2 equivalents will be fulfilled by making use of the project-based flexible mechanisms of the Kyoto Protocol (JI/CDM).

Table 7: Assessment of the Target

|  | MtCO2 equiv. | % of 1990 level (six gas basket) |
| --- | --- | --- |
| Base year (from projections) | 77.64 |  |
| Commitment | 67.55 | -13.0% |
| With existing P&Ms (trend 2010) | 84.4 | 8.7% |
| Gap (-ve means no gap) | 16.85 | 21.7% |
| Effect of additional P&Ms | 13.85 | -9.1% |

Source: Austrian Climate Strategy

Table 7a: Assessment of the Target

|  | Excl. sinks |  |
| --- | --- | --- |
|  | MtCO2 equiv. | % of base year level (six gas basket) |
| Base year emissions (from projections) | 78.4 | 100% |
| Kyoto Commitment/burden sharing | 68.2 | -13.0% |
| With existing P&Ms projections | 89.9 | 114.8% |
| Gap (-ve means no gap) | 21.8 | 27.8% |
| With additional P&Ms projections | 81.0 | 103.3% |
| Remaining gap | 12.8 | 16.3% |
| Effect of flexible mechanisms | 7.0 | 8.9% |
| Remaining gap (if target includes flex mechs) | 5.8 | 7.4% |
| Above table excludes sinks |  |  |
| Sinks will be covered in the main report, based on the questionnaire submissions |  |  |

Source: Austrian greenhouse gas emissions projections (2006)

- Description of Modelling Approach

This section refers to the model-based projections provided in the Third National Communication. Emissions from the energy sector are projected with the modelling framework of the Austrian Institute for Economic Research. The framework includes the energy model DEDALUS and the macroeconomic multisectoral model MULTIMAC. Resulting figures for energy demand in the Austrian economy are split according to the subsectors of the Austrian greenhouse gas emission inventory and emissions are calculated in accordance with inventory methodology.

The structure of the energy-economy-environment (E3) modelling framework is as follows: The energy model DEDALUS is integrated into the macro economic multisectoral model of the Austrian economy MULTIMAC III. The output of DEDALUS determines the energy sector variables, which is the energy-economy link. The outcome of the macroeconomic multisectoral model (GDP, output by industries, capital stock for different energy-relevant purposes) determines together with fully exogenous influences (energy prices, technology diffusion for renewables and district heating, transport equipment, demography, etc.) the energy use and CO2 emissions, which constitute the remaining environmental link.

DEDALUS consists of an econometric model for final energy demand of 13 sectors of the Austrian economy and an input-output model of energy transformation with varying technical coefficients. This model describes the Austrian energy system in sufficient detail to derive emission paths of 13 sectors and the energy transforming processes.

MULTIMAC III is input-output based at a medium aggregation level of 32 industries and combines econometric functions for goods and factor demand, prices, wages and the labour market with the input-output accounting framework.

Emission scenarios for the agriculture, industrial processes and waste sectors are in principle based on the Austrian Carbon Balance Model (ACBM). ACBM is a model that reflects the dynamic behaviour of the carbon cycle in Austria with the aim of establishing a full carbon accounting for Austria. For the current projections, the results of the ACBM scenarios had to be partially adapted according to results of the energy projections (e.g. activities in the industry sector) and to the latest expert judgements (e.g. development of livestock). The emission figures are based on emissions factors from the Austrian greenhouse gas inventory.

Table 8: Modelling parameters

| Parameter | 1990 | 1995 | 2000 | 2005 | 2010 | 2015 | 2020 | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GDP growth | +4.6 | +1.5 | +3.9 | +2.1 | +2.0 | +1.9 | +2.0 | annual average % (for previous 5 years) |
| Oil price Brent |  |  | 24.9 | 16.5 | 16.5 | 19.7 | 22.5 | EURO(1990/bbl |
| Population | 7.73 | 8.05 | 8.11 | 8.17 | 8.21 | 8.25 | 8.30 | million |
| Electricity import/export ratio |  |  | -2.6 | +2.6 | +2.0 | +2.0 | +2.0 | % |
| Number of dwellings | 2.97 |  | 3.26 | 3.36 | 3.44 | 3.51 | 3.59 | million |
| Private cars | 2.99 | 3.59 | 4.03 | 4.40 | 4.77 | 5.10 | 5.37 | million |
| Energy prices |  |  |  |  |  |  |  |  |
| Fuel oil |  |  | 100 | 78 | 76 | 79 | 84 | % of 2000 |
| Electricity |  |  | 100 | 92 | 102 | 103 | 106 | % of 2000 |
| Gasoline |  |  | 100 | 86 | 84 | 86 | 89 | % of 2000 |
| Final energy consumption | 833 | 880 | 945 | 1010 | 1049 | 1085 | 1121 | PJ |
| Steel production activity | 3922 | 4529 | 5366 | 5502 | 5502 | 5447 | 5393 | 1000 tons |
| Energy efficiency | +18% |  |  | +10% |  |  |  | % |
| Cattle | 2584 | 2326 | 2156 | 2092 | 2010 | 1941 | 1875 | 1000 head |
| Municipal waste deposition in landfills | 4115 | 3529 | 3640 | 3463 | 2700 | 2147 | 1783 | 1000 tons |

Note: Input parameters to the models are in bold

- Country Conclusions

The main documents used for this summary were the Third National Communication and the Austrian Climate Strategy. The level of detail and clarity of the documents from Austria is rather good. Details of the methodology for the model-based projections are available and contain a reasonable level of detail on the models and parameters. The policies and measures are summarised in a Table, which makes clear the potential greenhouse gas savings.

The projected increase from the base year with measures implemented and adopted is +8.7% in the basket of six greenhouse gases by 2010. Additional policies and measures are projected to reduce GHG emissions to 9.1% below base year levels by 2010. This reduction is still not enough in order to reach the 13% reduction agreed under the Burden Sharing. The remaining GHG reduction requirement of around 3 Mt CO2 equivalents will be fulfilled by making use of the project-based flexible mechanisms of the Kyoto Protocol (JI/CDM).

The projections in the Austrian Climate Strategy show that transport related emissions are projected to increase by 62% (with measures) and 32% (with additional measures). The largest decreases are projected for energy demand from small combustion with –28% in the with additional measures scenario. The emissions from agriculture and waste reduce in both, the trend 2010 scenario and the scenario with additional measures.

Updated projections reported in 2006 show that transport related emissions are projected to increase by 71% (with measures) and 27% (with additional measures). The largest decreases are projected for waste with -38% with measures and with additional measures.

AUSTRIA - Priority indicators (see Chapters A5.2 and A5.4 for more information on indicators)

AUSTRIA – Reported indicators

Priority indicators

Additional priority indicators

Supplementary indicators