FRANCE

Overview	2

Sources of Information	3

Quality and Transparency of Reporting	4

Evaluation of Projections	28

Changes compared to last year	33

Indicators	34

| FRANCE Share in total EU-15 GHG emissions 2004 13 % Emissions base year 567.1 Mt Emissions 2004 562.6 Mt Emissions base year (for projections) 564.0 Mt Projections 2010 with existing measures 600.0 Mt Projections 2010 with additional measures 567.0 Mt Kyoto target (absolute, based on latest inventory) 567.1 Mt Kyoto target (% from base year) 0 % Change base year to 2004 – 0.8 % Change 2003–04 + 0.3 % Change base year to 2010 with existing measures + 6.4 % Change base year to 2010 with additional measures + 0.5 % Distance to linear target path 2004 -1.2 (– 0.8) percentage pts. Use of Kyoto mechanisms n.a. Sinks (Articles 3.3. and 3.4) 3.23 Mt Emissions in 1990 (Article 3.7) n.a. |  | Note: ‘Target with Kyoto mechanisms’ is calculated by combining the agreed burden-sharing target with sinks according to Article 3.4. | Past emissions: France’s GHG emissions were 0.3 % above those of 2003 and 0.8 % below base-year levels in 2004. Main factors for increasing emissions with regard to the previous year were an increase in fossil fuel combustion in households and manufacturing industries. Between 1990 and 2004, road transport was by far the largest contributor to emission increases, followed by consumption of halocarbons mainly in refrigeration and air conditioning. These increases were offset by, among others, reduction measures in adipic acid production. Emission projections: Emissions in 2004 were 7 percentage points below the level projected in the ‘with measures’ scenario for 2010. France will achieve its Kyoto target with additional domestic measures. France plans to make use of carbon sinks according to Article 3.4. of about 3.2 Mt. No information on the potential use of Kyoto mechanisms has been provided so far. |
| --- | --- | --- | --- |

# France

Please note that the projections relating to the NAP II are excluded from all the EU Country Profiles as they were not available in time to be assessed for the 2006 Country Profiles.

- Sources of Information

French 4th National Communication to the United Nations Framework Convention on Climate Change; rapport de la France au titre du paragraphe 2 de l’article 3 de la décision n°28

0/2004/CE du Parlement européen et du Conseil du 11 février 2004.

Overview of CCPM implementation in MS

Source: MS responses to the CCPMs questionnaire in 2005

| CCPM | FR |
| --- | --- |
| Taxation of energy products 2003/96/EC | B |
| Emissions trading 2003/87/EC | N |
| Promotion of electricity from RE sources 2001/77/EC | R |
| Promotion of cogeneration 2004/8/EC | B |
| Directives on energy labelling of appliances | N |
| Motor challenge, voluntary EC programme |  |
| Energy performance of buildings 2002/91/EC | N |
| Eco-management & audit scheme (EMAS) EC 761/2001 | N |
| Efficiency of hot water boilers 92/42/EEC | N |
| Transport modal shift to rail 2001/12/EC etc. | B |
| Promotion of biofuels for transport 2003/30/EC | R |
| Consumer information on cars 1999/94/EC | N |
| Agreement with car manufacturers ACEA etc. | R |
| Support under CAP (1782/2003) | R |
| Support under CAP - ammendment (1783/2003) | R |
| Landfill directive 1999/31/EC | B |
| CODE |  |
| New National PAM implemented after CCPM was adopted | N |
| Existing National PAM re-enforced by CCPM | R |
| National PAM already in force before CCPM was adopted | B |
| Not reported |  |

- Quality and Transparency of Reporting

The 4th NC provides information on policies and measures in the different sectors and gives quantified effects for most existing and additional measures for 2010. Aggregate effects by sector are estimated for 2020 as well. Both with measures and with additional measures projections are provided disaggregated by gas and sector but sometimes at different level of aggregation depending on the gas. The policies in the two scenarios are specified.

Table 1:	Information Provided on Policies and Measures

| Information provided | Level provided | Comments |
| --- | --- | --- |
| Policy names | +++ | Policies are distinguished by short names. |
| Objectives of policies | +++ | In most cases a good description of the objectives. |
| Which GHGs? | +++ | All |
| Status of Implementation | +++ |  |
| Implementation body specified | ++ | The ministries and other bodies in charge are mentioned. |
| Quantitative assessment of implementation | ++ | Effects for most measures quantified for 2010. For 2020 only aggregate figures by sector are provided. |
| Interaction with other P&Ms discussed | + | Partly addressed. |

+, ++, +++ level of information available increases as the number of + signs increases

Table 2:	Information provided on Projections

| Category of Information | Level of Information Provided | Comments |
| --- | --- | --- |
| Scenarios considered | +++ | Distinction is made between the following scenarios Without measures With existing measures (measures that have been adopted before 1 July 2004) With additional measures (measures included in Climate Plan or adopted before 1 October 2005) With additional measures+ (technical potential of emission reductions) |
| Expressed relative to inventory for previous years | +++ | Absolute figures. |
| Starting year | 2001 |  |
| Split of projections | ++ | Total figures and detailed projections for most but not all sources; different disaggregation depending on GHG |
| Presentation of results | +++ | Tables and figures given |
| Description of model (level of detail, approach and assumptions) | ++ | The use of the different models is described in principle. |
| Discussion of uncertainty | ++ | Short discussion of sensitivity analysis. |
| Details of parameters and assumptions | ++ | The main assumptions are stated in the Fourth National Communication. |

+, ++, +++ level of information available increases as the number of + signs increasesAssessment of Policies and Measures

Table 3 gives an overview of the effects of national policies and measures.

Table 3:	Summary of the Effect of Policies and Measures Included in the 2010 Projections (Mt CO2)

|  | With measures | With additional measures |
| --- | --- | --- |
| Energy (total, excluding transport) | 4,6 | 15,8 |
| Electricity Production | 3,0 | 11,0 |
| District Heating | 0,0 | 0,0 |
| Refineries | 0,0 | 0,0 |
| Fuel transformation | 0,0 | 0,0 |
| Residential | 2,0 | 3,0 |
| Industry | 0,0 | 1,0 |
| Not specified | -0,4 | 0,8 |
| Transport (energy) | 18,0 | 10,0 |
| Road | 18,0 | 10,0 |
| Aviation | 0,0 | 0,0 |
| Water | 0,0 | 0,0 |
| Rail | 0,0 | 0,0 |
| Industry | 38,1 | 6,2 |
| Solvents | 0,0 | 0,0 |
| Agriculture | -0,1 | 0,7 |
| Waste | 14,4 | 2,3 |
| Solid Waste disposal | 14,4 | 1,7 |
| Incineration | 0,0 | 0,6 |
| Other | 0,0 | 0,0 |
| Total (excluding sinks) | 75,0 | 35,0 |

Source: Fourth National Communication

Table 4:	Detailed Information on Polices and Measures

Source: European Climate Change Programme (ECCP), Database on Policies and Measures in Europe

http://www.oeko.de/service/pam/index.php

| Member State | Sector | Name | Category | Type | GHG | Status | Absolute Reduction [kt (Gg) CO2 eq. p.a.] |  |  | Reduction in trading sector | Costs [EUR/t] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 2005 | 2010 | 2020 |  |  |
| France | ENS | Agreements Negotiated (Nuclear Fuel Cycle and Losses from Electrical Power Cables) | Nuclear | voluntary/ negotiated agreement | CO2 |  |  |  |  |  |  |
| France | ENS | Development of Renewable Energies | Renewables | Fiscal; Economic; Regulatory | CO2 |  |  |  |  |  |  |
| France | WAM | policies and measures in the with measure projection |  |  |  |  |  | 28300 |  |  |  |
| France | AGR | Improvement of fertilizer application techniques | N management (organic and inorganic) | Regulatory; Information | N2O | implemented |  |  |  | no effect |  |
| France | AGR | Water protection legislation against nitrate | N management (organic and inorganic) | Information | N2O |  |  |  |  | no effect |  |
| France | AGR | Development of the Usage of Timber in Construction |  | Information | CO2 | implemented |  | 2600 |  |  |  |
| France | ENC | Thermal Regulation | Buildings - energy efficiency | Regulatory | CO2 | adopted | 900 | 1500 | 2400 | low |  |
| France | ENC | Display of Energy Consumption in Building | Energy efficiency | Regulatory | CO2 | planned |  |  |  | no effect |  |
| France | ENC | Financial and Tax-Related Incentives for Work on Existing Buildings | Buildings - energy efficiency | Fiscal; Economic | CO2 | adopted | 2200 | 3700 | 6600 | low |  |
| France | ENC | Actions on State-owned Buildings | Energy efficiency - Public | Regulatory | CO2 | implemented | 700 | 700 | 700 | low |  |
| France | ENC | Reduced rate of VAT for Improvement of Old Buildings Classification of District | Buildings - energy efficiency | Fiscal | CO2 | planned |  |  |  | low |  |
| France | ENC | Classification of District Heating Systems | Buildings - heat supply | Regulatory | CO2 | planned |  |  |  | no effect |  |
| France | ENC | Improvement of Existing Housing. Institutional Actions Concerning Road Freight | Energy efficiency - Public | Economic | CO2 | implemented |  |  |  |  |  |
| France | ENC | Reinforcement of Regulation on Heating (Recent Buildings) | Buildings - heat supply | Regulatory | CO2 | adopted |  | 1100 |  | low |  |
| France | ENC | Technical Standards and Regulations on Components | Buildings - energy efficiency | Regulatory | CO2 | adopted |  | 850 |  | low |  |
| France | ENC | Audit of Buildings in the Existing Services Market (for sale or for rent) | Energy efficiency - Companies | Regulatory | CO2 | implemented |  |  |  |  |  |
| France | ENC | Strengthening of Inspection Tools and Procedures | Buildings - energy efficiency | Regulatory | CO2 | implemented |  |  |  | low |  |
| France | ENC | Voluntary Agreements | Energy efficiency - Industry; Energy efficiency | voluntary/ negotiated agreement | CO2 | adopted |  |  |  | moderate |  |
| France | ENC | Actions on Pilot Sector Buildings | Energy efficiency - Companies | Information | CO2 | implemented |  |  |  |  |  |
| France | ENC | Solar Heat Energy | Buildings - heat supply | Economic | CO2 | implemented |  | 40 |  | moderate |  |
| France | ENC | The Rental Market | Buildings - energy efficiency | Regulatory | CO2 | planned |  |  |  | low |  |
| France | ENC | Allocation of Heating Costs (condominiums and collective services) | Buildings - heat supply | Regulatory | CO2 | planned |  |  |  |  |  |
| France | ENC | Planned Initiatives for Improving Heating in Buildings | Buildings - energy efficiency | voluntary/ negotiated agreement | CO2 | adopted |  |  |  | low |  |
| France | ENC | Conditions for Receiving Grants for Real Estate Purchase | Buildings - energy efficiency | Regulatory; Economic | CO2 | implemented |  |  |  | low |  |
| France | ENC | Grant System for Services Buildings | Energy efficiency - Companies | Economic | CO2 | planned |  |  |  | low |  |
| France | ENC | Grants for Condensation Boilers for Collective Use | Buildings - heat supply | Economic | CO2 | adopted |  | 140 |  | no effect |  |
| France | ENC | Support for Quality Label Policy | Energy efficiency | Economic | CO2 | implemented |  |  |  |  |  |
| France | ENC | Reduced rate of VAT, Energy-Saving Products and Services | Energy efficiency | Fiscal | CO2 | planned |  |  |  |  |  |
| France | ENC | Quality Labels and Public Awareness | Energy efficiency | Information | CO2; HFCs | implemented |  |  |  |  |  |
| France | ENC | Environmental Quality of Products for Construction | Appliances | Information; education | CO2; HFCs | implemented |  |  |  |  |  |
| France | ENC | Reduction of the Negative Effects of the Standardisation of Electricity Rates | Appliances | Economic | CO2 | implemented |  |  |  | complete |  |
| France | ENC | Promotion of a European Regulation to Improve Electrical Appliances Intended for Public Sale | Energy efficiency | Regulatory; Information; voluntary/ negotiated agreement | CO2 | adopted |  | 1300 |  | complete |  |
| France | ENC; ENS | Grants for reduction of emissions from Industry; Promotion of Energy-Efficient Appliances | energy efficiency; Energy efficiency | Economic; Information | CO2 | implemented |  | cl | cl | cl; complete |  |
| France | ENC | Specific Regulation on Electricity and Heating | Energy efficiency | Regulatory | CO2 | adopted |  |  |  | complete |  |
| France | ENC | Reduced rate of VAT on Products and Services | Energy efficiency | Fiscal | CO2 | implemented; planned |  | 900 |  |  |  |
| France | ENC | Wood power programme for collective and individual use | Renewables | Research; Information; Economic | CO2 | implemented |  | 300 |  | no effect |  |
| France | ENC | Tax on Energy (TGAP) | Energy Efficiency | Fiscal | CO2 | expired |  | 7300 |  | high |  |
| France | ENC | Training and Qualification | Energy Efficiency | education | CH4; N2O; HFCs; PFCs; SF6; CO2 | implemented |  |  |  | moderate |  |
| France | ENS | Consumption in the Nuclear Fuel Cycle | Energy efficiency | Economic | CO2 | adopted |  |  |  | complete |  |
| France | ENS | Losses in Electricity Lines | Energy efficiency | Economic | CO2 | adopted |  |  |  | complete |  |
| France | ENS | Substituting Traditional Power Stations with Gas Combined Cycles (GCC) and Co-generation | Fuel switch; CHP | Economic; Fiscal | CO2 | implemented |  | 5000 |  | complete | 21-62 |
| France | ENS | Support for the Production of Wind Energy Electricity | Renewables | Economic | CO2 | implemented |  | 2600 |  | complete |  |
| France | ENS | Wood heating instead of of electric heating in rural areas | Renewables | Economic; Information | CO2 | implemented |  |  |  |  |  |
| France | ENS | Wood Energy for Collective Purposes, District Heating Systems | Renewables | Economic | CO2 | implemented |  | 1100 |  |  |  |
| France | ENS | Production of Electricity Using Wood | Renewables | Research; Economic | CO2 | implemented |  |  |  | complete |  |
| France | ENS | Programme on renewable energies for Overseas Territories and Departments and Corsica | Renewables | Economic | CO2 | implemented |  | 450 |  |  |  |
| France | ENS |  | Renewables |  |  |  |  |  |  |  |  |
| France | ENS | Use of Forest Products to Provide Power | Renewables | planning | CO2 | implemented |  |  |  |  |  |
| France | WAM | Spread the recuperation of heat in incinerators | waste incineration | Regulatory | CO2 | implemented |  |  |  | no effect |  |
| France | ENS | Geothermal Energy | Renewables | Economic | CO2 | planned |  | 70 |  |  |  |
| France | ENS | Heat Grids | Renewables | Economic | CO2 | implemented |  |  |  |  |  |
| France | ENS | Reduced rate of VAT on the Sale of Heat Produced by an ENR | Renewables | Fiscal | CO2 | planned |  |  |  | moderate |  |
| France | ENS | Nuclear Investments; Development of Co-generation | Nuclear; Renewables | Economic; Regulatory | CO2 | implemented | 4600 | 4600; 3700 | 4600; 4400 |  |  |
| France | ENS | Reduction of Peaks on Load Curve | Energy efficiency | Regulatory; Economic | CO2 | implemented |  | 2350 |  | moderate |  |
| France | ENS | Leakage from Natural Gas Grids | Methane abatement; gas distribution and transmission network | voluntary/ negotiated agreement | CH4 | implemented | 540 | 640 | 640 |  |  |
| France | ENS | Development of Wind Energy | Renewables | Regulatory | CO2 | implemented |  | 700 |  |  |  |
| France | ENS | Wood Energy | Renewables | Economic | CO2 | implemented | 200 | 200 | 200 |  |  |
| France | ENS | Bio-based Fuels | Renewables |  | CO2 | implemented |  |  |  |  |  |
| France | ENS | Increase in Incineration Capacity | Energy efficiency | Regulatory | CO2 | implemented |  |  |  |  |  |
| France | ENS | Regulation on industrial boiler output |  | Regulatory | CO2 | implemented |  | cl | cl | cl |  |
| France | IND | Voluntary Commitment Aluminum Industry | F-Gases | voluntary/ negotiated agreement | CO2; PFCs | implemented | cl | cl | cl | cl |  |
| France | IND | Voluntary Commitment Steel Industry | Steel Production | voluntary/ negotiated agreement | CO2 | implemented | cl | cl | cl | cl |  |
| France | IND | Voluntary Commitment Lime Industry | Lime Production | voluntary/ negotiated agreement | CO2 | implemented | cl | cl | cl | cl |  |
| France | IND | Voluntary Commitment Cement Industry | Cement Production | voluntary/ negotiated agreement | CO2 | implemented | cl | cl | cl | cl |  |
| France | IND | Voluntary Commitment Glass Industry | Glass Production | voluntary/ negotiated agreement | CO2 | implemented | cl | cl | cl | cl |  |
| France | IND | Industrial N2O emission regulation | N2O abatement | Regulatory | N2O | implemented | 22900 | 22900 | 22900 | no effect |  |
| France | IND | Generalised Tax on Polluting Activities | N2O abatement | Fiscal | N2O | implemented |  |  |  | no effect | 0,125 |
| France | IND | Increase in the Generalised Tax on Polluting Activities | N2O abatement | Fiscal | N2O | implemented |  | 1000 |  |  | 1,1 |
| France | IND | Help for ADEME/Frac funding decisions | Green investment | education | CO2; CH4; N2O; HFCs; PFCs; SF6 | implemented |  | cl |  | cl |  |
| France | IND | Research & development, 20 million French francs per year | Green investment | Research | CO2; CH4; N2O; HFCs; PFCs; SF6 | planned |  | cl |  | cl |  |
| France | IND | Technologic demonstration, 30 million French francs per year | Green investment | Research | CO2; CH4; N2O; HFCs; PFCs; SF6 | planned |  | cl |  | cl |  |
| France | IND | New financing methods | Green investment | Economic | CO2; CH4; N2O; HFCs; PFCs; SF6 | implemented |  | cl |  | cl |  |
| France | IND | Single procedures for ADEME and Frac procedures | Green investment | other | CO2; CH4; N2O; HFCs; PFCs; SF6 | implemented |  | cl |  | cl |  |
| France | IND | Strengthening of Regulatory Requirements within the Framework of the ICPE Legislation | N2O abatement | Regulatory | N2O | implemented | 1800 | 1800 |  | no effect |  |
| France | IND | PFCs (CF4) in the Production of Aluminium | F-Gases | Regulatory | PFCs | implemented |  | cl |  | cl | 9 |
| France | IND | SF6 and PFCs in the electronics Industry | F-Gases | Regulatory | PFCs; SF6 | implemented |  | cl |  | cl |  |
| France | IND | SF6 in Magnesium Foundries | F-Gases | Regulatory | SF6 | planned |  | cl |  | cl | 0,27 |
| France | IND | SF6 in Electrical Equipment | F-Gases | voluntary/ negotiated agreement | SF6 | planned |  | cl |  | cl | 18,7 |
| France | IND | HFCs in Foams, Aerosols and Fire-Extinguishing Materials | F-Gases | Regulatory | HFCs | planned |  | cl |  | cl |  |
| France | IND | Inspection requirements for HFC emissions | F-Gases | Regulatory | HFCs | planned |  | 700 |  | no effect |  |
| France | IND | Limiting Emissions from Air Conditioning Equipment in Motor Vehicles | F-Gases | education; Regulatory | HFCs | implemented |  | 700 |  | no effect |  |
| France | IND | Efforts toward Standardisation of piping systems | F-Gases | education | HFCs | implemented |  | 1100 |  | no effect |  |
| France | IND | Recovery of coolant of car air-conditioning | F-Gases | Economic | HFCs | implemented |  | 700 |  | no effect |  |
| France | IND | Training and Qualification of Companies Working on refrigeration or Air-Conditioning Equipment | F-Gases | Regulatory | HFCs | implemented |  | 600 |  | no effect |  |
| France | IND | Implementation of a tax on fluorinated gases | F-Gases | Fiscal | HFCs | planned |  | 1400 |  | no effect |  |
| France | IND | Research and Development | F-Gases | Research | HFCs | implemented |  |  |  | no effect |  |
| France | LUC | Afforestation of agricultural areas | afforestation | Economic | CO2 | planned |  | 2500 |  | no effect |  |
| France | TRA | T-0.2.1, T-3.2.2 Institutional actions concerning freight transport | freight - logistics and combined transport | Regulatory | CO2 | implemented |  |  |  |  |  |
| France | TRA | T-0.1.1, T-3.2.3 Catch up the tax rates on gas-oil and raise the minimum tax on fuels | taxation; change of modal split | Fiscal | CO2 | implemented |  |  |  |  |  |
| France | TRA | Technical measures concerning lorries | vehicle efficiency | Regulatory | CO2 | implemented | 1500 | 1500 | 1500 |  |  |
| France | TRA | EU strategy for reduced CO2 emissions from new cars (ACEA agreement) | vehicle efficiency | voluntary/ negotiated agreement | CO2 | implemented |  | 3000 |  |  |  |
| France | TRA | T-0.3.2 MOT of light-duty vehicles | emission control | Regulatory | CO2 | implemented |  | 3100 | 2900 |  |  |
| France | TRA | Renewal of car park | vehicle efficiency | Economic | CO2 | expired |  |  |  |  |  |
| France | TRA | T-0.3.3, T-1.4 Alternative vehicles | alternative fuels and engine technologies | Fiscal; other | CO2 | implemented |  | 1100 | 1800 |  |  |
| France | TRA | T-0.1.6, T-3.1.1, T-3.1.2, T-4.2, T-4.4 Urban transport | change of modal split | Regulatory | CO2 | implemented |  | 2600 | 400 |  |  |
| France | TRA | T-0.3.6 High-speed train networks | change of modal split - public transport | Economic | CO2 | implemented | 500 |  | 800 |  |  |
| France | TRA | Compliance with speed limits | road-traffic management | Regulatory; other | CO2 | implemented |  | 3000 |  |  |  |
| France | TRA | T-0.3.7 Specific emission reduction of air transport | air traffic | Regulatory; Economic | CO2 | implemented |  |  |  |  |  |
| France | TRA | T-1.1.2 Extension of voluntary agreements to light commercial vehicles and to two-wheeled vehicles | vehicle efficiency | voluntary/ negotiated agreement | CO2 | adopted |  |  |  |  |  |
| France | TRA | T-1.1.3 Incentive for the renewal of light vehicles | vehicle efficiency | Economic | CO2 | planned |  |  |  |  |  |
| France | TRA | Energy labels for cars | vehicle efficiency | Information | CO2 | planned |  | 200 |  |  |  |
| France | TRA | Improving land vehicles - clean vehicle programme | alternative fuels and engine technologies | other | CO2 | adopted |  | 400 |  |  |  |
| France | TRA | T-1.5 Specific emissions of rail transport | emission control | Economic | CO2 | implemented |  | 400 |  |  |  |
| France | TRA | T-1.6 N2O emissions of catalytic converters | emission control | other | N2O | planned |  |  |  |  |  |
| France | TRA | T-2.3.1 Management of the great interurban main roads | road-traffic management | other | CO2 | planned |  | 40 |  |  |  |
| France | TRA | T-2.3.2 Regulation of traffic lights and moderating progressive signal systems | road-traffic management | Regulatory; other | CO2 | planned |  | 300 |  |  |  |
| France | TRA | Development of collective urban transport systems | change of modal split - public transport | Economic | CO2 | planned |  | 200 |  |  |  |
| France | TRA | T-2.3.4 Regulation of urban fast lanes | road-traffic management | Regulatory | CO2 | planned |  | 200 |  |  |  |
| France | TRA | T-2.3.5 User information | road-traffic management | other | CO2 | implemented |  |  |  |  |  |
| France | TRA | Maritime highways - Facilitation of maritime coastal traffic | freight - logistics and combined transport | Economic | CO2 | implemented |  | 200 |  |  |  |
| France | TRA | T-3.1.2 Documents of town planning and localization of activities | traffic reduction | other | CO2 | adopted |  |  |  |  |  |
| France | TRA | T-3.1.3 Impact of the waste management system | change of modal split | other | CO2 | adopted |  |  |  |  |  |
| France | TRA | T-3.1.4 Combined transport and associate transporting firms | freight - logistics and combined transport | other | CO2 | adopted |  | 500 |  |  |  |
| France | TRA | T-3.2.1 Taxation of jet fuel | air traffic | Fiscal | CO2 | planned |  | 500 |  |  |  |
| France | TRA | T-3.2.2 Respect of work regulation in the road professions (follow-up of T-0.2.1) |  | Regulatory | CO2 | adopted |  | 500 |  |  |  |
| France | TRA | T-3.2.3 Catching up gas oil tax | taxation; change of modal split | Fiscal | CO2 | implemented |  | 2700 |  |  |  |
| France | TRA | T.3.3.1 Internalisation of carbon costs | taxation; change of modal split | Fiscal | CO2 | implemented |  | 3700 |  |  |  |
| France | TRA | T-3.3.2 Control of urban transport (reinforcement of T-0.1.5) | road pricing | Fiscal | CO2 | expired |  |  |  |  |  |
| France | TRA | T-4.2 Offer of interurban infrastructure - other aspects | change of modal split | other | CO2 | implemented |  | 3700 |  |  |  |
| France | TRA | Rail freight | freight - logistics and combined transport | other | CO2 | adopted |  |  |  |  |  |
| France | TRA | T-4.4 Common transport and alternative urban transport | change of modal split | other | CO2 | planned |  | 550 |  |  |  |
| France | TRA | T-5.1 Training of professional drivers | eco-driving | education | CO2 | adopted |  | 700 |  |  |  |
| France | TRA | Cooperate travel plans | traffic reduction | other | CO2 | adopted |  | 20 |  |  |  |
| France | TRA | Reduction of emissions due to air-conditioning and other auxiliary equipment | air-conditioning | Regulatory | HFCs | planned |  | 3500 |  |  |  |
| France | TRA | Implementation of biofuel Directive | biofuels | Economic | CO2 | implemented |  | 7000 |  |  |  |
| France | TRA | Energy labels for cars | vehicle efficiency | Information | CO2 | planned |  | 200 |  |  |  |
| France | TRA | Bonus/surcharge based on labelling | vehicle efficiency | Economic | CO2 |  |  | 1000 |  |  |  |
| France | WAM | Conversion of Waste to Energy | waste incineration | Regulatory | CO2; CH4 | implemented | 1300 | 1300 | 1300 | no effect |  |
| France | WAM | Ban on Dumping of Ordinary Waste | reduced disposal and stabilization of biodegradable waste on landfills | Regulatory | CH4 | implemented |  | 12200 | 15550 | no effect |  |
| France | WAM | Recovery of Methane at Dump site | methane recovery | Regulatory | CH4 | implemented |  |  |  | no effect |  |
| France | WAM | Control of Waste Production |  | Regulatory | CH4; CO2 | implemented |  | 4000 |  | no effect |  |
| France | WAM | Reinforcing Recycling of Materials or Organic Matter | waste prevention or recycling | Regulatory | CH4; CO2 | implemented |  |  |  | no effect |  |
| France | WAM | Efficiency of Gas Capture Systems in Dumps | methane recovery | Regulatory | CH4 | implemented |  |  |  | no effect |  |
| France | WAM | Biological Pre-Treamtment as an inhibitor for the period of exploitation | reduced disposal and stabilization of biodegradable waste on landfills | Regulatory | CH4 | adopted |  |  |  | no effect |  |
| France | WAM | Analysis and Control of Biochemical Reactions in Dumps | reduced disposal and stabilization of biodegradable waste on landfills | Regulatory | CH4 | adopted |  |  |  | no effect |  |
| France | WAM | Agronomic Recycling of Organic Waste | waste composting | Regulatory | CH4; CO2 | planned |  |  |  | no effect |  |

- Evaluation of Projections

The data in Tables 5-7 are based on information from the Fourth National Communication. The data presented in the document include the French overseas territories. The geographical coverage is different under the Kyoto Protocol but no detailed projections by sector are available for the Kyoto scope.

Table 5 shows the projections by greenhouse gas for 2010 and Table 6 summarises the projections by sector. The sectoral disaggregation for the energy sector in the fourth national communication does not add up to the totals provided. The difference has been included under ‘not specified’ to provide consistent figures. Total greenhouse gas emissions are projected to increase by 35 Mt CO2 eq. (+6.0 %) in the "With measures" projection (table 7).

Table 5:	Summary of projections by gas in 2010 (Mt CO2)

|  | Base year | no measures | with measures | with additional measures |
| --- | --- | --- | --- | --- |
| Carbon dioxide (excl. sinks) | 396,9 | 475,6 | 451,4 | 425,4 |
| Methane | 68,5 | 70,2 | 55,6 | 53,1 |
| Nitrous oxide | 93,1 | 99,4 | 73,5 | 70,6 |
| HFCs* | 3,6 | 23,8 | 20,2 | 16,8 |
| PFCs | 3,6 | 5,4 | 0,9 | 0,9 |
| SF6 | 2,3 | 3,6 | 1,4 | 1,2 |
| Total (excl. sinks) | 568,0 | 678,0 | 603,0 | 568,0 |
| % change relative to base year (excl. sinks) |  | 19,4% | 6,2% | 0,0% |

*Base year for F-gases is 1990
Source: Fourth National Communication

Table 6:	Summary of projections by sector in 2010 (Mt CO2)

|  | Base year | with measures | % change relative to 1990 | with additional measures | % change relative to 1990 |
| --- | --- | --- | --- | --- | --- |
| Energy (total, excl. Transport) | 263,6 | 290,7 | 10% | 274,9 | 4% |
| Electricity Production | 42,0 | 50,0 | 19% | 39,0 | -7% |
| District Heating | 6,0 | 8,0 | 33% | 8,0 | 33% |
| Refineries | 13,0 | 21,0 | 62% | 21,0 | 62% |
| Fuel transformation | 7,0 | 4,0 | -43% | 4,0 | -43% |
| Residential | 84,0 | 102,0 | 21% | 99,0 | 18% |
| Industry | 83,0 | 86,0 | 4% | 85,0 | 2% |
| Not specified | 28,6 | 19,7 | -31% | 18,9 | -34% |
| Transport | 120,1 | 150,7 | 25% | 140,7 | 17% |
| Road | 111,0 | 137,0 | 23% | 127,0 | 14% |
| Aviation | 4,5 | 5,9 | 31% | 5,9 | 31% |
| Water | 1,9 | 2,4 | 26% | 2,4 | 26% |
| Rail | 1,1 | 0,3 | -73% | 0,3 | -73% |
| Industry | 57,3 | 51,0 | -11% | 44,8 | -22% |
| Solvents | 1,8 | 1,0 | -44% | 1,0 | -44% |
| Agriculture | 107,7 | 92,5 | -14% | 91,8 | -15% |
| Waste | 16,0 | 15,8 | -1% | 13,5 | -16% |
| Solid Waste disposal | 12,1 | 9,7 | -20% | 8,0 | -34% |
| Incineration | 3,9 | 6,1 | 56% | 5,5 | 41% |
| Other | 1,5 | 1,3 | -13% | 1,3 | -13% |
| Total (excl. sinks) | 568,0 | 603,0 | 6% | 568,0 | 0% |

Source: Fourth National Communication

|  | Carbon Dioxide |  |  | Methane |  |  | Nitrous Oxide |  |  | F-gases |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Base year | With measures | With additional measures | Base year | With measures | With additional measures | Base year | With measures | With additional measures | Base year | With measures | With additional measures |
| Energy (total, excl. Transport) | 249,0 | 280,9 | 265,5 | 11,7 | 5,8 | 5,7 | 2,9 | 4,0 | 3,7 | 0,0 | 0,0 | 0,0 |
| Electricity Production | 42,0 | 50,0 | 39,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 |
| District Heating | 6,0 | 8,0 | 8,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 |
| Refineries | 13,0 | 21,0 | 21,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 |
| Fuel transformation | 7,0 | 4,0 | 4,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 |
| Residential | 84,0 | 102,0 | 99,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 |
| Industry | 83,0 | 86,0 | 85,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 |
| Not specified | 14,0 | 9,9 | 9,5 | 11,7 | 5,8 | 5,7 | 2,9 | 4,0 | 3,7 | 0,0 | 0,0 | 0,0 |
| Transport | 118,5 | 145,6 | 135,6 | 0,0 | 0,0 | 0,0 | 1,6 | 5,1 | 5,1 | 0,0 | 0,0 | 0,0 |
| Road | 111,0 | 137,0 | 127,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 |
| Aviation | 4,5 | 5,9 | 5,9 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 |
| Water | 1,9 | 2,4 | 2,4 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 |
| Rail | 1,1 | 0,3 | 0,3 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 |
| Industry | 23,7 | 17,8 | 17,8 | 0,0 | 0,0 | 0,0 | 24,1 | 10,7 | 8,1 | 9,5 | 22,5 | 18,9 |
| Solvents | 1,8 | 1,0 | 1,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 |
| Agriculture | 0,0 | 0,0 | 0,0 | 44,7 | 40,1 | 39,4 | 63,0 | 52,4 | 52,4 | 0,0 | 0,0 | 0,0 |
| Waste | 3,9 | 6,1 | 5,5 | 12,1 | 9,7 | 8,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 |
| Solid Waste disposal | 0,0 | 0,0 | 0,0 | 12,1 | 9,7 | 8,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 |
| Incineration | 3,9 | 6,1 | 5,5 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 |
| Other | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 1,5 | 1,3 | 1,3 | 0,0 | 0,0 | 0,0 |
| Total (excl. sinks) | 396,9 | 451,4 | 425,4 | 68,5 | 55,6 | 53,1 | 93,1 | 73,5 | 70,6 | 9,5 | 22,5 | 18,9 |

Source: Fourth National Communication

The "With measures" projections show that France is will not be able to reach its commitment of greenhouse gas stabilisation under the EU burden sharing agreement with the current policies and measures (Table 7). If additional measures are implemented and deliver as projected France expects to meet its target.

Table 7:	Assessment of the Target

|  | Excl. sinks |  |
| --- | --- | --- |
|  | MtCO2 equiv. | % of base year level (six gas basket) |
| Base year emissions (from projections) | 568,0 | 100% |
| Kyoto Commitment/burden sharing | 568,0 | 0,0% |
| With existing P&Ms projections | 603,0 | 106,2% |
| Gap (-ve means no gap) | 35,0 | 6,2% |
| With additional P&Ms projections | 568,0 | 100,0% |
| Remaining gap | 0,0 | 0,0% |

Source: Fourth National CommunicationDescription of Modelling Approach

The Fourth National Communication gives only some general explanations on the modelling approach applied. For the final energy demand by sector a technical-economic model has been used. For overseas departments historic energy demand was extrapolated. The effects of the EU emissions trading system are not included in the model but simulated based on different CO2 price assumptions. Starting year for the projections is 2001. For non-energy related projections are based on expert judgements and information from business associations. The parameters used in the projections are given in the Table below:

Table 8:	Modelling parameters

| Parameter | 2010 | 2020 | Unit |
| --- | --- | --- | --- |
| Population | 61.1 | 62.7 | Millions |
| GDP growth | 2.3 | 2.3 | %/yr |
| Oil price | 30 | 30 | USD (2003)/barrel |
| Average specific consumption of new light-duty vehicles | 84 | 72 | 1990 = 100 |
| Average specific consumption of truck park | 102 | 101 | 1990 = 100 |

Source: Fourth National CommunicationCountry Conclusions

The French fourth National Communication includes comprehensive information on policies and measures as well as projections. Compared to the information provided in the previous national communication the level of detail has increased and estimates for savings and total GHG emissions in 2020 included.

With the policies and measures included in the ‘with measures’ scenario France will not be able to meet its burden sharing commitment of stabilising 1990 emissions by 2010. The gap of 35 Mt CO2 eq. or about 6% of base year emissions is reduced to zero in the ‘with additional measures’ scenario. France has started to implement to institutional steps necessary for using the Kyoto mechanisms but not yet set a target or initiated tenders. For France to reach its target all additional policies and measures have to be adopted and deliver as estimated in the projections. This leaves no room for delays in the implementation of additional measures and requires swift action by the French government (Note: There is some uncertainty in the distance to target presented here as the geographical coverage of the projections is not consistent with scope of the French Kyoto commitment. An analysis based on the correct scope but with less sectoral detail is given in the main report and appendices).

- Changes compared to last year

In the third National Communication the gap between burden sharing commitment and the with measures scenario was estimated at 50 Mt CO2 e.g. In contrast in the with additional measures variant emissions were projected 9 Mt CO2 e.g. below 1990 levels in 2010. This is consistent with the assumption that at least some measures included in the with additional measures scenario in the 3rd NC have been adopted and included in the with measures scenario.

FRANCE - Priority indicators (see Chapters A5.2 and A5.4 for more information on indicators)