SNAP CODE:	1101

1102

1111

1112

SOURCE ACTIVITY TITLE:	Other Sources and Sinks

Non-managed deciduous forests

Non-managed coniferous forests

Managed deciduous forests

Managed coniferous forests

NOSE CODE:	11.01.04, 11.01.05, 301.01.(06-11), 301.01.(15-17)

301.02.(04-12), 301.02.(15-16)

301.11.(04-11), 301.11.(15-17)

301.12.(04-12), 301.12.(15-16)

NFR CODE:	11.C

# 1	Activities included

All types of foliar forest emissions will be considered, non-managed and managed, deciduous and coniferous. Forest foliage is primarily a source of volatile organic compounds (VOC), and we distinguish here between isoprene, monoterpenes, and ‘other VOC’. Emissions from forest soils are covered in the chapter dealing with activities 110117, 110216, 111117 and 111216. Note that for methane, the flux is believed to be from the atmosphere to the forest floor, so in any case a zero emission factor is recommended for this species. Emissions from forest fires are covered in Chapter 11.B Forest fires. Forest and other vegetation fires. Emissions from the forest undergrowth and root system have not yet been included, although may be added at a later stage. Emissions from shrub-like vegetation, maquis, garrique, or other vegetation types are covered in SNAP 1104, Natural grassland and other vegetation.

For emissions of greenhouse gases, users should refer to the appropriate guidelines developed by the Intergovernmental Panel on Climate Change (IPCC) www.ipcc-nggip.iges.or.jp/

# 2	Contributions to total emissions

According to Corinair90, forests (deciduous and coniferous) contributed 19 % of total European non-methane volatile organic compounds (NMVOC) emissions, 4.4 % of CH4 emissions, 14.3 % of N2O emissions and 0.8 % of NH3 emissions.

However, as noted below, the natural emission estimates for VOC as supplied for Corinair-1990 must now be regarded as outdated. Table 2–1 compares the more recent estimate of Simpson et al. (1995) and Guenther et al. (1995) with estimates of anthropogenic emissions.

Table 2–1: 	Comparison of estimated isoprene, OVOC, and monoterpene emissions from forests with man-made VOC. Units: ktonnes per year

| Country | Isoprene | OVOC | Monoterp. | Man-made |
| --- | --- | --- | --- | --- |
|  |  |  |  | VOC** |
| Albania | 6 | 9 | 16 | 30 |
| Austria | 32 | 78 | 30 | 418 |
| Belgium | 30 | 13 | 7 | 364 |
| Bulgarian | 135 | 44 | 41 | 178 |
| Czech Republic*** | 70 | 95 | 124 |  |
| Denmark | 7 | 7 | 4 | 167 |
| Finland | 82 | 354 | 398 | 209 |
| France | 480 | 216 | 215 | 2393 |
| Germany | 121 | 190 | 249 | 3154 |
| Greece | 21 | 35 | 62 | 293 |
| Hungary | 82 | 16 | 23 | 205 |
| Iceland | 0 | 0 | 7 | 6 |
| Ireland | 2 | 6 | 11 | 102 |
| Italy | 53 | 89 | 142 | 2080 |
| Luxembourg | 2 | 1 | 0.4 | 19 |
| Netherlands | 8 | 6 | 5 | 424 |
| Norway | 29 | 104 | 143 | 266 |
| Poland | 63 | 176 | 113 | 802 |
| Portugal | 36 | 61 | 70 | 202 |
| Romania | 154 | 83 | 55 | 567 |
| Spain | 137 | 248 | 272 | 1050 |
| Sweden | 108 | 389 | 370 | 528 |
| Switzerland | 5 | 17 | 30 | 284 |
| Turkey | 213 | 460 | 175 |  |
| Russia | 2006* | 3197* | 1060-3490[I] | 3566 |
| UK | 53 | 27 | 39 | 2287 |
| Sum | 4000 | 6000 | 3700-6100 | 20000 |

Notes:

- All isoprene and other VOC (OVOC) emissions are from Simpson et al., 1995.

- All monoterpene emissions (except Russia) are from Guenther et al., 1995, in ktonne carbon.

- * 1989 estimates were made for whole Soviet Union, however, Russia is expected to account for the majority of emissions.

- **Man-made emissions are unofficial estimates, generally derived by subtracting estimated Natural and Agricultural emissions from total emissions.

- *** Former Czechoslovakia.

- Other references: I - Isidorov, 1992, sum of pine+fir emissions.

These activities are not believed to be a significant source of PM2.5.

# 3	General

## 3.1	Description

The subject of emission inventories for emissions from vegetation is still very much in its infancy in Europe, and the design of an inventory procedure should reflect this. Indeed, NMVOC inventories prepared for the Corinair 1990 database have already been outdated by recent re-evaluations of the emission factors on which these have been based (Guenther et al., 1993, 1995, 1998, Simpson et al., 1995, Seufert et al., 1997). Generally, the mix of emissions varies greatly both in and between vegetation types, and knowledge of this mix is constantly being updated and in some cases completely revised.

This is especially true for NMVOC, as models are almost completely reliant on good land-use databases for their biogenic emissions estimates. Hopefully, the procedures suggested here will lead to a Europe-wide database, which will greatly improve emission estimates for model calculations and policy decisions.

Biogenic VOC is also a rather loose term for a wide range of compounds, of which only a few are generally of most interest. Isoprene is generally the compound of most importance for ozone modelling, for example, and it is particularly useful to inventory this compound. Emissions of the various terpenes may also be important, although there are great uncertainties associated with their atmospheric behaviour. Similarly, the remaining VOC species (‘other VOC’, or OVOC) doubtless play some role in atmospheric chemistry problems, but little is known about the chemistry of many components or the quantitative emissions of individual species. Emissions may be large, however.

A review of the sources and chemistry of biogenic VOCs has recently been given by Fehsenfeld et al., 1992. A special-issue Atmospheric Environment dealing with a large number of European measurements has recently been published (Seufert et al., 1997).

Emissions vary greatly from one tree species to another. And, as knowledge has increased, some species previously classified as non-isoprene emitters have actually now been found to emit isoprene in significant quantities. Conversely, oaks were previously thought to be always high isoprene emitters, whereas now it is recognised that some evergreen oaks emit little isoprene, but very high amounts of terpenes (Seufert et al., 1997).

These considerations have been reflected in the new SNAP codes adopted for this chapter, which assign codes to specific types of trees, rather than to ‘high-isoprene emitters’, etc.

Light and temperature controls on emissions

For all types of vegetation, an appropriate system describing the emissions flux on an hourly basis is that of Guenther et al. (1996):

Flux (g m-2 yr-1)   =       . D .    dt		(1)

where

	= average emission potential (g g-1 h-1) for any particular species,

D	= foliar biomass density (g dry weight foliage m-2), and

	= a unit less environmental correction factor representing the effects of short-term (e.g. hourly) temperature and solar radiation changes on emissions.

Guenther et al. (1991, 1993) showed that, to a very good approximation, the short-term (hourly) variations in emissions of isoprene could be described by the product of a light-dependent factor, CL,  and a temperature-dependant factor, CT. Thus, the so-called ISOG algorithm:

iso = CL . CT 						(2, ISOG)

The light factor, CL is given by:

(3)

where

 (= 0.0027), and

- (= 1.066) are empirical constants, and

- L is the PAR flux (µmol photons (400-700nm) m-2 s-1).

Temperature dependence  is described by:

CTiso=exp(CT1(T−TS)RTST)1+exp(CT2(T−TM)RTST)			(4)

where

R is the gas constant (= 8.314 J K-1 mol-1), and

- (= 95000 J mol-1),

- (= 230000 J mol-1), and

- TM (= 314 K) are empirical coefficients based upon measurements of three plant species: eucalyptus, aspen, and velvet bean, but which seem to be valid for a variety of different plant species (Guenther et al. 1993, Guenther 1997);

- TS (= 303 K) is the standard temperature.

The environmental correction factor for monoterpene emissions from most plants are parameterised using the following equation (Guenther et al. 1993):

mts = exp (ß  (T-TS)) 					(5a, MTS)

where

ß (= 0.09 K-1) is an empirical coefficient based on non-linear regression analysis of numerous measurements present in the literature. This type of emission is associated with vaporisation of terpenes from stores within the plant tissue, and this algorithm is referred to here as MTS.

Recently, it was shown that some evergreen oaks, and also Norway spruce, show a light-dependency of monoterpene emissions. At least for Q. ilex this dependency seems to be well described by the Guenther isoprene algorithms (Kesselmeier et al., 1996, Seufert et al., 1997). Denoting this behaviour by MTL, we have:

mtl = iso								(5b, MTL)

These emission algorithms represent our current knowledge of terpenoid emission by plants. These algorithms will likely need to be revised in the future, when a better biological understanding of the biosynthesis and emission of terpenoids is available, since there remains an uncertainty in the resulting emission estimates that is about a factor of three or more. This variation is mainly due to the (1) differences in the emissions from branch-to-branch and from tree-to-tree, (2) variation with season, (3) nutrient condition of the plant, (4) stress and (5) experimental errors. Suggestions for improved algorithms have been made by Schuh et al. (1997) and Schnitzler et al. (1997), but the generality of these suggestions need further testing before we can recommend a change from the basic Guenther algorithms.

The relationship between environmental conditions and emission of OVOC is even less understood than isoprene and monoterpenes. Emissions of some of these compounds, including a group of C6 unsaturates, are strongly influenced by external factors other than light and temperature, such as plant wounding by microbes, insects or mechanical stress. Given the lack of other information regarding the factors controlling oxygenated hydrocarbon emission, the use of equation (5) for parameterisation of oxygenated hydrocarbon emission is recommended (Guenther et al, 1994, Geron et al., 1994, König et al. 1995). i.e.:

ovoc = mts			(6)

These light and temperature dependencies are illustrated in Figures 3–1 and 3–2.

Figure 3–1:	Temperature dependency of isoprene (ISOG) and of terpene stores (MTS) emissions

Figure 3.2:	Light dependency for emissions of compounds, which are emitted as they are synthesised (e.g. isoprene)

In principal, calculation of annual emissions thus requires both temperature and radiation data over the whole year with appropriate spatial resolution. However, many simplifications are possible and the simpler methodology (section 4 of the present chapter) proposes a seasonal approach. The use of equations 1–6 above is covered in the detailed methodology (section 5 of the present chapter).

## 3.2	Definitions

Some relevant terms are explained below.

Forest — for the purposes of this Guidebook, the definition of forest should be as inclusive as possible. In theory, all trees should be included, although in practice other definitions of forest may be included in statistical definitions, e.g.:

United Nations Economic Commission for Europe (UNECE)/ Food and Agriculture Organization of the United Nations (FAO) Forest means land with tree crown cover of more than about 20 % of the area, with trees usually growing to more than about 7 m in height and able to produce wood. This includes both closed forest formations, where trees of various storeys and undergrowth cover a high proportion of the ground, and open forest formations, with a continuos grass layer in which tree synusia cover at least 10 % of the ground.

Branch-level — refers to emissions or measurements where the ambient radiation and temperature is an average over a whole branch, including both sun leaves and shade leaves.

Leaf-level — data refer to data appropriate to a single leaf. Leaf-level emission potentials are on average 1.75 times higher than branch-level rates because the latter are more shaded (Guenther et al., 1994). Emission potentials in this chapter are only given as branch level. (U.S. papers tend to give leaf-level, which requires modelling the shading within a forest canopy).

Coniferous — all trees classified botanically as Gymnospermae, generally referred to as softwoods or needle-leafed species.

Non-coniferous — all trees classified botanically as Angiospermae, generally referred to as hardwood or broad-leaved species. Note that such species can be either deciduous or evergreen.

Deciduous — all plants that shed leaves, usually in the autumn.

DW — dry weight of plants (used for emission rates), as opposed to fresh weight.

Foliar biomass densities — as used here give the mass of foliage per unit projected ground area, and must not be confused with total biomass densities, which have the same units (g m-2), but include wood mass.

OVOC — other volatile organic compound. Any non-methane VOC species other than isoprene or monoterpenes emitted by vegetation, including oxygenated VOC, but also non-oxygenated.

PAR — photosynthetically-active radiation, typically about 45–50 % of total global radiation, covering the wavelength range 400–700 nm.

## 3.3 	Techniques

## 3.4 	Emissions

Biogenic emissions consist of a wide variety of species. Attention has mainly focused on isoprene and the class of monoterpene compounds (alpha-pinene, beta-pinene, limonene, etc.). The remaining OVOC species consist of a large number of species including hydrocarbons and oxygenated compounds (alcohols, aldehydes, etc.), and have proven difficult to quantify in atmospheric samples. See section 9 of the present chapter.

## 3.5 	Controls

‘Control measures’ is not usually an applicable concept for forest emissions. However, it can be mentioned that much of the current forest cover in Europe is artificial, in the sense that the selection of species has been decided by human intervention. Thus, Sitka forest plantations in the U.K. represent an emissions increase over the coniferous forest, which they replaced, so control in terms of species selection could be envisaged. Such action to reduce ‘natural’ emissions has so far only been undertaken in California as far we are aware.

# 4 	Simpler methodology

All methodologies for calculating biogenic emissions essentially involve multiplying an emissions factor for a type of vegetation by a statistic giving the amount of vegetation in the country or grid square. Two major alternatives for this are (1) to perform these calculations at a genera or preferably species specific level (requiring for example separate statistics for Norway spruce, Douglas fir, etc.), or (2) to perform the calculations for different ecosystem types. In this latter method, each ecosystem is assumed to consist of a number of species, and the assigned emission rates attempt to give the average emissions from this category.

The rest of this chapter follows a species-orientated method (1) approach as far as possible. The main justification for this is that the recent European measurements have differed sufficiently from their American counterparts on an ecosystem basis where detailed species measurements should form the basis of the database where possible. Of course, data still does not exist for many vegetation types in Europe, in which case some ecosystem-assumptions are necessary anyway. These will be based as far as possible on knowledge of European species.

As noted in section 3 of the present chapter, an appropriate system describing the emissions flux on an hourly basis is that of Guenther et al. (1996):

Flux (g m-2 yr-1)   =       . D .    dt			(1)

where

 is the average emission potential (g g-1 h-1) for any particular species,

"D" is the foliar biomass density (g dry weight foliage m-2),

 is a unitless environmental correction factor representing the effects of short-term (e.g. hourly) temperature and solar radiation changes on emissions.

For isoprene emissions, and light-activated terpene emissions (so far only quantified for two forest species, Picea abies and Quercus ilex),  is a function of light and temperature, and is denoted -iso. Terpene and OVOC emissions from most vegetation types are simply dependent on temperature, in which case  is temperature-only dependant, and denoted -mt.

The simplified methodology consists of modifying equation (1) to be a seasonal rather than an hourly calculation.

F =  . D .  			(7)

Where  represents the integrated value of  over the growing season of the vegetation concerned.

Using meteorological data from the European Monitoring and Evaluation Programme (EMEP) MSC-W models, the integrated values, -iso and -mts, have been calculated for both six-monthly (May-October) and 12-monthly growing  seasons, as averages over each country. These have been calculated from hourly  values, and thus have units of hours. The  values are tabulated in Table 4–1. With this simplified methodology we could estimate, for example, the isoprene emissions from 1 km2 of deciduous oak (e.g. Q. robur) as simply:

emission  =  area x    . D . -iso

=  106 (m2)  x  60 (g g-1 h-1)  x   320 (g m-2)  x -iso (h)

For Austria, for example, Table 4–1 gives -iso for six-months as 452, therefore we have:

emission = 106 (m2)  x  60 (g g-1 h-1)  x   320 (g m-2) x 452 (h)  = 8.67 tonne km-2

Table 4–1: Country average values of integrated environmental correction factors, -iso and -mts for 6- and 12-month growing seasons (unit= hours)

|  | -mts = -ovoc |  | -iso |  |
| --- | --- | --- | --- | --- |
|  | 6-month | 12-month | 6-month | 12-month |
| Albania | 745 | 976 | 563 | 719 |
| Austria | 588 | 734 | 452 | 540 |
| Belarus | 753 | 895 | 581 | 684 |
| Belgium | 739 | 969 | 580 | 712 |
| Bosnia Herz. | 709 | 893 | 561 | 686 |
| Bulgaria | 824 | 1029 | 620 | 755 |
| Croatia | 883 | 1121 | 667 | 815 |
| Czech Republic | 712 | 885 | 533 | 633 |
| Denmark | 518 | 704 | 373 | 485 |
| Estonia | 565 | 669 | 422 | 491 |
| Finland | 458 | 523 | 339 | 379 |
| France | 840 | 1107 | 669 | 829 |
| Germany | 698 | 890 | 525 | 632 |
| Greece | 1076 | 1440 | 816 | 1057 |
| Hungary | 966 | 1188 | 730 | 874 |
| Ireland | 467 | 713 | 337 | 478 |
| Italy | 904 | 1208 | 711 | 902 |
| Latvia | 636 | 757 | 486 | 572 |
| Lithuania | 675 | 813 | 516 | 613 |
| Luxembourg | 786 | 1003 | 620 | 745 |
| Macedonia, F.Y.R. | 631 | 783 | 492 | 597 |
| Moldova, Rep. of | 858 | 1040 | 649 | 771 |
| Netherlands | 676 | 901 | 513 | 643 |
| Norway | 327 | 397 | 240 | 284 |
| Poland | 736 | 912 | 558 | 669 |
| Portugal | 1015 | 1388 | 853 | 1093 |
| Romania | 783 | 964 | 587 | 706 |
| Russia, Fed. | 808 | 917 | 637 | 717 |
| Slovakia | 797 | 977 | 607 | 724 |
| Slovenia | 745 | 940 | 562 | 682 |
| Spain | 982 | 1301 | 806 | 1004 |
| Sweden | 423 | 508 | 315 | 368 |
| Switzerland | 465 | 580 | 368 | 432 |
| Turkey | 976 | 1263 | 783 | 983 |
| United Kingdom | 493 | 720 | 358 | 492 |
| Ukraine | 856 | 1023 | 656 | 771 |
| Yugoslavia | 752 | 937 | 557 | 674 |

# 5	Detailed state-of-the-art methodology

The detailed methodology still relies on the basic equations (1–6) given above, but allows for the use of better input information and a more refined calculation if local meteorological data are available. We give details for calculations at either a monthly or hourly resolution.

## 5.1 	Monthly calculation

For the monthly calculation we make the following assumptions for the integration of the ISOG-type emissions:

1. 	the light-intensity variation given by equation 2 can be replaced by a simple step-function, where CL = 1 during most of the day and zero otherwise;

2.	the calculation of the temperature correction (Equasions 4, 5) need not be done every hour, but instead may be approximated by a monthly average daytime temperature;

3. 	ambient temperature and light-intensity provide a reasonable approximation to leaf-level light and temperature.

Approximation (1) is generally rather good, as light levels quickly reach 1 000 mol m-2 s-1 during the morning hours in most locations, even with moderate cloud cover. (200 mol m-2 s-1 is set as the cut-off for defining day length, as this corresponds to approximately CL = 0.5). Approximation (2) introduces larger errors, but only of order 20 % or so, which is much less than the uncertainties in the emission potentials. Approximation (3) has been tested by Simpson et al.(1995) and shown to introduce only moderate uncertainties for European conditions, again much less than those of the emission potentials.

The number of light-hours per day corresponding to the above definition can be calculated as a simple function of latitude and month, as seen in Table 5–1.

Table 5–1: Number of light-hours* per day (NL) as a function of latitude and month

| Lat | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug | Sep | Oct | Nov | Dec |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 80 | 0.0 | 0.0 | 0.0 | 13.1 | 24.0 | 24.0 | 24.0 | 15.2 | 6.1 | 0.0 | 0.0 | 0.0 |
| 78 | 0.0 | 0.0 | 4.4 | 12.9 | 20.5 | 24.0 | 24.0 | 14.6 | 7.2 | 0.0 | 0.0 | 0.0 |
| 76 | 0.0 | 0.0 | 5.8 | 12.7 | 18.6 | 24.0 | 20.2 | 14.1 | 7.9 | 0.0 | 0.0 | 0.0 |
| 74 | 0.0 | 0.0 | 6.8 | 12.6 | 17.5 | 20.9 | 18.6 | 13.8 | 8.5 | 0.0 | 0.0 | 0.0 |
| 72 | 0.0 | 0.0 | 7.4 | 12.5 | 16.7 | 19.1 | 17.6 | 13.6 | 8.9 | 0.0 | 0.0 | 0.0 |
| 70 | 0.0 | 0.0 | 7.9 | 12.4 | 16.1 | 18.0 | 16.8 | 13.4 | 9.2 | 2.5 | 0.0 | 0.0 |
| 68 | 0.0 | 1.6 | 8.4 | 12.3 | 15.6 | 17.2 | 16.2 | 13.2 | 9.4 | 4.2 | 0.0 | 0.0 |
| 66 | 0.0 | 3.6 | 8.7 | 12.2 | 15.2 | 16.6 | 15.8 | 13.0 | 9.6 | 5.2 | 0.0 | 0.0 |
| 64 | 0.0 | 4.7 | 8.9 | 12.2 | 14.9 | 16.1 | 15.4 | 12.9 | 9.8 | 5.9 | 0.0 | 0.0 |
| 62 | 0.0 | 5.4 | 9.1 | 12.1 | 14.6 | 15.7 | 15.0 | 12.8 | 9.9 | 6.4 | 1.5 | 0.0 |
| 60 | 2.4 | 6.1 | 9.4 | 12.1 | 14.3 | 15.4 | 14.7 | 12.7 | 10.1 | 6.9 | 3.3 | 0.0 |
| 58 | 3.7 | 6.6 | 9.5 | 12.0 | 14.1 | 15.0 | 14.4 | 12.6 | 10.2 | 7.3 | 4.3 | 2.2 |
| 56 | 4.6 | 7.0 | 9.7 | 12.0 | 13.9 | 14.7 | 14.2 | 12.5 | 10.3 | 7.7 | 5.1 | 3.5 |
| 54 | 5.3 | 7.3 | 9.8 | 11.9 | 13.7 | 14.5 | 14.0 | 12.4 | 10.4 | 7.9 | 5.7 | 4.4 |
| 52 | 5.8 | 7.7 | 9.9 | 11.9 | 13.5 | 14.2 | 13.8 | 12.3 | 10.4 | 8.2 | 6.2 | 5.1 |
| 50 | 6.3 | 7.9 | 10.0 | 11.9 | 13.4 | 14.0 | 13.6 | 12.2 | 10.5 | 8.4 | 6.6 | 5.7 |
| 48 | 6.7 | 8.2 | 10.1 | 11.8 | 13.2 | 13.8 | 13.4 | 12.2 | 10.6 | 8.6 | 7.0 | 6.2 |
| 46 | 7.1 | 8.4 | 10.2 | 11.8 | 13.1 | 13.6 | 13.3 | 12.1 | 10.6 | 8.8 | 7.3 | 6.6 |
| 44 | 7.4 | 8.6 | 10.2 | 11.8 | 12.9 | 13.5 | 13.1 | 12.1 | 10.6 | 9.0 | 7.6 | 6.9 |
| 42 | 7.7 | 8.8 | 10.3 | 11.7 | 12.8 | 13.3 | 13.0 | 12.0 | 10.7 | 9.1 | 7.9 | 7.3 |
| 40 | 7.9 | 9.0 | 10.4 | 11.7 | 12.7 | 13.1 | 12.9 | 11.9 | 10.7 | 9.3 | 8.1 | 7.6 |
| 38 | 8.2 | 9.1 | 10.4 | 11.6 | 12.6 | 13.0 | 12.8 | 11.9 | 10.8 | 9.4 | 8.4 | 7.8 |
| 36 | 8.4 | 9.3 | 10.5 | 11.6 | 12.5 | 12.9 | 12.6 | 11.8 | 10.8 | 9.6 | 8.6 | 8.1 |

Notes:

- Day-lengths (in hours) calculated for the 15th of each month from Latitudes 80 degrees N to 36 degrees N.

- *Period of light-hours defined for PAR> 200 mol m-2 s-1.

If we let mm1 and mm2 be the start and end of the growing season for a particular vegetation type , Nd (mm) be the number of days per month, NL (mm) be the number of light-hours per day (Table 5–1), and Tmm be the monthly mean temperature, for month ‘mm’, yearly emissions can be evaluated with:

Emis(isoprene) =  mm=mm1mm2A.D.giso(Tmm).Nd(mm).NL(mm)

Emissions of terpenes from species displaying MTL behaviour are also described by this equation.

For the yearly emissions of species displaying the MTS-type behaviour there is no light-dependency, and we perform the calculation for 24 hours per day:

Emis(monoterpenes) =  mm=mm1mm2A.D.gmt(Tmm).Nd(mm).24

Similarly,

Emis(OVOC) =  mm=mm1mm2A.D.govoc(Tmm).Nd(mm).24

## 5.2 	Hourly calculation

If desired, and appropriate meteorological data are available, the environmental correction factors () may be evaluated on an hourly basis using local surface temperature and sunlight conditions. The algorithms, temperature and light corrections, CT  and CL respectively, are as given in equations 1–7 above.

### Refinements:

Age distribution of forest

Isidorov et al. (1993) have pointed out that a land-use database built up with knowledge of not only the area, but also the age distribution within each region, can give a better description of the biomass densities. This approach requires more data, but helps to eliminate a potentially large area of uncertainty.

Seasonal variation

Foliar density varies markedly over the year, and this can be straightforwardly incorporated into the above calculations if data are available through the use of seasonal-dependent foliar biomass density.

Altitude temperature correction

Atmospheric temperature generally decreases with height at a rate of ca. 6 °C per km. Thus, data obtained from a meteorological station at a given height may be corrected to temperatures in another location (e.g. on a mountainside) before applying any of the detailed methodologies.

# 6	Relevant activity statistics

Vegetation coverage in terms of the vegetation types discussed in section 8 of the present chapter is required, together with foliar biomass estimates (D), and estimates of growing seasons. Commercial forestry at least is usually well documented. Other wooded land is a common category where definitions are more problematic.

For a good inventory it is actually most important to specify the correct foliar biomass density to accompany any given area of vegetation. This is because ‘area’ is an ill-defined quantity in many instances, e.g. 1 km2 of wooded area may include very dense forest with an average foliar biomass density of, say, 1 400 g m-2, or it may contain scattered trees with only 100 g m-2.

The new SNAP codes have been designed to encourage the use of data for each tree species separately for at least the most common trees. Very nice examples of this type of compilation are provided by Andreani-Aksoyoglu and Keller (1995) for Switzerland, and Ortiz and Dory (1990) for Spain, the latter tabulating area coverage and mean biomass factors for all 50 level III territorial units.

Categories such as mixed forest should be avoided, as this gives no information on species content. If species-specific data are not available, then genus-level data should be used. Only as a last resort should more general categories be supplied.

As pointed out by Veldt (1989), common vegetation names are often confusing, and care should be taken to provide Latin names of species as well as common names of all species. Translations of some common tree species names are included in Table 14–1, taken from EC (1996).

Foliar biomass densities

For the simpler methodology, seasonal average foliar biomass densities may be used. Default values are suggested below and in section 8 of the present chapter. These suggestions appear to fit a wide range of measurements quite well, but the variability of Mediterranean vegetation may cause some problems. For example, Ortiz & Dory (1990) mention a land-use class, Monte hueco, which consists of a mixture of species, with biomass densities as low as 100 g m-2. For coniferous forests, Veldt suggests densities of 700–1 400 g m-2 for different species < 60 ° N latitude, whereas Ortiz and Dory use 400 g m-2. Even further north, variations are great. Andreani-Aksoyoglu and Keller, 1995, quote a biomass factor for oak species of 530 g m-2. Some variations are systematic; Isidorov et al. (1993) points out that foliar biomass as a proportion of total tree biomass increases in harsher conditions and with age.

Therefore, it is strongly recommended that foliar biomass densities appropriate to the local vegetation are used. There may well be a factor of two or three difference to the default values.

Table 6–1: Default foliar biomass densities (adapted from Veldt, 1989)

|  |  | Foliar biomass |
| --- | --- | --- |
| Land Use Type |  | density, D |
|  |  | (g m-2) |
| Broadleaf: |  |  |
| Deciduous oaks |  | 320 |
| Birch (Betula) |  | 320 |
| Poplar, aspen (Populus) |  | 320 |
| Default deciduous, broadleaved |  | 300 |
| Evergreen, broadleaved |  | 500 |
| Conifers |  |  |
| Norway spruce (Picea abies) | > 60 N latitude | 800 |
|  | 55–60 N latitude | 1 400 |
|  | < 55 N latitude | 1 600 |
| Sitka spruce (Picea sitchensis) |  | 1 400 |
| Other spruce |  | 1 400 |
| Scots pine (Pinus sylvestris) | > 60 N latitude | 500 |
|  | < 60 N latitude | 700 |
| Other Pinus ssp. |  | 700 |
| (Fir) Abies ssp. |  | 1 400 |
| Douglas fir (Pseutotsuga menziessi) |  | 1 000 |
| Larch (Larix) |  | 300 |
| Other coniferous |  | 1 000 |

Comments on satellite data

Satellites provide a spatially comprehensive method of mapping vegetation with very high-resolution. Use of such data is encouraged, but a strong warning should be issued that ground-validation is essential if biogenic emissions are to be estimated. The apparent beauty and detail of a satellite image should not be mistaken for accuracy. Satellite data are easily misinterpreted (wrong species, problems with non-dominant vegetation, etc.) and even in the United States where biogenic emission inventories are very advanced, discrepancies of up to a factor of five are still found between satellite-derived isoprene emissions and ground-based determinations  (Lamb et al., 1997).

# 7 	Point source criteria

No point sources.

# 8 	Emission factors, quality codes and references

Emission potentials () are required separately for isoprene, monoterpenes and OVOC. Furthermore, for monoterpenes, two classes of behaviour are distinguished. For most trees, emissions are temperature-only dependant, controlled by the -mts environmental factor (equation 5a). For evergreen oaks, the MTL algorithm is used (equation 5b).

Emission potentials for a wide variety of species have recently been compiled by Guenther et al. (1994, 1997), Geron et al.(1994) for American woodlands, and by Steinbrecher (1997) and Seufert et al. (1997) for European species. Very little reliable experimental data on the emissions of OVOCs is available, and consequently Guenther et al. (1994) recommended the use of a uniform emission rate of 1.5 g g-1 h-1 for all tree species, recognising that this was a first order approximation to a ten-fold range (0.5–5 g g-1 h-1). The data of König et al.(1995) fall within this range, and so until further European data are available, 1.5 g g-1 h-1 also seems a reasonable choice for preliminary, first-order estimates of OVOC emissions in Europe.

The emission potentials are given in Table 8–1.

Table 8–1:	Standard emission potentials (g g-1 h-1 at 30 °C and PAR=1 000 mol m-2 s-1) for European trees

|  |  | Type | Foliar | Iso. | Terpenes |  | O- VOC | Additional refs. |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Common name | Latin name |  | biomass | -iso | -mtl | -mts | -ovoc | Iso. | Terp. |
| (example) |  |  | density, D |  |  |  |  |  |  |
|  |  |  | g m-2 |  |  |  |  |  |  |
| Fir | Abies | E | 1400 | 0 | 0 | 3 | 1.5 |  |  |
| Maple/sycamore*: | Acer | D | 320 | 0 | 0 | 3 | 1.5 | S93 | S93 |
| Common alder | Alnus | D | 320 | 0 | 0 | 1.5 | 1.5 | S93 | S93 |
| Birch | Betula | D | 320 | 0 | 0 | 0.2 | 1.5 | K | P,K |
| Hornbeam | Carpinus | D | 320 | 0 | 0 | 0.65 | 1.5 | K | K |
| Cedar | Cedrus | E | 700 | 0 | 0 | 1.5 | 1.5 |  |  |
| Orange | Citrus sp. | D | 320 | 0 | 0 | 1.5 | 1.5 |  |  |
| Italian cypress | Cupressus | D | 700 | 0 | 0 | 0.65 | 1.5 |  |  |
| Blue gum | Eucalyptus sp. | E | 400 | 20 | 0 | 3 | 1.5 | Str97b | H |
| European beech | Fagus | D | 320 | 0 | 0 | 0.65 | 1.5 | P,S93,K,Sh | K,Sh |
| Ash | Fraxinus | D | 320 | 0 | 0 | 0 | 1.5 | S93 | S93 |
| Walnut | Juglans | D | 320 | 0 | 0 | 3 | 1.5 |  |  |
| Common juniper | Juniperus | e | 700 | 0 | 0 | 0.65 | 1.5 | O | O |
| European larch | Larix | d,c | 300 | 0 | 0 | 1.5 | 1.5 | S93 | S93 |
| Olive | Olea | e | 200 | 0 | 0 | 0 | 1.5 |  |  |
| Date palm | Phoenix |  |  | 20 | 0 | 0 | 1.5 |  |  |
| Spruce | Picea sp. | e | Varies | 1 | 1.5 | 1.5 | 1.5 | As P.abies |  |
| Norway spruce | Picea abies | e | Varies | 1 | 1.5 | 1.5 | 1.5 | S94,Ke; | J,Ke, S94,LP |
|  | Picea omorika | e | Varies | 10 | 0 | 0.65 | 1.5 |  |  |
| Blue spruce | Picea pungens | e | Varies | 1 | 0 | 0.65 | 1.5 |  |  |
| Sitka spruce | Picea sitchensis | e | Varies | 6 | 0 | 3 | 1.5 | Str96,97b,Sm |  |
| Pines: | Pinus sp. | e | 700 | 0 | 0 | 3 | 1.5 |  | - |
| Aleppo pine | Pinus halepensis | e | 700 | 0 | 0 | 0.65 | 1.5 |  | H |
| Umbrella pine | Pinus pinea | e | 700 | 0 | 0 | 6 | 1.5 | Ks,Std,Str97a,Sf |  |
| Maritime pine | Pinus pinaster | e | 700 | 0 | 0 | 0.2 | 1.5 |  | Si |
| Scots pine | Pinus sylvestris | e | Varies | 0 | 0 | 1.5 | 1.5 |  | J |
| Pistachio | Pistacia sp. |  |  | 0 | 0 | 3 | 1.5 | H,Ha | H,Ha |
| Americ. sycamore* | Platanus | d | 320 | 34 | 0 | 0 | 1.5 |  |  |
| Poplar | Populus | d | 320 | 60 | 0 | 0 | 1.5 | H |  |
| Cherry#4 | Prunus | d | 300 | 0 | 0 | 0 | 1.5 |  |  |
| Douglas Fir | Pseudotsuga | e | 1000 | 0 | 0 | 1.5 | 1.5 |  | D |

Notes:

Isoprene and monoterpene emission potentials are taken from Guenther et al., 1994, 1997, or Geron et al., 1994, except where European measurements can provide a basis, as indicated by additional references. For terpenes, -mtl denote emissions controlled by light and temperature (using -mtl), whereas -mts denote emissions controlled by temperature only. All isoprene rates are branch-level, often derived from leaf-level U.S. estimates by division by 1.75.

| Oaks: | - |  |  | - | - | - |  | -; | - |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Default deciduous Oak#1 | - | d | 320 | 60 | 0 | 0.2 | 1.5 | Sf | Sf |
| Default evergreen Oak#2 | - | e | 500 | 0 | 20 | 0 | 1.5 | Sf | Sf, |
| Turkey oak | Quercus cerris | d | 320 | 0 | 0 | 1 | 1.5 | S97 | S97 |
| Kermes/holly oak | Quercus coccifera | e | 500 | 0 | 20 | 0 | 1.5 | SH | SH |
| Hungarian oak | Quercus frainetto | d | 320 | 100 | 0 | 0.2 | 1.5 | S97,Sf, | Sf |
| Holm oak | Quercus ilex | e | 500 | 0 | 20 | 0 | 1.5 | Be,Ks96,Str97,Sf |  |
| Sessile oak | Qercus petraea | d | 320 | 60 | 0 | 0.2 | 1.5 | K,S97,Str97b,Sf |  |
| Downy oak | Quercus pubescens | d | 320 | 60 | 0 | 0.2 | 1.5 | S97 | S97 |
| European oak#3 | Quercus robur | d | 320 | 60 | 0 | 0.2 | 1.5 | S93,I | S93 |
| Cork oak | Quercus suber | e | 500 | 0 | 0 | 0.2 | 1.5 | Sf | Sf |
| Locust | Robinia pseudoacacia | d | 320 | 10 | 0 | - | 1.5 |  |  |
| Willow | Salix | d | 150 | 34 | 0 | 0.2 | 1.5 | O | S93 |
| Saw-palmetto | Serenoa | d | 320 | 10 | 0 | 0 | 1.5 |  |  |
| Lime tree/basswood | Tilia | d | 320 | 0 | 0 | 0 | 1.5 |  |  |
| Elm | Ulmus | d | 320 | 0 | 0 | 0.2 | 1.5 |  |  |

Notes:

- Type gives evergreen (e),deciduous (d), or  (d,c) for Larix deciduous coniferous.

- #1 e.g. Q. rubra, Q.faginea, Q.lusitanica.

- #2 e.g. Q.rotundifolia, Q.callipiprinos,Q.ithhaburiensis, Q.coccifera.

- #3 also known as English oak, Pendunculate oak.

- #4 includes almond, apricot, blackthorn (sloe), peach.

- * Sycamore = Acer pseudoplatanus, not to be confused with the American sycamore, Platanus occidentalis.

- Refs:

Be Bertin et al, 1997; D. Duyzer, 1993; H. Hewitt, C.N. and Owen, S., personal communication; I. Isidorov et al., 1985; J. Janson, 1993; K. Koenig et al., 1995; Ks96,97 Kesselmeier et al., 1996, 1997; LP Lindskog and Potter, 1995; Ha Hanson et al., 1997; O Owen et al., 1997; P. Puxbaum, 1997; Sh Schuh et al., 1997; Si Simon et al., 1994; Sf Seufert et al., 1997; Sm Simpson et al., 1995; Std Staudt et al., 1997; S94 Steinbrecher, R., 1994; S93,97  Steinbrecher et al., 1993, 1997; Str96,97a,97b Street et al., 1996,1997a,1997b.

# 9	Species profiles

Emission () potentials have been given separately for isoprene, terpenes, and OVOC, and this division represents the most important level of speciation. However, there are many species represented within the class of terpenes and OVOC covering a wide range of chemical behaviour. This section attempts some guidance as to likely breakdowns among the monoterpene and OVOC classes.

Monoterpenes

Although many types of monoterpenes exist, most plants emit only two to three major species, with the reactive -pinene often dominating emissions from species such as Norway spruce and Scots pine (Janson, 1993). The ratio of one compound to another is very variable, both with season and temperature, so it is very difficult to specify the speciation in a quantitative way (Janson, 1993). In order to illustrate the major compounds, Table 9–1 compares the ratios of several monoterpenes to -pinene obtained from several studies. Table 9–2 groups a number of species in order of their relative frequency of emission.

Table 9–1: Relative composition of hydrocarbon-mix emitted by vegetation as reported by different authors, adapted from Duyzer (1993). Numbers in % are given relative to -pinene (-pinene is 100 %)

|  | Veldt:91 | Janson:93 | Janson:93 | Steinb:.93 | Simon:93 | Duyzer:93 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Average of several pines | Scots pine | Norway spruce | Norway spruce | Maritime pine | Douglas fir |
| -pinene | 40 | 33 | 5 | 17 | 105 | 40–100 |
| 3-carene | 30 | 111 | 6–800 |  | 50 | 30–80 |
| Limonene | 26 | 61 | 5–15 | 13 | 44 | 20–60 |

Table 9–2:	Examples of monoterpenes emitted by vegetation into the atmosphere (Zimmerman, 1979; Isidorov, 1985, as given by Guenther et al., 1994)

| Major | Frequent | Occasional |
| --- | --- | --- |
| 3 - Carene | Thujene | Fenchene |
| d-Limonene | Tricyclene | -Fenchene |
| Myrcene | Terpinolene | -Fenchene |
| -pinene | -Terpinene | -Fenchene |
| -pinene | -Terpinene | Bornylene |
| Sabinene | -Terpinene | Alloocimene |
| Camphene | p-Cymene | Methyl chavicol |
| 1,8-Cineole | -Phellandrene | p-Cymen-8-ol |
| -Phellandrene | trans-Ocimene | Linalool |
|  | cis-Ocimene | 2-Methyl-6-methylene-1,7-octadiene-3-one |
|  | 2-Carene | Pinocarvone |
|  |  | Verbenone |
|  |  | Fenchone |
|  |  | Thujone |
|  |  | Camphor |

OVOC

The identification and quantification of OVOC emissions from plants has proven one of the most difficult problems in evaluating total biogenic emissions. OVOC consists of a wide variety of compounds, many of which have been difficult to measure. Examples are alcohols, ketones, esters, ethers, aldehydes, alkenes and alkanes. Useful reviews can be found in Puxbaum (1997), Bode et al. (1997), Guenther et al. (1994) and Kotzias et al. (1997). The most extensive quantitative European data-set appears to be that of König et al. (1995); otherwise some screening studies are also available (Hewitt and Street, 1992, Steinbrecher, 1994, Isidorov, 1992, Goldstein et al., 1996, Arey et al., 1991a,b).

# 10	Uncertainty estimates

None of the biogenic emission inventories used in Europe can be compared in terms of complexity or accuracy with those generated in the U.S. All European methodologies have been severely limited by the availability of data on a European scale. Several key items are either missing or known to only a limited extent, necessitating some rather arbitrary choices.

Assessment of the uncertainties inherent in calculations of biogenic VOC emissions in Europe is rather difficult. As a starting point, estimates of the uncertainty of even recent U.S. inventories have suggested up to a factor of three for isoprene (Guenther et al., 1994). Furthermore, even though much progress has been made in emission potentials and  algorithms (Guenther et al., 1993, 1997, Seufert et al., 1997), awareness has grown of the large uncertainties associated with specifying land-cover for particular species. Even in the U.S., where land-use databases exist over the whole country in consistent format, uncertainties associated with specifying forest coverage are still significant (Guenther et al., 1994). In Europe, such uncertainties are very much greater because such coherent land-use data sets have not yet become available.

We discuss some of the important factors contributing to the total uncertainty of the European emission estimates below.

Emission potentials

Even with large campaigns such as BEMA (Seufert et al., 1997) emission factors for European species are very few and taken from a very limited set of conditions and samples. Genus-level potentials derived in the U.S. are often not appropriate for Europe because the species mix within a genus is often very different. It is clear that many more measurements are required before emissions in Europe can be described with any confidence, but meanwhile the first positive steps that can be taken are to collect good land-use data as a basis for any inventory.

Land-use data

The focus of most forest statistics appears to be the area of productive, coniferous forest, rather than the categories of most interest for biogenic inventories. Even for the coniferous forest category, definitions vary greatly; 1 km2 of coniferous forest appears to mean that 50 % of the stem-volume is coniferous in Finland and Norway, 70 % in Sweden, 80 % in Ireland, and 100 % in the U.K. (UNECE, 1985). Whichever definition is used, the aim should be to get the best description of foliar biomass for the area and tree species concerned.

Biomass data

Although the biomass data given in the simpler methodology can be used as default values if no other information is available, factor of two uncertainties can easily be introduced. Clearly the best solution is for each country to specify biomass densities appropriate to local conditions.

OVOC emissions

Guenther et al. (1993) noted that the recommended emission rate of 1.5 g g-1 h-1 is associated with a 10-fold range (0.5–5 g g-1 h-1) in possible emissions, and that even this may be an underestimation of some emissions.

Final remarks

It has been recognised that the minimum level of uncertainty in global biogenic emission estimates is a factor of three (Guenther et al., 1995), but this is likely to represent a lower limit for the accuracy of European emission estimates. Furthermore, this figure relates to estimates of annual emissions. Uncertainties for episodic calculations must obviously be substantially greater.

# 11	Weakest aspects/priority areas for improvement in current methodology

The emission factors and knowledge of land-use within each region are certainly the weakest aspects. The emission factors can only be improved with more measurements. The land-use problem is primarily one of data collection, as presumably forestry and agricultural Institutes hold quite detailed data for most countries. Collection of this land-use data is of the greatest priority.

# 12	Spatial disaggregation criteria for area sources

Follows from land-use and climate data.

# 13	Temporal disaggregation criteria

The details of hourly calculations are given in section 5 of the present chapter, Detailed state-of-the-art methodology.

It is worth noting that annual emissions of biogenic emissions give only a limited insight into the importance of these compounds. For assessing their impacts on photochemical ozone formation, it is the biogenic emissions during the warmest and sunniest days which are of interest. In practice, therefore, photochemical oxidant models all calculate their own biogenic emission rates internally, using short-period temperature and radiation data in conjunction with land-use data.

# 14	Additional comments

Recent developments and re-evaluations of previous methodologies have resulted in significant changes in the emission factors, which should be used in inventorying biogenic VOC emissions. This chapter has presented information on the new emission factors for a range of species derived from the latest American and European evaluations. In addition, much progress has been made in developing algorithms to describe the emission-temperature-sunlight relationships for isoprene, monoterpenes and other VOC. Still, these algorithms will certainly be changed in the future as knowledge of the underlying processes improves. Suggestions for modifications to include long-term (seasonal changes) to the emission potentials or other improvements have been presented by Guenther (1997), Schnitzler et al., 1997, and Schuh et al. (1997), although more work is needed to evaluate these algorithms before they can be recommended for the Guidebook.

Canopy models

It is possible to apply complex ‘canopy’ approaches in which forest canopy models are used to estimate levels of temperature and radiation at different heights within a canopy (e.g. Pierce and Waldruff, 1991, Lamb et al., 1993), and such an approach was tested in Simpson et al. (1995). Canopy approaches should be used together with ‘leaf-level’ emission factors, as opposed to the ‘branch-level’ factors given in section 8 of the present chapter. However, the difference in emissions estimates between a canopy model and simple use of branch-levels estimates is relatively small (up to 20 %). Given the much larger uncertainties in the emission potentials, uncertainties introduced by the forest-canopy model itself (e.g. in temperature profiles within the canopy), and the lack of evaluation of such models in European conditions, we do not recommend applying such a model for European emissions at this stage.

The emission factors given in section 8 of the present chapter are therefore exclusively for use where emission-canopy models are not used.

It should be noted that this section still presents a simpler methodology for calculating emissions than can be found in Guenther et al., 1995. We make no attempt to account for factors such as net primary production, leaf-area index, or vegetation index. No canopy radiative model is used. Such factors might improve the accuracy of the estimates somewhat, but until the basic emission factors for European vegetation are more firmly established, too much sophistication in the inventory procedure seems unnecessary. Groups having the data and resources to implement such methods are referred to Guenther et al., 1994, Guenther et al., 1995 or Geron et al., 1994.

Table 14–1: Generic names of tree species in different European languages

| Botanical Name | French | German | Greek | Italian |
| --- | --- | --- | --- | --- |
| Fagus sylvatica | Hêtre | Rotbuche | O  | Faggio |
| Quercus petraea | Chêne rouvre | Traubeneiche |   | Rovere |
| Quercus robur | Chêne pédonculé | Stieleiche |   | Farnia |
| Quercus ilex | Chêne vert | Steineche |  | Leccio |
| Quercus suber | Chêne liège | Korkeiche |  | Sughera |
| Pinus sylvestris | Pin sylvestre | Gemeine Kiefer |   | Pino silvestre |
| Pinus nigra | Pin noir | Schwarzkiefer | M  | Pino nero |
| Pinus pinaster | Pin maritime | Seestrandkiefer | O  | Pino marittimo |
| Pinus halepensis | Pin d’Alep | Aleppokiefer | X  | Pino d’Aleppo |
| Picea abies | Epicéa commun | Rotfichte | E  | Abete rosso |
| Picea sitchensis | Epicéa de Sitka | Sitkafichte | E | Picea di Sitka |
| Abies alba | Sapin pectiné | Weitanne |   | Abete bianco |
| Larix deciduas | Mélèze d’Europe | Europäische Lärche |   | Larice |

| Botanical Name | Portuguese | Russian | Spanish | Swedish |
| --- | --- | --- | --- | --- |
| Fagus sylvatica | Faia | áóê ëåñíîé | Haya | Bok |
| Quercus petraea | Carvalho branco Americano | äóá ñêàëúíûé | Roble albar | Bergek |
| Quercus robur | Carvalho roble | äóá ÷åðåùàòûé | Roble común | Ek |
| Quercus ilex | Azinheira | äóá êàìåííûé | Encina | Stenek |
| Quercus suber | Sobreiro | äóá ïðîáêîâûé | Alcornoque | Korkek |
| Pinus sylvestris | Pinheiro silvestre | ñîñíà îáûêíîâåííàÿ | Pino silvestre | Tall |
| Pinus nigra | Pinheiro Austriaco | ñîñíà ÷¸ðíàÿ | Pino laricio | Svarttall |
| Pinus pinaster | Pinheiro bravo | ñîñíà ïðèìîðñêàÿ | Pino negral | Terpentintall |
| Pinus halepensis | Pinheiro de alepo | ñîñíà àëåïïñêàÿ | Pino carrasco | Aleppotall |
| Picea abies | Picea | åëü åâðîïåéñêàÿ | Abeto rojo | Gran |
| Picea sitchensis | Picea de Sitka | åëü ñèòõèíñêàÿ | Picea de Sitka | Sitkagran |
| Abies alba | Abeto branco | ïèõòà áåëàÿ | Abeto común | Sivergran |
| Larix deciduas | Larício Europeu | ëèñòâåííèöà åâðîïåéñêàÿ | Alerce | Europeisklärk |

| Botanical Name | Danish | Dutch | English | Finnish |
| --- | --- | --- | --- | --- |
| Fagus sylvatica | Bøg | Beuk | Common beech | Pyökki |
| Quercus petraea | Vintereg | Wintereik | Sessile oak | Talvitammi |
| Quercus robur | Stilkeg | Zomereik | European oak | Metsätammi |
| Quercus ilex | Steneg | Steeneik | Holm oak | Rautatammi |
| Quercus suber | Korkeg | Kurkeik | Cork oak | Korkkitammi |
| Pinus sylvestris | Skovfyr | Grove den | Scots pine | Metsämänty |
| Pinus nigra | Østrisk fyr | Oostenrijkse/ Corsicaanse zwarte den | Corsican/Austrian black pine | Euroopanmusta-mänty |
| Pinus pinaster | Strandfyr | Zeeden | Maritime pine | Rannikkomänty |
| Pinus halepensis | Aleppofyr | Aleppoden | Aleppo pine | Aleponmänty |
| Picea abies | Rødgran | Fijnspar | Norway spruce | Metsäkuusi |
| Picea sitchensis | Sitkagran | Sitkaspar | Sitka spruce | Sitkankuusi |
| Abies alba | Ædelgran | Zilverden | Silver fir | Saksanpihta |
| Larix deciduas | Lærk | Europese lariks | European larch | Euroopanlehti-kuusi |

# 15	Supplementary documents

The American Biogenic Emission Inventory System (BEIS) has resulted in extensive lists of emission potentials. The latest published version is Geron et al. (1994). The updated BEIS-3 version is currently under preparation by Guenther et al. (1998). (Some of these rates have been already adopted in Table 8–1).

A qualitative list of isoprene and monoterpene emitting species is held at:

Hewitt, C. N., Street R.A. and Scholefield P.A. (1998):

Isoprene and monoterpene-Emitting Species Survey 1998.

www.es.lancs.ac.uk/es/people/pg/pas/download.html

# 16 	Verification procedures

If satellite data have been used in the land-use mapping process it is essential that these be independently verified by on-the-ground surveys. Large errors are possible in the identification of vegetation types and biomass from remote sensing methods.

In general, all of the emission potentials are built upon very few data. More measurements are required of at least the major sources, and several different measurement techniques need to be applied in order to eliminate the artefacts (usually enhanced emissions) easily generated by disturbances to the vegetation.

# 17 	References

Andreani-Aksoyoglu S. and Keller J., 1995. ‘Estimates of monoterpenes and isoprene emissions from the forests of Switzerland’, Journal of Atmospheric Chemistry, 20, pp. 71–87.

Arey J., Winer A., Atkinson R., Aschman S., Long W., Morrison C. and Olszyk D. (1991). ‘Terpenes emitted from agricultural species found in California's central valley’, Journal of Geophysical Research, 96D, pp. 9329–9336.

Arey J., Winer A. M., Atkinson R., Aschmann S. M., Long W. D. and Morrison C. L. (1991). ‘The emission of (z)-3-hexen-1-ol, (z)-3-hexenylacetate and other oxygenated hydrocarbons from agricultural plant species’, Atmospheric Environment 25A, pp. 1063–1075.

Bode K., Helas G. and Kesselmeier J. (1997). ‘Biogenic contribution to atmospheric organic acids’. In: Biogenic volatile organic compounds in the atmosphere, Helas G., Slanina J. and Steinbrecher R. (eds.), SPB Academic Publishing bv Amsterdam, pp. 157–170.

Bertin N., Staudt M., Hansen U., Seufert G., Ciccioli P., Foster P., Fugit J.L. and Torres, L. (1997). ‘Diurnal and seasonal course of monoterpene emissions from Quercus ilex (L.) under natural conditions — applications of light and temperature algorithms’, Atmospheric Environment, 31, S1, pp. 135–144.

Boissard C., Cao X. L., Street R. A., Shuttleworth S. M., Juan C.-Y., Duckham S. C., S. C. Hewitt S. C., Beverland I. J., ONeil D. H., Moncrieff J. B., Milne R. and Fowler D. (1996). ‘Quantification of Non-methane Hydrocarbon Emissions from Two Terrestrial Ecosystems in the UK’. In: Proceedings of EUROTRAC Symposium 96, Borrell P. M., Borrell P., Cvitas T., Kelly K. and Seiler W. (eds.), Computational Mechanics Publications, Southampton, pp. 163–167.

Corchno, S.B., Arey J. and Atkinson R. (1992). ‘Hydrocarbon emission from  twelve urban shade trees of the Los Angeles Basin’, Atmospheric Environment 3 (26B), pp. 339–348.

Duyzer J., 1993. Measurements of the emissions of monoterpenes from Douglas fir forest. Technical report IMW-R 93/312, TNO, Delft, NL.

EC, 1996. European programme for the intensive monitoring of forest ecosystems. General information on the permanent observation plots in Europe (level II). European Commission, DG VI, Brussels.

Evans R. C., Tingey D. T. and Gumpertz M. L. (1985). ‘Interspecies variation in terpenoid emissions from Engelmann and Sitka spruce seedlings’, Forest Science 31, pp. 132–142.

Evans R.C., Tingey D.T., Gumpertz M.L., and Burns W.F. (1982). ‘Estimates of isoprene and monoterpene emission rates in plants’, Botanical Gazette, 143, pp. 304–310.

Fehsenfeld F., Calvert J., Fall R., Goldan P., Guenther A.B., Hewitt C.N., Lamb B., Liu S., Trainer M., Westberg H. and Zimmerman P. (1992). ‚Emissions of volatile organic compounds from vegetation and the implications for atmospheric chemistry’, Global Biogeochemical Cycles, No 6, pp. 389–430.

Geron C., Guenther A. and Pierce T. (1994). ‘An improved model for estimating emissions of volatile organic compounds from forests in the Eastern United States’, Journal Geophysical Research, 99, 12773–12792.

Geron C.D., Pierce T.E. and Guenther A.B. (1995). ‘Reassessment of biogenic volatile organic compound emissions in the Atlanta area’, Atmospheric Environment, 29, No 13, pp. 1569–1578.

Goldstein A. H., Fan S. M., Goulden M. L., Munger J. W. and Wofsy S. C. (1996). ‘Emissions of ethene, propene, and 1-butene by a midlatitude forest’, Journal of Geophysical Research 101(D4), pp. 9149–9157.

Guenther A., Greenberg J., Helmig D., Klinger L., Vierling L., Zimmerman P, abd Geron C. (1996). ‘Leaf, branch, stand and landscape scale measurements of volatile organic compound fluxes from U.S. woodlands’, Tree Physiology, 16, pp. 17–24.

Guenther A.B., Monson R.K. and Fall R. (1991). ‘Isoprene and monoterpene rate variability: observations with Eucalyptus and emission rate algorithm development’, Journal Geophysical Research, 96, No D6, pp. 10799–10808.

Guenther A.B., Zimmerman P.R., Harley P.C., Monson R.K. and Fall R. (1993). ‘Isoprene and monoterpene rate variability: model evaluations and sensitivity analyses’, Journal of Geophysical Research, 98, No D7, pp. 12609–12617.

Guenther A., Zimmerman P. and Wildermuth M. (1994). ‘Natural volatile organic compound emission rate estimates for U.S. woodland landscapes’, Atmospheric Environment, 28, pp. 1197–1210.

Guenther A., Hewitt C.N., Erickson D., Fall R., Geron C., Graedel T., Harley P., Klinger L., Lerdau M., McKay W.A., Pierce T., Scholes R., Steinbrecher R., Tallamraju R., Taylor J. and Zimmerman P. , 1995. ‘A global model of natural volatile organic compound emissions’, Journal of Geophysical Research, 100, No D5, pp. 8873–8892.

Guenther A., Geron C., Pierce T., Lamb B., Harley P. and Fall R. (1998) (in preparation). Natural emissions of volatile organic compounds, carbon monoxide, and oxides of nitrogen from North America.

Hanson U. et al. (1997). ‘Biogenic emissions and CO2 gas exchange investigated on four Mediterranean shrubs’, Atmospheric Environment, 31, S1, pp. 157–166.

Hewitt C.N. and Street R.A. (1992). ‘A qualitative assessment of the emissions of non-methane hydrocarbon compounds from the biosphere to the atmosphere in the U.K.: Present knowledge and uncertainties’, Atmospheric Environment, 26A, No 17, pp. 3069–3077.

Hewitt C. N., Street R.A. and Scholefield P.A. (1997). Isoprene and Monoterpene-Emitting Species Survey 1997. www.es.lancs.ac.uk/es/people/pg/pas/download.html

Isidorov V.A., Zenkevich I.G. and Ioffe B.V. (1985). ‘Volatile organic compound in the atmosphere of forests’, Atmospheric Environment, Vol. 19, No 1, pp. 1–8.

Isidorov V.A. (1992). ‘Non-methane hydrocarbons in the atmosphere of boreal forests: Composition, emission rates, estimation of regional emission and photocatalytic transformation’, Ecological Bulletins, Vol. 42, pp. 71–76.

Isidorov V.A., Povarov V.G., Klokova E.M., Prilepsky E.B., Churilova, Yu.Yu., (1993). ‘Estimation of photochemically active VOC emission by forests of the European part of the Former USSR’. In: Proc. Sixth Europ. Symp.  Phys.Chem.Behav.Atmos.Pollut., Varese, Italy, 18–22.10.1993, Vol. 1, pp. 31–40.

Janson R. W. (1993). ‘Monoterpenes emissions from Scots pine and Norwegian spruce’, Journal of Geophysical Research, 98, No D2, pp. 2839–2850.

Kempf K., Allwine E., Westberg H., Claiborn C. and B. Lamb (1996). ‘Hydrocarbon emissions from spruce species using environmental chamber and branch enclosure methodes’, Atmospheric Environment 30(9), pp. 1381–1389.

Kesselmeier J., Schäfer L., Ciccioli P., Brancaleoni E., Cecinato A., Frattoni M., Foster P., Jacob V., Denis J., Fugit J.L., Dutaur L. and Torres L. (1996). ‘Emission of monoterpenes and isoprene from a Mediterranean oak species Quercus ilex L. measured within the BEMA (Biogenic emissions in the Mediterranean area) project’, Atmospheric Environment, 30, Nos 10/11, pp. 1841–1850.

Kesselmeier J. et al. (1997). ‘Emissions of short chained organic acids, aldehydes and monoterpenes from Quercus ilex L.  and Pinus pinea L. in relation to physiological activities, carbon budgets, and emission algorithms’, Atmospheric Environment, 31, S1, pp. 119–134.

König G., Brunda M., Puxbaum H., Hewitt C.N., Duckham S.C. and Rudolph J. (1995). ‘Relative contribution of oxygenated hydrocarbons to the total biogenic VOC emissions of selected mid-European agricultural and natural plant species’, Atmospheric Environment, 29, No 8, pp. 861–874.

Lamb B., Gay D., Westberg H. and Pierce T. (1993). ‘A biogenic hydrocarbon emission inventory for the U.S.A. using a simple forest canopy model’, Atmospheric Environment, 27, No 11, pp. 1673–1690.

Lamb B., Hopkins B., Westberg H. and Zimmerman P. (1997). Evaluation of biogenic emission estimates using ambient VOC concentrations in Western Washington. Presented at workshop on biogenic hydrocarbons in the atmospheric boundary layer, 24–27.8.1997, University of Virginia.

Lindskog A. and Potter A. (1995). ‘Terpene emissions and ozone stress’, Chemosphere, 30, No 6, pp. 1171–1181.

Lübkert B. and Schöpp W. (1989). A model to calculate natural VOC emissions from forests in Europe, International Institute for Applied Systems Analysis. Working paper WP-89-082.

Ortiz A. and Dory M.A.G. (1990). The estimation of non methane VOC emissions from nature in Spain for Corinair inventory. In: Pacyna J. M. and Joerss K.E., editors, EMEP Workshop on emission inventory techniques, Regensburg, Germany, 3–6.7.1990. Norwegian Institute for Air Research, Kjeller, Norway, EMEP/CCC-Report 7/90.

Owen S., Boissard C., Street R.A., Duckham C. Csiky O., Hewitt C.N. (1997). ‘The BEMA project: Screening of 18 Mediterranean plant species for volatile organic compound emissions’, Atmospheric Environment, 31, No S1, pp. 101–118.

Pierce T.E. and Waldruff P.S. (1991). ‘PC-BEIS: a personal computer version of the biogenic emissions inventory system’, Journal of the Air and Waste Management Association, 41, No 7, pp. 937–941.

Pierce T.E. (1991). User's guide to the personal computer version of the biogenic emissions inventory system (PC-BEIS), Atmospheric research and exposure assessment laboratory, U.S. E.P.A., Research Triangle Park, NC. Report EPA/600/8-90/084.

Pio C., Nunes T., Valente A. and Brito S. (1994). Forest emissions of hydrocarbons, annual report 1993, Part 4 BIATEX, published by Eurotrac.

International scientific secretariat, Fraunhofer Institute (IFU), Garmisch-Partenkirchen, Germany, July 1994.

Puxbaum H. (1997). ‘Biogenic emissions of alcohols, esters, ether and higher aldehydes’. In: Biogenic volatile organic compounds in the atmosphere, G. Helas, J. Slanina, and R. Steinbrecher (eds.), SPB Academic Publishing bv Amsterdam, pp. 79–99.

Rasmussen R. A. (1978). Isoprene plant species list. Special report of air pollution Research Section Washington State University.

Rudolph J., Plass-Dülmer C., Benning L., Brandenburger U., Brauers T., Dorn H.-P., Hausmann M., Hofzumahaus A., Holland F., Parusel E., Ramacher B., Wahner A. Wedel A., Duckham C., Hewitt N., König G. and Puxbaum H. (1998). The Popcorn campaign 1994, an intensive field study of biogenic and man made volatile organic compounds in the atmosphere: an overview. Journal of Atmospheric Chemistry. In press.

Schnitzler J.-P., Lehning A., Steinbrecher R. (1997). ‘Seasonal pattern of isoprene sysnthase activity in Quercus robur leaves and its impact on modeling isoprene emission rates’, Botanica Acta 110, pp. 240–243.

Schuh et al. (1997). ‘Emissions of volatile organic compounds from  sunflower and beech: dependence on temperature and light intensity’, Journal of Atmospheric Chemistry, 27, pp. 291–318.

Seufert G., Bartzis J., Bomboi T., Ciccioli P., Cieslik S., Dlugi R., Foster P., Hewitt N., Kesselmeier J., Kotzias D., Lenz R., Manes F., Perez-Pastor R., Steinbrecher R., Torres L., Valentini R. and Versino B. (1997). ‘The BEMA-project: An overview of the Castelporziano experiments’, Atmospheric Environment 31, No S1, pp. 5–18.

Simon V., Clement B., Riba M.L. and Torres L. (1994). ‘The Landes Experiment: monoterpenes emitted from maritime pine’, Journal of Geophysical Research 99(D8), pp. 16501–16510.

Simpson D., Guenther A., Hewitt C.N. and Steinbrecher R. (1995). ‘Biogenic emissions in Europe 1. Estimates and uncertainties’, Journal of Geophysical Research, 100, No D11, pp. 22875–22890.

Simpson D. (1995). ‘Biogenic emissions in Europe 2: Implications for ozone control strategies’, Journal of Geophysical Research, 100, No D11, pp. 22891–22906.

Staudt et al. (1997). ‘Seasonal and diurnal patterns of monoterpene emissions from Pinus pinea (L.) under field conditions’, Atmospheric Environment, 31, S1, pp. 145–156.

Steinbrecher R. (1994). ‘Emission of VOCs from selected European ecosystems: the state-of-the-art’. In: et al., Borrel P., editor, Transport and Transformation of Pollutants in the Troposphere, Proceedings Eurotrac symposium 1994, pp. 448–455. SPB Acad. Publish. bv., the Hague, Netherlands.

Steinbrecher R. (1996). Reaktive organische Luftkomponenten (C6-C12) anthropogenen und biogenen Ursprungs in Laub- und Nadelwldern. Abschlubericht BMBF-Forschungsvorhaben 07 EU 816/7, p. 88.

Steinbrecher R. (1997). Emission factor table of BVOC of plant species in Europe, IFU-Garmisch (version December 1997).

Steinbrecher R., Hahn J., Stahl K., Eichstdter G., Lederle K., Rabong R., Schreiner A.-M. and Slemr J. (1997). ‚Investigations on emissions of low molecular weight compounds (C2-C10) from vegetation’. In: Biosphere-Atmosphere exchange of pollutants and trace substances, S. Slanina (ed.) Springer Verlag Berlin, Vol. 4, pp. 342–351.

Steinbrecher R., Schürmann W., Schreiner A-M. and Ziegler H. (1993). Terpenoid emissions from Common oak (Quercus robur L.) and Norway spruce (Picea abies [l.] karst, In Slanina J. et al., editor, Proceedings of the Joint CEC/BIATEX Workshop, Aveiro (P), 4–7 May 1993 on the General Assessment of Biogenic Emissions and Deposition of Nitrogen Compounds, Sulfur compounds and Oxidants in Europe, pages 251–261. CEC Research Programme Report 47.

Street R.A. (1995). Emissions of non-methane hydrocarbons from three forest ecosystems, PhD thesis, Lancaster Univ., Lancaster, England.

Street R. A., Duckham S. C., Boussard C. and Hewitt C. N. (1997). Emissions of VOCs from Stressed and Unstressed Vegetation. In: Biosphere-Atmosphere Exchange of Pollutants and Trace Substances, Slanina S. (ed.), Springer-Verlag Berlin, Heidelberg 1997, pp. 366–371.

Street R.A., Hewitt C.N. and Mennicken S. (1997). ‘Isoprene and monoterpene emissions from a Eucalyptus plantation in Portugal’, Journal of Geophysical Research, D13, 102, pp. 15875–15887.

UNECE, 1992. The environment in Europe and North America: annotated statistics 1992.

Veldt C. (1988). Inventorying natural VOC emissions for the Corinair project, Apeldoorn, The Netherlands, MT-TNO report 88–275; Also published in Coriniar Technical Annexes Volume 2, Default emission factor handbook, European Commission EUR 12586/2, pp. 101–128.

Veldt C. (1989). Leaf biomass data for the estimation of biogenic VOC emissions, Apeldoorn, The Netherlands, MT-TNO report pp. 89–306.

Veldt C. (1991). The use of biogenic VOC measurements in emission inventories, Apeldoorn, The Netherlands, MT-TNO report pp. 91–323.

Winer A. M., Fitz D. R., Miller P. R., Atkinson R., Brown D. E., Carter W. P. L., Dodd M. C., Johnson C. W., Myers M. A., Neisess K. R., Poe M. P. and Stephens E. R. (1983). Investigation of the role of natural hydrocarbons in photochemical smog formation in California. Final report, contract No AO-056-32, California Air Resources Board, Statewide Air Pollution Research Center, University of California, Riverside, California 92521.

Winer A.M., Arey J., Atkinson R., Aschmann S.M., Long W.D., Morrison C.L. and Olszyk, D. (1992). ‘Emission rates of organics from vegetation in California´s central valley’, Atmospheric Environment 26A, pp. 2647–2659.

Zimmerman P. (1979). Testing of hydrocarbon emissions of vegetation, leaf litter and aquatic surfaces and development of a methodology for compiling biogenic emission inventories, EPA 450/4-79-004.

# 18	Bibliography

Guenther A.B., Zimmerman P.R., Harley P.C., Monson R.K. and Fall R. (1993). ‘Isoprene and monoterpene rate variability: Model evaluations and sensitivity analyses’, Journal of Geophysical Research, 98, No D7, pp. 12609–12617.

Guenther A., Zimmerman P. and Wildermuth M. (1994). ‘Natural volatile organic compound emission rate estimates for U.S. woodland landscapes’, Atmospheric Environment, 28, pp. 1197–1210.

Guenther A., Hewitt C.N., Erickson D., Fall R., Geron C., Graedel T., Harley P., Klinger L., Lerdau M., McKay W.A., Pierce T., Scholes R., Steinbrecher R., Tallamraju R., Taylor J. and Zimmerman P. (1995). ‘A global model of natural volatile organic compound emissions’, Journal of Geophysical Research, 100, No D5, pp. 8873–8892.

Hewitt C. N., Street R.A. and Scholefield P.A. (1997). Isoprene and Monoterpene-Emitting Species Survey 1997: www.es.lancs.ac.uk/es/people/pg/pas/download.html .

Simpson D., Guenther A., Hewitt C.N. and Steinbrecher R. (1995). ‘Biogenic emissions in Europe 1. Estimates and uncertainties’, Journal of Geophysical Research, 100, No D11, pp. 22875–22890.

Simpson D., Winiwarter W., Börjesson G., Cinderby S., Ferreiro A., Guenther A., Hewitt C.N., Janson R., Khalil M.A.K., Owen S., Pierce T.E., Puxbaum H., Shearer M., Steinbrecher R., Tarrason L. and Öquist M.G. Inventorying emissions from Nature in Europe, submitted.

Steinbrecher R. (1997). Emission factor table of BVOC of plant species in Europe, IFU Garmisch (version December 1997).

Veldt C. (1989). Leaf biomass data for the estimation of biogenic VOC emissions, Apeldoorn, The Netherlands, MT-TNO report 89-306.

# Point of enquiry

Enquiries concerning this chapter should be directed to the relevant leader(s) of the Task Force on Emission Inventories and Projection’s expert panel on Agriculture and Nature. Please refer to the TFEIP website (www.tfeip-secretariat.org/) for the contact details of the current expert panel leaders.