## Changes to the UK Greenhouse GAS Inventory Since The Provisional Inventory

This note describes the methodological changes and changes in activity data made to the inventory since the provisional 2003 Submission to EUMM in December 2002.  Where appropriate, emissions from 1990 onwards are recalculated to include the changes and to provide a consistent time series.  Only larger changes are noted below.  Full methodological details are given in UK Greenhouse Gas Inventory, 1990 to 2000. Baggott, S, et al, Appendix 1 to 7.NETCEN, AEA Technology, (2003)

Fuel Combustion - Reference Approach (1A)

Carbon emission factors (t C/TJ) for Anthracite and Coking Coal have been revised based on updated data from the UK Department of Trade and Industry.  This has resulted in small changes to the CO2 emissions calculated according to the IPCC reference approach.

Fuel Combustion - Reference Approach (1A)

Due the late provision of sulphur concentrations in liquid fuels, the estimates for of SO2 from liquid fuel combustion have been revised.

Manufacturing Industry and Construction (1A2)

Emissions from fuel used in the manufacture of lime have been estimated separately for the first time, rather than included in the category 'other industry'.  Emissions of carbon, sulphur dioxide and NMVOC are not affected by this change.  Emission estimates for NOx and CO are based on Pollution Inventory and the overall impact is to increase emissions of both pollutants.

Since the December 2002,we have finalised the methodology for estimating combustion from the lime process.  These emissions have now been included in the CRF, but were excluded in the December 2002 submission.

Energy Industries - Manufacturing Industries and Construction 'other industry' (1A2f)

Emissions of NMVOC and SO2 have been revised slightly based on updated data from the UK Environment Agency.

Chemical Industry 'Other' (Sulphuric Acid & Pigment Production) (2B5)

Emissions of SO2 have been revised slightly from 1990 to 2001 based on updated data from the UK Environment Agency.

Chemical Industry 'Other' (Organic Chemicals) (2B5)

Emissions of CO and NMVOC have been revised slightly from 1990 to 2001 based on updated data from the UK Environment Agency.

Halocarbons (2E and 2F)

The model used to estimate emissions of halocarbons has been revised, and small revisions have been made to emission estimates of SF6 and HFCs.

Agriculture (4)

We detected an inconsistency with animal numbers provided, and have revised the background data entered in to the CRF, and the estimates of enteric methane from 1990 onwards.  In the December 2002 submission, only the 2001 and 2002 CRF had been corrected.

J D Watterson and J Goodwin

National Environmental Technology Centre

Culham

OX14 3ED

Ref: 47052001/2001/CD1550/JW

28,03,02