Appendix 5 Agriculture

CONTENTS

1	Livestock	2

1.1	Enteric Emissions of Methane	2

1.2	Methane Emissions from Animal Manures.	4

1.3	Emissions of Nitrous Oxide from Animal Waste
Management Systems	5

2	Agricultural Soils	9

2.1	Inorganic Fertiliser	9

2.2	Biological Fixation of Nitrogen by Crops	9

2.3	Crop Residues	10

2.4	Histosols	11

2.5	Grazing Animals	11

2.6	Organic Fertilizers	11

2.7	Atmospheric Deposition of NOx and NH3	12

2.8	Leaching and Runoff	13

3	Field Burning.	14

4	Quality Assurance	14

5	REFERENCES	15

# Livestock

The NAEI estimates emissions of methane from farm animals resulting from enteric fermentation and the storage and spreading of animal manures and slurries.  The methane emission estimates were supplied by Defra (2002a).

## Enteric Emissions of Methane

Methane is produced in herbivores as a by-product of enteric fermentation, a digestive process by which carbohydrates are broken down by micro-organisms.  Emissions are calculated from animal population data collected in the June Agricultural Census and published in Defra (2002b) and the appropriate emission factors.  Data for earlier years are often revised so information was taken from the Defra agricultural statistics database.  Table 1 shows the emission factors used.  Apart from cattle, lambs and deer, the methane emission factors are IPCC Tier I defaults (IPCC, 1997) and do not change from year to year.  The dairy cattle emission factors are estimated following the IPCC Tier 2 procedure (IPCC, 1997) and vary from year to year.  For dairy cattle, the calculations are based on the population of the ‘dairy breeding herd’ rather than ‘dairy cattle in milk’ because the latter definition includes ‘cows in calf but not in milk’.  The emission factors for beef and other cattle were also calculated using the IPCC Tier 2 procedure (Table 3), but do not vary from year to year.  The enteric emission factors for beef cattle were almost identical to the IPCC Tier I default so the default was used in the estimates.  The base data and emission factors for 1990-2001 are given in Tables 2 and 3.  The emission factor for lambs is assumed to be 40% of that for adult sheep.  In using the animal population data, it is assumed that the reported number of animals are alive for that whole year.  The exception is the treatment of sheep where it is normal practice to slaughter lambs and other non-breeding sheep after 6 to 9 months.  Hence it is assumed that breeding sheep are alive the whole year but that lambs and other non-breeding sheep are only alive 6 months of a given year.  The sheep emission factors in Table 1 are reported on the basis that the animals are alive the whole year.

Table 1	Methane Emission Factors for Livestock Emissions

|  | Enteric methanea kg CH4/head/year | Methane from manuresa kg CH4/head/year |
| --- | --- | --- |
| Dairy Breeding Herd | 117b | 13.2b |
| Beef Herd | 48 | 2.74 |
| Other Cattle >1 year, Dairy Heiffers | 48 | 6 |
| Other Cattle <1 year | 32.8 | 2.96 |
| Pigs | 1.5 | 3 |
| Breeding Sheep | 8 | 0.19 |
| Other Sheep | 8e | 0.19e |
| Lambs < 1 year | 3.2ce | 0.076ce |
| Goats | 5 | 0.12 |
| Horses | 18 | 1.4 |
| Deer: Stags & Hinds | 10.4c | 0.26c |
| Deer: Calves | 5.2c | 0.13c |
| Poultryd | 0 | 0.078 |

a	IPCC (1997)

b	Emission factor for year 2001

c	Sneath et al. (1997)

d	Chickens, turkeys, geese, ducks and guinea fowl

e	Factor quoted assumes animal lives for a year; emission calculation assumes animal lives for 6 months

Table 2	 Dairy Cattle Methane Emission Factorsa

|  | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Average Weight of cow (kg)b | 550 | 556 | 561 | 567 | 572 | 578 | 584 | 590 | 596 | 602 | 608 | 614 |
| Average Rate of Milk Production (liter/d) | 14.3 | 14.2 | 14.5 | 14.7 | 14.7 | 15.0 | 15.1 | 15.9 | 16.1 | 16.4 | 16.6 | 16.7 |
| Average Fat Content (%) | 4.01 | 4.04 | 4.06 | 4.07 | 4.05 | 4.05 | 4.08 | 4.07 | 4.07 | 4.03 | 4.03 | 4.01 |
| Enteric Emission Factor (kg CH4/head/y) | 104 | 104 | 106 | 107 | 107 | 109 | 110 | 113 | 114 | 115 | 116 | 117 |
| Manure Emission Factor (kg CH4/head/y) | 11.7 | 11.7 | 12.0 | 12.1 | 12.1 | 12.3 | 12.4 | 12.7 | 12.9 | 13.0 | 13.1 | 13.2 |

a	43% of animals graze on good quality pasture, rest confined

Gestation period 281 days

Digestible energy 65%

Methane conversion rate 6%

Ash content of manure 8%

Methane producing capacity of manure 0.24 m3/kg VS

b	Weight assuming annual growth of 1% from 1990

Table 3	Beef and Other Cattle Methane Emission Factorsa

|  | Beef Cattle | Other Cattle |
| --- | --- | --- |
| Average Weight of Animal (kg) | 500 | 180 |
| Time Spent Grazing | 50% | 46% |
| GE (MJ/d) | 123.3 | 83.4 |
| Enteric Emission Factor (kg CH4/head/y) | 48.5b | 32.8 |
| Manure Emission Factor (kg CH4/head/y) | 2.74 | 2.96 |

a	Digestible Energy 65%

Ash content of manure 8%

Methane producing capacity of manure 0.24 m3/kg VS

b	IPCC (1997) default (48 kg/head/y) used since calculated factor is very close to default and the difference under the Tier II method will not affect the accuracy of the emission factor at the required level of precision

## Methane Emissions from Animal Manures.

Methane is produced from the decomposition of manure under anaerobic conditions.  When manure is stored or treated as a liquid in a lagoon, pond or tank it tends to decompose anaerobically and produce a significant quantity of methane.  When manure is handled as a solid or when it is deposited on pastures, it tends to decompose aerobically and little or no methane is produced.  Hence the system of manure management used affects emission rates.  Emissions of methane from animal manures are calculated from animal population data (Defra, 2002b) in the same way as the enteric emissions.  The emission factors are listed in Table 1.  Apart from cattle, lambs and deer, these are all IPCC Tier I defaults (IPCC, 1997) and do not change from year to year.  The emission factors for lambs are assumed to be 40% of that for adult sheep.  Emission factors for dairy cattle were calculated from the IPCC Tier 2 procedure using data shown in Tables 2 and 4 (Defra, 2002a).  There was a revision (in 2002) of the allocation of manure to the different management systems based on new data. This is detailed in Section 1.3.  For dairy cattle, the calculations are based on the population of the ‘dairy breeding herd’ rather than ‘dairy cattle in milk’ used in earlier inventories as the latter definition includes ‘cows in calf but not in milk’.  The waste factors used for beef and other cattle are now calculated from the IPCC Tier 2 procedure but do not vary from year to year.  Emission factors and base data for beef and other cattle are given in Table 3.

Table 4 	Cattle Manure Management Systems in the UK

| Manure Handling System | Methane Conversion Factor %a | Fraction of manure handled using manure system % | Fraction of manure handled using manure system % |
| --- | --- | --- | --- |
|  |  | Dairy | Beef and Other |
| Pasture Range | 1 | 45.5 | 50.5 |
| Liquid System | 10 | 30.6 | 6 |
| Solid Storage | 1 | 9.8 | 20.7 |
| Daily Spread | 0.1 | 14.1 | 23 |

a	IPCC (1997)

## Emissions of Nitrous Oxide from Animal Waste Management Systems

Animals are assumed not to give rise to nitrous oxide emissions directly, but emissions from their manures during storage are calculated for a number of animal waste management systems (AWMS) defined by IPCC.  Emissions from the following AWMS are reported under the Manure Management IPCC category:

- Flushing anaerobic lagoons. These are assumed not to be in use in the UK.

- Liquid systems

- Solid storage and dry lot (including farm-yard manure)

- Other systems (including poultry litter, stables)

According to IPCC(1997) guidelines, the following AWMS are reported in the Agricultural Soils category:

- All applied animal manures and slurries

- Pasture range and paddock

Emissions from the combustion of poultry litter for electricity generation are reported under power stations.

The IPCC (1997) method for calculating emissions of N2O from animal waste management systems can be expressed as:

N2O(AWMS)	=	44/28 .  N(T) . Nex(T) . AWMS(T) . EF(AWMS)

where

N2O(AWMS)	= 	N2O emissions from animal waste management systems (kg N2O/yr)

N(T)	= 	Number of animals of type T

Nex(T)	= 	N excretion of animals of type T (kg N/animal/yr)

AWMS(W)	= 	Fraction of Nex that is managed in one of the different

waste management systems of type W

EF(AWMS)	=	N2O emission factor for an AWMS (kg N2O-N/kg of Nex in AWMS)

The summation takes place over all animal types and the AWMS of interest.  Animal population data are taken from Agricultural Statistics (Defra, 2002b).  Table 5 shows emission factors for nitrogen excretion per head for domestic livestock in the UK (Nex).  These are based on a balance by Smith (1998).

The UK methodology assumes that 20% of the total N emitted by livestock volatilises as NOx and NH3 and therefore does not contribute to N2O emissions from AWMS.  This is because in the absence of a more detailed split of NH3 losses at the different stages of the manure handling process it has been assumed that NH3 loss occurs prior to major N2O losses.  Thus, the Nex factors used in the AWMS estimates (and those reported in Tables 5 and 6) exclude the fraction of N volatilising and are 20% less than if they were reported on the same basis as the ‘total’ Nex factors reported in the IPCC Guidelines.  Values of total N excreted shown in the Common Reporting Format are not corrected in this way and are estimates of total N excreted from livestock.  Nex factors for dairy cattle take account of the assumed growth in the average cow weight by 1% per annum and are shown in Table 6.

The conversion of excreted N into N2O emissions is determined by the type of manure management system used.  The distribution of waste management systems for each animal type (AWMS(T)) is given in Table 7.  The distributions used were revised for cattle and poultry in the 2000 Inventory.  The change related to the way that data on ‘no significant storage capacity’ of farm yard manure (FYM) were allocated.  This could have a large effect on emissions because it amounted to around 50% of manure and the ‘Daily spread (DS)’ category has an emission factor of zero, compared to 0.02 for the ‘Solid storage and dry lot (SSD)’ category.  However, we are advised (Smith, 2002) that:

In terms of slurry, it seems likely that where a proportion of the estimated slurry production is attributed with “nil” or little storage (<1 month capacity), as above, it can be assumed that  such units will rely on a significant amount of daily – weekly spreading activity, according to land availability and trafficability, throughout.  With FYM and poultry manure, however, significant storage capacity exists within the house and so, “no storage” generally implies that manure is cleared from the house/straw littered yard and spread direct on land.  Storage capacity within the house or yard might comprise between 7 weeks – 12 months (poultry) or several months (cattle) and is unlikely to require “daily” spreading activity.

Therefore, assigning this ‘stored in house’ manure to ‘daily spread’ is acceptable only if emissions from the housing phase are thought to be very small.  Calculations were performed with the N2O Inventory of Farmed Livestock to compare housing and storage phases (Sneath et al. 1997).  For pigs and poultry, the emission factor for housing is the same as or greater than that of storage.  It would therefore lead to significant underestimation to use the daily spread emission factor.  All of the FYM in this case has therefore been re-allocated to SSD.

For dairy and non-dairy cattle, the emission factor for the housing phase is around 10% of the storage phase, so the non-stored FYM has been split between SSD and DS to account for this.

Table 8 gives the N2O emission factor for each animal waste management system (EF(AWMS)).  These are expressed as the emission of N2O-N per mass of excreted N processed by the waste management system.

Emissions from grazing animals (pasture range and paddock) and daily spread are calculated in the same way as the other AWMS.  However, emissions from land spreading of manure that has previously been stored in a) liquid systems, b) solid storage and dry lot and c) other systems, are treated differently.  These are discussed in Section 2.6 on Organic Fertilizer.

Table 5	Nitrogen Excretion Factors for Animals in the UKa

| Animal Type | Emission Factor kg N/animal/yearb |
| --- | --- |
| Dairy Cows | 96.0c |
| Other Cattle > 2 year | 60 |
| Other Cattle 1-2 year | 47 |
| Other Cattle <1 year | 11.8 |
| Pigs < 20kg | 3.0 |
| Other Pigs 20-50 kg | 7.1 |
| Fattening & Other Pigs > 50 kg | 10.7 |
| Breeding Pigs > 50 kg | 14.3 |
| Breeding Sheep | 9.2 |
| Other Sheep <1 year | 9.2e |
| Lambs | 3.36e |
| Goats | 7.2 |
| Broilers | 0.495 |
| Broiler Breeders | 0.899 |
| Layers | 0.589 |
| Ducks, | 0.984 |
| Turkeys | 1.052 |
| Growing Pullets | 0.106 |
| Horses | 32 |
| Deer: Stagsd | 17.5 |
| Deer: Hindsd | 11.7 |
| Deer: Calvesd | 8.64 |

a	Smith (1998)

b	Nex  factors exclude 20% N volatilising as NOx and NH3

c	Estimate for year 2001

d	Sneath et al, (1997)

e	Factor quoted assumes animal lives for a year.  Emission calculation assumes animal lives for 6 months.

Table 6	 Nitrogen Excretion Factors for Dairy Cattlea

|  | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Emission Factor kg N/animal/year | 84.8 | 85.8 | 86.8 | 87.7 | 88.7 | 89.8 | 90.8 | 91.8 | 92.9 | 93.8 | 94.9 | 96.0 |

a	Nex factors exclude 20% N volatilising as NOx and NH3

Table 7	Distribution of Animal Waste Management Systems used for Different Animal typesc

| Animal Type | Liquid System | Daily Spread | Solid Storage and Dry Lota | Pasture Range and Paddock | Otherb | Fuel |
| --- | --- | --- | --- | --- | --- | --- |
| Dairy Cows | 30.6 | 14.1 | 9.8 | 45.5 |  |  |
| Other Cattle >1 year | 6.0 | 23.0 | 20.4 | 50.5 |  |  |
| Other Cattle <1 year |  | 22.9 | 22.3 | 54.8 |  |  |
| Fattening & Other Pigs > 20 kg, | 29.2 | 5.8 | 64.0 | 1.0 |  |  |
| Breeding sows | 35.5 | 7.1 | 28 | 29.3 |  |  |
| Pigs <20 kg | 38.3 | 7.7 | 46.0 | 8.0 |  |  |
| Sheep |  |  | 2.0 | 98.0 |  |  |
| Goats |  |  |  | 96.0 | 4.0 |  |
| Broilers & Table Fowl (1999) |  |  |  | 1.0 | 66.0 | 33.0 |
| Breeders (1999) |  |  |  | 1.0 | 99.0 |  |
| Layerse |  |  |  | 10.0 | 90.0 |  |
| Pulletse |  |  |  | 10.0 | 90.0 |  |
| Ducks, Geese & Guinea Fowle |  |  |  | 50.0 | 50.0 |  |
| Turkeyse |  |  |  | 8.0 | 92.0 |  |
| Horses |  |  |  | 96.0 | 4.0 |  |
| Deer: Stagsd |  |  |  | 100 |  |  |
| Deer: Hinds & Calvesd |  |  |  | 75.0 | 25.0 |  |

a	Farmyard manure

b	Poultry litter, Stables

c	ADAS (1995a)

d	Sneath et al. (1997)

e	Tucker (1997)

Table 8	Nitrous Oxide Emission Factors for Animal Waste Handling Systemsa

| Waste Handling System | Emission Factor kg N2O-N per kg N excreted |
| --- | --- |
| Liquid System | 0.001 |
| Daily Spreadb | 0 |
| Solid Storage and Dry Lot | 0.02 |
| Pasture, Range and Paddockb | 0.02 |
| Fuel | - |
| Other | 0.005 |

a	IPCC (1997)

b	Reported under Agricultural Soils

# Agricultural Soils

Direct emissions of nitrous oxide from agricultural soils are estimated using the IPCC recommended methodology (IPCC, 1997) but incorporating some UK specific parameters.  The IPCC method involves estimating contributions from:

(i)	The use of inorganic fertilizer

(ii)	Biological fixation of nitrogen by crops

(iii) 	Ploughing in crop residues

(iv)	Cultivation of Histosols (organic soils)

(v)	Spreading animal manures on land

(vi)	Manures dropped by animals grazing in the field

In addition to these, the following indirect emission sources are estimated:

(vii)	Emission of N2O from atmospheric deposition of agricultural NOx and NH3

(viii)	Emission of N2O from leaching of agricultural nitrate and runoff

Descriptions of the methods used follow.

## Inorganic Fertiliser

Emissions from the application of inorganic fertilizer are calculated using the IPCC (1997) methodology and IPCC default emission factors. They are given by:

N2O(SN) 	=	44/28 . N(FERT) . (1-Frac(GASF)) . EF1

where

N2O(SN)	=	Emission of N2O from synthetic fertiliser application

(kg N2O/yr)

N(FERT)	=	Total use of synthetic fertiliser (kg N/yr)

Frac(GASF)	=	Fraction of synthetic fertiliser emitted as NOx  + NH3

=	0.1 kg NH3-N+NOx -N / kg synthetic N applied

EF1	=	Emission Factor for direct soil emissions

=  	0.0125 kg N2O-N/kg N input

Annual consumption of synthetic fertilizer is estimated based on crop areas (DEFRA, 2002b) and fertilizer application rates (BSFP, 2001).

## Biological Fixation of Nitrogen by Crops

Emissions of nitrous oxide from the biological fixation of nitrogen by crops are calculated using the IPCC (1997) methodology and IPCC default emission factors. They are given by:

N2O(BF)	=	44/28 . 2 . Crop(BF) . Frac(NCRBF) . EF1

where

N2O(BF)	=	Emission of N2O from biological fixation (kg N2O/yr)

Crop(BF)	=	Production of legumes (kg dry mass/yr)

Frac(NCRBF)	=	Fraction of nitrogen in N fixing crop

=	0.03 kg N/ kg dry mass

EF1	=	Emission Factor for direct soil emissions

=	0.0125 kg N2O-N/kg N input

The factor of 2 converts the edible portion of the crop reported in agricultural statistics to the total biomass.  The fraction of dry mass for the crops considered is given in Table 9.

Table 9	Dry Mass Content and Residue Fraction of UK Crops

| Crop Type | Fraction dry massb | Residue/Crop |
| --- | --- | --- |
| Broad Beans, Green Peas | 0.08 | 1.1 |
| Field Beand, Peas(harvest dry) | 0.86 | 1.1 |
| Rye, Mixed corn, Triticale | 0.855a | 1.6 |
| Wheat, Oats | 0.855a | 1.3 |
| Barley | 0.855a | 1.2 |
| Oilseed Rape, Linseed | 0.91a | 1.2 |
| Maize | 0.50 | 1 |
| Hopsc | 0.20 | 1.2 |
| Potatoes | 0.20 | 0.4 |
| Roots, Onions | 0.07 | 1.2 |
| Brassicas | 0.06 | 1.2 |
| Sugar Beet | 0.1 | 0.2 |
| Other | 0.05 | 1.2 |
| Phaseolus beans | 0.08 | 1.2 |

a	Defra (2002b)

b	Burton (1982), Nix (1997) or Defra estimates

c	Hops dry mass from Brewers Licensed Retail Association (1998)

d	Field beans dry mass from PGRE (1998)

The data for the ratio residue/crop are default values found under Agricultural Soils  or derived from Table 4.17 in Field Burning of Agricultural Residues (IPCC, 1997). Crop production data are taken from DEFRA (2002b, 2000c). The total nitrous oxide emission reported also includes a contribution from improved grass calculated using a fixation rate of 4 kg N/ha/year (Lord, 1997).

## Crop Residues

Emissions of nitrous oxide from the ploughing in of crop residues are calculated using the IPCC (1997) methodology and IPCC default emission factors. They are given by:

N2O(CR)	=	44/28 . 2 . (CropO . Frac(NCRO) .+ Crop(BF) . Frac(NCRBF)) . (1-FracR) . (1-FracB) . EF1

where

N2O(CR)	=	Emission of N2O from crop residues (kg N2O/yr)

CropO	=	Production of non-N fixing crops (kg dry mass/yr)

Frac(NCRO)	=	Fraction of nitrogen in non-N fixing crops

=	0.015 kg N/ kg dry mass

FracR	=	Fraction of crop that is remove from field as crop

FracB	=	Fraction of crop residue that is burnt rather than left on field

EF1	=	Emission Factor for direct soil emissions

=	0.0125 kg N2O-N/kg N input

Crop(BF)	=	Production of legumes (kg dry mass/year)

Frac(NCRBF)	=	Fraction of nitrogen in N fixing crop

= 0.03 kg N/ kg dry mass

Production data of crops are taken from Defra (2001b, 2001c).  The dry mass fraction of crops and fraction of crop removed from the field are given in Table 9.  Field burning has largely ceased in the UK since 1993.  For years prior to 1993, field burning data were taken from the annual MAFF Straw Disposal Survey (MAFF, 1995).

## Histosols

Emissions from Histosols were estimated using the IPCC (1997) default factor of 5 kg N2O-N/ha/yr.  The area of cultivated Histosols is assumed to be equal to that of eutric organic soils in the UK and is based on a FAO soil map figure supplied by SSLRC.

## Grazing Animals

Emissions from manure deposited by grazing animals are reported under agricultural soils by IPCC.  The method of calculation is the same as that for AWMS (Section 1.3), using factors for pasture range and paddock.

## Organic Fertilizers

Emissions from animal manures and slurries used as organic fertilizers are reported under agricultural soils by IPCC.  The calculation involves estimating the amount of nitrogen applied to the land and applying IPCC emission factors.  For daily spreading of manure, the emission is given by:

N2O(DS)	=	44/28 .  NT . Nex(T) . AWMS(DS) . EF1

where

N2O(DS)	= 	N2O emissions from daily spreading of wastes (kg N2O/yr)

NT	= 	Number of animals of type T

Nex(T)	= 	N excretion of animals of type T (kg N/animal/yr)

AWMS(DS)	= 	Fraction of Nex that is daily spread

EF1	=	Emission Factor for direct soil emissions

=	0.0125 kg N2O-N/kg N input

For the application of previously stored manures to land, a correction is applied to account for previous N2O losses during storage.

N2O(FAW)	=	44/28 .  (NT . Nex(T) . AWMS(T) - N(AWMS) ) . EF1

where

N2O(FAW)	=	N2O emission from organic fertiliser application

NT	= 	Number of animals of type T

Nex(T)	= 	N excretion of animals of type T (kg N/animal/yr)

AWMS(W)	= 	Fraction of Nex that is managed in one of the different

waste management systems of type W

N(AWMS)	= 	N2O emissions from animal waste management systems as

nitrogen (kg N2O-N/yr)

The summation is for all animal types and manure previously stored in categories defined as a) liquid, b) solid storage and dry lot and c) other.

## Atmospheric Deposition of NOx and NH3

Indirect emissions of N2O from the atmospheric deposition of ammonia and NOx are estimated according to the IPCC (1997) methodology but with corrections to avoid double counting N.  The sources of ammonia and NOx considered, are synthetic fertiliser application and animal manures applied as fertiliser.

The contribution from synthetic fertilisers is given by:

N2O(DSN)	=	44/28 . (N(FERT) - N(SN)) . Frac(GASF) . EF4

where

N2O(DSN)	=	Atmospheric deposition emission of N2O arising from synthetic fertiliser application (kg N2O/yr)

N(FERT)	=	Total mass of nitrogen applied as synthetic fertiliser (kg N/yr)

N(SN)	=	Direct emission of N2 O(SN) as nitrogen (kg N2O-N)

Frac(GASF)	=	Fraction of total synthetic fertiliser nitrogen that is emitted

as NOx + NH3

=	0.1 kg N/ kg N

EF4	=	N deposition emission factor

=	0.01 kg N2O-N/kg NH3-N and NOx-N emitted

The estimate includes a correction to avoid double counting N2O emitted from synthetic fertiliser use.

The indirect contribution from waste management systems is given by

N2O(DWS)	=	44/28. (N(EX)/(1-Frac(GASM)) -N(F) ) . Frac(GASM) . EF4

where

N2O(DWS)	=	Atmospheric deposition emission of N2O arising from animal wastes(kg N2O/yr)

N(EX)	=	Total N excreted by animals (kg N/yr)

Frac(GASM)	=	Fraction of livestock nitrogen excretion that volatilises as

NH3  and NOx

=	0.2 kg N/kg N

N(F) 	=	Total N content of wastes used as fuel (kg N/yr)

The equation corrects for the N content of manures used as fuel but no longer for the N lost in the direct emission of N2O from animal manures as previously.  The nitrogen excretion data in Table 5 already exclude volatilisation losses and hence a correction is included for this.

## Leaching and Runoff

Indirect emissions of N2O from leaching and runoff are estimated according the IPCC methodology but with corrections to avoid double counting N.  The sources of nitrogen considered, are synthetic fertiliser application and animal manures applied as fertiliser.

The contribution from synthetic fertilisers is given by:

N2O(LSN)	=	44/28 . (N(FERT) . (1-Frac(GASF) )- N(SN)) . Frac(LEACH) . EF5

where

N2O(LSN)	=	Leaching and runoff emission of N2O arising from synthetic fertiliser application (kg N2O/yr)

N(FERT)	=	Total mass of nitrogen applied as synthetic fertiliser (kg N/yr)

N(SN)	=	Direct emission of N2O(SN) as nitrogen (kg N2O-N/yr)

Frac(GASF)	=	Fraction of total synthetic fertiliser nitrogen that is emitted

as NOx + NH3

=	0.1 kg N/ kg N

Frac(LEACH)	=	Fraction of nitrogen input to soils that is lost through leaching and runoff

=	0.3 kg N/ kg fertiliser or manure N

EF5	=	Nitrogen leaching/runoff factor

=	0.025 kg N2O-N /kg N leaching/runoff

The estimate includes a correction to avoid double counting N2O emitted from synthetic fertiliser use.

The indirect contribution from waste management systems is given by

N2O(LWS)	=	44/28. (N(EX)-N(F) -N(WS) ) . Frac(LEACH) . EF5

where

N2O(LWS)	=	Leaching and runoff emission of N2O from animal wastes (kg N2O/yr)

N(EX)	=	Total N excreted by animals (kg N/yr)

N(F) 	=	Total N content of wastes used as fuel (kg N/yr)

N(AWMS)  	=	Total N content of N2O emissions from waste management systems including daily spread and pasture range and paddock (kg N2O-N/yr)

Frac(LEACH)	=	Fraction of nitrogen input to soils that is lost through leaching and runoff

=	0.3 kg N/ kg fertiliser or manure N

EF5	=	Nitrogen leaching/runoff factor

=	0.025 kg N2O-N /kg N leaching/runoff

The equation corrects both for the N lost in the direct emission of N2O from animal wastes and the N content of wastes used as fuel.

# Field Burning

The National Atmospheric Emissions Inventory reports emissions from field burning under the category agricultural incineration.  The estimates are derived from emission factors calculated according to IPCC (1997) and from USEPA (1997) shown in Table 10.

Table 10	Emission Factors for Field Burning (kg/t)

|  | CH4 | CO | NOx | N2O | NMVOC |
| --- | --- | --- | --- | --- | --- |
| Barley | 3.05a | 63.9a | 2.18a | 0.060a | 7.5b |
| Other | 3.24a | 67.9a | 2.32a | 0.064a | 9.0b |

a	IPCC (1997)

b	USEPA (1997)

The estimates of the masses of residue burnt of barley, oats, wheat and linseed are based on crop production data (Defra, 2001c) and data on the fraction of crop residues burnt (MAFF, 1997; ADAS, 1995b).  Field burning ceased in 1993 in England and Wales.  Burning in Scotland and Northern Ireland is considered negligible, as is grouse moor burning, so no estimates are reported from 1993 onwards.  The carbon dioxide emissions are not estimated because these are part of the annual carbon cycle.

# Quality Assurance

The livestock activity data used for constructing the inventory are supplied annually from the June census by the Defra Economics and Statistics Group, who adhere to documented QA procedures.  Activity data on mineral fertiliser are calculated using application rates from Defra's annual British Survey of Fertiliser Practice (BSFP, 2001) multiplied by crop areas in Defra's Survey of Farming Incomes (June Census).  Data from the June Census, in the form of *.PDF files, can be downloaded from the Defra website (www.defra.gov.uk) and incorporated into inventory spreadsheets without the need for manual data entry, eliminating the need for “double entry” procedures.  Annual comparisons of emission factors and other coefficients used are made by contractors compiling the inventory on behalf of Defra and by Defra itself.  Any changes are documented in the spreadsheet and in the accompanying chapter of the National Inventory Report.  Hardcopies of the submitted inventories, associated emails and copies of activity data are filed in Government secure files adhering to Government rules on document management.

Defra contractors who work on compiling the agricultural inventory, IGER, operate strict internal quality assurance systems with a management team for each project overseen by an experienced scientist with expertise in the topic area.  A Laboratory Notebook scheme provides quality control through all phases of the research and these are archived in secure facilities at the end of the project.  All experiments are approved by a consultant statistician at each of the planning, data analysis and interpretation and synthesis stages.  A range of internal checks exist to ensure that projects run to schedule, and internal and external (viz. visiting group procedures, etc.) reviews ensure the quality of the outputs.

# References

ADAS, (1995a).  Personal communication to A. Croxford (MAFF).  Distribution of animal waste management systems for cattle.

ADAS, (1995b).  Personal communication to A. Croxford MAFF.  Linseed burning data.

Brewers Licensed Retail Association, (1998).  Personal communication to R. Gerry (MAFF).  Estimate of dry matter content of hops.

BSFP, (2001).  British Survey of Fertiliser Practice: Fertiliser Use on Farm Crops for Crop Year 2000, The BSFP Authority, Peterborough ISBN 1-86190-110-0.  Data for preceding years comes from earlier versions of the same publication.

Burton, (1982).  Post-Harvest Physiology of Crops, Longman, London, ISBN 0-582-46038-7.

Defra, (2002a).  Personal communications from M. Rose, Land Management Improvement Division.

Defra, (2002b).  Agriculture in the UK 2001, The Stationary Office, London.

Defra, (2002c). Basic Horticultural Statistics for the United Kingdom: Calendar and Crop Years 1991/92 – 2001/02, The Stationary Office, London.

IPCC, (1997).  IPCC Revised 1996 Guidelines for National Greenhouse Gas Inventories, Volume 3, Greenhouse Gas Inventory Reference Manual, IPCC WGI Technical Support Unit, Hadley Centre, Meteorological Office, Bracknell, UK.

Lord, E (1997).  Personal communication to S. Gleave (MAFF).  Factor for biological nitrogen fixation by improved grass.

MAFF, (1995).  Straw Disposal Survey.  Data for earlier years taken from previous editions of the survey.  Contact: Government Buildings, Epsom Rd., Guildford, Surrey, GU1 2LD for copies.

MAFF, (1997).  Farm Incomes in the UK 1995/96, The Stationery Office, London, ISBN 0-11-2543026-0.

Nix, J, (1997).  Farm Management Pocket Book 1998, 28th ed., Wye College Press, Ashford, UK.

PGRE, (1998).  Personal communication to R. Gerry (MAFF).  Estimate of dry matter content of field beans.

Smith, K, (1998).  Personal communication from K. Smith, ADAS.

Smith, K, (2002).  Personal communication from K Smith, ADAS.

Sneath, RW, Chadwick DR, Phillips VR & Pain BF, (1997).  A UK Inventory of Methane/Nitrous Oxide Emissions from Farmed Livestock.  Contract reports (2) to MAFF, projects WA0604/5, SRI, IGER & ADAS.

Tucker, S & Canning, P, (1997).  Personal communication to D. Wilkins (MAFF).

USEPA, (1997).  Compilation of Air Pollutant Emission Factors, volume 1, 5th ed., AP-42, United States Environmental Protection Agency, North Carolina.

Appendix 6 Land Use Change and Forestry

CONTENTS

1	Introduction	2

2	Changes in Forests and Other Woody Biomass
Stocks (5A)	2

3	FOREST & GRASSLAND COVNERSION (5B)	7

4	CO2 Emissions and Removals from Soils (5D)	8

4.1	LAND USE change 	8

4.2	Set Aside	14

4.3	Emissions of CO2 from soil due to liming	14

5	Other sources and sinks (5E)	15

5.1	Changes in Crop Biomass	15

5.2	Peat Extraction	16

5.3	Lowland (fen) peat drainage	16

5.4	Upland (forestry) peat drainage	17

6	data presentation formats	18

7	Quality Assurance Methods and Standards	23

8	REFERENCES	24

# Introduction

The estimates for Land Use Change and Forestry are from work carried out by the Centre for Ecology & Hydrology (Cannell et al 1999, Milne and Brown 1999).  The data is reported under IPCC categories 5A (Changes in Forests and Other Woody Biomass, 5C (CO2 Emissions from Soils) and 5E (Other). No data is presently included for Categories 5B (Forest and Grassland Conversion) or 5C (Abandonment of Managed Lands) as these have been considered to be negligible, or not occurring, in the UK. The situation for deforestation has recently been reviewed and preliminary results relevant to Category 5B are presented below and, after final review, will be included in a later Inventory.

Here we present descriptions of the methods used for the Categories included and compare the format for data presentation used in this National Inventory Report (NIR) to that used for the UNFCCC Common Reporting Format (CRF).

# Changes in Forests and Other Woody Biomass Stocks (SA)

The estimates are based on data for the areas of forest plantation published by the UK Forestry Commission and the Northern Ireland Department of Agriculture.  The carbon uptake is calculated by a carbon accounting model (Dewar and Cannell 1992, Cannell and Dewar 1995, Milne et al. 1998) as the net change in pools of carbon in standing trees, litter, soil in broadleaf forests and products. Restocking is assumed in all forests.  It should be noted that part of the net uptake by litter and soils is reported in category 5D in the UNFCCC CRF submission.  The values of these removals are provided in Section 6 to allow comparison with data from countries which report only changes in woody biomass and include soils etc. elsewhere and to detail the differences between reporting here and in the UNFCCC CRF.

The carbon accounting model of Dewar and Cannell (1992) calculated the mass of carbon in trees, litter, soil and wood products from harvested material in new even-aged plantations that were clearfelled and then replanted at the time of Maximum Area Increment (MAI). Two types of input data and two parameter sets were required for the model (Cannell and Dewar, 1995). The input data are a) areas of new forest planted in each year in the past and b) the stemwood growth rate and harvesting pattern. Parameter values were required to estimate i) stemwood, foliage, branch and root masses from the stemwood volume and ii) the decomposition rates of litter, soil carbon and wood products.

For the estimates described here we used the combined area of new private and state planting from 1921 to 1996 for England, Scotland, Wales and Northern Ireland sub-divided into conifers and broadleaves (Milne et al. 1998). Restocking was dealt with in the model through the second and subsequent rotations for the 'new' areas and hence areas restocked each year did not need to be considered separately.

The carbon flow model uses Forestry Commission Yield Tables (Edwards and Christie, 1981) to describe forest growth. It was assumed that all new conifer plantations have the same growth characteristics as Sitka spruce (Picea sitchensis (Bong.) Carr.) under an intermediate thinning management. Milne et al. (1998) have shown that mean Yield Class for Sitka spruce varied across Great Britain from 10 to 16 m3 ha-1 a-1 but with no obvious geographical pattern and that this variation had a less than 10% effect on estimated carbon uptake. The Inventory data has therefore been estimated by assuming all conifers in Great Britain followed the growth pattern of Yield Class 12 m3 ha-1 a-1, but in Northern Ireland Yield Class 14 m3 ha-1 a-1, Sitka spruce. Milne et al. (1998) also showed little effect of different assumptions on broadleaf species. It is assumed here, that broadleaf forests had the characteristics of beech (Fagus sylvatica L.) of Yield Class 6 m3 ha-1 a-1. Data in the most recent inventory of British woodlands (Forestry Commission 2002) shows that beech is only about 8% of broadleaf forest (all ages). Although sensitivity analysis of the carbon accounting model shows that different assumptions on the broadleaf species planted has little effect on overall carbon uptake the assumption of using beech as the representative species will be reviewed. Using oak or the sycamore-ash-birch group Yield Class data instead of beech data is likely to have a less than 10% effect on the value of removal of carbon to UK forests. Irrespective of the assumption on representative species the variation in removals from 1990 to the present is determined by the afforestation rate in earlier decades and the effect this has on the age structure in the present forest estate and hence the average growth rate. This afforestation is all on ground that has not been wooded for many decades. Table 1 shows the afforestation rate since 1922 and the present age structure. In addition to these planted forests there are about 850 kha of woodland planted prior to 1922 or not of commercial importance. This area is not included for the purposes of the Greenhouse Gas Inventory. Variation from year to year in the reported removals to woody biomass, soils and harvested products reflect the changing pattern of afforestation over the period of available data. For example, there are increases in removals to harvested products about 50 years (the conifer forest rotation cycle) after a period of increased planting of conifers.  It can be shown that if forest expansion continues at the present rate then removals of atmospheric carbon will continue to increase until about 2005 and then will begin to decrease, reflecting the reduction in afforestation rate after the 1970s.

Table 1: Afforestation rate and age distribution of conifers and broadleaves in the United Kingdom.

|  | Planting rate (kha a-1) |  | Age distribution |  |
| --- | --- | --- | --- | --- |
|  | Conifers | Broadleaves | Conifers | Broadleaves |
| 1922-1929 | 4.9 | 2.4 | 2.9% | 7% |
| 1930-1939 | 7.2 | 2.2 | 5.4% | 9% |
| 1940-1949 | 6.3 | 1.9 | 4.7% | 7% |
| 1950-1959 | 20.0 | 3.0 | 14.9% | 12% |
| 1960-1969 | 28.4 | 2.9 | 21.1% | 12% |
| 1970-1979 | 33.2 | 1.5 | 24.8% | 6% |
| 1980-1989 | 22.5 | 1.4 | 16.8% | 5% |
| 1990 | 26.8 | 3.1 | 2.0% | 1% |
| 1991 | 15.4 | 5.8 | 1.1% | 2% |
| 1992 | 13.4 | 6.8 | 1.0% | 3% |
| 1993 | 11.6 | 6.5 | 0.9% | 3% |
| 1994 | 10.1 | 8.9 | 0.8% | 3% |
| 1995 | 7.4 | 11.2 | 0.6% | 4% |
| 1996 | 9.5 | 10.5 | 0.7% | 4% |
| 1997 | 7.4 | 8.9 | 0.6% | 4% |
| 1998 | 7.0 | 9.7 | 0.5% | 4% |
| 1999 | 6.6 | 10.1 | 0.5% | 4% |
| 2000 | 6.5 | 10.9 | 0.5% | 4% |
| 2001 | 4.9 | 13.4 | 0.4% | 5% |

Increases in stemwood volume were based on standard Yield Tables, as in Dewar and Cannell (1992) and Cannell and Dewar (1995), and the mass of carbon in a forest was calculated from this volume by multiplying by wood density, stem to branch and root mass ratios and the fraction of carbon in wood (0.5 assumed). The values used for these parameters for conifers and broadleaves are given in Table 2. The pattern of increases in stemwood volume between plating and first thinning is presently under review and newly developed curve will be used in the submissions and report for 2002. The change to the estimated Removals will however be small.

Table 2: Main parameters for forest carbon flow model for species used to estimates carbon uptake by planting of forests of Sitka spruce (P. sitchensis and beech (F. sylvatica) in United Kingdom (data from Dewar & Cannell, 1992).

|  |  | P. sitchensis | P. sitchensis | F. sylvatica |
| --- | --- | --- | --- | --- |
|  |  | YC12 | YC14 | YC6 |
|  | Rotation (years) | 59 | 57 | 92 |
|  | Initial spacing (m) | 2 | 2 | 1.2 |
|  | Year of first thinning | 25 | 23 | 30 |
|  | Stemwood density (t m-3) | 0.36 | 0.35 | 0.55 |
|  | Max. carbon in foliage (t ha-1) | 5.4 | 6.3 | 1.8 |
|  | Max. carbon in fine roots (t ha-1) | 2.7 | 2.7 | 2.7 |
|  | Fraction of wood in branches | 0.09 | 0.09 | 0.18 |
|  | Fraction of wood in woody roots | 0.19 | 0.19 | 0.16 |
|  | Max. foliage litterfall (t ha-1 a-1) | 1.1 | 1.3 | 2 |
|  | Max. fine root litter loss (t ha-1 a-1) | 2.7 | 2.7 | 2.7 |
|  | Dead foliage decay rate (a-1) | 1 | 1 | 3 |
|  | Dead wood decay rate (a-1) | 0.06 | 0.06 | 0.04 |
|  | Dead fine root decay rate (a-1) | 1.5 | 1.5 | 1.5 |
|  | Soil organic carbon decay rate (a-1) | 0.03 | 0.03 | 0.03 |
|  | Fraction of litter lost to soil organic matter | 0.5 | 0.5 | 0.5 |
|  | Lifetime of wood products | 57 | 59 | 92 |

The parameters controlling the transfer of carbon into the litter pools and its subsequent decay are given in Table 2. Litter transfer rate from foliage and fine roots increased to a maximum at canopy closure. A fraction of the litter was assumed to decay each year, half of which added to the soil organic matter pool that then decayed at a slower rate. Tree species and Yield Class, but not other factors which varied with location, were assumed to control the decay of litter and soil matter. Additional litter was generated at times of thinning and felling.

As in Cannell and Dewar (1995) it was assumed that conifer forests increased the amount of organic carbon in litter but did not increase the net amount of carbon in soil due to gains from the new forest being balanced by loss due to the disturbance at planting.  Most conifer afforestation in the UK is on soils with high organic carbon content. Until recently little has been known about carbon losses from such soils when ploughed for forest planting. It was generally believed that, especially for peats, the losses would be large and continue for very long periods (many decades). So, although the new forest would add new organic carbon to the existing soil, carbon would also continue to be lost from the existing soil. In the absence of detailed measurements it was therefore that for most conifer planting the gains and losses would balance but in the case of deep peats there would be a continuing net loss. The rate of this loss was estimated from some preliminary measurements on recently disturbed peats to be about 2 tC ha-1 a-1. These estimates of losses on afforested deep peats are reported under Category 5D (Emissions from Soils) in the CRF or Category 5E (Other) in the National Inventory Report (NIR)  (see below). Broadleaved forests were assumed to increase the net amount of carbon in litter and soil and, since normally planted on mineral soils, the change in emissions from pre-existing soils would be negligible Analysis of measurements taken at a deep peat moorland, locations covering afforestation of peats from 1 to 9 years previously and at a 26 year old conifer forest have recently been completed (Hargreaves et al. 2003) and suggest that long term losses from afforested peatlands are not as great as had been previously thought, settling to about 0.3 tC ha-1 a-1 thirty years after afforestation.. In addition a short burst of regrowth of moorland plant species occurs before forest canopy closure. The pattern of carbon loss and gain from afforested deep peat moorland is summarized in Table 3.

Table 3: Emissions of carbon from deep peat due to ploughing for afforestation. Negative values mean uptake of carbon from the atmosphere. Here this is due to temporary re-growth of moorland plants between ploughing and forest canopy closure. (Based on work of Hargreaves et al. 2003).

| Years after afforestation | Carbon loss (tC ha-1 a-1) |
| --- | --- |
| 0 | 0.0 |
| 1 | 2.2 |
| 2 | 3.8 |
| 3 | 2.5 |
| 4 | 1.1 |
| 5 | -0.3 |
| 6 | -1.2 |
| 7 | -1.6 |
| 8 | -1.6 |
| 9 | -1.3 |
| 10 | -1.1 |
| 15 | -0.2 |
| 20 | 0.1 |
| 25 | 0.2 |
| 30 | 0.3 |

The data reported for Category 5A and 5D/E is therefore under review in order to include these newer estimates of long-term carbon loss. The revision will also take into account estimates of the areas of deep peat afforested in the decades since 1920. The revised numbers will be reported in the CRF submission and NIR for 2002 and will include, as well as lower losses from deep peat, the carbon incorporated into the soil under the new conifer forests that were, until now, assumed to be offset by losses from the pre-existing soil. This will lead to a significant increase in the general level of removals of atmospheric carbon dioxide to Forests and Woody Biomass. Table 4 summarises the likely changes.

Table 4: Summary of effect of incorporation of new data on losses from afforested deep peats to be introduced in future CRF submissions and National Inventory Reports.  Data indicate likely scale of changes to Removals of CO2 to Forest and Woody Biomass (5A) and associated Emissions (5D/5E) due to planting on deep peats.

|  | Gg CO2 | 2001 CRF & NIR | 2002 CRF & NIR |
| --- | --- | --- | --- |
| 1990 | Removals1 | -9456 | -14261 |
| 1990 | Emissions2 | 1467 | 133 |
| 1995 | Removals1 | -10431 | -15150 |
| 1995 | Emissions2 | 1467 | 3 |
| 2001 | Removals1 | -10515 | -15245 |
| 2001 | Emissions2 | 1467 | 111 |

- Removals from atmosphere due to build up of carbon in new trees, litter and soil organic matter

- Emissions from deep peat soil pre-existing forest establishment

It is assumed in the carbon accounting model harvested material from thinning and felling is made into wood products. These products are then assumed to decay over a period equal to the rotation of the forest, conifer or broadleaf as appropriate, since products from broadleaves (e.g. furniture) will decay more slowly than those from conifers (e.g. paper, building timber). The net change in the carbon in this pool of wood products is reported in Category 5A. Calculated in this way that part of the total wood products pool from UK forests is presently increasing due to continuing expansion in forest area. Dewar and Cannell (1992) and Cannell and Dewar (1995) provided a detailed description of all the assumptions in the model.

# Forest and Grassland Conversion (5B)

In this, and previous National Inventory Reports, it has been assumed that permanent conversion of forest to non-forest in the UK has been negligible. This assumption was based on stringent government guidelines against deforestation, including the need for approval for any permanent forest felling from the Forestry Commission or equivalent in Northern Ireland. Review of this assumption suggests that some deforestation is happening where urban development is encroaching on old woodlands. This situation is covered by a different set of guidelines and, due to the need for new housing, permission for felling is more readily obtained. Only local planning authorities hold documentation for allowed felling and this makes estimating the national total difficult. However, in England, The Ordnance Survey (national mapping agency) makes an annual assessment of land use change (Office of The Deputy Prime Minister, 2003) from data it collects for map updating. These data suggest that over the period from 1985 to 1999 about 500 ha per annum of woodland was being converted to urban use, mostly to outdoor recreation and housing. Scaling by urban areas in Scotland, Wales and Northern Ireland suggests that an additional 100 ha per annum would be converting in those regions. Table 5 shows the range of GHG emissions that would result from deforestation in the range 500 to 1000 ha per annum. Appropriate data will therefore be included in future CRF submissions and NIRs on completion of data analysis.

Table 5: Range of possible emissions due to deforestation in the UK. Method based on IPCC 1996 Guidelines.

|  |  | LOW | MID | HIGH |  |
| --- | --- | --- | --- | --- | --- |
| Carbon in standing forest = |  | 1201 | 1201 | 1201 | tC ha-1 |
| ##### Fraction in products ~ |  | 0.62 | 0.62 | 0.62 |  |
| Fraction burned on site ~ |  | 0.4 | 0.4 | 0.4 |  |
| Area cleared = |  | 500 | 750 | 1000 | ha a-1 |
| Fraction oxidised = |  | 0.9 | 0.9 | 0.9 | (1996 Guidelines) |
| Immediate emissions of Carbon~ |  | 21.6 | 32.4 | 43.2 | ktC a-1 |
| ##### Immediate emissions of Nitrogen~ |  | 0.2 | 0.3 | 0.4 | ktN a-1 |
| Non-CO2 emissions | Factor (1996 Guidelines) |  |  |  |  |
| CH4 | 0.012 | 0.3 | 0.4 | 0.5 | ktC a-1 |
| CO | 0.06 | 1.3 | 1.9 | 2.6 | ktC a-1 |
| N2O | 0.007 | 0.001 | 0.002 | 0.003 | ktN a-1 |
| NOx | 0.121 | 0.026 | 0.039 | 0.052 | ktN a-1 |

- Typical for broadleaf woodland.

- Based on data used in estimating Forest Biomass Removals

# CO2 Emissions and Removals from Soils (5D)

Three processes are reported in this category: changes in soil stocks due to land use change, change in soil stocks due specifically to the change in land use from arable in Set Aside schemes and emissions due to the application of lime and dolomite.

## land use change

The basic method for assessing changes in soil carbon due to land use change is to use a matrix of change from surveys of land linked to a dynamic model of gain or loss of carbon.  In the latest version of the method matrices from the Monitoring Landscape Change (MLC) data from 1947 & 1980 and the DETR/ITE Countryside Surveys (CS) of 1984 & 1990 are used. Land use in the UK can be placed into 4 broad groups – (Semi) Natural, Farming, Woodland and Urban – and hence the more detailed categories for the two surveys were combined as shown in Table 7a for MLC and 6b for CS.  In both cases only unimproved grassland is included in the Natural category.  For the CS the different types of grass are shown in Table 8.

A database of soil carbon density for the UK has been constructed (Milne and Brown 1995, Cruickshank et al. 1998) from information provided by the Soil Survey and Land Research Centre, the Macaulay Land Use Research Institute and Queen’s University Belfast on soil type, land cover and carbon content of soil cores. These densities include carbon to a depth of 1 m or to bedrock whichever is the shallower, for mineral and peaty/mineral soils. Deep peats in the North of Scotland are identified separately and depths to 5 m are included but these play a minor role in relation to land use change. MLURI reviewed and revised downwards the values of soil carbon density for some peaty soils types in Scotland for the 1999 Inventory. Table 6 shows average values of soils carbon density for different land covers in the four devolved areas of the UK. The data of Table 6 shows no strong evidence of a major difference in the soil carbon density of tilled cropland or actively managed grass hence the inclusion of both uses within the Farm category.

Table 6: Average soil carbon density (t C ha-1) for different land cover in the UK

| Region Cover | England | Scotland | Wales | N. Ireland |
| --- | --- | --- | --- | --- |
| Natural | 487 | 1048 | 305 | 551 |
| Woodland | 217 | 580 | 228 | 563 |
| Arable | 153 | 156 | 93 | 151 |
| Pasture | 170 | 192 | 200 | 178 |
| Other | 33 | 141 | 43 | 102 |

Table 7a: Grouping of MLC land cover types for soil carbon change modelling.

| FARM | NATURAL | WOODLAND | URBAN |
| --- | --- | --- | --- |
| Crops | Upland heath | Broadleaved wood | Built up |
| Market garden | Upland smooth grass | Conifer wood | Urban open |
| Improved grassland | Upland coarse grass | Mixed wood | Transport |
| Rough pasture | Blanket bog |  | Mineral workings |
|  | Bracken |  | Derelict |
|  | Lowland rough grass |  |  |
|  | Lowland heather |  |  |
|  | Neglected grassland |  |  |
|  | Marsh |  |  |

Table 7b: Grouping of CS land cover types for soil carbon change modelling. For Managed grass (I) signifies “Improved”, usually by ploughing and seeding, (U) signifies “Unimproved” by such means.

| FARM | NATURAL | WOODLAND | URBAN |
| --- | --- | --- | --- |
| Tilled land | Rough grass/marsh | Broadleaved/mixed | Communications |
| Managed grass(I) | Managed grass (U) | Coniferous | Built up |
|  | Dense bracken |  | Inland bare (Hard areas) |
|  | Moorland grass |  |  |
|  | Dense heath |  |  |
|  | Open heath |  |  |

Table 8: Different types of CS land cover included in the “Improved” and “Unimproved” groups for soil carbon modelling.

| Managed grass (I) | Managed grass (U) |
| --- | --- |
| Recreational | Non-agricultural improved |
| Recently sown | Calcareous |
| Pure rye | Upland |
| Well managed |  |
| Weedy swards |  |

Table 9: Area and change data sources for different periods in estimation of changes in soil carbon. (1) Stamp (1962), (2) MLC (1986), (3) Barr et al. (1993).

| Year or Period | Area data | Change matrix or data |
| --- | --- | --- |
| 1930 | Land use Survey (1) |  |
| 1930 – 1947 | Interpolated | MLC 1947->MLC1980 |
| 1947 | MLC (2) |  |
| 1947-1980 | Interpolated | MLC 1947->MLC1980 |
| 1980 | MLC (2) |  |
| 1980-1984 | Interpolated | Interpolated |
| 1984 | CS1984 (3) |  |
| 1984-1990 | Interpolated | CS1984->CS1990 |
| 1990 | CS1990 (3) |  |
| 1990-2010 | Extrapolated from 84->90 | CS1984->CS1990 |

Area data exist for the period 1930 to 1990 and those from 1984 to 1990 are used to extrapolate forward for the years 1991 to 1998. Land use change matrices for the periods 1947 to 1980 and 1984 to 1990 are used. See Table 9 for the sources of information for land use and matrices of change.

The core equation describing changes in soil carbon with time for any land use transition is

Ct=Cf−(Cf−C0)e−kt

Ct is carbon density at time t

C0 is carbon density initially

Cf  carbon density after change to new land use

k is time constant of change

If the inventory year is 1990 and AT is area in a particular land use transition in year T considered from 1930 onwards then total carbon lost or gained from 1930 to 1990 (X1990) and from 1930 to 1989 (X1989) is given by

X1990=T=1930t=1990AT(C0−Cf)(1−e−k(1990−T))

X1989=T=1930T=1989AT(C0−Cf)(1−e−k(1989−T))

Hence flux of carbon in 1990 is given by difference:

F1990=X1990−X1989

The land use transitions considered are each of those between the (Semi) Natural, Farm, Woodland and Urban categories. Scotland, England and Wales are treated separately. Northern Ireland does not yet have a matrix of land use change and changes in soil carbon are calculated by a method based on that recommended by the IPCC (1997b, c). The area data for Great Britain are shown in Table 10. The data from the CS has had a small adjustment applied to account for one of the detailed land types (Non-cropped arable) actually bridging the main Natural and Farm categories.

Table 10a:Area of land in England for each use category from field and area surveys (1) Stamp (1962), (2) MLC (1986), (3) Barr et al. (1993).

|  |  | Area (ha) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Source | Year | Farm | Natural | Urban | Woodland |
| lus (1) | 1930 | 9,542,340 | 1,543,000 | 1,034,858 | 843,800 |
| mlc (2) | 1947 | 9,242,777 | 1,639,511 | 823,665 | 865,370 |
| mlc (2) | 1980 | 9,013,401 | 1,307,178 | 1,301,965 | 948,779 |
| cis (3) | 1984 | 8,670,815 | 1,908,436 | 1,249,383 | 1,303,455 |
| cis (3) | 1990 | 8,336,428 | 2,120,609 | 1,323,084 | 1,353,399 |

Table 10b: Area of land in Wales for each use category from field and area surveys (1) Stamp (1962), (2) MLC (1986), (3) Barr et al. (1993).

|  |  | Area (ha) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Source | Year | Farm | Natural | Urban | Woodland |
| lus (1) | 1930 | 1,094,187 | 771,520 | 77,298 | 120,439 |
| mlc (2) | 1947 | 1,061,571 | 701,347 | 71,422 | 160,077 |
| mlc (2) | 1980 | 1,148,150 | 521,131 | 121,459 | 203,677 |
| cis (3) | 1984 | 1,155,174 | 585,248 | 176,112 | 221,521 |
| cis (3) | 1990 | 1,132,768 | 593,918 | 188,628 | 222,953 |

Table 10c: Area of land in Scotland for each use category from field and area surveys (1) Stamp (1962), (2) MLC (1986), (3) Barr et al. (1993).

|  |  | Area(ha) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Source | Year | Farm | Natural | Urban | Woodland |
| lus (1) | 1930 | 1,861,215 | 5,265,673 | 146,906 | 443,187 |
| mlc (2) | 1947 | 2,037,860 | 5,209,630 | 260,313 | 447,753 |
| mlc (2) | 1980 | 2,100,125 | 4,667,711 | 297,076 | 890,644 |
| cis (3) | 1984 | 2,109,333 | 4,940,892 | 287,471 | 1,019,931 |
| cis (3) | 1990 | 2,059,553 | 4,935,184 | 294,291 | 1,068,543 |

In the model, the change is required in equilibrium carbon density from the initial to the final land use during a transition.  Here, these are calculated for each land use category as averages for Scotland, England and Wales.  In order to account for variation in carbon density and Land Use Change in different soil types these averages are weighted by the area of soil groups used by IPCC (1997c).  They define five groups, which are represented in Great Britain, on the basis of their carbon content and activity namely: aquic, high activity clay, and low activity clay, sandy and organic.  In Great Britain few clay soils truly fall into the ‘high activity’ class so the total clay content is used to divide these soils into ‘high’ and ‘low’ groups.  For Great Britain all soil types not falling into these five types an ‘undefined’ groups is used.  Mean soil carbon density change are calculated as:

Cijc=s=16(CsijcLsijc)s=16Lsijc

which is the weighted mean, for each country, of change in equilibrium soil carbon when land use changes and

i = initial land use (Natural, Farm, Woods, Urban)

j = new land use   (Natural, Farm, Woods, Urban)

c = country   (Scotland, England & Wales)

s = soil group (High clay, low clay, aquic, organic, sandy, undefined)

Csijc is change in equilibrium soil carbon for a specific land use transition

within a soil group region in a specific country

Lsijc is area change (1984 to 1990) for a specific land use transition within a soil group region in a specific country.

The rate of loss or gain of carbon is dependent on the type of land use transition (Table 12). For transitions where carbon is lost e.g. transition from Natural to Farm land, a ‘fast’ rate is applied whilst a transition that gains carbon occurs much more slowly. This ’slow’ rate had in the 1998, and earlier, GHG Inventories been set such that 99% of the change occurred in 100 years throughout GB as had been observed at Rothamsted (Howard et al. 1994). However, it was observed that due to the high carbon densities in Scottish soils that the uptake rates of carbon in that country were unreasonably large when land moved to the Natural class from the Farm class. For the 1998 Inventory the rate of uptake was therefore reduced until the uptake of soil carbon in such transitions was less than the order of net primary productivity for cold temperate grasslands (about 300 g m-2 a-1). Thus a rate of soil carbon accumulation in Scotland that took the equivalent of 800 years to reach 99% of the new values was used. Since the 1999 Inventory, a different approach to taking account of the uncertainty in such rates of transition has been adopted. A literature search for information on measured rates of changes of soil carbon due to land use was carried out and, in combination with expert judgement, ranges of possible times for completion of different transitions were selected. These are shown in Table 11.

Table 11: Range of times for soil carbon to reach 99% of a new value after a change in land use in England (E), Scotland (S) and Wales (W).

|  | Low (years) | High (years) |
| --- | --- | --- |
| Carbon loss (“fast”) E, S, W. | 50 | 150 |
| Carbon gain (“slow”) E, W. | 100 | 300 |
| Carbon gain (“slow”) S. | 300 | 750 |

Table 12: Rates of change of soil carbon for land use change transitions. (“Fast” & “Slow” refer to 99% of change occurring in times shown in Table 11.

|  |  | 1984 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Farm | Natural | Urban | Woods |
| 1990 | ###### Farm |  | fast | slow | fast |
|  | Natural | slow |  | slow | fast |
|  | Urban | fast | fast |  | fast |
|  | Woods | slow | slow | slow |  |

The model of change was then run 500 times with the time constant for change in soil carbon being selected separately using a Monte Carlo approach for England, Scotland and Wales from within the ranges of Table 11. The mean carbon flux for each region resulting from this imposed random variation was then reported as the estimate for the Inventory. An adjustment was made to these calculations for each country to remove increases in soil carbon due to afforestation, as the value for this was considered to be better estimated by the C-Flow model used for the Changes in Forests and Other Woody Biomass Stocks (5A) category.

Variations from year to year in the reported net emissions reflect the trend in land use change as described by the matrix of change between 1984 and 1990. New survey data covering changes between 1990 and 1998 has recently become available and this will be used to improve estimation of the trend of emissions and removals in future Inventories

## set aside

The estimation of changes in soil carbon calculated by the matrix method for all transitions does not fully include the effects of the policy of Set Aside from production of arable areas.  This is the case because although the schemes were introduced in 1988 there was a slow rate of acceptance by farmers and it was not until after 1990 that significant area is recorded in the Annual Farm Census.  In this post-1990 period the matrix method uses an extrapolation of the CS field data from 1984 to 1990 therefore a separate estimate of the effect of Set Aside on soil carbon for these later years has been made.  Data reported in inventories prior to 1997 were based on the observation from the Annual Farm Census that Set Aside was continuing to increase in total area.  However from more recent Census data it would seem that the total area has now passed its maximum and is beginning to fall.  This reflects the fact that the Schemes will be phased out, to be replaced with others with different objectives.  The data reported here therefore take into account not only the effect of soil carbon increasing in areas where land is not used for arable purposes but the subsequent loss of the extra accumulated carbon from the soil when land is returned to arable use.

Set Aside areas are taken from the Annual Farm Census for Scotland and England & Wales separately.  Scottish soils coming out of arable use are assumed to be able to take up 300 t/ha but that this happens at a rate that would only allow 99% of that change to occur in 500 years.  For English & Welsh soils it is assumed that the change in equilibrium soil carbon density would be 60 t/ha and that 99% of this change would occur in 200 years. These times fall in the middle of the ranges used in the main calculation for the effect of land use change causing an increase in soil carbon. The new areas of land in Set Aside are calculated from the increases in area up to the maximum total recorded area (in 1995 throughout GB).  The emission of carbon from these areas are calculated for years up until 1999 when it is assumed that all land will have returned to arable.  To compensate for the reducing area, two assumptions were made: a) the area lost in each year from 1995 onwards was assumed to have been in Set Aside for 3 years and b) the carbon gained in these 3 years would be lost at a rate which would cause 99% of the change to occur in 20 years.  The 3-year assumption is made, as there is no clear indication of how long any area does remain in Set Aside. This value is not unreasonable but may be low given that some Set Aside could have existed from 1988. Prior to the 1998 Inventory it was assumed that all Set Aside was simply abandoned but between 30 and 50% is actually managed by cutting etc.  Such areas will not be very different from other rotational pasture situations that we have already shown to have similar soil carbon to arable areas.  Hence such areas have been excluded from estimates of the effect of Set Aside reported here.

Thus for the estimates reported here the assumptions are: Set Aside area rises to a maximum in 1995 then falls away to zero by 1999, uptake occurs slowly in Scotland and 50% of areas in the Agricultural Census are in rotational form of management are excluded. Northern Ireland has negligible change in soil carbon due to Set Aside.

## emissions of CO2 from soild due to liming

Emissions of carbon dioxide from the application of limestone, chalk and dolomite to agricultural soils were estimated.  Data on the use of limestone, chalk and dolomite for agricultural purposes is reported in BGS (2002). Estimates of these individual materials had to be made this year as only their total was published because of commercial confidentiality rules for small quantities. It is assumed that all the carbon contained in the lime is released in the year of use.  For limestone and chalk, a factor of 120 t C/kt is used, and for dolomite application, 130 t C/kt.  These factors are based on the stoichiometry of the reaction and assume pure limestone and dolomite.

# Other Sources and Sinks (5E)

These are:

A sink

- Changes in crop biomass

and 3 sources

- Drainage of upland deep peat

- Drainage of lowland wetlands

- Peat extraction

The activity data and carbon fluxes are based on data from Adger and Subak (1996) for the sink and from Bradley (1997), Cannell et al. (1993), Cruickshank et al. (1997), Hargreaves and Fowler (1997) for the sources.

## Changes in crop biomass

Adger and Subak (1996) estimated recent changes in carbon storage in biomass on non-forest lands in the U.K., including land used for agriculture, horticulture and urbanization. The land area converted to forest was specifically excluded to avoid overlap with estimates for Category 5A. They used agricultural census statistics for the period 1988-1992 published by the Ministry of Agriculture, Fisheries and Food. These statistics are strongly correlated with agricultural land cover data in 1984 and 1990 U.K. Countryside Surveys, which were used to calculate changes in soil carbon on non-forest lands, so the two estimates are considered to be compatible.

Two carbon sinks were quantified. First, 0.23 MtC a-1 was estimated to be accumulating in biomass as a result, mainly, of (i) the transfer of land from arable crops with 2.2 tC ha-1 biomass to set-aside land with 5.0 tC ha-1 biomass, (ii) the establishment of woodlands on farms in response to financial incentives (Farm Woodland Scheme and Farm Woodland Premium), assuming that these woodlands increased in biomass by 2.8 tC ha-1 a-1, (iii) the transfer of agricultural land to urban uses, assuming that urban land has an average carbon density of 3 tC ha-1  and (iv) the transfer of rough grass to permanent grass.

Second, 0.14 MtC a-1 was estimated to be accumulating on agricultural land, without a change in crop type, on the assumption that the annual average standing biomass has increased linearly with yield. Most of this component was due to increases in cereal yields.

Thus, the total increase in biomass on agricultural land was estimated to be 0.37 MtC a-1.  However, this is an upper bound, because some of the farm woodlands were also counted in Forestry Commission statistics which were used to calculate the forest biomass carbon for Category 5A, and because increases in `harvest index' mean that crop biomass generally increases proportionately less than yield. Thus, the lower estimate for this component of 0.3 MtC a-1 ±30% has been adopted. From the 1998 Inventory onwards more recent data from the Agricultural Census were considered but did not support any change to the existing estimate. This rate is therefore reported for all years from 1990 to 2000.

## peat extraction

Trends in peat extraction in Scotland and England over period 1990 to 2000 are estimated from activity data taken from the UK Minerals Handbook (BGS 2002). In Northern Ireland no new data on use of peat for horticultural use was available and a recent survey of extraction for fuel use suggested that there is no significant trend for this purpose. The contribution of emissions due to peat extraction in this region is therefore incorporated as constant from 1990 to 2001. Peat extraction is negligible in Wales. Emissions factors are from Cruickshank et al. (1997) and are shown in Table 13.

Table 13: Emission Factors for Peat Extraction (GB Great Britain, NI Northern Ireland)

|  | Emission | Factor |
| --- | --- | --- |
|  | kg C m-3 | Gg C/Gg |
| GB Horticultural Peat | 55.7 | - |
| GB Fuel Peat | 55.7 | - |
| NI Horticultural Peat | 44.1 | - |
| NI Fuel Peat | - | 0.3 |

## lowland (fen) peat drainage (reported in Category 5D in CRF)

The baseline (1990) for the area of drained lowland wetland for the UK was taken as 150 kha. This represents all of the East Anglian Fen and Skirtland and limited areas in the rest of England. This total consists of 24 kha of land with thick peat (more than 1m deep) and the rest with thinner peats. Different loss rates were assumed for these two thicknesses as shown in Table 14.

Table 14: Area and carbon loss rates of UK fen wetland in 1990.

|  | Area | Organic carbon content | Bulk density kg m-3 | Volume loss rate m3 m-2 a-1 | Carbon mass loss GgC a-1 | Implied emission factor gC m-2 a-1 |
| --- | --- | --- | --- | --- | --- | --- |
| ‘Thick’ peat | 24x107 m2 (24 kha) | 21% | 480 | 0.0127 | 307 | 109 |
| ‘Thin’ peat | 126x107 m2 (126 kha) | 12% | 480 | 0.0019 | 138 | 1280 |
| Total | 150x107 m2 (150 kha) |  |  |  | 445 | 297 |

The trend in emissions after 1990 was estimated on the assumption that no more area has been drained since then but the existing areas have continued to lose carbon. The annual loss decreases for a specific location in proportion to the amount of carbon remaining. But, in addition to this, as the peat loses carbon it will become more mineral in structure. Burton (1995) provides data on how these soil structure changes proceed with time. The Century model of plant and soil carbon was used to average the carbon losses for the areas of component soils as they thinned to lose peat, become humose and possibly even mineral (Bradley 1997)

## upland (forestry) peat drainage (reported in Category 5D in CRF)

The data included in this and previous year’s CRF submissions and NIR’s for emissions from drainage of peat for forestry are based on areas of planted forest in Cannell et al. (1993) (see Table 15) and an emission rate measured by Hargreaves and Fowler 1997 and Hargreaves et al. (2003) measured in the field one year after forest planting. The value for emission is assumed to continue indefinitely at about at this early rate. The continuing rate is taken to be 2tC ha-1 a-1 (The resulting emission is therefore the same for each year of the Inventory. (Table 15).

This approach is under review following analysis of field data measured for forests with ages up to 26 years after planting (Hargreaves et al. 2003) The likely effect on emissions and removals is discussed earlier in this Appendix (Section 2).

Table 15: Activity and Emission Factor Data for Upland Drainage

|  | Afforested peat (kha) | Emission rate (tC ha-1 a-1) | Annual loss (ktC) |
| --- | --- | --- | --- |
| England | 20 | 2 | 40 |
| Wales | 10 | 2 | 20 |
| Scotland | 160 | 2 | 320 |
| Northern Ireland | 10 | 2 | 20 |
| UK | 200 | 2 | 400 |

# Data Presentation Formats

Emissions and removals of CO2 for the Land Use Change and Forestry Sector are presented here in a different format that that used for the Common Reporting Format (CRF) submission to the UNFCCC. This has arisen since clarification was given in the guidance for use of the CRF on how emissions and removals for soils should be treated. However, to maintain historical consistency, the format for reporting in the LUCF Sector used in this National Inventory Report has not been changed from that used previously. Table 16 shows the processes affected. Tables 17, 18a and 18b give more detail on the effect of the different formats on the reported emissions and removals. The reported net emission for the LUCF Sector is not affected by the use of the different formats.

Table 16: Categories used for reporting soils emissions and removals in National Inventory Report and UNFCCC Common Reporting Format

| Process | National Inventory Report | Common Reporting Format |
| --- | --- | --- |
| Removals to forest soils and litter | 5A2 (Removal) | 5D (Removal) |
| Removals to agricultural soils due to Set Aside | As part of a net emission in 5D (Emission) | 5D (Removal) |
| Emissions from soils due to upland drainage | 5E (Emission) | 5D (Emission) |
| Emissions from soils due to lowland drainage | 5E (Emission) | 5D (Emission) |

Table 17 Emissions and removals of carbon dioxide by processes in Land Use Change and Forestry Sector. The reporting categories used in the National Inventory Report and for the UNFCCC Common Reporting Format are also shown. (IE Included Elsewhere.)

| Process | Gg CO2 | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | NIR | CRF |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperate forest | Removal | -5731.00 | -5819.00 | -6321.33 | -6864.00 | -7095.00 | -7604.67 | -7271.00 | -7205.00 | -6985.00 | -6827.33 | -7157.33 | -6845.67 | 5A2 | 5A2 |
| Other (Harvested wood) | Removal | -1573.00 | -1576.67 | -1349.33 | -1118.33 | -1096.33 | -913.00 | -1155.00 | -1111.00 | -1199.00 | -1294.33 | -1151.33 | -1298.00 | 5A5 | 5A5 |
| Forest soils | Removal | 14186.61 | 14144.58 | 13879.86 | 13231.45 | 13438.55 | 12809.63 | 12842.92 | 12315.72 | 12106.06 | 12197.79 | 11743.74 | 11649.24 | 5A2 | 5D |
| Mineral soils | Emission | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE | 5D | 5D |
| Mineral soils | Removal | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE | 5D | 5D |
| Organic soils | Emission | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE | 5D | 5D |
| Organic soils | Removal | 1430.45 | 1771.95 | 1809.58 | 1130.32 | 1269.82 | 1529.44 | 1515.16 | 1346.38 | 1058.05 | 886.93 | 794.18 | 729.34 | 5D | 5D |
| Liming | Emission | -2152.33 | -2170.67 | -2075.33 | -1991.00 | -1994.67 | -1910.33 | -2086.33 | -2141.33 | -2244.00 | -2317.33 | -2244.00 | -2372.33 | 5D | 5D |
| Other (Set Aside) | Removal | -177.75 | -263.86 | -351.14 | -465.25 | -1897.02 | -2006.88 | -1542.40 | -1006.69 | -136.08 | -298.09 | -1071.21 | -1124.96 | 5D | 5D |
| Other (Set Aside) | Emission | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 5D | 5D |
| Other (Lowland Drainage) | Emission | 1650.00 | 1613.33 | 1576.67 | 1540.00 | 1503.33 | 1466.67 | 1430.00 | 1393.33 | 1356.67 | 1320.00 | 1283.33 | 1261.33 | 5E | 5D |
| Other (Upland Drainage) | Emission | 1466.67 | 1466.67 | 1466.67 | 1466.67 | 1466.67 | 1466.67 | 1466.67 | 1466.67 | 1466.67 | 1466.67 | 1466.67 | 1466.67 | 5E | 5D |
| Peat extraction | Emission | 791.57 | 802.60 | 791.57 | 780.54 | 888.79 | 949.85 | 868.98 | 814.85 | 703.55 | 821.59 | 816.08 | 854.68 | 5E | 5E |
| Changes in crop biomass | Removal | -1100.00 | -1100.00 | -1100.00 | -1100.00 | -1100.00 | -1100.00 | -1100.00 | -1100.00 | -1100.00 | -1100.00 | -1100.00 | -1100.00 | 5E | 5E |

Table 18a Emissions and removals in categories with the Land Use Change and Forestry Sector as reported in the format used for the National

Inventory Report. (IE Included Elsewhere.)

| NIR | Gg Carbon | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | Category |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperate forest | Removal | -7883.33 | -7989.67 | -8396.67 | -8855.00 | -9089.67 | -9515.00 | -9357.33 | -9346.33 | -9229.00 | -9144.67 | -9401.33 | -9218.00 | 5A2 | Sum of removals to forest biomass, forest litter and forest soil |
| Harvested wood | Removal | -1573.00 | -1576.67 | -1349.33 | -1118.33 | -1096.33 | -913.00 | -1155.00 | -1111.00 | -1199.00 | -1294.33 | -1151.33 | -1298.00 | 5A5 | Removals to Harvested wood |
| Soils | Emission | 15439.30 | 15652.66 | 15338.31 | 13896.53 | 12811.34 | 12332.19 | 12815.68 | 12655.41 | 13028.03 | 12786.63 | 11466.71 | 11253.62 | 5D | Sum of emissions from soils and removals to soils due to land use change (not forestry), Set Aside and liming of agricultural land |
| Soils | Removal | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE | IE |  | 5D |  |
| Sub Total | Emission | 3908.24 | 3882.60 | 3834.90 | 3787.21 | 3858.79 | 3883.19 | 3765.64 | 3674.85 | 3526.88 | 3608.26 | 3566.08 | 3582.68 | 5E | Sum of emissions from soils due to upland drainage, lowland drainage and peat extraction |
| Sub Total | Removal | -1100.00 | -1100.00 | -1100.00 | -1100.00 | -1100.00 | -1100.00 | -1100.00 | -1100.00 | -1100.00 | -1100.00 | -1100.00 | -1100.00 | 5E | Increases in crop biomass |
| Total | Emission | 19347.54 | 19535.26 | 19173.21 | 17683.73 | 16670.13 | 16215.38 | 16581.32 | 16330.26 | 16554.91 | 16394.89 | 15032.79 | 14836.30 | 5 | Gross LUCF Emissions |
| Total | Removal | -10556.3 | -10666.3 | -10846.0 | -11073.3 | -11286.0 | -11528.0 | -11612.3 | -11557.3 | -11528.0 | -11539.0 | -11652.7 | -11616.0 | 5 | Gross LUCF Removals |
| Total | Net | 8791.21 | 8868.93 | 8327.21 | 6610.40 | 5384.13 | 4687.38 | 4968.99 | 4772.93 | 5026.91 | 4855.89 | 3380.13 | 3220.30 | 5 | Net LUCF Emissions |

| Table 18b Emissions and removals in categories with the Land Use Change and Forestry Sector as reported in the format used for the UNFCCC Common Reporting Format. (IE Included Elsewhere.) CRF | Gg Carbon | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | Category |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperate forest | Removal | -5731.00 | -5819.00 | -6321.33 | -6864.00 | -7095.00 | -7604.67 | -7271.00 | -7205.00 | -6985.00 | -6827.33 | -7157.33 | -6845.67 | 5A2 | Removals to forest biomass. |
| Harvested wood | Removal | -1573.00 | -1576.67 | -1349.33 | -1118.33 | -1096.33 | -913.00 | -1155.00 | -1111.00 | -1199.00 | -1294.33 | -1151.33 | -1298.00 | 5A5 | Removals to Harvested wood |
| Soils | Emission | 18733.72 | 18996.53 | 18732.78 | 17368.44 | 17678.37 | 17272.40 | 17254.75 | 16522.10 | 15987.44 | 15871.38 | 15287.92 | 15106.59 | 5D | Sum of emissions from soils due to land use change on agricultural soils (net emissions), upland drainage, lowland drainage and liming of agricultural land |
| Soils | Removal | -2330.08 | -2434.53 | -2426.47 | -2456.25 | -3891.69 | -3917.21 | -3628.74 | -3148.03 | -2380.08 | -2615.42 | -3315.21 | -3497.30 | 5D | Sum of removals to forest soils and litter and to agricultural soils due to Set Aside. |
| Other | Emission | 791.57 | 802.60 | 791.57 | 780.54 | 888.79 | 949.85 | 868.98 | 814.85 | 703.55 | 821.59 | 816.08 | 854.68 | 5E | Emissions from soils due to peat extraction |
| Other | Removal | -1100.00 | -1100.00 | -1100.00 | -1100.00 | -1100.00 | -1100.00 | -1100.00 | -1100.00 | -1100.00 | -1100.00 | -1100.00 | -1100.00 | 5E | Removals due to increases in crop biomass |
| Total | Emission | 19525.29 | 19799.13 | 19524.35 | 18148.98 | 18567.15 | 18222.25 | 18123.73 | 17336.95 | 16690.99 | 16692.98 | 16104.00 | 15961.27 | 5 | Gross LUCF Emissions |
| Total | Removal | -10734.08 | -10930.20 | -11197.14 | -11538.58 | -13183.02 | -13534.88 | -13154.74 | -12564.03 | -11664.08 | -11837.09 | -12723.87 | -12740.96 | 5 | Gross LUCF Removals |
| Total | Net | 8791.21 | 8868.93 | 8327.21 | 6610.40 | 5384.13 | 4687.38 | 4968.99 | 4772.93 | 5026.91 | 4855.89 | 3380.13 | 3220.30 | 5 | Net LUCF Emissions |

# Quality Assurance Methods and Standards

CEH has put in place high quality assurance standards, and selects subcontractors from professional organisations who meet those standards. The general standards are:

The use of professionally qualified staff.

The application of rigorous quality control procedures.

The use of modern equipment.

The use of validated methods.

The quality control and curation of databases.

The establishment of management procedures to ensure compliance.

The particular quality control measures relevant to this report are as follows.

Databases

The databases used to calculate carbon sources and sinks are all quality controlled at source by the responsible organisation, e.g. CEH for land use, SSLRC and MLURI for soils and FC for forestry statistics.

Models

All modelling is done by trained staff who now have many years’ experience of simulating changes in soil and biomass carbon. The output of models is checked against quality assured data. Predictions of future sources and sinks are bench marked against predictions made by other researchers in Europe through a COST E21, other research meetings and the scientific literature.

Output

The integrity of results, the quality of the reports, the relationship to contracted deliverables and the punctuality of reporting, are all subject to management vetting and tracking within CEH, through the Heads of Sections, Directors of Sites and the Finance Administration. Additionally, all staff are required to publish as much non-confidential scientific information in the peer reviewed scientific literature, with the prior approval of the customer and customer acknowledgement.

Field measurements of sources and sinks

All fieldwork, sampling and data handling is done by experienced and trained staff to defined protocols agreed to meet the objectives of the work. The procedures for flux measurement are fully documented and instruments are calibrated directly with primary standards.

Chemical analysis

All chemical analyses are done at CEH, Merlewood and are supported by full quality assurance and control procedures under BS 5750. The integrity of results is checked by conducting bi-monthly inter-laboratory comparisons (Aquachecks and the International Soil Exchange Scheme).

# References

Adger, WN & Subak, S. (1996) Estimating above-ground carbon fluxes from U.K. agricultural land, Geographical Journal, Vol. 162, pp 191-204.

Barr, CJ, Bunce, RGH, Clarke, RT, Fuller, RM., Furse, MT, Gillespie, MK., Groom, GB, Hallam, C.J, Hornung, M, Howard, DC, Ness, MJ, (1993) Countryside Survey 1990, Main Report. London, Department of the Environment.

BGS (2001), United Kingdom Minerals Yearbook 1, British Geological Survey, Natural Environment Research Council

Bradley, I. (1997) Carbon loss from drained lowland fens. In: Carbon Sequestration in Vegetation and Soils (Ed. by MGR Cannell), DOE/ITE Contract EPG 1/1/3. Final Report March 1997. Department of Environment, London.

Burton, R.G.O. (1995) Evaluating organic matter dynamics in cultivated organic topsoils – use of historical analytical data. MAFF Contract to SSLRC No. LE0203 –81/3830.

Cannell, MGR, Dewar, RC. (1995), The carbon sink provided by plantation forests and their products in Britain, Forestry, Vol. 68, No. 1, pp 35-48.

Cannell, MGR, Milne, R, Hargreaves, KJ, Brown, TAW, Cruickshank, MM, Bradley, RI, Spencer, T, Hope, D, Billett, MF, Adger, WN & Subak, S (1999) National inventories of terrestrial carbon sources and sinks: the UK experience. Climatic Change 42, 505-530

Cruickshank, MM, Tomlinson, RW, Devine, PM, Milne, R (1997) Effects of agricultural, management on carbon storage in Northern Ireland. In: Carbon Sequestration in Vegetation and Soils (Ed. by MGR Cannell), DOE Contract EPG 1/1/3. Final Report March 1997. Department of Environment, London.

Cruickshank, MM, Tomlinson, RW (1997) Carbon loss from UK peatlands for fuel and horticulture. In: Carbon Sequestration in Vegetation and Soils (Ed. by MGR Cannell), DOE, Contract EPG 1/1/3. Final Report March 1997. Department of Environment, London.

Cruickshank, MM, Tomlinson, RW, Devine, PM and Milne, R (1998) Carbon in the vegetation and soils of Northern Ireland. Proceedings of the Royal Irish Academy, 98B, 9 - 21.

Dewar, RC, Cannell, MGR, (1992) Carbon sequestration in the trees, products and soils of forest plantations: an analysis using UK examples. Tree Physiology  Vol. 11, pp49 - 72.

DOE, (1997) Climate Change: The UK Programme; United Kingdom’s Second Report under the Framework Convention on Climate Change, The Stationary Office, London.

Forestry Commission (2002) National Inventory of Woodland and Trees http://www.forestry.gov.uk/forestry/hcou-54pg4d

Hargreaves, K and Fowler, D (1997) Short-term CO2 fluxes over peatland. In: Carbon Sequestration in Vegetation and Soils (Ed. by MGR Cannell), DOE Contract EPG 1/1/3. Final Report March 1997. Department of Environment, London

Hargreaves, K.J., Milne, R. and Cannell, M.G.R. (2003)  Carbon balance of afforested peatland in Scotland. Forestry (In press).

Howard, PJA and  Howard, DM (1994) Modelling the effects of land use change and climate change on soil organic carbon stores. In: Carbon sequestration by soils in the UK, Report to the Department of the Environment, Contract No. PECD 7/12/80, March 1994.

IPCC, (1997a), IPCC Revised 1996 Guidelines for National Greenhouse Gas Inventories, Volume 1, Greenhouse Gas Inventory Reporting Instructions, IPCC WGI Technical Support Unit, Hadley Centre, Meteorological Office, Bracknell, UK.

IPCC, (1997b), IPCC Revised 1996 Guidelines for National Greenhouse Gas Inventories, Volume 2, Greenhouse Gas Inventory Workbook, IPCC WGI Technical Support Unit, Hadley Centre, Meteorological Office, Bracknell, UK.

IPCC, (1997c), IPCC Revised 1996 Guidelines for National Greenhouse Gas Inventories, Volume 3, Greenhouse Gas Inventory Reference Manual, IPCC WGI Technical Support Unit, Hadley Centre, Meteorological Office, Bracknell, UK.

Milne, R and Brown, TA (1997) Carbon in the vegetation and soils of Great Britain. Journal of Environmental Management, 49, 413 - 433.

Milne, R, and Brown, TA (1999) Methods and data for Land Use Change and Forestry Sector in the 1997 IPCC Greenhouse Gas Inventory. In.: Carbon Sequestration in Vegetation and Soils (Ed. by R. Milne), DETR Contract EPG 1/1/39. Interim Report April 1999.

Milne, R, Brown, TAW and Murray, TD (1998)  The effect of geographical variation in planting rate on the uptake of carbon by new forests of Great Britain. Forestry, 71, 298 – 309.

MLC (1986) Monitoring Landscape Change Vols. 1, 1A & 10. Report prepared by Hunting Surveys & Consultants Ltd for Department of the Environment and the Countryside Commission.

Office of The Deputy Prime Minister (2003) Land Use Change Statistics Index.  http://www.planning.odpm.gov.uk/luc/lucindex.htm

Stamp, LD (1962) The Land of Britain: Its Use and Misuse. London, Longman.

The Land Use Change and Forestry chapter was provided by R.Milne

Centre for Ecology & Hydrology, Bush Estate, Penicuik, EH26 0QB