# Greenhouse Gas Emissions Inventories Report from Spain
1990-2002

communication to the european commission
(decision 2004/280/eC)

Directorate-General for Environmental 
Quality and Assessment

Ministry of the Environment

Madrid, March 2004

### contents

1.- PRESENTAtion	1

2.- GENERAL METHODOLOGY	3

3.- DEvelopment of the inventory principles	7

4.- RECALCULATIONS	10

5.- RESULTS AND KEY SOURCES ANALYSIS	24

6.- RESULTS COMPARISON (1990-2001) ed. 2004 vs. ed. 2003	54

7. KEY SOURCES ANALYSIS	65

APPENDIX methodology for carbon dioxide estimation in the forestry sector	143

##### 1.- PRESENTAtion

The present document is the edition corresponding to 2004 of the report on the 1990-2002 inventories of Greenhouse Gases (GHG) presented by Spain to the Commission of the European Union by virtue of the provisions of Council Decision 2004/280/EC, repealing that of 93/389/EEC, regarding a Monitoring Mechanism of Community Carbon Dioxide and other greenhouse gas emissions.

The presentation of the inventories is carried out in the Common Reporting Format (CRF) defined in document FCCC/CP/2002/8 drawn up by the United Nations Secretariat on the Framework Convention on Climatic Change.

As for the substances referred to in the inventory, the data in the CRF tables are given both in units of mass for each gas and in terms of CO2-equivalent for each of the compounds/groups considered in Annex A of Kyoto Protocol and that includes the six categories below with a direct effect on global warming: carbon dioxide (CO2), methane (CH4), nitrous oxide (N2O), hydrofluorocarbons (HFC); perfluorocarbons (PFC), and sulphur hexafluoride (SF6). In addition, the estimated emissions of three gases with an indirect warming effect: nitrogen oxides (NOX), carbon monoxide (CO), non-methane volatile organic compounds (NMVOC), as well as those of sulphur oxides (SOX) are given in terms of mass. The amounts sequestered by sinks have been estimated solely with reference to the CO2 fixed as carbon by tree-forest biomass.

The current edition of the inventories updates, and revises in some cases, the estimates provided in the previous edition for the years in the period 1990-2001, at the same time as it extends the time series to 2002. Where necessary, the revision of the estimates for certain items in the inventories has been due to various factors including: a) the review of the input data and statistics themselves, b) changes in the estimation methodologies (methods choices, factors and algorithms) as a result of the improved awareness of the emissions-generating processes, and c) in some cases, the correction of errors detected. The following sections 2, “General Methodology” and 3, “Development of Inventory Principles” summarize the methodologies and approaches used to estimate the emissions and describe to which extent the recommended good practices have been followed for the compilation of these inventories. Revisions to the previous inventories from 1990-2001 are described in section 4, “Recalculations”; these are included in this edition with relation to the estimates presented in the previous edition. In section 5 “Summary of Results, Key Sources Identification and Uncertainty Analysis”, the results as accumulated inventory, by type of gas and activity sector are presented first, and continue by identifying which activities have the greatest repercussion be it in absolute values of emissions (level) or trend (tendency) of the series of inventories, then the presentation continues with the identification of keys sources both in levels as in trends, finishing this section with the Tier 1 inventory uncertainty analysis. Section 6 “Results comparison (1990-2001) current ed. vs. previous ed.”, present and comment on the primary changes registered in the emissions estimates with regard to the previous edition. Finally, in secction 7 “Analysis by Source Categories” a detailed presentation is made on the primary information gathered, the estimation procedures and results obtained by the main inventory source categories following the CRF nomenclature.

NB. Important Notice. It is to be clearly stated that for easy of correspondence between the Spanish and English versions of this report, the numbers appearing in the tables and figures of this English version are written with the following separators:

- point “.” is used as the thousand separator

ii)	comma “,” is used as the unit fraction separator

for numbers in the text the usual English version separators are, nevertheless, used.

##### 2.- GENERAL METHODOLOGY

The data shown in the set of CRF tables for this edition contain all of the relevant information on the direct and indirect emission/uptake of greenhouse gases produced in Spain during the period 1990 to 2002.

The fulfilment of the CRF tables is effected on the basis of the pertinent data available in the National Emissions Inventories into the Atmosphere (Inventarios Nacionales de Emisiones a la Atmósfera), maintained and developed by the Directorate-General for Environmental Quality and Assessment at the Spanish Ministry of the Environment.

In the present edition of the greenhouse gas inventory, the fulfilment of the CRF tables has been completed following the bottom-up hierarchic link structure of the CRF software itself to produce automatically from the background tables both the derived sectoral tables with the emissions estimated by IPCC activity category and also, at a level of greater aggregation, the summary tables for levels and trends.

A brief description is given below of the work carried out to draw up the greenhouse gas inventory and particularly how the CRF tables are completed on the basis of the data from the National Air Emissions Inventory System.

The approaches for the emissions estimations recommended in both the “IPCC Guidelines for national greenhouse gas inventories, 1996 revised version” and the “IPCC Good practice guidance and uncertainty management in national greenhouse gas emissions, ed 2000” were adopted for all those activities for which the said approaches were considered the most precise, taking into account the resources and data available. In those cases where a national approach was available and was considered more appropriate than the alternative IPCC approach, the national approach was adopted in line with the recommendations of the IPCC itself. Thus, with respect to the crosstabs of gas type and emission-producing activity, the approaches adopted have been those indicated in the following sub-section.

2.1.- METHODOLOGIES APPLIED BY IPCC ACTIVITY CATEGORY

Energy: Combustion Processes

The carbon mass balance has been used for the estimation of the CO2 emissions, taking the most specific national parameters as the fuel characterization, either provided by the emissions producers themselves, as in the case of the Large Point Sources, or else derived from the standard fuel specifications.

The following approaches have been used for the other pollutants:

-	Estimations based on available direct measurements, mainly in the case of SO2 and NOX at the Large Point Sources.

-	Estimations based on mass balances, mainly in the case of SO2 in the emissions from mobile sources and from stationary sources without desulphurisation technologies.

-	Estimations based on the EMEP/CORINAIR methods, including COPERT III for traffic, as these were considered more specific for the combustion installations under consideration than those proposed at the IPCC. This group generally includes the estimations of NMVOC, CH4, CO and N2O.

Energy: Fugitive Emissions

In this activity category national methods have been used when, as in the cases below, it has been possible to call on information regarding processes, emission factors or estimation algorithms considered better adapted to the sector’s activity in Spain:

-	CH4 emissions in the mining and use of coal.

-	CH4, NMVOC and CO2 emissions in the transportation and distribution of natural gas and other gaseous fuels (air with added methane/propane, propane, gas works gas).

-	CO2 emissions in the (non combustive) processes for the transformation of fuels, mainly in coke plants and oil refineries.

-	NMVOC emissions in the transportation of oil-based products.

For the remaining activities in this sector, the IPCC or EMEP/CORINAIR emission factors have been used, depending on which one was considered more representative. To this category belong, for example:

-	The emissions of CH4 and NMVOC generated in the national oil and natural gas production activities; as well as the fugitive emissions of both pollutants generated in the loading-unloading and storage operations for crude oil and oil-based products at the sea terminals.

Industrial Processes

The emissions of the three main greenhouse gases (CO2, CH4, N2O) from the activities in this sector have been estimated in accordance with the IPCC methodology.

For the indirect greenhouse gases, as well as for sulphur dioxide, the approach used has been a combination of national methods complemented, when these data were missing, by EMEP/CORINAIR emission factors.

For the estimation of the emissions of fluorinated gases (HFC, PFC and SF6), the IPCC actual approach methodology was adopted. The complementary potential approach was not considered feasible as, at the maximum level of disaggregation of the foreign trade nomenclature, it was not possible to identify the commercial transactions (and it was also tremendously complex to do so in the chain of importers-distributors-exporters) by individual gas type, in other words, as:

- HFC-23; HFC-32, HFC-125; HFC-134a; HFC-143a; HFC-227ea, HFC-236fa in the HFC group;

- CF4; C2F6; C3F8; C4F10; in the PFC group; and

- SF6.

Use of Solvents and Other Products

Within this group, in which the IPCC methodology itself refers to EMEP/CORINAIR for a large number of activities, the national methods have been used and complemented by EMEP/CORINAIR factors.

Agriculture

Within the group of agricultural activities, it is necessary to distinguish the methodological handling by sub-sectors and by gas type if appropriate. We thus have:

-	With regard to the CH4 emissions from enteric fermentation in cattle, the IPCC methodology has been followed, with an advanced (tier 2) approach for cattle and sheep.

-	For the estimation of the CH4 and N2O emissions from handling manure, the IPCC methodology has been followed, supported by national manure generation and management parameters. Similar treatment (IPCC methodology supported by national emission factors) has been applied to the estimation of CH4 emissions in rice growing.

-	In the case of the N2O emissions from agricultural soils, the IPCC methodology has been followed, supported with regard to the determination of basic activity parameters and variables on national studies.

-	The pollutant emissions generated in the burning of agricultural waste have been estimated: a) for CH4 and CO, in accordance with the IPCC methodology; and b) for N2O and NOx, using this same methodology supported by national parameters.

-	In the estimation of soil NOx emissions and NMVOC emissions from agricultural land and the burning of agricultural waste, the EMEP/CORINAIR methodology has been used as there is no alternative in IPCC.

Changes in Land Use and Forestry

The estimations of the net uptake of CO2 corresponding to this IPCC category are obtained by the difference between the uptakes of the forest tree biomass and the losses from that same biomass through logging . The calculation, shown in Table 5 A of the CRF, has been effected following the IPCC methodology, using data taken from the Second and Third National Forest Inventories, in what concerns net growth of tree-forest biomass, from the Agricultural Statitiscs Yearly Book, in what concerns wood and firewood cuttings, and from other national data sources , in what concerns tree-related characterizing parameters and forestry management  and monitoring practices. A more in-depth description of the methodology applied for this source category is presented in the Annex titled “Methodology for estimating net CO2 uptakes by tree-forest biomass”, that can be found at the end of this volume.

- Waste

The emissions of the two main substances involved in waste treatment and disposal activities (CH4 and N2O) have been estimated using the IPCC methodology. The most significant activity for these emissions has been the disposal of municipal solid waste in either controlled landfills or uncontrolled tips. The estimation of methane emissions due to anaerobic degradation of the organic fraction in the waste has been estimated using the first-order kinetic equation. The second most important activity is the treatment of municipal and industrial waste water, where the IPCC guidelines have also been followed for the estimation of the methane emissions on the sewage treatment lines and on the sludge treatment lines. Other sources estimated in this sector refer to the emissions of N2O due to human consumption of proteins as well as waste incineration.

2.2.- TREATMENT OF CARBON STORED IN FEEDSTOCKS

The consumption of fuel products for non-energy uses appears in the fuel balance sheets under the homonymous heading. The amounts of all the types of fuel listed under this heading are included in the reference approach analysis, with each of these amounts being appropriately broken down into two fractions: a) the fraction that is stored in products; and b) the fraction that is presumably released in the short-term, giving rise to the corresponding CO2 emissions in accordance with the aforesaid reference approach.

2.3.- TREATMENT OF INTERNATIONAL BUNKERS

For the purposes of pro-memo reporting, that is to say outwith the national total, for the emissions corresponding to international sea and air traffic, the activity variable have been taken to be the fuel consumption figures appearing in the energy balance sheets as allocated to the respective international traffic flows: a) international marine bunkers; and b) international aviation, respectively.

##### 3.- DEvelopment of the inventory principles

This section comments on the developments in this 2004 edition to the principles that must be taken into account in the drafting of the inventories, and their reference to the CRF tables, as indicated in the FCCC/CP/2002/8 document of the Secretariat for the UNFCCC.

3.1.- TIME SERIES CONSISTENCY

An important feature of the compilation process has been the emphasis placed on guaranteeing that, as far as possible, the time series 1990-2002 of the country was consistently estimated over the years in accordance with the updated Revised 1996 IPCC Guidelines (and corresponding nomenclature) and the complementary document “IPCC Good practice guidance and uncertainty management in national greenhouse gas inventories”, published in 2000. The estimated figures for by-gas have been converted in terms of CO2-equivalent using the 1995 revised IPCC global warming potentials (100 years time horizon).

3.2.- RECALCULATIONS PERFORMED

The consistency objective just mentioned forced a recalculation of previously submitted, edition 2003, estimations of emissions and removals covering the period 1990-2001. It is firmly believed that this revision of the previous estimates, that is shown in Tables 8(a) of CRF, has significantly contributed to an improvement in the accuracy and completeness of the inventories series and the emissions trends derived therefrom. Basic information for the year 2002 is partly provisional (it uses projected activity data for some sectors), so it is envisaged that estimations for this year will presumably be recalculated when definite data are available.

3.3.- COHERENCE

The coherence of the estimated CO2 emissions originating from fossil fuel combustion activities has systematically been taken into account through the whole inventorying process of such emitting activities. The use of fuels for these energy purposes has essentially been matched with the national energy balances as reported in the “Energy Balance Sheets” publication of EUROSTAT and “Energy Statistics” of the International Energy Agency. CO2 emissions from biomass fuels combustion is also reported within the specified “Memo” items, although not taken into account in the national CO2 emissions estimated figure. The reference approach shown in Tables 1.A(b) and 1.A(c) can be seen as a coherence checking for CO2 emissions from fuel combustion activities, although, by the very definition of its activity variable coverage, the emissions estimated according to this approach yield normally higher figures as the ones obtained following the national approach.

3.4.- COMPLETENESS

Completeness of the inventories has been assessed according to IPCC estimating status typology: NO (not occurring), NE (not estimated); NA (not available); IE (included elsewhere); C (confidential), 0 (less than half a unit). It is believed that the completeness has been satisfactorily met, with the major caveat that CO2 removals by sinks have only been estimated for IPCC source “Changes in forests and other woody biomass stocks” and that no estimates of potential emissions have been calculated for fluorinated gases (HFC, PFC, SF6).

In short, the list of IPCC categories not covered in the inventories, ordered by gas species affected, is:

Carbon dioxide:

Sector: Land use change and forestry; categories: 5B-Forest and grassland conversion, 5C-Abandonment of managed lands, 5D-CO2 emissions and removals from soil.

All these three categories have not been estimated due mainly to lack of relevant statistical data. Additionally, the IPCC methodology, as it stands in the Revised 1996 IPCC Reference Manual raises, for some items, significant practical difficulties. It is expected that the expected new IPCC Guidelines for this group contribute to clarifying concepts and facilitating their implementation. For the case of Spain, categories 5B and 5D seem a priori the most relevant, category 5C having in principle a limited potential for the net balance of emissions. In any case, additional efforts are to be allocated to all categories of the “Land use change and forestry” sector once the IPCC estimating methodology under way receives approval.

Fluorinated gases, potential emissions

Sector: Industrial processes – potential emissions of fluorinated gases.

It has not been possible to estimate the potential emissions due to the reasons explained in section 2.1 regarding the limitation of data sources. This shortage of estimations only affects Table 2(II) Sheet 2/2.

3.5.- UNCERTAINTY/ESTIMATION QUALITY

The assessment of uncertainty, or symmetrically the estimation quality, has been shown in Table 7 of the CRF using the quality codes H (high), M (medium), and L (low) for each pertinent crosstab of gas type and IPCC activity category. This ordinal classification of quality is only a first stage in the analysis of the uncertainty associated with the inventory estimations. Work is now in progress for the implementation of a quantitative estimation of uncertainty in accordance with the approach recommended in the “IPCC Good Practice Guidelines” document.

3.6.- TRANSPARENCY

From a formal standpoint, the fulfilment of the CRF background tables with the inclusion of the activity variables, estimated emissions and implicit emission factors, as well as the complementary information included in the said tables, as appropriate, represents the most significant advance towards the achievement of data transparency in the preparation of the inventories. In addition, the transparency requirements are dealt with through the documentation and filing of the background information sources which, over and above what can logically be reflected in the background tables, have been used in preparing the inventories.

##### 4.- RECALCULATIONS

Recalculations have been performed for the period 1990-2001, for a group of activities which are subject to comment on an individual basis further below in section 4.2, “Comments by Activity”. These comments have been included also in the first part of tables 8(b) of the CRF format, and are motivated by the changes in the estimation methodology, which in turn, normally leads to the revision of the representative activity rates and of the emission factors, or by corrections to previous errors detected.

For year 2001 we should point out that, in addition, new calculations have been made in all those cases when an updated version of the activity rates was available; these may have been caused by the refreshing of estimates in the original sources, or simply projections used in the previous edition. In section 4.1, titled “General Comments”, the most relevant general sources of information that affect this type of estimates are enumerated. These comments are also included in the second part of the tables 8(b) of the CRF format.

4.1. GENERAL COMMENTS

The changes within the categories of greatest emission potential are motivated primarily by changes in some of the following sources of information:

4.1.1.	Large Point Sources Questionnaires (GFP)

In the case of the Large Point Sources, for which information is gathered through a specific questionnaire, data related to the year 2002 has been included in its entirety to the current edition and simultaneously corrected errors detected in the questionnaires corresponding to years 1990-2001. These changes could have affected plants that belong to the following sectors:

Utility Power Stations.

Oil Refineries.

Integrated Iron/Steel plants.

Alumina and Primary Aluminium plants.

Paper Pulp plants.

Large Vehicle Paint plants.

MSW Incineration plants.

4.1.2.	Activity Statistics

The statistical activity data has been revised for the main following sectors:

Forestry biomass:

For estimation of net uptake of CO2 by tree forest relevant information has been taken from the Third National Forest Inventory (in addition to the Second one that was the only one used in the previous submission).

4.1.3.	Energy Balances with special reference to CO2

Although the tallying of the fuels balance was performed in the previous version as well the current, there are some notable discrepancies, especially in the year 2001 fuels consumption data between one version and the other. It must be noted that due to the late availability of the Fuels Balance of 2001 the figures of fuels consumption for some activities had to be estimated. A similar situation occurred at the time of compiling the previous inventory edition with regard to the Fuels Balance of 2000. This motivated the need to operate with projected figures in the previous version, which has provoked a recalculation for the year 2001 in the category 1.A, “Fuel Combustion”.

4.2. COMMENTS BY ACTIVITY

Recalculations by activity category, discussed in this sub-section, are sorted by CRF source category for those activities whose estimated emissions have experienced significant changes for the period 1990-2001 with regard to the estimations provided in the previous edition.

Within each CRF source categories the paragraph headings refer to:

- Greenhouse gases concerned

- Methods

- Emission Factors

- Activity Data

- Addition/removal/replacement of source/sink categories

1.A	Combustion activities:

Gases Concerned: CO2, CH4, N2O

Activity Data:

Fuel balance 2001 revision: AR. The fuel balance for 2001 has been revised in accordance with the updated information received from the International Energy Agency and EUROSTAT (this information was not available in the previous edition).

1.A.1a 	Energy industries: Public electricity and heat production

Gases Concerned: CO2, CH4, N2O

Addition/removal/replacement of source/sink categories:

Public electricity and heat production: RA. MSW incineration plants without energy recovery have been reallocated to CRF source category 6C (in the previous submission they were allocated to CRF source category 1A1 together with those with energy recovery).

1.A.1b	Energy industries: Petroleum refining

Gases Concerned: CO2, CH4, N2O

Petroleum refining: ME. Revision of the default CO2 EFs for the year 1990 to re-express them in terms of ultimate CO2 as required by IPCC instead of in terms of end-of-pipe CO2 (discounting carbon emitted as NMVOC, CO, CH4, and PM).

Emission Factors:

Petroleum refining: EM.

a) Revision of an input mistake in a directly reported CO2 emissions figure from one refinery unit for the year 1997: the error was three of magnitude in the reported figure (126 tonnes corrected to 126 kilotonnes).

b) Revision of a very low directly reported CO2 emissions figure from one refinery unit that was not consistent with the reported activity data (fuel consumption). For the revision an EF derived from C content of the fuel was applied.

Petroleum refining: EF.

a) Revision of the default CO2 EF for refinery gas used in refinery furnaces units as the EF figure used in the previous submission (55 kg/GJ) was identified as an anomalous low value. The new value used (60 kg/GJ) is referenced in EMEP/CORINAIR Guidebook (table 6, page B136-9).

b) Revision of the default CO2 EFs for year 1990 to express them in terms of ultimate CO2 (as required by IPCC) instead of in terms of end-of-pipe CO2 as in the previous submission.

Activity Data:

Petroleum refining: AR.

a) Introduction of a cogeneration unit for the period 1996-2001 at one oil refinery (this unit was not reported in the previous submission).

b) Revision of a fuel type consumption in one oil refinery for the year 2001 reported in the previous submission as hydrogen, when in fact it corresponds to a refinery gas enriched with hydrogen.

1.A.1c	Manufacture of Solid Fuels and Other Energy Industries

Gases Concerned: CO2, CH4, N2O

Emission Factors:

Coke plants: EM. Revision of the estimated emission figures for 1992 and 1993 after detection of an error in the activity data rate (see explanation for activity data).

Activity Data:

Coke plants: AR. Revision of the activity data in terms of mass of coke produced for years 1992 and 1993 in collieries not located in iron and steel integrated plants. This revision does not affect activity data in terms of energy, as reported in the CRF tables, but in terms of mass as indicated above. This change has consequences on emissions estimates (because emissions are calculated for these plants as coke mass production times the respective EF).

1.A.2	Combustion in manufacturing industries and construction

Gases Concerned: CO2, CH4, N2O

Emission Factors:

Petroleum coke fuel characteristics: EF. As a consequence of the revision of petroleum coke carbon content (t C/TJ), the CO2 EF has been changed for the whole period 1990-2001 in the following industrial sectors categories: cement, lime and plaster production, chemical industry, machinery industries, and other non metallic minerals industries.

Activity Data:

Petroleum coke fuel characteristics: AR. As a consequence of the revision of petroleum coke LHV, the activity rate expressed in terms of energy (TJ) for the whole period 1990-2001 has been changed in the following industrial sectors categories: cement, lime and plaster production, chemical industry, machinery industries, and other non metallic minerals industries.

Addition/removal/replacement of source/sink categories:

Combustion in manufacturing industries: RA. This reference to recalculation is motivated by the reallocation for the whole period 1990-2001 of a set of activities related to combustion in a miscellaneous subset of industrial activities that were assigned in the previous submission to category 1A2f: 1) Lime productions in iron and paper pulp industries have been reallocated to categories 1A2a and 1A2d respectively; 2) Galvanizing furnace in iron industry has been reallocated to category 1A2a; 3) Anode baking furnaces in aluminium industry have been reallocated to category 1A2b.

1.A.2a	Fuel conbustion in iron and steel

Gases Concerned: CO2, CH4, N2O

Emission Factors:

Sinterization: EF. Revision of 2001 CO2 and N2O EF for the sinterization unit in one plant that were incorrectly assimilated to the corresponding 2000 EF values.

Activity Data:

Iron and steel: AR. Revision of steel plant furnace gas consumption in one integrated steel plant for the period 1991-1993. The activity data was assumed to be expressed in mass units (tonnes) when in fact it was expressed in the previous submission in volume units (kNm3); for the recalculation the appropriate density coefficient has been applied.

1A2c	Fuel combustion in chemicals

Gases Concerned: CO2, CH4, N2O

Emission Factors:

Chemicals: EF. Correction of the emissions estimates for the year 2000, after detection of the omission of the approppriate EF for hard coal consumption (activity data had been included but not the corresponding EF so that no emissions were calculated for this fuel).

1.A.2d	Fuel combustion in pulp, paper, and print

Gases Concerned: CO2, CH4, N2O

Activity Data:

Pulp, paper and print: AR. Revision of activity data for the year 2001 in one paper pulp plant for natural gas consumption (the LHV applied was that corresponding to propane).

1.A.2f	Combustion in manufacturing industries and construction

Gases Concerned: CO2, CH4, N2O

Emission Factors:

Cement and lime kilns: EF. Within this CRF category the N2O EF has been revised for the whole period 1990-2001 in cement and lime kilns, in consideration of the furnace operating conditions (mainly their high temperatures).

Bricks and tiles: EF. Revision of natural gas N2O EF after detection of incorrect figures for the whole period 1990-2001.

Activity Data:

Lime kilns: AR. Provisional data (fuel consumption) for 2001 in the previous submission has been revised with information supplied by the lime manufacturing entrepeneurial association (ANCADE).

Fine ceramic industry: AR. Fuel consumption for the year 2001 in the previous submission has been revised with information supplied by the manufacturing entrepeneurial association (ASCER).

Other glass manufacturing: AR. Fuel consumption for the years 2000 and 2001 have been revised after detection of incorrect figures in the previous submission.

Industrial mobile machinery: AR. A revision of diesel oil LHV has been made for for the whole period 1990-2001. This revision only affects to IEF, not the emissions themshelves.

1.A.3c	Railways

Gases Concerned: CO2, CH4, N2O

Activity Data:

Railways: AR. Fuel consumption, in energy terms (TJ NCV) as the LHV has been revised after verification of its appropiated value. This change has not implications on the emissions figures, which remain unchanged, but affects the IEFs.

1.A.4a, 1.A.4b	Fuel combustion in comercial/institutional and residential sectors

Gases Concerned: CO2, CH4, N2O

Emission Factors:

Petroleum coke fuel characteristics: EF. As a consequence of the revision of petroleum coke carbon content (t C/TJ), the CO2 EF has been changed for the whole period 1990-2001.

Activity Data:

Petroleum coke fuel characteristics: AR. As a consequence of the revision of petroleum coke LHV, the activity rate expressed in terms of energy (TJ) for the whole period 1990-2001 has been changed.

Residential combustion: AR. Due to a mistake in the previous submission a consumption of "Other fuels" was reported for 1999 in the residential sector but this consumption does not in fact exist. This has been fixed in the present submission.

1.A.4c	Fuel combustion in Agriculture/Forestry/Fisheries

Gases Concerned: CO2, CH4, N2O

Activity Data:

Agricultural and Forestry mobile machinery: AR. Some minor errors in fuel consumption diesel oil and gasoline in this type of machinery identified in the previous submission for reference years 1998-2001 have been fixed.

1.B.1	Fugitive emissions from fuels – Solid fuels

Gases Concerned: CO2, CH4

Activity Data:

Amount of fuel produced: AR. Revision of the mass units in which the production of fuel were expressed. In the previous submission it was mistakenly expressed in thousand of tonnes instead of million of tonnes. This error has been fixed in the present submission. This revision only affects IEF, not to the emissions themselves.

1.B.1b	Solid fuel transformation

Gases Concerned: CO2

Methods:

Coke plants: ME. A major revision of the CO2 estimating methodology has been conducted for the present submission. CO2 emissions have been re-estimated for the wole period using a full C-content material balance, with information on basic parameters provided by the main manufacturer. This is the recalculation with the greatest implication in CO2 emission differences between the present and previous submissions.

Emission Factors:

Coke plants: EM. Revision of the emissions estimated data for 1992 and 1993 after detection of an error in the activity data actually entered in the emissions estimation algorithm (please note that the CRF reported activity data was correct although it was incorrectly entered in the algorithm).

Activity Data:

Coke plants: AR. Revision of the coke production data for 1990 (corresponding to one plant) that in previous submission had been incorrectly typed. This revision does not affect emissions.

1.B.2a	Fugitive emissions from fuels: Oil

Gases Concerned: CO2

Emission Factors:

Refining/Storage: EM. Revision of the CO2 emissions for the whole period 1990-2001 after detection of a double counting in the hydrogen plant at an oil refinery.

1.B.2b	Fugitive emissions from fuels: Natural gas - Transmission and distribution

Gases Concerned: CO2, CH4

Activity Data:

Transmission and distribution of gas: AR. Revision of activity data for the year 1992 that were incorrectly typed for the previous submission. This revision only affects IEF not the emissions themshelves.

1.B.2c	Fugitive emisions from fuels: Flaring

Gases Concerned: CO2

Emission Factors:

Flaring - Oil: EF and EM. Revision of the CO2 emission factor for the year 2001 at an oil refinery plant, after detection of an incorrect emission factor used in the process of emissions estimation (CO EF was mistakenly entered instead of CO2).

Activity Data:

Flaring - Oil and Gas: AR. Revision of 1996 activity data for oil (oil refined) and gas (gas consumed) that were incorrectly typed for the previous submission (they correspond to 1992). This revision only affects IEF not the emissions themshelves.

2.A.2	Lime production

Gases Concerned: CO2

Activity Data:

Lime production: AR. Provisional data for 2001 in previous submission has been revised with information supplied by lime manufacturing entrepeneurial association (ANCADE).

2.A.4	Soda Ash Production

Gases Concerned: CO2

Emission Factors:

Soda ash production: EF. Revision of the whole time series of soda ash CO2 emission factors according to the information provided by the sole plant manufacturer.

Activity Data:

Soda ash production: AR. Revision of the whole time series of soda ash production according to the information provided by the sole plant manufacturer.

2.A.7	Mineral products: Other

Gases Concerned: CO2

Methods:

Glass processes - decarbonazing: ME. Small rounding errors in CO2 emissions for the whole period 1990-2001 in the previous submission have been fixed.

Activity Data:

Glass processes - decarbonazing: AR. Activity rates for the years 2000 and 2001 have been revised for the "Other glass" sub-sector after detection of incorrect figures in the previous submission.

Addition/removal/replacement of source/sink categories:

Magnesite production: AD. This corresponds to the inclusion of this activity for the first time in the emission inventory submission 2004. With regard to industrial processes, the emissions of CO2 originate from raw materials decarbonization.

2.B.1	Ammonia production

Gases Concerned: CO2

Emission Factors:

Ammonia production: EF. The CO2 emission factors have been revised for the period 1995-2001 in order to achieved spatial consistency (bottom-up for provinces to national scale) and time series consistency (over the whole period 1990-2002).

Activity Data:

Ammonia production: AR. Revision of the ammonia production data for years 2000 and 2001 according to new information provided by the Chemical Industry Federation (FEIQUE).

2.B.2	Nitric Acid production

Gases Concerned: N2O

Activity Data:

Nitric acid production: AR. Revision of nitric acid production data for 1995 after detection of the incorrectly typed figure in the previous submission. This only affects IEF, not to the emissions themselves.

2.B.5	Chemical industry other

Gases Concerned: CO2, CH4

Activity Data:

Activity variables: AR. The figures for activity variables provided by Ministry for Science and Technology and the Chemical Industry Federation in the 2002 edition of “Industria Química en España” (“Chemistry Industry in Spain”) have been revised (the new edition includes revised figures for some of the products for which preliminary statistics for 2001 were given in the previous edition).

2.C.1	Iron and Steel Production

Gases Concerned: CH4

Emission Factors:

Pig iron production: EM. In the previous submission, the 1994 CH4 emission figure directly entered into the CRF cell was incorrect (it corresponded to 1992). This mistake has been fixed.

2.C.2	Ferroalloys production

Gases Concerned: CO2

Activity Data:

Ferroalloys production: AR. Revision of ferroalloys production data for 1997, 1998 and 1999 after detection of the incorrectly typed figures in the previous submission. This only affects IEF, not the emissions themselves.

3.A	Paint application

Gases Concerned: CO2

Activity Data:

Paint application: AR. Revision of the paint application data for 2001, according to new information provided by the Paint Manufactures Entrepeneurial Association (ASEFAPI).

3.B	Degreasing and dry cleaning

Gases Concerned: CO2

Activity Data:

Dregreasing and dry cleaning: AR. Revision of the solvent consumption data for the whole period as the previous submission only included the figures corresponding to main halogenated solvents. Data in the current submission included also non-halogenated solvents. This only affects IEF, not the emissions themselves.

3.A, 3.D	Paint application and Other solvent and product use

Gases Concerned: CO2

Activity Data:

Paint application and other solvent use: AR, EF and EM. Reported data of activity rates and associated emissions for 1997 and 1998 related to a decommissioned car manufacturing plant have been discounted from data in the previous submission. The revision has the effect of avoiding double counting (the plant was moved to another location).

4.A	Enteric Fermentation

Gases Concerned: CH4

Activity Data:

Milk production: AR. The figures for milk production reported in the previous submission for 2000 and 2001 have been revised (for year 2000 the recently published data have been entered instead of those derived from 1999 data projections, for 2001 the new reported figure is based on projection from 2000 instead of projections from 1999). This revision affects the gross energy (GE) estimates of dairy cattle and consequently their contributed emissions.

4.B	Manure Management

Gases Concerned: CH4

Activity Data:

Milk Production: AR. The revision for milk production data in years 2000 and 2001 as indicated above imply changes in gross energy (GE) and consequently in Volatile Solids (VS). Changes in this last parameter are thus reflected in emissions changes. Emissions changes in this source category are nevertheless minor.

4.C	Rice Cultivation

Gases Concerned: CH4

Activity Data:

Rice Cultivated Surface: AR. Rice "harvested area", as appears in table 4.C, was in previous submission reported in a incorrect unit for all years in the inventoried period 1990-2001. The present submission has fixed this "units" reporting error. The revision does not affect the reported emissions themshelves (only the activity data and the implied emission factor have changed).

4.D	Agricultural Soils

Gases Concerned: N2O

Activity Data:

Crop residues: AR. Changes in estimated activity data for crop residues in 1999, 2000 and 2001 are reflected in minor variations for these years in N contained in crop residues. These changes affect N2O crop residue emissions and N2O emissions from field burning of agricultural residues.

4.F	Field Burning of Agricultural Residues

Gases Concerned: N2O

Activity Data:

See comment to 4.D.

5.A	Changes in Forest and Other Woody Biomass Stocks

Gases Concerned: CO2

Methods:

Net CO2 uptake by forest trees: ME. An in-depth revision of methodology has been implemented for the present submission. The main features of this revision are as follows:

a)	Use information from both the Second and Third National Forest Inventory instead of only the Second one used in the previous submission.

b)	Use of a yearly time series for 1990-2000 (with extrapolations for the years 2001 and 2002) of wood and firewood cuttings instead of only data for year 1990 used in the previous submission.

c)	Use of species-specific factors to expand the tree trunk volume to total tree living volume biomass, instead of only a general default expansion factor for all species.

d)	No discounting of individualized estimates of CO2 emissions from forest fires as the Third National Forest Inventory already takes into account the loss of CO2 from forest fires.

In summary, a dynamic time series is provided for the first time in the present submission for sub-category 5A.

6.C	Waste incineration

Gases Concerned: CO2, CH4, N2O

Activity Data:

Flaring in steel industry: AR. LHV for steel plant furnace gas flared has been revised according to plant specific information provided via questionnaire for the whole period 1990-2001. Additionally, for 2001, the activity rate previously expressed in terms of kNm3 has been reconverted to mass units (tonnes) in order to apply the LHV coefficient.

Addition/removal/replacement of source/sink categories:

Incineration of MSW: RA. MSW incineration plants without energy recovery have been reallocated to CRF source category 6C (in the previous submission they were allocated to CRF source category 1A1 together with those with energy recovery).

##### 5.- RESULTS AND KEY SOURCES ANALYSIS

For the analysis of the implications on the potential of global atmospheric warming, the emissions figures are estimated with weightings based on the 1995 IPCC global warming factors (100-year time horizon) for each gas and shown in terms of gigagrammes (Gg) of CO2-equivalent. For the case of Spain, the base period taken for reference purposes is 1990 for CO2, CH4 and N2O and 1995 for the fluorinated gases, HFC, PFC and SF6. The time interval under consideration is from 1990 to 2002.

5.1.- AGGREGATE ANALYSIS

Table 5.1 shows in both absolute terms (Gg CO2-equivalent) and in terms of the time index (100 in the reference year) the values corresponding to the total gross emissions. The graphic representation of this index is shown in Figure 5.1. From the preceding data, it can be seen that the total emissions in 2002 are 39.4% above the reference year, whereas the percentage increase reduces to 31.1% when the average of the last quinquennium, 1998-2002, is compared with the same reference year. Overall, the index has shown sustained growth during the inventoried period, except for 1993, 1996 and 2001 when the levels fell by comparison with the preceding years. In terms of the slope of the curve, the interval 1990-1996 is characterized by a more moderate growth than that corresponding to the interval 1996-2002. This variability in the evolution seems to be linked from time to time (annual peaks/troughs) with the greater or lesser degree of rainfall in the year in question and its sequel on the thermal electricity generation, although another series of additional factors such as the general expansion in fuel consumption and industrial activity are at the base of change of slope observable between the two intervals indicated above, 1990-1996 and 1996-2002.

TablE 5.1.
EvoluTION OF AGGREGATE EMISSIONS

Absolute Values (Gg CO2 equivalent)

| Base year | 1990 | 1998 | 1999 | 2000 | 2001 | 2002 |
| --- | --- | --- | --- | --- | --- | --- |
| 286.797,91 | 284.555,86 | 340.823,76 | 370.377,38 | 385.202,87 | 383.459,67 | 399.731,53 |

Annual Index

| Base year | 1990 | 1998 | 1999 | 2000 | 2001 | 2002 |
| --- | --- | --- | --- | --- | --- | --- |
| 100 | 99,22 | 118,84 | 129,14 | 134,31 | 133,70 | 139,38 |

Mean five-yearly index

| Base year | 1990-1994 | 1991-1995 | 1992-1996 | 1993-1997 | 1994-1998 | 1995-1999 | 1996-2000 | 1997-2001 | 1998-2002 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 100 | 102,40 | 104,62 | 105,94 | 108,07 | 111,71 | 116,31 | 121,11 | 126,25 | 131,07 |

FigurE 5.1.
Index of the Evolution

5.2.- ANALYSIS BY GAS

Table 5.2 gives the emissions estimates by type of gas for each of the six groups or species indicated above as having a direct warming effect: CO2, CH4, N2O, HFC, PFC and SF6. The top of the table shows the emissions in absolute terms (Gg CO2-equivalent), the medium part shows the percentage contributions to total CO2 equivalent gross emissions, and the bottom part gives the trend in terms of the time index (100 in the reference year).

Table 5.2.
Evolution of emissions by type of gaS

Absolute Values (Gg CO2 equivalent)

| GAS | Base year | 1990 | 1998 | 1999 | 2000 | 2001 | 2002 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO2 | 224.751,23 | 224.751,23 | 268.775,66 | 295.260,27 | 306.829,65 | 308.278,41 | 325.448,17 |
| CH4 | 30.244,07 | 30.244,07 | 37.682,86 | 38.064,81 | 39.265,40 | 40.312,18 | 41.136,13 |
| N2O | 26.273,22 | 26.273,22 | 27.665,96 | 29.007,40 | 30.320,52 | 29.140,11 | 28.755,38 |
| HFC | 4.645,44 | 2.403,18 | 5.809,01 | 7.163,91 | 8.171,44 | 5.287,84 | 3.896,11 |
| PFC | 790,37 | 828,41 | 749,69 | 695,53 | 404,84 | 228,82 | 257,05 |
| SF6 | 93,58 | 55,75 | 140,57 | 185,46 | 211,02 | 212,31 | 238,69 |
| TOTAL GASES | 286.797,91 | 284.555,86 | 340.823,76 | 370.377,38 | 385.202,87 | 383.459,67 | 399.731,53 |

Percentage Contribution To Total CO2 equivalent

| GAS | Base year | 1990 | 1998 | 1999 | 2000 | 2001 | 2002 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO2 | 78,37 | 78,98 | 78,86 | 79,72 | 79,65 | 80,39 | 81,42 |
| CH4 | 10,55 | 10,63 | 11,06 | 10,28 | 10,19 | 10,51 | 10,29 |
| N2O | 9,16 | 9,23 | 8,12 | 7,83 | 7,87 | 7,60 | 7,19 |
| HFC | 1,62 | 0,84 | 1,70 | 1,93 | 2,12 | 1,38 | 0,97 |
| PFC | 0,28 | 0,29 | 0,22 | 0,19 | 0,11 | 0,06 | 0,06 |
| SF6 | 0,03 | 0,02 | 0,04 | 0,05 | 0,05 | 0,06 | 0,06 |
| TOTAL GASES | 100,00 | 100,00 | 100,00 | 100,00 | 100,00 | 100,00 | 100,00 |

Annual Index

| GAS | Base year | 1990 | 1998 | 1999 | 2000 | 2001 | 2002 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO2 | 100 | 100 | 119,59 | 131,37 | 136,52 | 137,16 | 144,80 |
| CH4 | 100 | 100 | 124,60 | 125,86 | 129,83 | 133,29 | 136,01 |
| N2O | 100 | 100 | 105,30 | 110,41 | 115,40 | 110,91 | 109,45 |
| HFC | 100 | 51,73 | 125,05 | 154,21 | 175,90 | 113,83 | 83,87 |
| PFC | 100 | 104,81 | 94,85 | 88,00 | 51,22 | 28,95 | 32,52 |
| SF6 | 100 | 59,57 | 150,21 | 198,17 | 225,48 | 226,87 | 255,06 |
| TOTAL GASES | 100 | 99,22 | 118,84 | 129,14 | 134,31 | 133,70 | 139,38 |

With reference to Figure 5.2, it is pointed out the dioxide of carbon as the main compound in the absolute weight of the emissions (around 79.5%). The pair formed by the methane and the nitrous oxide, with values varying, depending on the year, between 19.7% and 17.5%, is seen as the second most important block. Finally, there comes the group of fluorinated compounds with a range of contribution between 1.1% and 1.9%. By comparing the three blocks in Figure 5.2, it can be seen that the contribution by CO2 has shown an 3% increase,  from 78.4% in the base year -to 81.4% in year 2002, while the average for the quinquenium 1998-2002 was at 80.1%. By contrast a small variation in the percentage contribution  can be seen for CH4, with a range of variation of less than 0.5, as the extreme values in this period are 10.3%-10.6%. N2O reflects a drop in its relative contribution from 9.2%in the reference year to 7.7% in the period 1998-2002 and to 7.2% in year 2002. For gases containing fluor, there a steady reduction in PFC from 1990 to 2001, growing slightly in 2002, a increase in SF6 and a larger contribution of HFC along the period 1990-2000 that drops sharply in 2001 and 2002, although all of these remain at low (very low) absolute levels even in 2002: HFC (0.97%), PFC (0.06%) and SF6 (0.06%).

fIGURe 5.2.
Contribution to emissions by GAS

Figure 5.3 presents the variations over time, with the top part indicating the indices corresponding to the three gases with the greatest contribution and the lower part those containing fluor. Observing the evolution of CO2 in the upper section, the relative minimum values for 1993 and 1996 can be seen, as can the increase in the slope when going from the sub-interval 1990-1996 to 1996-2002, as patterns reflected to a large extent in the trend commented above for the aggregate index. The evolution in CH4 is more uniform throughout the period. N2O continues to follow a different trend from the two gases above, with a slight drop during the sub-interval 1990-1995, then growing in later years till 2000, when the time series shows a steady slight decay in the two following years. As for the fluorinated gases, differences can be seen in the evolution of PFC, stable-decreasing with a significant decrease from 1999 to 2001 and stabilization in 2001-2002., and that of HFC and SF6, for which a first stable period, 1990-1993, is followed by parallel growth in the intervals 1994-1997, more pronounced growth in 1998-2000 for SF6, being their trends different from 2000 to 2002, with an increase for SF6 and a sharp reduction for HFC.

fIGURE 5.3.
INDICES of EVOLUtion of EMIsSIONS by GAS

The heterogeneity of the HFC and PFC sub-groups in terms of the individual gases they comprise makes it particularly difficult to give a general comment. However, with respect to future prospects, the figures seem to point to a stabilization in the emissions from production processes and a  growth in emissions from consumption activities. For the three remaining gases, the increases seen between 1990 and 2002 for CO2 (44.8%) and CH4, (36.0%) are only partly offset by the moderate increase in N2O (9.5%) when it comes to determining the weighted variation rate (40.6%) for the aggregate of the emissions.

5.3.- ANALYSIS BY SECTORS

Table 5.3 gives the emissions estimates by activity sectors, distinguishing between the following groups in the IPCC nomenclature: Energy, industrial processes, use of solvents and other products, agriculture, and waste treatment and disposal. A pro-memoria summary is given for the group including changes in land-use and forestry, as these no do not include gross emissions figures but net sequestration of CO2. The top of the table shows the emissions in absolute terms (Gg CO2-equivalent), the medium part shows the percentage contributions to total CO2 equivalent gross emissions, and the lower part gives the evolution over time (100 in the reference year) for each group under consideration.

TablE 5.3.
Evolution of emissions by SECTOR

Absolute Values (Gg CO2 equivalent)

| SECTOR | Base Year | 1990 | 1998 | 1999 | 2000 | 2001 | 2002 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1. Energy | 213.595,91 | 213.595,91 | 257.023,69 | 282.954,46 | 294.353,40 | 295.494,29 | 312.303,82 |
| 2. Industrial Processes | 25.013,68 | 22.771,63 | 27.634,87 | 29.860,84 | 30.976,09 | 28.201,38 | 27.417,54 |
| 3. Solvent and Other Product Use | 1.329,83 | 1.329,83 | 1.635,51 | 1.673,56 | 1.706,90 | 1.616,43 | 1.693,51 |
| 4. Agriculture | 37.373,21 | 37.373,21 | 40.943,27 | 41.922,26 | 43.618,90 | 42.989,97 | 42.650,34 |
| 6. Waste | 9.485,29 | 9.485,29 | 13.586,41 | 13.966,27 | 14.547,57 | 15.157,60 | 15.666,31 |
| TOTAL SECTORS | 286.797,91 | 284.555,86 | 340.823,76 | 370.377,38 | 385.202,87 | 383.459,67 | 399.731,53 |
| 5. Land Use Change and Forestry | -9.456,00 | -9.456,00 | -23.870,79 | -26.757,65 | -31.746,09 | -31.476,51 | -35.301,49 |

# Percentage Contribution To Total CO2 equivalent

| SECTOR | Base year | 1990 | 1998 | 1999 | 2000 | 2001 | 2002 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1. Energy | 74,48 | 75,06 | 75,41 | 76,40 | 76,42 | 77,06 | 78,13 |
| 2. Industrial Processes | 8,72 | 8,00 | 8,11 | 8,06 | 8,04 | 7,35 | 6,86 |
| 3. Solvent and Other Product Use | 0,46 | 0,47 | 0,48 | 0,45 | 0,44 | 0,42 | 0,42 |
| 4. Agriculture | 13,03 | 13,13 | 12,01 | 11,32 | 11,32 | 11,21 | 10,67 |
| 6. Waste | 3,31 | 3,33 | 3,99 | 3,77 | 3,78 | 3,95 | 3,92 |
| TOTAL SECTORS | 100,00 | 100,00 | 100,00 | 100,00 | 100,00 | 100,00 | 100,00 |

# Annual Index

| SECTOR | Base year | 1990 | 1998 | 1999 | 2000 | 2001 | 2002 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1. Energy | 100 | 100 | 120,33 | 132,47 | 137,81 | 138,34 | 146,21 |
| 2. Industrial Processes | 100 | 91,04 | 110,48 | 119,38 | 123,84 | 112,74 | 109,61 |
| 3. Solvent and Other Product Use | 100 | 100 | 122,99 | 125,85 | 128,36 | 121,55 | 127,35 |
| 4. Agriculture | 100 | 100 | 109,55 | 112,17 | 116,71 | 115,03 | 114,12 |
| 6. Waste | 100 | 100 | 143,24 | 147,24 | 153,37 | 159,80 | 165,16 |
| TOTAL SECTORS | 100 | 99,22 | 118,84 | 129,14 | 134,31 | 133,70 | 139,38 |

With reference to Figure 5.4, the first comment to be pointed out when examining the data by sectors is the dominant contribution by the energy group with a weighting that varies between 74.5% in the reference year  and 78.1% in 2002, with an intermediate value of 76.7% for the period 1998-2002. It must be remembered that this IPCC group includes, in addition to the emissions from combustion in stationary and mobile sources, the fugitive emissions from activities dealing with mining, transport and distribution of fuels, which are of great relevance for certain gases such as NMVOC and CH4. Secondly, although lagging far behind the previous group, there is the contribution by agriculture, with values varying around 13% in the reference year and between 11.3%for the period 1998-2002 and 10.7% in year 2002. The third most important group comprises physical and chemical industrial processes (excluding combustion activities that are listed in the energy group), with a contribution that goes down from 8.7% in the reference year, through 7.7% as average for the period 1998-2002, to 6.9% in year 2002. The group dealing with waste treatment and disposal shows a relative growth, with a contribution rising from 3.3% in the reference year to figures of around 3.9% in the mean and in the last year of the sub-interval 1998-2002 Finally, mention must be made of the marginal and stable contribution, 0.4%-0.5%, of the group comprising the use of solvents and other products, which is relatively unrelated to the direct greenhouse gas effect as the essential contribution comes from NMVOC.

fIGURE 5.4.
Contributions to emissions by sector

Figure 5.5 presents the variations over time, with the top part indicating the indices corresponding to the sectors of energy processing and agriculture and the lower part the industrial processes (excluding combustion), waste treatment and disposal, and the use of solvents and other products. With respect to the energy sector, this reproduces to a large extent the patterns commented on above in connection with aggregate emissions (Figure 5.1) and CO2 (Figure 5.3), justified by the close connection between the emissions of CO2 and the energy sector and between both of these and the aggregate emissions. In the agriculture sector, a period of stability can be seen between 1990 and 1995, followed, in the period 1997-2000, by a monotonous upwards trend, followed with a decay from 2000 to 2002. The waste treatment and disposal sector is the one showing the most intense and uniform upward tendency over the entire period 1990-2002. Lastly, the sector for the use of solvents and other products, after the stable phase during the years 1990-1992, shows a sustained increase over the period 1993-2000, falling in 2001 and then growing in 2002, although its extremely low absolute level makes it irrelevant in terms of the evolution of the aggregate. In short, a distinction can be drawn between the evolution on the one hand of the energy and industrial processes groups, which in 2002 are around 42.4% above the reference year, then the waste treatment group reaching a level 65% higher than the reference year, and on the other hand the agricultural sector, which is less than these and able to offset slightly their contribution to the increase in the aggregate emissions, with a increase of around 14.1% in relation to the reference year.

FigurE 5.5.
Indices of the Evolution of emissions by sector

5.4.- KEY SOURCES ANALYSIS

Aiming to improve the precision and accuracy of the emission inventory estimates, it is necessary to consolidate the quality assurance and quality control procedures on the activities which could be considered key or high-priority from the perspective of their contribution to uncertainty of the estimates.

In this sense the IPCC Good Practice Guidance (GPG) document defines an emission source (alternatively a sink) as key if such source could exert a significant influence in the estimates, be it as a level (level assessment) or trend (trend assessment) within the inventory emissions aggregate.

From an operational perspective the GPG establishes two approaches (tiers) to identify the key sources. The first approach, tier 1, is focused on determining the influence, be it by level or trend, a source of emission could exert on the uncertainty of the global inventory estimates but without requiring the use of more formal procedures of uncertainty analysis. The second approach, tier 2, proposes a similar procedure but expects that prior information derived from a formal uncertainty analysis is available. Considering that the National Emission Inventories have not yet implemented a formal uncertainty analysis (although it is being developed) the following analysis has been developed according to tier 1.

The tier 1 approach distinguishes between the identification of a source as key be it as a level or trend or both concepts. For level identification, a threshold is first defined (usually at 95%) for the cumulative distribution function of the activities contributing to the aggregate inventory emissions; having sorted these activities by descending order of contribution to the inventory emissions (in terms of CO2-e). One can consider that the activities selected, in relation to the cumulative distribution threshold, can take account of approximately 90% of the inventory uncertainty. To identify the trend, tier 1 also specifies a threshold of 95% but defined in this case on the contribution of activities to the trend metrics.

Once the key source analysis approach has been described, the obtained results for the level of the initial (1990) and final (2002) years are presented in tables 5.4 and 5.5, and for the trend of the year 2002 in table 5.6. A results summary for the corresponding period is presented in table 5.7.

Apart from the classification of a source, where appropriate, as a key source in quantitative terms with respect to its level and/or trend as expressed in the preceding tables, qualitative criteria are in some cases also of interest for those source activities that deserve attention due to other (qualitative) motives. The latter, influencing these significant activities, are (among others): i) actions to control and or reduce emissions; ii) important changes experienced during the considered period; iii) display evidence of great uncertainties; iv) show unexpectedly high or low emissions; and v) subject to important methodology changes in the emission estimates. This is true, among others, for the following categories: i) The use of SF6 in the manufacture of semiconductors: as has already been mentioned in other sections of this report, the occurrence of this phenomenon in Spain is pending confirmation and, if it does in fact occur, the estimation of its emissions is pending, so there might eventually be a considerable change in the SF6 emissions estimated for this activity; ii) The use of HFC in air conditioning and refrigeration activities for which scant data are available about the activity variable which might also give rise, in a future edition of the Inventory, to a considerable revision of the estimated emissions; iii) The production of lime in other sectors not yet included in the Inventory, for instance in the food industry, has so far not been considered and this might also lead to a revision of the estimated emissions; iv) Other uses for limestone and dolomite not considered in the Inventory and also subject to revision.

With regard to what was reported in this section in the previous edition of the Inventory Report, the criteria adopted to select the list of key sources considered in the present report has taken into acccount more narrowly the principles established in the IPCC Good Practice document (that in any case gives allowance to include national consideration when adopting such criteria). So the list of the selected key sources evidences the following changes with regard to the one selected in the previous submission:

Refering to CH4 and N2O emissions for the Energy Sector the criteria adopted have been :

To differenciate by transport mode within the transport sector

To lump together all stationary sources (although differenciated by pollutant type)

With regard to CO2 emissions from stationary combustion within the Energy Sector the breakdown includes the crossing of fuel type with the following activity subcategories: power stations (1A1a), oil refineries (1A1b) , coke plants (1A1c) and other sources (1A2+1A4).

Fugitive emissions of CO2 and CH4 from Energy Sector have been considered within the following subcategories: Solid fuels  (1B1) and Oil and gas(1B2).

Within the Industrial Processes group category, the CH4 emissions originate in subgroups 2B (Chemical Industry) and 2C (Metal Production) have been grouped in only one item. Also, the CO2 emissions for Chemical Industry has not been distinghished from the rest of activities within Industrial Processes.

Source category 4D has been further broken down into the following subcategories:

4D1 Direct Soils Emissions

4D2 Animal Production

4D3 Indirect Soils Emissions

A similar approach has been followed within source category Waste gathering together the emissions for Industrial Wastewater Treatment (6B1) and Domestic and Commercial Wastewater Treatment (6B2) for each the two gases CH4 and N2O.

To fully automate the application of the selected criteria for establishing the list and the contributing estimates of the selected key sources a software programme has been developed. This programme greatly facilitates the identification and analysis of all key sources according to the criteria previously adopted.

TABLE 5.4.
LEVEL CONTRIBUTION BY ACTIVITIES – YEAR 1990

| IPCC SOURCE CATEGORIES |  | Fuel | Gas | Gg CO2 equivalent | Contribution (A) | Cumulative (B) |
| --- | --- | --- | --- | --- | --- | --- |
| 1A1a | Public Electricity and Heat Production- Coal | Coal | CO2 | 57.779,09 | 0,203 | 0,203 |
| 1A2+1A4 | Stationary Combustion - Other sources - Oil | Oil | CO2 | 45.247,12 | 0,159 | 0,362 |
| 1A3b | Road Transportation- Gasoline | Gasoline | CO2 | 25.928,33 | 0,091 | 0,453 |
| 1A3b | Road Transportation -Diesel Oil | Diesel Oil | CO2 | 25.383,12 | 0,089 | 0,542 |
| 1A2+1A4 | Stationary Combustion - Other sources - Coal | Coal | CO2 | 15.078,76 | 0,053 | 0,595 |
| 4A | Enteric Fermentation |  | CH4 | 12.651,18 | 0,044 | 0,640 |
| 2A1 | Cement Production |  | CO2 | 12.534,33 | 0,044 | 0,684 |
| 1A1b | Petroleum refining- Oil | Oil | CO2 | 10.862,00 | 0,038 | 0,722 |
| 1A2+1A4 | Stationary Combustion - Other sources - Gas | Gas | CO2 | 9.465,71 | 0,033 | 0,755 |
| 4D1 | Agricultural Soils - Direct Emissions |  | N2O | 8.535,01 | 0,030 | 0,785 |
| 4B | Manure Management |  | CH4 | 6.220,81 | 0,022 | 0,807 |
| 1A1a | Public Electricity and Heat Production- Oil | Oil | CO2 | 6.015,03 | 0,021 | 0,828 |
| 6A | Solid Waste Disposal on Land |  | CH4 | 5.390,92 | 0,019 | 0,847 |
| 4D3 | Agricultural Soils - Indirect Emissions |  | N2O | 4.836,27 | 0,017 | 0,864 |
| 1A3a | Domestic Civil Aviation |  | CO2 | 4.135,39 | 0,015 | 0,879 |
| 1A1+1A2+1A4 | Stationary Combustion |  | N2O | 2.895,33 | 0,010 | 0,889 |
| 2B2 | Nitric Acid Production |  | N2O | 2.884,16 | 0,010 | 0,899 |
| 4D2 | Agricultural Soils - Animal Production |  | N2O | 2.793,99 | 0,010 | 0,909 |
| 2E1 | HCFC-22 Manufacture (HFC-23 Emissions) |  | HFC | 2.403,18 | 0,008 | 0,917 |
| 2-2A1-2A2-2C1 | Other Industrial Processes |  | CO2 | 2.398,78 | 0,008 | 0,926 |
| 1B1 | Fugitive Emissions from Solid fuels |  | CH4 | 1.788,93 | 0,006 | 0,932 |
| 1B2 | Fugitive Emissions from Oil and Natural gas |  | CO2 | 1.743,41 | 0,006 | 0,938 |
| 1A1c | Manufacture of Solid fuels and Other Energy Industries | Coal | CO2 | 1.664,03 | 0,006 | 0,944 |
| 4B | Manure Management |  | N2O | 1.631,73 | 0,006 | 0,950 |
| 1A3d | National navigation |  | CO2 | 1.536,50 | 0,005 | 0,955 |
| SUBTOTAL |  |  |  | 271.803,10 |  |  |
| TOTAL EMISSIONS |  |  |  | 284.555,86 |  |  |
| PERCENT OF TOTAL |  |  |  | 95,52 |  |  |

(A). Percentage of the activity categories emissions with relation to total inventory emissions.

(B). Cumulative percentage of the activity categories emissions with relation to total inventory emissions.

TABLE 5.5.
LEVEL CONTRIBUTION BY ACTIVITIES – YEAR 2002

| IPCC SOURCE CATEGORIES |  | Fuel | Gas | Gg CO2 equivalent | Contribution (A) | Cumulative (B) |
| --- | --- | --- | --- | --- | --- | --- |
| 1A1a | Public Electricity and Heat Production- Coal | Coal | CO2 | 78.418,78 | 0,196 | 0,196 |
| 1A3b | Road Transportation -Diesel Oil | Diesel Oil | CO2 | 57.209,14 | 0,143 | 0,339 |
| 1A2+1A4 | Stationary Combustion - Other sources - Oil | Oil | CO2 | 53.102,09 | 0,133 | 0,472 |
| 1A2+1A4 | Stationary Combustion - Other sources - Gas | Gas | CO2 | 37.288,43 | 0,093 | 0,565 |
| 1A3b | Road Transportation- Gasoline | Gasoline | CO2 | 25.982,17 | 0,065 | 0,630 |
| 2A1 | Cement Production |  | CO2 | 15.853,10 | 0,040 | 0,670 |
| 4A | Enteric Fermentation |  | CH4 | 14.720,06 | 0,037 | 0,707 |
| 1A1a | Public Electricity and Heat Production- Oil | Oil | CO2 | 14.097,41 | 0,035 | 0,742 |
| 1A1b | Petroleum refining- Oil | Oil | CO2 | 11.903,22 | 0,030 | 0,772 |
| 6A | Solid Waste Disposal on Land |  | CH4 | 10.872,77 | 0,027 | 0,799 |
| 4B | Manure Management |  | CH4 | 8.626,77 | 0,022 | 0,821 |
| 4D1 | Agricultural Soils - Direct Emissions |  | N2O | 8.357,39 | 0,021 | 0,842 |
| 1A1a | Public Electricity and Heat Production- Gas | Gas | CO2 | 5.900,37 | 0,015 | 0,856 |
| 1A2+1A4 | Stationary Combustion - Other sources - Coal | Coal | CO2 | 5.694,35 | 0,014 | 0,871 |
| 4D3 | Agricultural Soils - Indirect Emissions |  | N2O | 5.204,67 | 0,013 | 0,884 |
| 1A3a | Domestic Civil Aviation |  | CO2 | 5.084,17 | 0,013 | 0,896 |
| 1A1+1A2+1A4 | Stationary Combustion |  | N2O | 3.888,17 | 0,010 | 0,906 |
| 4D2 | Agricultural Soils - Animal Production |  | N2O | 3.305,79 | 0,008 | 0,914 |
| 2-2A1-2A2-2C1 | Other Industrial Processes |  | CO2 | 2.876,66 | 0,007 | 0,922 |
| 2F | Consumption of Halocarbons and SF6 (except HCFC-22) |  | HFC & PFC | 2.790,71 | 0,007 | 0,929 |
| 6B | Wastewater Handling |  | CH4 | 2.427,63 | 0,006 | 0,935 |
| 1A3d | National navigation |  | CO2 | 2.338,07 | 0,006 | 0,940 |
| 1A3b | Road Transportation |  | N2O | 2.197,32 | 0,005 | 0,946 |
| 1B2 | Fugitive Emissions from Oil and Natural gas |  | CO2 | 2.148,94 | 0,005 | 0,951 |
| SUBTOTAL |  |  |  | 380.288,22 |  |  |
| TOTAL EMISSIONS |  |  |  | 399.731,53 |  |  |
| PERCENT OF TOTAL |  |  |  | 95,14 |  |  |

(A). Percentage of the activity categories emissions with relation to total inventory emissions.

(B). Cumulative percentage of the activity categories emissions with relation to total inventory emissions.

TABLE 5.6.
TREND CONTRIBUTION BY ACTIVITIES – YEAR 2002

| IPCC SOURCE CATEGORIES |  | Fuel | Gas | Gg CO2-e 1990 | Gg CO2-e 2002 | Trend (A) | Contribution (B) | Cumulative (C) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1ª2+1A4 | Stationary Combustion - Other sources - Gas | Gas | CO2 | 9.465,71 | 37.288,43 | 0,043 | 0,180 | 0,180 |
| 1A3b | Road Transportation -Diesel Oil | Diesel Oil | CO2 | 25.383,12 | 57.209,14 | 0,038 | 0,162 | 0,342 |
| 1A2+1A4 | Stationary Combustion - Other sources - Coal | Coal | CO2 | 15.078,76 | 5.694,35 | 0,028 | 0,116 | 0,458 |
| 1A2+1A4 | Stationary Combustion - Other sources - Oil | Oil | CO2 | 45.247,12 | 53.102,09 | 0,019 | 0,078 | 0,536 |
| 1A3b | Road Transportation- Gasoline | Gasoline | CO2 | 25.928,33 | 25.982,17 | 0,019 | 0,078 | 0,615 |
| 1A1a | Public Electricity and Heat Production- Oil | Oil | CO2 | 6.015,03 | 14.097,41 | 0,010 | 0,042 | 0,657 |
| 1A1a | Public Electricity and Heat Production- Gas | Gas | CO2 | 427,14 | 5.900,37 | 0,009 | 0,040 | 0,697 |
| 4D1 | Agricultural Soils - Direct Emissions |  | N2O | 8.535,01 | 8.357,39 | 0,006 | 0,027 | 0,724 |
| 1A1b | Petroleum refining- Oil | Oil | CO2 | 10.862,00 | 11.903,22 | 0,006 | 0,025 | 0,749 |
| 6A | Solid Waste Disposal on Land |  | CH4 | 5.390,92 | 10.872,77 | 0,006 | 0,025 | 0,774 |
| 4A | Enteric Fermentation |  | CH4 | 12.651,18 | 14.720,06 | 0,005 | 0,023 | 0,797 |
| 2F | Consumption of Halocarbons and SF6 (except HCFC-22) |  | HFC & PFC | 0,00 | 2.790,71 | 0,005 | 0,021 | 0,818 |
| 1A1a | Public Electricity and Heat Production- Coal | Coal | CO2 | 57.779,09 | 78.418,78 | 0,005 | 0,021 | 0,838 |
| 2E1 | HCFC-22 Manufacture (HFC-23 Emissions) |  | HFC | 2.403,18 | 1.028,66 | 0,004 | 0,018 | 0,856 |
| 2B2 | Nitric Acid Production |  | N2O | 2.884,16 | 1.945,10 | 0,004 | 0,016 | 0,872 |
| 2A1 | Cement Production |  | CO2 | 12.534,33 | 15.853,10 | 0,003 | 0,013 | 0,885 |
| 4D3 | Agricultural Soils - Indirect Emissions |  | N2O | 4.836,27 | 5.204,67 | 0,003 | 0,012 | 0,897 |
| 1A1c | Manufacture of Solid fuels and Other Energy Industries | Coal | CO2 | 1.664,03 | 792,12 | 0,003 | 0,012 | 0,908 |
| 1B1 | Fugitive Emissions from Solid fuels |  | CH4 | 1.788,93 | 1.019,04 | 0,003 | 0,011 | 0,920 |
| 1A3b | Road Transportation |  | N2O | 672,98 | 2.197,32 | 0,002 | 0,009 | 0,929 |
| 2C3 | Aluminium Production |  | PFC | 828,41 | 191,81 | 0,002 | 0,007 | 0,936 |
| 1A1b | Petroleum refining- Gas | Gas | CO2 | 45,08 | 834,53 | 0,001 | 0,006 | 0,942 |
| 1A3a | Domestic Civil Aviation |  | CO2 | 4.135,39 | 5.084,17 | 0,001 | 0,005 | 0,947 |
| 4B | Manure Management |  | N2O | 1.631,73 | 1.632,77 | 0,001 | 0,005 | 0,952 |
| SUBTOTAL |  |  |  | 256.187,90 | 362.120,21 | 0,226 |  |  |
| TOTAL EMISSIONS |  |  |  | 284.555,86 | 399.731,53 | 0,237 |  |  |
| PERCENT OF TOTAL |  |  |  | 90,03 | 90,59 | 95,24 |  |  |

(A). See the formula (2) from the previous footnote 2 previous.

(B). Percentage of the activity categories emissions with relation to total inventory emissions.

(C). Cumulative percentage of the activity categories emissions with relation to total inventory emissions.

TABLE 5.7.
SUMMARY OF THE ACTIVITIES CONTRIBUTION TO THE INVENTORY

| IPCC SOURCE CATEGORIES |  | Fuel | Gas | Key Source | Level criteria | Trend criteria | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ENERGY |  |  |  |  |  |  |  |  |
| 1A1a | Public Electricity and Heat Production- Gas | Gas | CO2 | YES | YES | YES | Level in 1997-2002 | Trend in 1991, 1993-1995, 1997-2002 |
| 1A1a | Public Electricity and Heat Production- Coal | Coal | CO2 | YES | YES | YES | Level in 1990-2002 | Trend in 1991-2002 |
| 1A1a | Public Electricity and Heat Production- Oil | Oil | CO2 | YES | YES | YES | Level in 1990-2002 | Trend in 1991-1997, 1999-2002 |
| 1A1a | Public Electricity and Heat Production- Other | Other | CO2 |  |  |  |  |  |
| 1A1b | Petroleum refining- Gas | Gas | CO2 | YES |  | YES |  | Trend in 1997-2000, 2002 |
| 1A1b | Petroleum refining- Oil | Oil | CO2 | YES | YES | YES | Level in 1990-2002 | Trend in 1991-1996, 1998-2002 |
| 1A1c | Manufacture of Solid fuels and Other Energy Industries | Gas | CO2 | YES |  | YES |  | Trend in 1991-1992, 1996-1998 |
| 1A1c | Manufacture of Solid fuels and Other Energy Industries | Coal | CO2 | YES | YES | YES | Level in 1990-1991 | Trend in 1992-2002 |
| 1A1c | Manufacture of Solid fuels and Other Energy Industries | Oil | CO2 |  |  |  |  |  |
| 1A2+1A4 | Stationary Combustion - Other sources - Gas | Gas | CO2 | YES | YES | YES | Level in 1990-2002 | Trend in 1991-2002 |
| 1A2+1A4 | Stationary Combustion - Other sources - Coal | Coal | CO2 | YES | YES | YES | Level in 1990-2002 | Trend in 1991-2002 |
| 1A2+1A4 | Stationary Combustion - Other sources - Oil | Oil | CO2 | YES | YES | YES | Level in 1990-2002 | Trend in 1991-2002 |
| 1A2+1A4 | Stationary Combustion - Other sources - Other | Other | CO2 |  |  |  |  |  |
| 1A1+1A2+1A4 | Stationary Combustion |  | CH4 | YES |  | YES |  | Trend in 1995 |
| 1A1+1A2+1A4 | Stationary Combustion |  | N2O | YES | YES | YES | Level in 1990-2002 | Trend in 1991-1992, 1995 |
| 1A3a | Domestic Civil Aviation |  | CO2 | YES | YES | YES | Level in 1990-2002 | Trend in 1991-1997, 2002 |
| 1A3a | Domestic Civil Aviation |  | CH4 |  |  |  |  |  |
| 1A3a | Domestic Civil Aviation |  | N2O |  |  |  |  |  |
| 1A3b | Road Transportation- Gasoline | Gasoline | CO2 | YES | YES | YES | Level in 1990-2002 | Trend in 1991-2002 |
| 1A3b | Road Transportation -Diesel Oil | Diesel Oil | CO2 | YES | YES | YES | Level in 1990-2002 | Trend in 1991-2002 |
| 1A3b | Road Transportation |  | CH4 |  |  |  |  |  |
| 1A3b | Road Transportation |  | N2O | YES | YES | YES | Level in 1998-2000, 2002 | Trend in 1993-2002 |
| 1A3d | National navigation |  | CO2 | YES | YES | YES | Level in 1990, 1993-1996, 1999-2002 | Trend in 1992, 1994, 1997-1998 |
| 1A3d | National navigation |  | CH4 |  |  |  |  |  |
| 1A3d | National navigation |  | N2O |  |  |  |  |  |
| 1A3c+1A3e | Other transport |  | CO2 |  |  |  |  |  |
| 1A3c+1A3e | Other transport |  | CH4 |  |  |  |  |  |
| 1A3c+1A3e | Other transport |  | N2O |  |  |  |  |  |
| 1B1 | Fugitive Emissions from Solid fuels |  | CO2 |  |  |  |  |  |
| 1B1 | Fugitive Emissions from Solid fuels |  | CH4 | YES | YES | YES | Level in 1990-1992, 1997 | Trend in 1991-2002 |
| 1B2 | Fugitive Emissions from Oil and Natural gas |  | CO2 | YES | YES | YES | Level in 1990-2002 | Trend in 1991, 1993-1994 |
| 1B2 | Fugitive Emissions from Oil and Natural gas |  | CH4 |  |  |  |  |  |

TABLE 5.7. (Cont.)
SUMMARY OF THE ACTIVITIES CONTRIBUTION TO THE INVENTORY

| IPCC SOURCE CATEGORIES |  | Fuel | Gas | Key Source | Level criteria | Trend criteria | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| INDUSTRIAL PROCESSES |  |  |  |  |  |  |  |  |
| 2A1 | Cement Production |  | CO2 | YES | YES | YES | Level in 1990-2002 | Trend in 1991-2002 |
| 2A2 | Lime Production |  | CO2 | YES |  | YES |  | Trend in 1994 |
| 2C1 | Iron and Steel Production |  | CO2 |  |  |  |  |  |
| 2-2A1-2A2-2C1 | Other Industrial Processes |  | CO2 | YES | YES | YES | Level in 1990-2002 | Trend in 1992-1994, 1999 |
| 2 | Industrial Processes |  | CH4 |  |  |  |  |  |
| 2B2 | Nitric Acid Production |  | N2O | YES | YES | YES | Level in 1990-2001 | Trend in 1991-2002 |
| 2C3 | Aluminium Production |  | PFC | YES |  | YES |  | Trend in 1991, 2000-2002 |
| 2E1 | HCFC-22 Manufacture (HFC-23 Emissions) |  | HFC | YES | YES | YES | Level in 1990-2001 | Trend in 1991-2000, 2002 |
| 2E2+2E3 | Production of Halocarbons and SF6 (except HCFC-22) |  | HFC & PFC |  |  |  |  |  |
| 2F | Consumption of Halocarbons and SF6 (except HCFC-22) |  | HFC & PFC | YES | YES | YES | Level in 2001-2002 | Trend in 1997-2002 |
| 2F7 | Electrical Equipment |  | SF6 |  |  |  |  |  |
| SOLVENT AND OTHER PRODUCT USE |  |  |  |  |  |  |  |  |
| 3 | Solvent and Other Product Use |  | CO2 |  |  |  |  |  |
| 3 | Solvent and Other Product Use |  | N2O |  |  |  |  |  |
| AGRICULTURE |  |  |  |  |  |  |  |  |
| 4A | Enteric Fermentation |  | CH4 | YES | YES | YES | Level in 1990-2002 | Trend in 1991-1995, 1997-2002 |
| 4B | Manure Management |  | CH4 | YES | YES | YES | Level in 1990-2002 | Trend in 1991, 1993-1994, 1996, 1998 |
| 4B | Manure Management |  | N2O | YES | YES | YES | Level in 1990-1994, 1996-1998 | Trend in 1994-1995, 1997-2002 |
| 4C | Rice Cultivation |  | CH4 |  |  |  |  |  |
| 4D1 | Agricultural Soils - Direct Emissions |  | N2O | YES | YES | YES | Level in 1990-2002 | Trend in 1991-2002 |
| 4D2 | Agricultural Soils - Animal Production |  | N2O | YES | YES | YES | Level in 1990-2002 | Trend in 1992, 1995, 1998-1999 |
| 4D3 | Agricultural Soils - Indirect Emissions |  | N2O | YES | YES | YES | Level in 1990-2002 | Trend in 1991-1995, 1997-2002 |
| 4F | Field Burning of Agricultural Wastes |  | CH4 |  |  |  |  |  |
| 4F | Field Burning of Agricultural Wastes |  | N2O |  |  |  |  |  |
| WASTE |  |  |  |  |  |  |  |  |
| 6A | Solid Waste Disposal on Land |  | CO2 | YES |  | YES |  | Trend in 1995 |
| 6A | Solid Waste Disposal on Land |  | CH4 | YES | YES | YES | Level in 1990-2002 | Trend in 1991-2002 |
| 6B | Wastewater Handling |  | CH4 | YES | YES | YES | Level in 1991-2002 | Trend in 1996 |
| 6B | Wastewater Handling |  | N2O |  |  |  |  |  |
| 6C | Waste Incineration |  | CO2 | YES |  | YES |  | Trend in 1995-2001 |
| 6C | Waste Incineration |  | CH4 |  |  |  |  |  |
| 6C | Waste Incineration |  | N2O |  |  |  |  |  |

- 5.5.- UNCERTAINTY ANALYSIS: TIER 1

The results presented in the preceding sections of this chapter refer to the estimates for the central values of the emissions corresponding to the combinations of activities and pollutants. The aggregation of the emissions, weighted according to the 1995 IPCC (100 years horizon) warming potentials for each gas under consideration, are reflected in the estimation of the central value of the overall emission of the inventory. The central value is, however, only an indication of the level of the random variable that is the estimated emission of each pollutant source and gas. In order to characterize the precission of the estimation, it is necessary to establish methods to determine its uncertainty. The IPCC Good Practice Guidance offers two approaches or levels (Tiers) for quantifying uncertainty in each activity and gas, as well as for the determination of the uncertainty of the inventory’s weighted emission. The Tier 1 approach, the one adopted for estimating the uncertainty in this edition of the inventory, deals with the determination of uncertainty by using two-stage error propagation equations.

In the first stage, an approximate estimate is made of the uncertainty of the emissions in a source category and gas bearing in mind that this emission can be represented as the product of an activity variable and an emission factor, and taking into account the combination of the uncertainty of these two component factors as expressed in the following equation:

UE=UA2+UF2	[5.5.1]

where:

UE	represents the uncertainty associated to the emission

UA	represents the uncertainty associated to the activity variable

UF	represents the uncertainty associated to the emission factor

and where UE, UA and UF express, as a percentage, the ratios (uncertainty coefficients) whose numerator is one half of the 95% confidence interval of the variable considered and the denominator is the expected value of the variable.

In the second stage an approximate estimate is made of the uncertainty of an aggregate as the weighted sum of the component uncertainties, combinations of activities and pollutants, making up the inventory. This estimation of the uncertainty is effected both in terms of the level and in terms of the trend, the difference between emissions in the reference year and the base year, as expressed in equation [5.5.2].

UEtotal=UE1E12+UE2E22+...+UEnEn2E1+E2+...+En	[5.5.2]

where:

UEtotal	represents the uncertainty associated to the aggregated emissions

UEi	represents the uncertainty associated to the component emission

Ei	represents the expected value of each component emission

and where UEtotal and UEi express, as a percentage, the ratios (uncertainty coefficients) whose numerator is one half of the 95% confidence interval of the variable considered and the denominator is the expected value of the variable.

The Tier 1 procedure has been applied according to the definition and relations between variables (columns) of Table 6.1, Section 6.3.2, of the IPCC Good Practice Guidance implemented on a spreadsheet, which is also provided separately.

For the estimation of the trend’s uncertainty, the difference between the reference year and the base year, two types of sensitivity have been defined to assess these differences:

-	Type A Sensitivity.

This represents the change in the difference in the inventory emissions between the base year and the reference year, expressed as a percentage, resulting from an increase of 1% in the emissions of a given source and gas in the base year and in the reference year.

-	Type B Sensitivity.

This represents the change in the difference in the inventory emissions between the base year and the reference year, expressed as a percentage, resulting from an increase of 1% in the emissions of a given source and gas only in the reference year.

Conceptually, the Type A sensitivity arises from uncertainties affecting both the base year and the reference year, whereas the Type B sensitivity arises from uncertainties that only affect  the reference year. The uncertainties that correlate over the years are normally associated with Type A sensitivity, whereas the uncertainties that do not correlate over the years are associated with Type B sensitivity. These two types of sensitivity introduce simplifications into the correlation analysis. To make the algorithm operative, it is assumed, by default, that the uncertainties of the emission factors correspond to Type A sensitivity, as they are normally correlated over the years; whereas the activity variables correspond to Type B sensitivity, as they are not normally correlated over time, unless otherwise mentioned as will be seen below in the application of the algorithm to the case of the present inventory. Once the emissions uncertainties have been calculated according to each of the two types of sensitivity indicated, they can be added together in a weighted manner using the propagation error equation (equation [5.5.2]) to obtain the overall uncertainty for the trend.

The calculation procedure is carried out, as indicated above, using the spreadsheet which reproduces the concepts and formulas of columns A to M in Table 6.1, Section 6.3.2, of the aforementioned IPCC Good Practices Guidance.

In order to make the estimation procedure operational, it is necessary to introduce the data corresponding to the following columns as the primary information for the spreadsheet:

A-B	Respectively listing the source categories considered in the analysis (A) and the emitted gas (B). These combinations of source and gas categories are sorted by their importance in the inventory for the table’s reference year. The reference years taken for this analysis, and with respect to the base year, have been 2000 and 2001, as the most recent ones among the latest ones available, except for 2002 for which a very considerable part of the base information such as the national fuels balance is still provisional as of the date of the drafting of the inventory.

C	Emissions by source and gas category, in CO2-equivalent terms, in the base year. The data introduced are the central values of the estimated emissions for the inventory of the base year, 1990 for CO2, CH4 and N2O, and 1995 for the fluorinated gases.

D	Emissions by source and gas category, in CO2-equivalent terms, in the years 2000 and 2001. The data introduced are the central values of the estimated emissions for the inventory of 2000 and 2001. The information is presented on separate worksheets for each reference year, one for 2000 and another for 2001.

E	Uncertainty associated to the activity variable, expressed as a percentage based on a ratio, the uncertainty coefficient, where the numerator is one half of the 95% confidence interval of the variable considered and the denominator is the expected value of the variable.

F	Uncertainty associated to the emission factor, expressed as a percentage based on a ratio, the uncertainty coefficient, where the numerator is one half of the 95% confidence interval of the emission factor considered and the denominator is the expected value of the emission factor.

S/N	This column indicates, with an S for Yes, those source categories in which it is felt that the activity variable correlates over time. The N for No indicates the opposite case, that is to say, there is no correlation of the activity variable over time. Thus, the source categories marked with an S are the exception to Type B sensitivity in the activity variables.

With the foregoing information, the remaining columns in the table are calculated in accordance with the formulas specified in Section 6.3.2 of Table 6.1 of the aforesaid IPCC Good Practice Guidance.

The data introduced into columns E and F come from a variety of sources:

-	The main references considered have been the revised IPCC Guidelines from 1996 and the Good Practice Guidance from 2000.

-	The estimates considered most credible by the inventory working team include information from another series of sources:

·	For issues regarding fuel activity variables, analysis of the fuel balance variability (supply versus demand and statistical errors) and that coming from the individual questionnaires to the plants.

·	For CO2 emission factors in combustion, from the analysis of variability of the heating values and carbon contents of the fuels per energy unit or by unit of mass or volume. Furthermore, criteria have been introduced on the presumable uncertainty in the oxidation coefficients, parameters on which some information has been obtained from experts in the energy sector. For the CH4 and N2O factors in combustion, wide bands have been taken to cover the variability observed in the IPCC and EMEP/CORINAIR references and for both stationary and mobile combustion.

·	For the fugitive emissions of energy, regard has also been had for the variability indicated in the IPCC and EMEP/CORINAIR references.

·	For the emissions generated in industrial processes, the information in the IPCC Guidelines has been complemented by information received by the inventory working team from experts in the sector and, where appropriate, from the individual questionnaires sent to plants.

·	For the determination of the uncertainty for activity variables and the emission factors for CH4 and N2O coming from agriculture, the information available in the IPCC Guidelines has been complemented by the hypotheses assumed by the inventory working team regarding uncertainty in the assignation of husbandry management systems and farming practices.

·	For the determination of the uncertainty for activity variables and the emission factors for CH4 and N2O coming from waste treatment, the information available in the IPCC Guidelines has been complemented by the hypotheses assumed by the inventory working team regarding uncertainty in the waste management systems and pertinent parameters for the treatment processes.

·	For the activity variables and emission factors for the production of fluorinated hydrocarbons (emissions of HFC-23, a sub-product of the manufacture of HCFC-22), consideration has been given to the upper limits for the presumable accuracy in the estimation procedures used by the manufacturing plants. For the activity variables and emission factors in the consumption of HFCs and SF6, factors of 2 and 3 have been taken respectively in the light of the lack of precision and coverage of some of the data sources used.

·	For a wide set of activities*gases the references taken have come from the information obtained in the comparative analysis of the inventories by other European Union Member States.

Tables 5.5.2 and 5.5.3 show, respectively for 2000 and 2001, the primary data and the results of the estimation procedure described above. As a synthesis of the results of the procedure for quantifying uncertainty, Table 5.5.1 allows the following conclusions to be drawn:

- The confidence interval at 95% for the level of the emissions of the inventory aggregate is in the order of 17% around the central value for the reference year, specifically 17.5% for 2000 and 16.6% for 2001.

- The confidence interval at 95% for the emissions evolution index in the reference year with respect to the base year is in the order of 2.5%, specifically 2.2% for 2000 and 2.5% for 2001.

Table 5.5.1
95% confidence intervals for the level
and trend in inventory emissions

| Year | Absolute values (kt CO2-e) |  |  |  |  | Evolution index over base year = 100 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Central value | Lower limit |  | Upper limit |  | Central value | Lower limit |  | Upper limit |  |
|  |  | Value | % | Value | % |  | Value | % | Value | % |
| Base year | 286,798 |  |  |  |  |  |  |  |  |  |
| 2000 | 385,203 | 317,792 | -17.5 | 452,614 | 17.5 | 134.3 | 131.4 | -2.2 | 137.3 | 2.2 |
| 2001 | 383,460 | 319,806 | -16.6 | 447,114 | 16.6 | 133.7 | 130.4 | -2.5 | 137.0 | 2.5 |

TABLE 5.5.2
UNCERTAINTY ANALYSIS OF GREENHOUSE GASES EMISSIONS. TIER 1. YEAR 2000

| A |  | B | C | D |  |  | E | F | G |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Key Sources (Year 2000) |  | Gas | Base year Emissions (Gg CO2-e) | Year 2000 Emissions (Gg CO2-e) | Contribution Level 2000 (%) | Cumulative Level 2000 (%) | AD Uncertainty (%) | EF Uncertainty (%) | Combined Uncertainty (%) |
| IPCC Code | Description of IPCC category |  |  |  |  |  |  |  |  |
| 1A1a | Public Electricity and Heat Production- Coal | CO2 | 57.779 | 75.441 | 19,6 | 20 | 2 | 4 | 4,5 |
| 1A2+1A4 | Stationary Combustion - Other sources - Oil | CO2 | 45.247 | 55.281 | 14,4 | 34 | 3,5 | 5 | 6,1 |
| 1A3b | Road Transportation -Diesel Oil | CO2 | 25.383 | 49.944 | 13,0 | 47 | 5 | 2,2 | 5,5 |
| 1A2 +1A4 | Stationary Combustion - Other sources - Gas | CO2 | 9.466 | 32.392 | 8,4 | 55 | 3 | 3 | 4,2 |
| 1A3b | Road Transportation- Gasoline | CO2 | 25.928 | 27.144 | 7,0 | 62 | 3,5 | 2,1 | 4,1 |
| 2A1 | Cement Production | CO2 | 12.534 | 15.034 | 3,9 | 66 | 1,5 | 8,2 | 8,3 |
| 4A | HCFC-22 Manufacture (HFC-23 Emissions) | CH4 | 12.651 | 14.231 | 3,7 | 70 | 10 | 20 | 22,4 |
| 1A1b | Petroleum refining- Oil | CO2 | 10.862 | 12.389 | 3,2 | 73 | 2,5 | 2,7 | 3,7 |
| 1A1a | Public Electricity and Heat Production- Oil | CO2 | 6.015 | 11.163 | 2,9 | 76 | 1,5 | 2 | 2,5 |
| 6A | Solid Waste Disposal on Land | CH4 | 5.391 | 10.098 | 2,6 | 79 | 30 | 70,4 | 76,5 |
| 4D1 | Agricultural Soils - Direct Emissions | N2O | 8.535 | 9.736 | 2,5 | 81 | 7,5 | 380 | 380,1 |
| 4B | Manure Management | CH4 | 6.221 | 8.439 | 2,2 | 83 | 35 | 200 | 203,0 |
| 2E1 | HCFC-22 Manufacture (HFC-23 Emissions) | HFC | 4.638 | 6.323 | 1,6 | 85 | 2 | 20 | 20,1 |
| 1A2 +1A4 | Stationary Combustion - Other sources - Coal | CO2 | 15.079 | 5.939 | 1,5 | 87 | 12 | 15 | 19,2 |
| 4D3 | Agricultural Soils - Indirect Emissions | N2O | 4.836 | 5.609 | 1,5 | 88 | 100 | 900 | 905,5 |
| 1A3a | Domestic Civil Aviation | CO2 | 4.135 | 5.497 | 1,4 | 89 | 35 | 2 | 35,1 |
| 1A1+1A2+1A4 | Stationary Combustion | N2O | 2.895 | 3.792 | 1,0 | 90 | 4 | 75 | 75,1 |
| 4D2 | Agricultural Soils - Animal Production | N2O | 2.794 | 3.210 | 0,8 | 91 | 50 | 100 | 111,8 |
| 1A1a | Public Electricity and Heat Production- Gas | CO2 | 427 | 2.747 | 0,7 | 92 | 1,75 | 1,5 | 2,3 |
| 2-2A1-2A2-2C1 | Other Industrial Processes | CO2 | 2.399 | 2.676 | 0,7 | 93 | 100 | 20 | 102,0 |
| 2B2 | Nitric Acid Production | N2O | 2.884 | 2.307 | 0,6 | 93 | 2 | 10 | 10,2 |
| 6B | Wastewater Handling | CH4 | 1.518 | 2.241 | 0,6 | 94 | 50 | 100 | 111,8 |
| 1B2 | Fugitive Emissions from Oil and Natural gas | CO2 | 1.743 | 2.197 | 0,6 | 94 | 10 | 25 | 26,9 |
| 1A3b | Road Transportation | N2O | 673 | 1.876 | 0,5 | 95 | 5 | 100 | 100,1 |
| 1A3d | National navigation | CO2 | 1.536 | 1.850 | 0,5 | 95 | 50 | 2,7 | 50,1 |
| 2F | Consumption of Halocarbons and SF6 (except HCFC-22) | HFC & PFC | 8 | 1.818 | 0,5 | 96 | 100 | 200 | 223,6 |
| 4B | Manure Management | N2O | 1.632 | 1.605 | 0,4 | 96 | 35 | 200 | 203,0 |
| 2A2 | Lime Production | CO2 | 917 | 1.277 | 0,3 | 97 | 20 | 2,3 | 20,1 |
| 3 | Solvent and Other Product Use | CO2 | 964 | 1.267 | 0,3 | 97 | 100 | 25 | 103,1 |
| 1B1 | Fugitive Emissions from Solid fuels | CH4 | 1.789 | 1.202 | 0,3 | 97 | 10 | 100 | 100,5 |
| 6B | Wastewater Handling | N2O | 1.002 | 1.079 | 0,3 | 98 | 30 | 100 | 104,4 |
| 1A1c | Manufacture of Solid fuels and Other Energy Industries | CO2 | 1.664 | 851 | 0,2 | 98 | 5 | 15,1 | 15,9 |
| 1B2 | Fugitive Emissions from Oil and Natural gas | CH4 | 584 | 830 | 0,2 | 98 | 50 | 100 | 111,8 |
| *** | Other emission sources | *** | 6.667 | 7.717 | 2,0 | 100 | 100 | 100 | 141,4 |
| Total emissions (without LUCF) |  |  | 286.798 | 385.203 |  |  |  |  |  |

TABLE 5.5.2 (Cont.)
UNCERTAINTY ANALYSIS OF GREENHOUSE GASES EMISSIONS. TIER 1. YEAR 2000

| A |  | B | H | YN | I | J | K | L | M |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Key Sources (Year 2000) |  | Gas | % Combined Uncertainty (% Emisiones totales 2000) | AD correlated between years (Y/N) | Type A Sensibility | Type B Sensibility | Uncertainty in trend by EF uncertainty (%) | Uncertainty in trend by AD uncertainty (%) | Uncertainty in trend (%) |
| IPCC Code | Description of IPCC category |  |  |  |  |  |  |  |  |
| 1A1a | Public Electricity and Heat Production- Coal | CO2 | 0,9 | N | -0,008 | 0,263 | -0,03 | 0,74 | 0,74 |
| 1A2+1A4 | Stationary Combustion - Other sources - Oil | CO2 | 0,9 | N | -0,019 | 0,193 | -0,10 | 0,95 | 0,96 |
| 1A3b | Road Transportation -Diesel Oil | CO2 | 0,7 | N | 0,055 | 0,174 | 0,12 | 1,23 | 1,24 |
| 1A2 +1A4 | Stationary Combustion - Other sources - Gas | CO2 | 0,4 | N | 0,069 | 0,113 | 0,21 | 0,48 | 0,52 |
| 1A3b | Road Transportation- Gasoline | CO2 | 0,3 | N | -0,027 | 0,095 | -0,06 | 0,47 | 0,47 |
| 2A1 | Cement Production | CO2 | 0,3 | N | -0,006 | 0,052 | -0,05 | 0,11 | 0,12 |
| 4A | HCFC-22 Manufacture (HFC-23 Emissions) | CH4 | 0,8 | N | -0,010 | 0,050 | -0,19 | 0,70 | 0,73 |
| 1A1b | Petroleum refining- Oil | CO2 | 0,1 | N | -0,008 | 0,043 | -0,02 | 0,15 | 0,15 |
| 1A1a | Public Electricity and Heat Production- Oil | CO2 | 0,1 | N | 0,011 | 0,039 | 0,02 | 0,08 | 0,09 |
| 6A | Solid Waste Disposal on Land | CH4 | 2,0 | Y | 0,010 | 0,035 | 0,70 | 0,30 | 0,76 |
| 4D1 | Agricultural Soils - Direct Emissions | N2O | 9,6 | N | -0,006 | 0,034 | -2,29 | 0,36 | 2,32 |
| 4B | Manure Management | CH4 | 4,4 | Y | 0,000 | 0,029 | 0,06 | 0,01 | 0,06 |
| 2E1 | HCFC-22 Manufacture (HFC-23 Emissions) | HFC | 0,3 | N | 0,000 | 0,022 | 0,01 | 0,06 | 0,06 |
| 1A2 +1A4 | Stationary Combustion - Other sources - Coal | CO2 | 0,3 | N | -0,050 | 0,021 | -0,75 | 0,35 | 0,83 |
| 4D3 | Agricultural Soils - Indirect Emissions | N2O | 13,2 | Y | -0,003 | 0,020 | -2,78 | -0,31 | 2,80 |
| 1A3a | Domestic Civil Aviation | CO2 | 0,5 | N | 0,000 | 0,019 | 0,00 | 0,95 | 0,95 |
| 1A1+1A2+1A4 | Stationary Combustion | N2O | 0,7 | N | 0,000 | 0,013 | -0,03 | 0,07 | 0,08 |
| 4D2 | Agricultural Soils - Animal Production | N2O | 0,9 | N | -0,002 | 0,011 | -0,19 | 0,79 | 0,81 |
| 1A1a | Public Electricity and Heat Production- Gas | CO2 | 0,0 | N | 0,008 | 0,010 | 0,01 | 0,02 | 0,03 |
| 2-2A1-2A2-2C1 | Other Industrial Processes | CO2 | 0,7 | N | -0,002 | 0,009 | -0,04 | 1,32 | 1,32 |
| 2B2 | Nitric Acid Production | N2O | 0,1 | N | -0,005 | 0,008 | -0,05 | 0,02 | 0,06 |
| 6B | Wastewater Handling | CH4 | 0,7 | N | 0,001 | 0,008 | 0,07 | 0,55 | 0,56 |
| 1B2 | Fugitive Emissions from Oil and Natural gas | CO2 | 0,2 | N | -0,001 | 0,008 | -0,01 | 0,11 | 0,11 |
| 1A3b | Road Transportation | N2O | 0,5 | N | 0,003 | 0,007 | 0,34 | 0,05 | 0,34 |
| 1A3d | National navigation | CO2 | 0,2 | N | -0,001 | 0,006 | 0,00 | 0,46 | 0,46 |
| 2F | Consumption of Halocarbons and SF6 (except HCFC-22) | HFC & PFC | 1,1 | Y | 0,006 | 0,006 | 1,26 | 0,63 | 1,41 |
| 4B | Manure Management | N2O | 0,8 | Y | -0,002 | 0,006 | -0,41 | -0,07 | 0,42 |
| 2A2 | Lime Production | CO2 | 0,1 | N | 0,000 | 0,004 | 0,00 | 0,13 | 0,13 |
| 3 | Solvent and Other Product Use | CO2 | 0,3 | N | 0,000 | 0,004 | 0,00 | 0,62 | 0,62 |
| 1B1 | Fugitive Emissions from Solid fuels | CH4 | 0,3 | N | -0,004 | 0,004 | -0,42 | 0,06 | 0,42 |
| 6B | Wastewater Handling | N2O | 0,3 | N | -0,001 | 0,004 | -0,09 | 0,16 | 0,18 |
| 1A1c | Manufacture of Solid fuels and Other Energy Industries | CO2 | 0,0 | N | -0,005 | 0,003 | -0,07 | 0,02 | 0,08 |
| 1B2 | Fugitive Emissions from Oil and Natural gas | CH4 | 0,2 | N | 0,000 | 0,003 | 0,02 | 0,20 | 0,21 |
| *** | Other emission sources | *** | 2,8 | N | -0,004 | 0,027 | -0,43 | 3,81 | 3,83 |
| Uncertainty in total emissions |  |  | 17,5 | In trend (difference between years): |  |  |  |  | 6,30 |
|  |  |  |  | In trend (% of mean total emissions in the base year): |  |  |  |  | 2,20 |

TABLE 5.5.3
UNCERTAINTY ANALYSIS OF GREENHOUSE GASES EMISSIONS. TIER 1. YEAR 2001

| A |  | B | C | D |  |  | E | F | G |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Key Sources (Year 2001) |  | Gas | Base year Emissions (Gg CO2-e) | Year 2001 Emissions (Gg CO2-e) | Contribution Level 2001 (%) | Cumulative Level 2001 (%) | AD Uncertainty (%) | EF Uncertainty (%) | Combined Uncertainty (%) |
| IPCC Code | Description of IPCC category |  |  |  |  |  |  |  |  |
| 1A1a | Public Electricity and Heat Production- Coal | CO2 | 57.779 | 68.309 | 17,8 | 18 | 2 | 4 | 4,5 |
| 1A2+1A4 | Stationary Combustion - Other sources - Oil | CO2 | 45.247 | 55.977 | 14,6 | 32 | 3,5 | 5 | 6,1 |
| 1A3b | Road Transportation -Diesel Oil | CO2 | 25.383 | 53.850 | 14,0 | 46 | 5 | 2,2 | 5,5 |
| 1A2 +1A4 | Stationary Combustion - Other sources - Gas | CO2 | 9.466 | 34.135 | 8,9 | 55 | 3 | 3 | 4,2 |
| 1A3b | Road Transportation- Gasoline | CO2 | 25.928 | 26.995 | 7,0 | 62 | 3,5 | 2,1 | 4,1 |
| 2A1 | Cement Production | CO2 | 12.534 | 15.327 | 4,0 | 66 | 1,5 | 8,2 | 8,3 |
| 4A | HCFC-22 Manufacture (HFC-23 Emissions) | CH4 | 12.651 | 14.599 | 3,8 | 70 | 10 | 20 | 22,4 |
| 1A1b | Petroleum refining- Oil | CO2 | 10.862 | 12.394 | 3,2 | 73 | 2,5 | 2,7 | 3,7 |
| 1A1a | Public Electricity and Heat Production- Oil | CO2 | 6.015 | 12.325 | 3,2 | 77 | 1,5 | 2 | 2,5 |
| 6A | Solid Waste Disposal on Land | CH4 | 5.391 | 10.485 | 2,7 | 79 | 30 | 70,4 | 76,5 |
| 4D1 | Agricultural Soils - Direct Emissions | N2O | 8.535 | 8.789 | 2,3 | 82 | 7,5 | 380 | 380,1 |
| 4B | Manure Management | CH4 | 6.221 | 8.583 | 2,2 | 84 | 35 | 200 | 203,0 |
| 1A2 +1A4 | Stationary Combustion - Other sources - Coal | CO2 | 15.079 | 5.992 | 1,6 | 85 | 12 | 15 | 19,2 |
| 1A3a | Domestic Civil Aviation | CO2 | 4.135 | 5.618 | 1,5 | 87 | 35 | 2 | 35,1 |
| 4D3 | Agricultural Soils - Indirect Emissions | N2O | 4.836 | 5.318 | 1,4 | 88 | 100 | 900 | 905,5 |
| 1A1+1A2+1A4 | Stationary Combustion | N2O | 2.895 | 3.874 | 1,0 | 89 | 4 | 75 | 75,1 |
| 4D2 | Agricultural Soils - Animal Production | N2O | 2.794 | 3.290 | 0,9 | 90 | 50 | 100 | 111,8 |
| 1A1a | Public Electricity and Heat Production- Gas | CO2 | 427 | 3.168 | 0,8 | 91 | 1,75 | 1,5 | 2,3 |
| 2E1 | HCFC-22 Manufacture (HFC-23 Emissions) | HFC | 4.638 | 2.903 | 0,8 | 92 | 2 | 20 | 20,1 |
| 2-2A1-2A2-2C1 | Other Industrial Processes | CO2 | 2.399 | 2.839 | 0,7 | 93 | 100 | 20 | 102,0 |
| 2F | Consumption of Halocarbons and SF6 (except HCFC-22) | HFC & PFC | 8 | 2.348 | 0,6 | 93 | 100 | 200 | 223,6 |
| 6B | Wastewater Handling | CH4 | 1.518 | 2.327 | 0,6 | 94 | 50 | 100 | 111,8 |
| 1A3d | National navigation | CO2 | 1.536 | 2.098 | 0,5 | 94 | 50 | 2,7 | 50,1 |
| 1B2 | Fugitive Emissions from Oil and Natural gas | CO2 | 1.743 | 2.067 | 0,5 | 95 | 10 | 25 | 26,9 |
| 2B2 | Nitric Acid Production | N2O | 2.884 | 2.044 | 0,5 | 95 | 2 | 10 | 10,2 |
| 1A3b | Road Transportation | N2O | 673 | 2.039 | 0,5 | 96 | 5 | 100 | 100,1 |
| 4B | Manure Management | N2O | 1.632 | 1.614 | 0,4 | 96 | 35 | 200 | 203,0 |
| 2A2 | Lime Production | CO2 | 917 | 1.335 | 0,3 | 97 | 20 | 2,3 | 20,1 |
| 3 | Solvent and Other Product Use | CO2 | 964 | 1.216 | 0,3 | 97 | 100 | 25 | 103,1 |
| 6B | Wastewater Handling | N2O | 1.002 | 1.085 | 0,3 | 97 | 30 | 100 | 104,4 |
| 1B1 | Fugitive Emissions from Solid fuels | CH4 | 1.789 | 1.057 | 0,3 | 98 | 10 | 100 | 100,5 |
| 1B2 | Fugitive Emissions from Oil and Natural gas | CH4 | 584 | 993 | 0,3 | 98 | 50 | 100 | 111,8 |
| 1A1c | Manufacture of Solid fuels and Other Energy Industries | CO2 | 1.664 | 752 | 0,2 | 98 | 5 | 15,1 | 15,9 |
| *** | Other emission sources | *** | 6.667 | 7.715 | 2,0 | 100 | 100 | 100 | 141,4 |
| Total emissions (without LUCF) |  |  | 286.798 | 383.460 |  |  |  |  |  |

TABLE 5.5.3 (Cont.)
UNCERTAINTY ANALYSIS OF GREENHOUSE GASES EMISSIONS. TIER 1. YEAR 2001

| A |  | B | H | YN | I | J | K | L | M |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Key Sources (Year 2001) |  | Gas | % Combined Uncertainty (% Emisiones totales 2001) | AD correlated between years (Y/N) | Type A Sensibility | Type B Sensibility | Uncertainty in trend by EF uncertainty (%) | Uncertainty in trend by AD uncertainty (%) | Uncertainty in trend (%) |
| IPCC Code | Description of IPCC category |  |  |  |  |  |  |  |  |
| 1A1a | Public Electricity and Heat Production- Coal | CO2 | 0,8 | N | -0,031 | 0,238 | -0,12 | 0,67 | 0,69 |
| 1A2+1A4 | Stationary Combustion - Other sources - Oil | CO2 | 0,9 | N | -0,016 | 0,195 | -0,08 | 0,97 | 0,97 |
| 1A3b | Road Transportation -Diesel Oil | CO2 | 0,8 | N | 0,069 | 0,188 | 0,15 | 1,33 | 1,34 |
| 1A2 +1A4 | Stationary Combustion - Other sources - Gas | CO2 | 0,4 | N | 0,075 | 0,119 | 0,22 | 0,50 | 0,55 |
| 1A3b | Road Transportation- Gasoline | CO2 | 0,3 | N | -0,027 | 0,094 | -0,06 | 0,47 | 0,47 |
| 2A1 | Cement Production | CO2 | 0,3 | N | -0,005 | 0,053 | -0,04 | 0,11 | 0,12 |
| 4A | HCFC-22 Manufacture (HFC-23 Emissions) | CH4 | 0,9 | N | -0,008 | 0,051 | -0,16 | 0,72 | 0,74 |
| 1A1b | Petroleum refining- Oil | CO2 | 0,1 | N | -0,007 | 0,043 | -0,02 | 0,15 | 0,15 |
| 1A1a | Public Electricity and Heat Production- Oil | CO2 | 0,1 | N | 0,015 | 0,043 | 0,03 | 0,09 | 0,10 |
| 6A | Solid Waste Disposal on Land | CH4 | 2,1 | Y | 0,011 | 0,037 | 0,80 | 0,34 | 0,87 |
| 4D1 | Agricultural Soils - Direct Emissions | N2O | 8,7 | N | -0,009 | 0,031 | -3,47 | 0,33 | 3,49 |
| 4B | Manure Management | CH4 | 4,5 | Y | 0,001 | 0,030 | 0,19 | 0,03 | 0,19 |
| 1A2 +1A4 | Stationary Combustion - Other sources - Coal | CO2 | 0,3 | N | -0,049 | 0,021 | -0,74 | 0,35 | 0,82 |
| 1A3a | Domestic Civil Aviation | CO2 | 0,5 | N | 0,000 | 0,020 | 0,00 | 0,97 | 0,97 |
| 4D3 | Agricultural Soils - Indirect Emissions | N2O | 12,6 | Y | -0,004 | 0,019 | -3,60 | -0,40 | 3,62 |
| 1A1+1A2+1A4 | Stationary Combustion | N2O | 0,8 | N | 0,000 | 0,014 | 0,00 | 0,08 | 0,08 |
| 4D2 | Agricultural Soils - Animal Production | N2O | 1,0 | N | -0,002 | 0,011 | -0,16 | 0,81 | 0,83 |
| 1A1a | Public Electricity and Heat Production- Gas | CO2 | 0,0 | N | 0,009 | 0,011 | 0,01 | 0,03 | 0,03 |
| 2E1 | HCFC-22 Manufacture (HFC-23 Emissions) | HFC | 0,2 | N | -0,011 | 0,010 | -0,23 | 0,03 | 0,23 |
| 2-2A1-2A2-2C1 | Other Industrial Processes | CO2 | 0,8 | N | -0,001 | 0,010 | -0,03 | 1,40 | 1,40 |
| 2F | Consumption of Halocarbons and SF6 (except HCFC-22) | HFC & PFC | 1,4 | Y | 0,008 | 0,008 | 1,63 | 0,82 | 1,82 |
| 6B | Wastewater Handling | CH4 | 0,7 | N | 0,001 | 0,008 | 0,10 | 0,57 | 0,58 |
| 1A3d | National navigation | CO2 | 0,3 | N | 0,000 | 0,007 | 0,00 | 0,52 | 0,52 |
| 1B2 | Fugitive Emissions from Oil and Natural gas | CO2 | 0,1 | N | -0,001 | 0,007 | -0,02 | 0,10 | 0,10 |
| 2B2 | Nitric Acid Production | N2O | 0,1 | N | -0,006 | 0,007 | -0,06 | 0,02 | 0,07 |
| 1A3b | Road Transportation | N2O | 0,5 | N | 0,004 | 0,007 | 0,40 | 0,05 | 0,40 |
| 4B | Manure Management | N2O | 0,9 | Y | -0,002 | 0,006 | -0,40 | -0,07 | 0,40 |
| 2A2 | Lime Production | CO2 | 0,1 | N | 0,000 | 0,005 | 0,00 | 0,13 | 0,13 |
| 3 | Solvent and Other Product Use | CO2 | 0,3 | N | 0,000 | 0,004 | -0,01 | 0,60 | 0,60 |
| 6B | Wastewater Handling | N2O | 0,3 | N | -0,001 | 0,004 | -0,09 | 0,16 | 0,18 |
| 1B1 | Fugitive Emissions from Solid fuels | CH4 | 0,3 | N | -0,005 | 0,004 | -0,47 | 0,05 | 0,47 |
| 1B2 | Fugitive Emissions from Oil and Natural gas | CH4 | 0,3 | N | 0,001 | 0,003 | 0,07 | 0,24 | 0,26 |
| 1A1c | Manufacture of Solid fuels and Other Energy Industries | CO2 | 0,0 | N | -0,005 | 0,003 | -0,08 | 0,02 | 0,08 |
| *** | Other emission sources | *** | 2,8 | N | -0,004 | 0,027 | -0,42 | 3,80 | 3,83 |
| Uncertainty in total emissions |  |  | 16,6 | In trend (difference between years): |  |  |  |  | 7,4 |
|  |  |  |  | In trend (% of mean total emissions in the base year): |  |  |  |  | 2,5 |

##### 6.- RESULTS COMPARISON (1990-2001)
ed. 2004 vs. ed. 2003

In this section, the comparative analysis of the results obtained for the period 1990-2001 in the current edition, 2004, of the inventories compared to the corresponding results for the same period in previous edition, 2003, are presented. Please refer to section 4 above for the motivating reasons of the recalculations.

The presentation is based on a series of graphs, figures 6.1 to 6.9, where each one shows the time profiles, as absolute value emissions (upper panel) or as indexed numbers (lower panel) of both editions. The absolute emission data are expressed homogeneously in CO2-equivalent units; and the indices take the value 100 for the base year.

Figure 6.1 shows the evolution of the emissions inventory aggregate. It is apparent that the estimate in the edition 2004, compared with that in the previous edition, has meant a decrease (except for year 2001) of no more than three and a half thousand kilo-tonnes in terms of CO2-equivalent (less than 1.2 in percentage points). The almost parallel scaled evolution of the two editions series (absolute emissions numbers) in the upper panel for the period 1990-1998 is reflected in the lower panel in similar (almost identical) values of the corresponding indexed numbers in both editions series and, consequently, the comparatively tighter matching of the two absolute emissions numbers series from 1998 to 2001 is reflected in a relative wider breach for those years in the indices numbers series of the lower panel.

Figure 6.2 shows the evolution of energy sector. The trend profiles of both the upper and lower panels parallel very closely those of their respective counterparts in figure 6.1. The explanation for this similarity between the time profiles of figures 6.1 and 6.2 traces back to the fact that the main important change (recalculation) contributing to the CO2 differences between present and previous inventory editions has been the revision of fugitive CO2 emissions from coke oven plants which are included within the Energy group.

Figure 6.3 shows the evolution of the industrial processes emissions not related to the energy consumption. From the upper  panel it is evidenced that the new time series evolves in absolute terms fairly parallel to the previous time series. The difference between the two time series is explained by: i) the introduction of magnesite production for the first time in the present edition of the emission inventory, and ii) the (net upwards) revision of some emissions factors and activity rates for a miscellaneous group of industrial activities.

Figure 6.4 shows the emissions evolution from the use of solvents and other products. With the only exception of year 2001 no changes appear between the new and the previous series. The differences for 2001 are accounted by a revision in the basic activity data of paint application, as reported by the entrepreneurial paint manufacturers’ association.

Figure 6.5 shows the evolution in the agriculture sector emissions. Only negligible differences, mainly due to rounding of basic data, can be detected.

Figure 6.6 shows the emissions evolution of the treatment and disposal of wastes. The two editions series are very similar in absolute emissions numbers, with discrepancies less than some 0.07 thousand kilo tonnes CO2 equivalent, or equivalently 0.7 in percentage points. The main reason for the changes has been the reallocation of the Municipal Solid Waste (MSW) Incinerating Plants without energy recover to their appropriate 6C CRF category, which in the previous edition were assigned to CRF category 1A1.The wider breach in the lower panel for years 1994 to 2001 is mainly caused by the differences in absolute emissions numbers in the upper panel in the early years (and reference year) of the inventoried period.

The results comparison between the current edition of the inventories and the previous are completed with the following 3 figures (6.7 to 6.9); the evolution in both series reflect the estimates of the three primary gases in the inventory: CO2, CH4 and N2O.

Hence, the evolution in the group of activities that produce CO2 emissions is shown in figure 6.7. The profile is very similar to the one in figure 6.2, which corroborates the close relationship between CO2 emissions and the energy sector.

The evolution in the group of activities that produce CH4 emissions is presented in figure 6.8. The differences between the new and the previous series are very small lesser than 0.05 thosand kilo-tonnes CO2 equivalent, roughly 0.2%, rending the two series almost undistinguishable in both the upper and lower panels. The causes of such differences are minor revisions in the activity data of the agricultural sector (such as in milk production statistics) as well as also minor revisions of emissions factors and activity data in the energy sector (such as in petroleum coke characteristics).

Figure 6.9 shows the evolution in the N2O emissions for the group of activities that produce this gas. One can appreciate in the upper panel  the relatively lower level of the new edition series as compared to the previous one, by an amount between 0.3 and 0.4 kilo-tonnes of CO2 equivalent, roughly between 1.2% and 1.4% This increase is mainly a consequence of the (downwards) revisions of some N2O combustion emissions factors in specific categories of the industrial sector. The drawings of the two series in the lower panel are almost undistinguishable.

Figure 6.1.
total emissions evolution

Figure 6.2.
energy evolution

Figure 6.3.
industrial processes evolution

Figure 6.4.
solvent and other product use evolution

Figure 6.5.
agriculture evolution

Figure 6.6.
waste evolution

Figure 6.7.
co2 Evolution

Figure 6.8.
cH4 Evolution

Figure 6.9.
n2o Evolution

##### 7. KEY SOURCES ANALYSIS

This chapter presents a detailed description of the most significant activities, the key activities, for their contribution to greenhouse gas emissions in the inventory as a whole. The chapter is intended to give an extended view of the level and evolution of emissions for the main activities in the inventory.

The chapter has been structured into sections in accordance with the main groups of CRF sources. Each section deals with one of these groups:

7.1	Energy

7.2	Industrial Processes

7.3	Solvent and Other Product Use

7.4	Agriculture

7.5	Land-Use Change and Forestry

7.6	Waste

7.1. ENERGY

SECTOR OVERVIEW

In 2002, energy emissions represented 78.13% of the total emissions of CO2 equivalent in Spain, implying an increase over the base year when these represented 74.48% of the total. On the other hand, the emissions of CO2 equivalent in this sector have seen an increase of 46.21% throughout the period inventoried 1990-2002, passing from 213,596 kilotonnes of CO2 equivalent in 1990 to 312,304 kilotonnes in 2002. Figure 7.1.1 shows the evolution of the in terms of CO2 equivalent throughout the period 1990-2002 by component categories according to the CRF nomenclature.

FIGURE 7.1.1
EVOLUTION OF CO2 EQUIVALENT EMISSIONS

a)	Basic data sources (activity variables, algorithms and emission factors)

The most significant activity variables for this sector are fuel consumption and their allocation to the different categories within the sector.

The inventory assumes a principle of consistency with the national fuel balance sheet, in the versions of EUROSTAT and the International Energy Agency, for the totals of each type of fuel. In the sectorial breakdown, notwithstanding, the inventory has drawn on its own information, derived, in a great many cases, from direct information coming from the power plants for the sectors to which a personalized questionnaire was sent; the total fuel consumption for each type using this route take preference over that obtained by other means and more aggregate data. Furthermore, for certain other sectors, an estimation was made of the consumption of fuels starting with their specific consumption ratios or taking into account the data provided on their activity variables (non-ferrous metalworking, glass industries, consumption by the domestic fishing fleet, agricultural and forestry machinery, among others.)

As far as the types of fuel are concerned, the NAPFUE nomenclature of the EMEP/CORINAIR is used, supplemented where necessary by that of the IPCC. In addition, for certain very significant fuels in the calculation of CO2 emissions, there is an individualized analysis by energy-industrial plants, thus allowing the emissions to be estimated by means of the carbon balance, including at times specific information on the oxidation factor.

With respect to the emissions estimation algorithms, the starting point is, as far as possible, the carbon balance in order to estimate the CO2 emissions (IPCC methodology based on the fuel’s carbon content and the oxidation factor of the carbon to CO2), although when this information is not available for a fuel, an approximation is taken for the final CO2 factor based on the standard characteristics of the fuels (basically the lower heating value). For CH4 and N2O, where the mass balance methodology is not operational, emission factors have been taken mainly from the EMEP/CORINAIR Guidebook and the IPCC Guidelines (revised guidelines from 1996) and from the Good Practice Guidance dated 2000.

A specific mention of the data sources for the key activities in this sector is given below in the presentation of each particular activity.

b)	Completeness, transparency and time-series consistency.

The estimated emissions of the three main gases with a direct atmospheric warming effect (CO2, CH4 and N2O) cover the vast majority of the CRF categories in the energy sector. The sector also includes emissions from municipal solid waste incineration plants when there is energy recovery. Notwithstanding, the following limitations on the application of the principle of completeness must be considered: there is no differentiated inclusion of the emissions corresponding to the military use of energy, the emissions of CO2 from the extraction and manipulation of coal, the emissions (flares and releases) of oil and natural gas production, nor the emissions from the incineration of industrial waste, although part of this kind of waste is accounted for in the emissions from those plants where energy is recovered from this waste stream.

In the energy sector, there is intensive application of the use of personalized questionnaires for data collection at the plants considered to be large point sources, including those of the following sectors: public service power stations, municipal solid waste incinerators, oil refineries, transformation of solid fuels (coke plants in integrated iron and steel plants), integrated steel and iron works, manufacture of alumina and primary aluminium and paper pulp factories. For such plants the base information used allows the detailed analysis of fuel consumptions and their composition in terms of carbon contents and heating power. This information can be traced, except in cases of a requirement of confidentiality that some sectors have raised for the revision of the national inventories. For the sectors and activities in which the plant data have been processed using non-individualized sources, in most significant cases there is a sectorial and provincial breakdown (NUTS3) of the same, allowing a detailed analysis of the data in accordance with the bottom-up approach.

The current edition of the inventory has extended the series inventoried to 2002, now covering the period 1990-2002, and whenever there have been changes in the methodology or coverage of the activity variables and emission factors chosen as representative with respect to the previous edition, the series of estimated emissions has been back-projected consistently to 1990. In this same line, whenever evidence has been found of a need for recalculation (for instance, due to the detection of errors), the entire set of the time series has also been revised.

REFERENCE AND SECTORAL APPROACHES

The reference approach gives a first approximation to CO2 combustion emissions (IPCC category 1A), dealing only with data on indigenous production of primary products, net external (foreign) balance and stock changes of fuels as well as their non-energy use aggregated at the national level (top-down approach).

This process is useful as a method for contrasting the estimations made with the sectorial approach (bottom-up approach). The discrepancies between both methods are relatively low in the national inventory, only slightly exceeding 2% in CO2 emitted in some years (the threshold above which the IPCC requests justification for the discrepancies). The differences are shown in the table 7.1.1, expressed in terms of both energy consumption and CO2  emissions:

TABLE 7.1.1
Difference Reference Approach/Sectorial Approach (%)

|  | Difference Reference Approach / Sectorial Approach (%) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 |
| Energy | 11,51 | 12,45 | 10,34 | 9,29 | 8,45 | 12,54 | 12,12 | 12,15 | 12,96 | 10,31 | 10,26 | 8,59 | 7,80 |
| CO2 Emissions | 2,18 | 1,85 | 1,78 | 1,45 | 1,77 | 2,17 | 3,46 | 2,40 | 2,21 | 0,93 | 1,14 | -0,95 | 0,68 |

Methodology.

The reference approach has been developed following IPCC methodological criteria (please refer to the explanatory diagram in Figure 7.1.2). Only a minor modification of the approach described by the IPCC about variables for the non-energy use of fuels needs to be explained. For “Coal oils and tars”, benzol, pitch and tar production in coke ovens has been taken as the activity variable, whereas for “Bitumen and Tars”, the non-energy consumption of the above-mentioned products was used directly. .

Figure 7.1.2
IPCC methodological criteria

- Socio-economic variables

- The variables involved in this estimation are associated with fossil fuels, namely:

- Origin-destination flows: these include exportation, importation, international bunkers and domestic stock changes. For primary fuels indigenous production is also considered.

- Non-energy uses.

All socio-economic data comes from the fuel balance drawn up as a part of the emissions inventory. Certain discrepancies were observed after the comparison with the data included in National Energy Balances published by international organizations like the IEA-OECD or Eurostat. These are basically caused by:

a)	Multiplicity of information sources consulted: Data from diverse sources were captured in the development and validation of this energy balance, with the IEA-OECD and Eurostat as two of the most important sources used. Data on the non-energy use of “coal oils and tars” has been taken from “Statistics on Production of Coking Paste, Coke Oven and Blast Furnace Gas” published by Spain’s Ministry of Economy, whereas for the other non-energy products (feedstocks) and international bunkers (for marine and aviation use), data have been taken from energy balances published by IEA-OECD and/or Eurostat.

b)	Provisional nature of the National Inventory Balance for the year (t-2). The final values of the annual balances are the result of compiling and contrasting all of the data available in the process of preparing the current inventory. The National Balance for year (t-2) is only partially estimated because national statistics and National Energy Balances of the IEA-OECD and Eurostat are not available.

Other parameters in fuels:

Certain characteristics of fossil fuels are also taken into account in the estimation algorithm, namely: a) net calorific values, b) carbon contents, c) fraction of unoxidized carbon and d) carbon fraction stored in feedstocks.

National Inventory includes specific data on the energy capacity and chemical composition of fuels, including their carbon content, obtained at the sectorial level or from facilities through questionnaires sent to relevant emission categories like refineries, thermal power stations, integrated metalworking plants or the transport and distribution of natural gas. Standard characteristics have been assigned to fuels consumed in other activities.

At the present time, as there are no national data available on the fraction of carbon oxidized or the carbon fraction stored in feedstocks, the quantities suggested by the IPCC have been used by default.

Net Calorific Values (NCV):

In the reference approach, the origin-destination movements of the solid and liquid fossil fuels are expressed in mass terms, reproducing the same numbers as in the fuel balance for the emission inventory. The apparent consumption of these fuels is later converted to energy units (TJ of net calorific value) by applying a representative national NCV.

In the case of fossil fuels at the sectorial level, the mean annual factor was used in the reference approach. This factor is obtained by weighting the NCV applied to each activity A (NCVA) in terms of the corresponding fuel consumption in mass terms, MA.:

NCVR:A,t=ANCVA,t. MA,tAMA,t             t=t0,...,tn

When the national fuel consumption is exclusively through a non-energy use and in consequence it has not been included in any combustion activity, then the NCV suggested by IPCC has been adopted directly,. The source and the annual factors used for the liquid and solid fuels in the reference approach are shown in the table 7.1.2.

TABLE 7.1.2
source and the annual factors in the reference approach

|  | NCV selected for Reference Approach (GJ/t) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Source (*) | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 |
| Oil |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Crude Oil | S | 41,86 | 41,86 | 41,86 | 41,86 | 41,86 | 41,86 | 41,86 | 41,86 | 41,86 | 41,86 | 41,86 | 41,86 | 41,86 |
| Gasoline | P | 44,37 | 44,38 | 44,39 | 44,42 | 44,44 | 44,49 | 44,51 | 44,54 | 44,56 | 44,59 | 44,63 | 44,67 | 44,78 |
| Jet Kerosene | P | 43,36 | 43,36 | 43,36 | 43,36 | 43,36 | 43,36 | 43,36 | 43,36 | 43,36 | 43,36 | 43,36 | 43,36 | 43,36 |
| Other Kerosene | P | 43,36 | 43,36 | 43,36 | 43,36 | 43,36 | 43,36 | 43,36 | 43,36 | 43,36 | 43,36 | 43,36 | 43,36 | 43,36 |
| Gas / Diesel Oil | P | 43,06 | 43,06 | 43,06 | 43,06 | 43,06 | 43,06 | 43,05 | 43,05 | 43,06 | 43,06 | 43,05 | 43,05 | 43,06 |
| Residual Fuel Oil | P | 40,18 | 40,18 | 40,18 | 40,18 | 40,18 | 40,18 | 40,18 | 40,18 | 40,18 | 40,18 | 40,18 | 40,18 | 40,18 |
| LPG | P | 44,78 | 44,78 | 44,79 | 44,79 | 44,79 | 44,79 | 44,79 | 44,79 | 44,79 | 44,79 | 44,79 | 44,79 | 44,79 |
| Naphtha | P(**) | 44,68 | 44,68 | 44,68 | 44,68 | 44,68 | 44,68 | 44,68 | 44,78 | 44,89 | 44,99 | 44,99 | 44,99 | 44,99 |
| Bitumen | D | 40,19 | 40,19 | 40,19 | 40,19 | 40,19 | 40,19 | 40,19 | 40,19 | 40,19 | 40,19 | 40,19 | 40,19 | 40,19 |
| Lubricants | D | 40,19 | 40,19 | 40,19 | 40,19 | 40,19 | 40,19 | 40,19 | 40,19 | 40,19 | 40,19 | 40,19 | 40,19 | 40,19 |
| Petroleum Coke | P | 32,49 | 32,50 | 32,50 | 32,50 | 32,50 | 32,50 | 32,50 | 32,50 | 32,49 | 32,50 | 32,51 | 32,49 | 32,50 |
| Refinery Feedstocks | D | 44,80 | 44,80 | 44,80 | 44,80 | 44,80 | 44,80 | 44,80 | 44,80 | 44,80 | 44,80 | 44,80 | 44,80 | 44,80 |
| Other Oil | D | 40,19 | 40,19 | 40,19 | 40,19 | 40,19 | 40,19 | 40,19 | 40,19 | 40,19 | 40,19 | 40,19 | 40,19 | 40,19 |
| Coal |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Coking Coal | P | 30,44 | 30,44 | 30,44 | 30,44 | 30,44 | 30,44 | 30,44 | 30,44 | 30,44 | 30,44 | 30,44 | 30,44 | 30,44 |
| Other Bit. Coal | P | 21,95 | 21,83 | 22,11 | 22,24 | 22,36 | 22,50 | 22,16 | 21,37 | 22,13 | 22,07 | 22,48 | 22,58 | 22,70 |
| Sub-bit. Coal | P | 13,05 | 13,31 | 13,58 | 14,95 | 13,72 | 13,87 | 13,66 | 13,46 | 13,52 | 12,82 | 12,63 | 12,79 | 12,55 |
| Lignite | P | 6,90 | 6,88 | 6,93 | 7,19 | 7,02 | 7,16 | 7,29 | 7,49 | 7,52 | 7,74 | 7,82 | 7,81 | 8,19 |
| BKB & Patent Fuel | P | 19,30 | 16,60 | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Coke Oven/Gas Coke | P | 29,80 | 29,80 | 29,80 | 29,80 | 29,80 | 29,80 | 29,80 | 29,80 | 29,80 | 29,80 | 29,80 | 29,80 | 29,80 |

(*) S: Conutry-Standard l factor;  D: IPCC default factor; P: Country-specific average factor.based on mass of fuel used.º

(**) Serie of average factors from NCV reported by refining plants has been completed using estimate (porojections).

Carbon content (C):

When choosing carbon content, the criterion observed has been to favour the contrast with the Sectorial Approach. So in the reference approach, the annual carbon content implicit in the Sectorial Approach, CSA, was assigned to the fuels, starting from the associated carbon emission and the alleged fuel consumption:

CR.A,t=CS.A,t=CEmissionsS.A,tEnergyConsumptionS.A,t=1244.1COxidized.COeEmissionsS.A,tEnergyConsumptionS.A,t       t=t0,...,tn

Developing the above formula with the CO2 emissions and consumptions for emission activity A, the equation would be expressed as follows:

CR.A,t=1244.1COxidized.ACO2EmissionsA,tNCVR.A,tAMA,t            t=t0,....,tn

As with the net calorific values, this algorithm is not applicable when the annual consumption of a fuel is solely for non-energy purposes. Default IPCC values are used in this case. See table 7.1.3.

TABLE 7.1.3
carbon content in the reference approach

|  | Carbon content selected for Reference Approach (tC/TJ) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Source(*) | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 |
| Oil |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Crude Oil | D | 20,00 | 20,00 | 20,00 | 20,00 | 20,00 | 20,00 | 20,00 | 20,00 | 20,00 | 20,00 | 20,00 | 20,00 | 20,00 |
| Gasoline | P | 19,76 | 19,76 | 19,76 | 19,74 | 19,73 | 19,71 | 19,70 | 19,69 | 19,68 | 19,67 | 19,65 | 19,63 | 19,58 |
| Jet Kerosene | P | 20,04 | 20,01 | 20,01 | 20,01 | 20,01 | 20,01 | 20,01 | 20,01 | 20,01 | 20,01 | 20,01 | 20,01 | 20,01 |
| Other Kerosene | P | 20,11 | 20,11 | 20,11 | 20,11 | 20,11 | 20,11 | 20,11 | 20,11 | 20,11 | 20,11 | 20,11 | 20,11 | 20,11 |
| Gas / Diesel Oil | P | 20,08 | 20,09 | 20,09 | 20,08 | 20,07 | 20,07 | 20,07 | 20,07 | 20,06 | 20,06 | 20,06 | 20,06 | 20,05 |
| Residual Fuel Oil | P | 21,02 | 21,02 | 21,02 | 21,02 | 21,02 | 21,00 | 21,03 | 21,01 | 21,00 | 21,01 | 21,03 | 21,08 | 21,13 |
| LPG | P | 17,82 | 17,82 | 17,83 | 17,83 | 17,83 | 17,83 | 17,83 | 17,84 | 17,84 | 17,83 | 17,83 | 17,83 | 17,84 |
| Naphtha | P(**) | 20,00 | 18,44 | 18,44 | 18,44 | 18,68 | 19,74 | 19,92 | 19,89 | 19,86 | 19,83 | 19,43 | 19,75 | 19,75 |
| Bitumen | D | 22,00 | 22,00 | 22,00 | 22,00 | 22,00 | 22,00 | 22,00 | 22,00 | 22,00 | 22,00 | 22,00 | 22,00 | 22,00 |
| Lubricants | D | 20,00 | 20,00 | 20,00 | 20,00 | 20,00 | 20,00 | 20,00 | 20,00 | 20,00 | 20,00 | 20,00 | 20,00 | 20,00 |
| Petroleum Coke | P | 27,08 | 27,08 | 27,08 | 27,08 | 27,08 | 27,08 | 27,08 | 27,08 | 27,08 | 27,07 | 27,06 | 27,17 | 27,05 |
| Refinery Feedstocks | D | 20,00 | 20,00 | 20,00 | 20,00 | 20,00 | 20,00 | 20,00 | 20,00 | 20,00 | 20,00 | 20,00 | 20,00 | 20,00 |
| Other Oil | D | 20,00 | 20,00 | 20,00 | 20,00 | 20,00 | 20,00 | 20,00 | 20,00 | 20,00 | 20,00 | 20,00 | 20,00 | 20,00 |
| Coal |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Coking Coal | P | 26,16 | 26,16 | 26,16 | 26,16 | 26,16 | 26,16 | 26,16 | 26,16 | 26,16 | 26,16 | 26,16 | 26,16 | 26,16 |
| Other Bit. Coal | P | 27,34 | 27,76 | 27,97 | 27,69 | 27,32 | 27,34 | 27,27 | 27,71 | 27,46 | 28,26 | 27,03 | 27,25 | 27,28 |
| Sub-bit. Coal | P | 27,37 | 27,24 | 27,92 | 29,73 | 28,02 | 27,98 | 27,71 | 27,88 | 28,08 | 27,93 | 27,46 | 27,15 | 27,22 |
| Lignite | P | 29,99 | 27,20 | 27,86 | 30,08 | 27,92 | 27,36 | 27,61 | 28,13 | 26,86 | 26,77 | 34,70 | 32,03 | 30,58 |
| BKB & Patent Fuel | P | 27,68 | 23,68 | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO | NO |
| Coke Oven Coke | P | 29,22 | 29,22 | 29,22 | 29,22 | 29,22 | 29,22 | 29,22 | 29,22 | 29,22 | 29,22 | 29,22 | 29,04 | 29,22 |
| Coke Oven Gas | P | NO | NO | NO | 12,33 | 12,33 | 12,33 | 12,33 | 12,33 | 12,33 | 12,33 | 12,33 | 12,33 | NO |
| Blast Furnace Gas | P | NO | NO | NO | 50,97 | 45,00 | 45,00 | 45,00 | 45,00 | 45,00 | 35,93 | 45,00 | 45,00 | NO |
| Gas |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Natural Gas | P | 15,08 | 15,08 | 15,35 | 15,35 | 15,35 | 15,35 | 15,35 | 15,35 | 15,35 | 15,35 | 15,35 | 13,35 | 15,35 |

(*)D: IPCC default factor; P: Country-specific average factor.based on energy.

(**) Serie has been completed using estimate (porojections).

Fraction of carbon stored in feedstocks:

As already indicated in the sub-chapter entitled “Other parameters”, the main information source was the IPCC Manual. The values given in the corresponding CRF table have been respected, except for the value for coal oils and tars (coming from coking coal), modified in line with expert judgement. Estimations for this factor were assigned to the other products not considered by the IPCC (petroleum coke, other petroleum products, coke oven gas and blast furnace gas). See table 7.1.4.

TABLE 7.1.4
fraction of carbon stored in feedstocks

| Feedstock | Fraction C stored |
| --- | --- |
| Naphtha | 0,8 |
| Lubricants | 0,5 |
| Bitumen | 1 |
| Coal Oils and Tars (from Coking Coal) | 0,9 |
| Natural Gas | 0,33 |
| Petroleum coke | 0,8 |
| Other petroleum products | 0,8 |
| Coke Oven Gas | 0,5 |
| Blast Furnace Gas | 0,5 |

Differences between the reference approach and the sectorial approach

The evolutions and annual variation rates in the year on year estimations (energy and CO2 emissions) obtained by applying both approaches are shown in the figures 7.1.3 and 7.1.4.

FIGURE 7.1.3
EVOLUTION OF THE ESTIMATION

FIGURE 7.1.4
VARIATION RATES OF THE ESTIMATION

The variations observed in the estimations by both methods are justified by two fundamental aspects: a) structural differences between the approaches and b) considerations about the parameters.

Structural differences between the approaches:

This group includes the following points, among others:

- List of fuels: Only the fossil fuels available for the year (gaseous, liquids and solids) are included in the reference approach, and this is extended in the sectorial approach through the incorporation, on occasion, of waste feedstocks counted in previous years but that subsequently appear as fuels in the form of industrial and/or domestic waste (used tyres burnt as fuel and incineration of plastic packaging).

- List of source activities: The sectorial approach covers all the combustion activities included in IPCC category 1A. On the other hand, the reference approach extends its coverage to all activities where the emissions are directly generated by the use of fuels: categories  IPCC 1A and 1B, in part (for example: by fugitive emissions in the transport and distribution of natural gas, or in coke oven door leakage and extinction).

- It is very difficult to evaluate the repercussions of this aspect because there is no documentation about the figures reported by the bodies drafting the energy balances. It is assumed to be of little significance.

- Estimation methodology for emitted CO2: In the reference approach, a carbon mass balance is applied using mean factors. There is, however, no uniformity regarding the technique applied in the sectorial approach (carbon balance, mean factors based on energy, measured emissions,..)

- The selection of weighted calorific values and carbon contents minimizes the effects of this point.

Some considerations associated with the parameters:

- Statistical differences in the fuel balance in the emission inventory.

- Negative differences in feedstocks (consumption exceeds the fuel available) would indicate double counting and therefore an underestimation in the reference approach estimations because of the discounting of carbon content stored in feedstocks. In the same way, if the consumption was for energy purposes, the emissions of the sectorial approach will be inflated due to an increase in the emissions associated with that fuel.

- Lack of information about the use of non-energy products including data on fuels which, as part of the manufacturing process for non-energy products, have energy uses (combustion), at least in part, in addition to acting as components of the products.

International bunkers:

The international bunker emissions are not included in the total emissions of national inventory, but have been estimated following the same methodology applied in the national source category (internal navigation and domestic civil aviation).

KEY SOURCES ANALYSIS

The key sources analysis for the period 1990-2002 has identified the following in category 1 “Energy Processing”:

-	Public service power and heating plants (1A1a) for emissions of CO2. For solid and liquid fuels for their emissions level throughout the period 1990-2002, and for gas fuels for their level in the period 1997-2002. Because of their profile, for solid fuels in the period 1991-2002; for liquid fuels in the periods 1991-1997 and 1999-2002; and for gas fuels in 1991, 1993-1995 and 1997-2002.

-	Oil refineries (1A1b) for emissions of CO2. For liquid fuels for their level of emissions during the period 1990-2002 and for their profile in the years 1991-1996 and 1998-2002; and for gas fuels for their profile in the years 1997-2000 and 2002.

-	Transformation of solid fuels and other energy industries (1A1c) for CO2 emissions. For solid fuels for their level of emissions in 1990 and 1991 and for their profile in the period 1992-2002, and for gas fuels for their profile in 1991-1992 and 1996-1998.

-	Stationary combustion (1A2 and 1A4) for the level of CO2 emissions throughout the period 1990-2002 and for its profile in the period 1991-2002, whether for solid, liquid or gas fuels.

-	Domestic air traffic (1A3a) for its level of CO2 emissions throughout the period 1990-2002 and for its profile in 1991-1997 and 2002.

-	Road transport (1A3b): for the levels of CO2 emissions throughout the period 1990-2002 and for its profile in 1991-2002, both for petrol and for diesel; and for N2O emissions at the levels of 1998-2000 and 2002 and for its profile in 1993-2002.

-	Domestic maritime traffic (1A3d) for its level of CO2 emissions in 1990, 1993-1996 and 1999-2002 and for its profile in 1992, 1994, 1997 and 1998.

-	Stationary combustion (1A1, 1A2 and 1A4) for its level of N2O emissions in 1990-2002 and for its profile in 1991, 1992 and 1995; and for its CH4 profile in 1995.

-	Fugitive emissions from solid fuels (1B1): for CH4 due to its level of emissions in 1990-1992 and 1997, and for its profile in the period 1991-2002.

-	Fugitive emissions – oil and natural gas (1B2): for CO2 due to their level in all the years of the period 1990-2002, and for their profile in 1991, 1993 and 1994.

Table 7.1.5 below summarizes the foregoing and gives the contribution of the emissions level and the trend for the key categories in this sector, as well as the absolute values in terms of CO2 equivalent, all referring to 2002.

TABLE 7.1.5
KEY SOURCES: LEVEL AND TREND CONTRIBUTION

| IPCC Activity |  | Gas | CO2-e (Gg) (2002) | Level contribution (2002) |  |  | Trend contribution (2002) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Code | Description |  |  | % | Key Source | Range | % | Key Source | Range |
| 1A1a | Public electricity and heat production – Solid fuels | CO2 | 78.419 | 19,62 | YES | 1 | 2,06 | YES | 13 |
| 1A1a | Public electricity and heat production – Liquid fuels | CO2 | 14.097 | 3,53 | YES | 8 | 4,24 | YES | 6 |
| 1A1a | Public electricity and heat production – Gaseous fuels | CO2 | 5.900 | 1,48 | YES | 13 | 3,98 | YES | 7 |
| 1A1b | Petroleum refining – Liquid fuels | CO2 | 11.903 | 2,98 | YES | 9 | 2,52 | YES | 9 |
| 1A1b | Petroleum refining – Gaseous fuels | CO2 | 835 | 0,21 | NO | 34 | 0,58 | YES | 22 |
| 1A1c | Manufacture of solid fuels and other energy industries – Solid fuels | CO2 | 792 | 0,20 | NO | 35 | 1,16 | YES | 18 |
| 1A1c | Manufacture of solid fuels and other energy industries – Gaseous fuels | CO2 | 249 | 0,06 | NO | 44 | 0,03 | NO | 50 |
| 1A3a | Civil aviation | CO2 | 5.084 | 1,27 | YES | 16 | 0,54 | YES | 23 |
| 1A3b | Road transportation – Gasoline | CO2 | 25.982 | 6,50 | YES | 5 | 7,83 | YES | 5 |
| 1A3b | Road transportation – Diesel oil | CO2 | 57.209 | 14,31 | YES | 2 | 16,17 | YES | 2 |
| 1A3b | Road transportation | N2O | 2.197 | 0,55 | YES | 23 | 0,94 | YES | 20 |
| 1A3d | Navigation | CO2 | 2.338 | 0,58 | YES | 22 | 0,13 | NO | 35 |
| 1A2 + 1A4 | Stationary combustion in industry and other sectors – Solid fuels | CO2 | 5.694 | 1,42 | YES | 14 | 11,62 | YES | 3 |
| 1A2 + 1A4 | Stationary combustion in industry and other sectors – Liquid fuels | CO2 | 53.102 | 13,28 | YES | 3 | 7,85 | YES | 4 |
| 1A2 + 1A4 | Stationary combustion in industry and other sectors – Gaseous fuels | CO2 | 37.288 | 9,33 | YES | 4 | 18,00 | YES | 1 |
| 1A1 + 1A2 + 1A4 | Stationary combustion | N2O | 3.888 | 0,97 | YES | 17 | 0,13 | NO | 36 |
| 1B1 | Fugitive emissions – Solid fuels | CH4 | 1.019 | 0,25 | NO | 32 | 0,27 | YES | 19 |
| 1B2 | Fugitive emissions – Oil and natural gas | CO2 | 2.149 | 0,54 | YES | 27 | 0,05 | NO | 33 |

The most outstanding characteristics of each of the source categories indicated above are now characterized below.

A) 1A1a. Public service electricity and heat production

This section comprises the public service electricity and heat production plants which represent, albeit small in number (54 thermal power stations and 9 incinerators in 2002), one of the main contributions to the overall emissions of the inventory. It should be noted that alongside the conventional thermal plants, this section includes the municipal solid waste incineration plants in which energy is recovered (through co-generation of electricity).

At thermal power stations, the dominant technology involves boiler installations and, among these, ones with a power in excess of 300 MWt. In addition to the boilers, there are significant installations of gas turbines and stationary engines, and these have increased greatly in 2002 with the commissioning of six combined cycle power stations.

As for the fuels, it is clear that there is a clear predominance of solid fuels (national and imported coal), and by types of fuel, brown coal and anthracites followed by brown lignite and sub-bituminous coal are common, and then to a lesser degree the gases derived from primary solid fuels (coke gas and furnace gas). Among the liquid fuels, the main consumption corresponds to fuel-oil with a complementary contribution by gas-oil. With respect to gaseous fuels, there is an increase already mentioned in the consumption of natural gas in 2002 as a result of the use of this fuel in the new kind of power stations mentioned above. Finally, the group of other fuels includes consumption due to the use of municipal solid waste in incinerators.

Table 7.1.6.a shows the activity variable, fuel consumption, expressed in terms of energy (TJ of lower heating value, LHV). This information is derived from a calculation starting with the consumption in physical units (tonnes or m3) and their corresponding heating values. The information on fuel consumption and characteristics obtained by means of the individualized questionnaire to the thermal power stations included their composition and, apart from the LHV parameter indicated above, their characteristics also included the contents of carbon, sulphur, ashes, etc. as determined by means of analyses with results that provide the mean annual values for these parameters.

TABLE 7.1.6.a
FUEL CONSUMPTION

|  | FUEL CONSUMPTION (TJLHV) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Type | 1990 | 1998 | 1999 | 2000 | 2001 | 2002 |
| Liquid fuels | 79.772 | 97.992 | 144.113 | 141.255 | 158.593 | 181.993 |
| Solid fuels | 581.243 | 611.116 | 710.496 | 759.041 | 685.106 | 789.899 |
| Gaseous fuels | 7.337 | 23.592 | 28.000 | 39.788 | 50.174 | 95.728 |
| Biomass |  | 19 | 12 | 3 |  |  |
| Other fuels | 3.103 | 10.960 | 11.725 | 11.741 | 11.987 | 13.148 |
| Total | 671.455 | 743.679 | 894.347 | 951.829 | 905.860 | 1.080.767 |

For the estimation of the CO2 emissions, preference is given to the calculation procedure starting from the carbon content of each fuel used, and this is complemented by the stoichiometric calculation of the figure raised to mass of CO2 through the inclusion of the oxidation factor. This latter value uses the default values proposed by the IPCC. For the estimation of CH4 and N2O, the emissions applied are the factors selected from the different methodological guidelines (EMEP/CORINAIR, IPCC) regarding the energy activity variable (GJ) in terms of LHV. This same procedure was followed to obtain the estimation of other pollutants considered in the CRF (NMVOC and CO), whereas for SO2 and NOX, preference is given to the measured emissions provided by the plants.

Table 7.1.6.b shows the emissions of CO2 for each type of fuel, with this gas giving the source its nature as a key source. Table 7.1.6.c complements the previous information by including the emissions associated with CH4 and N2O combustion expressing all three gases’ emissions in terms of CO2 equivalent. This same table gives the index for the evolution over time (base 100 in 1990) of the CO2 equivalent emissions, and the contributions of the CO2 equivalent emissions of this category to the total of the inventory and the “Energy” sector.

TABLE 7.1.6.b
CO2 EMISSIONS

|  | CO2 EMISSIONS (Gg) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Type | 1990 | 1998 | 1999 | 2000 | 2001 | 2002 |
| Liquid fuels | 6.015 | 7.475 | 11.270 | 11.163 | 12.325 | 14.097 |
| Solid fuels | 57.779 | 60.163 | 71.811 | 75.441 | 68.309 | 78.419 |
| Gaseous fuels | 427 | 1.481 | 1.886 | 2.747 | 3.168 | 5.900 |
| Biomass |  |  |  |  |  |  |
| Other fuels | 120 | 398 | 422 | 425 | 449 | 485 |
| Total | 64.341 | 69.516 | 85.389 | 89.776 | 84.252 | 98.901 |

TABLE 7.1.6.c
CO2 EQUIVALENT EMISSIONS: INDICES AND RATIOS

|  | 1990 | 1998 | 1999 | 2000 | 2001 | 2002 |
| --- | --- | --- | --- | --- | --- | --- |
| CO2-eq (Gg) | 64.804 | 70.096 | 86.185 | 90.582 | 85.129 | 99.930 |
| Index CO2-eq | 100,0 | 108,2 | 133,0 | 139,8 | 131,4 | 154,2 |
| % CO2-eq of total | 22,77 | 20,57 | 23,27 | 23,52 | 22,20 | 25,00 |
| % CO2-eq of energy | 30,34 | 27,27 | 30,46 | 30,77 | 28,81 | 32,00 |

B) 1A1b. Oil Refineries

Of the set of oil refinery activities, this section includes those corresponding to combustion processes. These distinguish between boilers, gas turbines, stationary engines and non-contact process kilns. The first three types of installations are intended to generate electricity, steam or heat in accordance with the requirements of the refining plants, and do not present any peculiarity with respect to the installations of this kind that there may be in other sectors. However, process furnaces are specific to this sector, as a series of physical-chemical reactions take place on the crude oil, such as distillation, catalytic reforming, hydro-treatment, catalytic cracking, alkylation, hydrocracking, etc., that give rise to the various fractions of crude oil into which it breaks down. There is no contact between the flame or combustion gases and the crude oil or its resulting fractions in these furnaces. It should be mentioned that the emissions these furnaces might generate through the non-combustive processes taking place inside them are included in category 1B2a. The emissions from the waste gas flares are not included either, as they are listed in category 1B2c.

Table 7.1.7.a shows the fuel consumptions, expressed in energy terms (TJ of lower heating power), used as the activity variable for the emission estimations. The data on these consumptions, as well as the fuels’ characteristics, have been collected by means of an individualized questionnaire to each of the ten refineries in Spain. The fuels consumed are basically fuel-oil and refinery gas, with notably lower amounts of gas-oil and natural gas, and practically testimonial amounts of LPG and naphtha. The increase observed in gas fuel consumption (natural gas) is due to the gradual installation of co-generation units at the refineries.

TABLE 7.1.7.a
FUEL CONSUMPTION

|  | FUEL CONSUMPTION (TJLHV) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Type | 1990 | 1998 | 1999 | 2000 | 2001 | 2002 |
| Liquid fuels | 155.424 | 180.400 | 182.620 | 182.559 | 184.833 | 176.839 |
| Gaseous fuels | 820 | 11.050 | 10.761 | 12.166 | 10.251 | 14.839 |
| Total | 156.244 | 191.450 | 193.381 | 194.724 | 195.084 | 191.678 |

For the estimation of CO2 emissions, preference is given, providing that the pertinent data has been made available, to the calculation procedure starting from the carbon content of each fuel used, and complemented by the stoichiometric calculation raised to the mass of CO2 with inclusion of the oxidation factor. For this latter figure, the default values proposed by the IPCC are taken. For the estimation of CH4 and N2O emissions, the factors selected in the various methodological guidelines (EMEP/CORINAIR, IPCC) are taken for the energy activity variable (GJ) in LHV terms. This same procedure is followed for the estimation of other pollutants considered in the CRF (NOX, NMVOC and CO), whereas for SO2 preference is given to the estimated emissions provided by the plants (normally by mass balance).

Table 7.1.7.b shows the emissions of CO2 for each type of fuel, with this gas giving the source its nature as a key source. Table 7.1.7.c complements the previous information by including the emissions associated with combustion of CH4 and N2O, with all of the emissions of the three gases being expressed in terms of CO2 equivalent. This same table gives the index for the evolution over time (base 100 in 1990) of the CO2 equivalent emissions; the contributions of the CO2 equivalent emissions of this category to the total of the inventory and the “Energy” sector; and the ratio of the CO2 equivalent emissions to the crude oil processed.

TABLE 7.1.7.b
CO2 EMISSIONS

|  | CO2 EMISSIONS (Gg) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Type | 1990 | 1998 | 1999 | 2000 | 2001 | 2002 |
| Liquid fuels | 10.862 | 12.335 | 12.403 | 12.389 | 12.394 | 11.903 |
| Gaseous fuels | 45 | 617 | 599 | 672 | 542 | 835 |
| Total | 10.907 | 12.951 | 13.002 | 13.061 | 12.936 | 12.738 |

TABLE 7.1.7.c
CO2 EQUIVALENT EMISSIONS: INDICES AND RATIOS

|  | 1990 | 1998 | 1999 | 2000 | 2001 | 2002 |
| --- | --- | --- | --- | --- | --- | --- |
| CO2-eq (Gg) | 11.296 | 12.838 | 12.910 | 12.899 | 12.894 | 12.385 |
| Index CO2-eq | 100,0 | 113,7 | 114,3 | 114,2 | 114,2 | 109,6 |
| % CO2-eq of total | 3,97 | 3,77 | 3,49 | 3,35 | 3,36 | 3,10 |
| % CO2-eq of energy | 5,29 | 4,99 | 4,56 | 4,38 | 4,36 | 3,97 |

|  | 1990 | 1998 | 1999 | 2000 | 2001 | 2002 |
| --- | --- | --- | --- | --- | --- | --- |
| Crude oil processed (kt) | 53.556 | 60.859 | 60.023 | 59.174 | 57.320 | 57.892 |
| kt CO2-eq / kt crude oil | 0,211 | 0,211 | 0,215 | 0,218 | 0,225 | 0,214 |

C) 1A1c. Manufacture of solid fuels and other energy industries

This category includes the emissions generated in the transformation of solid fuels (coke plants, coal gasification), as well as those generated at non-specific combustion installations both in this sector and in coal mining and the production of oil and natural gas.

The basic activity variable used is fuel consumption. In the cases of coke plants located in integrated iron and steel plants and coal gasification, the data were collected by means of individual questionnaires to the plants carrying out these activities. For the remaining activities in this category, including the coke plants not included above, the information has been based on the data from the International Energy Agency. The main fuels used in this category are coke gas and blast furnace gas among solid fuels, fuel-oil and gas-oil among the liquid fuels and natural gas as a gaseous fuel. Table 7.1.8.a presents the fuel consumptions expressed in energy terms (TJ of lower heating power).

TABLE 7.1.8.a
FUEL CONSUMPTION

|  | FUEL CONSUMPTION (TJLHV) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Type | 1990 | 1998 | 1999 | 2000 | 2001 | 2002 |
| Liquid fuels | 2.754 | 3.787 | 4.337 | 3.693 | 3.633 | 4.628 |
| Solid fuels | 15.656 | 11.148 | 10.230 | 11.536 | 10.862 | 11.337 |
| Gaseous fuels | 3.824 | 9.680 | 9.253 | 8.518 | 9.753 | 4.444 |
| Total | 22.234 | 24.615 | 23.820 | 23.747 | 24.249 | 20.409 |

For the estimation of CO2 emissions, preference is given, providing that the pertinent data has been made available, to the calculation procedure starting from the carbon content of each fuel used, and complemented by the stoichiometric calculation raised to the mass of CO2 with inclusion of the oxidation factor. For this latter figure, the default values proposed by the IPCC are taken. For the estimation of CH4 and N2O emissions, the factors selected in the various methodological guidelines (EMEP/CORINAIR, IPCC) are taken for the energy activity variable (GJ) in LHV terms. This same procedure is followed for the estimation of other pollutants considered in the CRF (NOX, NMVOC and CO), except for those cases where the plants have provided measured emissions (particularly for SO2 and NOX).

Table 7.1.8.b shows the emissions of CO2 for each type of fuel, with this gas giving the source its nature as a key source. Table 7.1.8.c complements the previous information by including the emissions associated with combustion of CH4 and N2O, with all of the emissions of the three gases being expressed in terms of CO2 equivalent. This same table gives the index for the evolution over time (base 100 in 1990) of the CO2 equivalent emissions, and the contributions of the CO2 equivalent emissions of this category to the total of the inventory and the “Energy” sector.

TABLE 7.1.8.b
CO2 EMISSIONS

|  | CO2 EMISSIONS (Gg) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Type | 1990 | 1998 | 1999 | 2000 | 2001 | 2002 |
| Liquid fuels | 203 | 279 | 303 | 357 | 317 | 455 |
| Solid fuels | 1.664 | 859 | 775 | 851 | 752 | 792 |
| Gaseous fuels | 210 | 542 | 518 | 477 | 546 | 249 |
| Total | 2.078 | 1.680 | 1.596 | 1.685 | 1.615 | 1.496 |

TABLE 7.1.8.c
CO2 EQUIVALENT EMISSIONS: INDICES AND RATIOS

|  | 1990 | 1998 | 1999 | 2000 | 2001 | 2002 |
| --- | --- | --- | --- | --- | --- | --- |
| CO2-eq (Gg) | 2.142 | 1.729 | 1.641 | 1.737 | 1.658 | 1.543 |
| Index CO2-eq | 100,0 | 80,7 | 76,6 | 81,1 | 77,4 | 72,1 |
| % CO2-eq of total | 0,75 | 0,51 | 0,44 | 0,45 | 0,43 | 0,39 |
| % CO2-eq of energy | 1,00 | 0,67 | 0,58 | 0,59 | 0,56 | 0,49 |

D) 1A2 and 1A4. Stationary combustion in industry and other sectors

These categories, which jointly make up a key source in CO2 emissions for solid, liquid and gaseous fuels, include a wide variety of combustion processes performed either in industry or in other sectors (commerce, institutions, residences, as well as in agriculture, forestry and fishing).

The basic data sources for the activity variables (fuel consumptions) have been:

-	For the industrial sector (category 1A2) the national fuel balance, complemented with data from the main sectorial associations (cement, lime, bricks and tiles, ceramics, etc.) and direct information from individualized questionnaires sent to the plants. In general the aggregate consumptions of the national fuel balance have been respected but in some cases their sectorial distribution has been revised to make it consistent with the direct information coming for some sectors from the business associations and the individual questionnaires sent to the plants.

-	For the commercial, institutional and residential sectors (categories 1A4a and 1A4b), the information on fossil fuels has basically been taken from the national fuel balance, and for biomass an estimation has been made of the consumptions starting from the data provided by the Institute for Energy Diversification and Saving (IDAE).

-	For the agriculture, forestry and fishing sector (category 1A4c), the estimation has been made starting from the knowledge of the activity patterns and the energy requirements associated with them, assuming that practically all of the fuel is gas-oil. As for the information on activity patterns in the sub-sectors, deep-sea fishing, agricultural and forestry machinery, the following process has been followed:

*	Deep-sea fishing: the data collected includes information on the power of the fishing fleet provided by the Sub-Directorate General for the Fishing Fleet, and hypotheses on the duration of their stop-overs in port and operations at sea.

*	Agricultural and forestry machinery: for agricultural machinery the initial information has been the installed power of the fleet of machines considered to be operational by type of machinery (tractors, harvesters, etc.) according to data provided by the Sub-Directorate General for Agricultural Production Resources at the Spanish Ministry of Agriculture, Food and Fishing (MAPA in its Spanish acronym) and with certain hypotheses on the number of effective hours/year for each type of machine and its energy requirements per hour of standard operation. A similar process has been followed for forestry machinery, but taking into account that the information has been provided by experts in the sector.

Finally, the data for stationary combustion in the agricultural sector (motors and other installations) are taken from the information on this in the national fuel balance except for gas-oil for which a proportion of the consumption effected in moving agricultural machinery is estimated.

Table 7.1.9.a presents the estimated fuel consumptions for these two categories taken together.

TABLE 7.1.9.a
FUEL CONSUMPTION

|  | FUEL CONSUMPTION (TJLHV) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Type | 1990 | 1998 | 1999 | 2000 | 2001 | 2002 |
| Liquid fuels | 605.701 | 700.332 | 721.286 | 722.116 | 748.344 | 711.175 |
| Solid fuels | 155.679 | 74.141 | 66.417 | 66.201 | 64.921 | 66.798 |
| Gaseous fuels | 172.262 | 427.080 | 506.155 | 575.724 | 636.834 | 693.227 |
| Biomass | 157.767 | 154.118 | 155.969 | 157.338 | 157.753 | 158.023 |
| Other fuels |  | 407 | 998 | 1.106 | 1.035 | 1.423 |
| Total | 1.091.408 | 1.356.079 | 1.450.825 | 1.522.485 | 1.608.887 | 1.630.646 |

In order to produce the estimation of the CO2 emissions default emission factors have been used for the fuel type, taken from the main references of the IPCC Good Practice Guidance and the EMEP/CORINAIR Guidebook. This general rule has, however, been qualified when the specific characteristics of the fuels are available, as is the case, among others, of the industrial sectors for integrated steel and iron working, manufacture of paper and pulp, and manufacture of aluminium, where this information is available through the individual questionnaire sent to the plants. In certain cases where the basic information about fuel consumptions was not available, but the amounts of products manufactured or main raw material used were known, the emission factors for the product or raw material as indicated in the EMEP/CORINAIR Guidebook have been converted to fuel factors assuming an energy requirement per unit of product or raw material and a “mix” of standard fuels in the sector of reference. This has been the case mainly in the non-ferrous metalworking industry and in the glass industry.

Table 7.1.9.b shows the emissions of CO2 for each type of fuel, with this gas giving the source its nature as a key source. Table 7.1.9.c complements the previous information by including the emissions associated with combustion of CH4 and N2O, with all of the emissions of the three gases being expressed in terms of CO2 equivalent. This same table gives the index for the evolution over time (base 100 in 1990) of the CO2 equivalent emissions, and the contributions of the CO2 equivalent emissions of this category to the total of the inventory and the “Energy” sector.

TABLE 7.1.9.b
CO2 EMISSIONS

|  | CO2 EMISSIONS (Gg) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Type | 1990 | 1998 | 1999 | 2000 | 2001 | 2002 |
| Liquid fuels | 45.247 | 53.484 | 55.137 | 55.281 | 55.977 | 53.102 |
| Solid fuels | 15.079 | 7.140 | 6.132 | 5.939 | 5.992 | 5.694 |
| Gaseous fuels | 9.466 | 23.963 | 28.425 | 32.392 | 34.135 | 37.288 |
| Biomass |  |  |  |  |  |  |
| Other fuels |  | 31 | 79 | 89 | 85 | 118 |
| Total | 69.792 | 84.618 | 89.773 | 93.700 | 96.189 | 96.203 |

TABLE 7.1.9.c
CO2 EQUIVALENT EMISSIONS: INDICES AND RATIOS

|  | 1990 | 1998 | 1999 | 2000 | 2001 | 2002 |
| --- | --- | --- | --- | --- | --- | --- |
| CO2-eq (Gg) | 72.668 | 87.728 | 92.969 | 96.896 | 99.411 | 99.294 |
| Index CO2-eq | 100,0 | 120,7 | 127,9 | 133,3 | 136,8 | 136,6 |
| % CO2-eq of total | 25,54 | 25,74 | 25,10 | 25,15 | 25,92 | 24,84 |
| % CO2-eq of energy | 34,02 | 34,13 | 32,86 | 32,92 | 33,64 | 31,79 |

E) 1A3a. Civil aviation

This category includes the transport activities carried out by aircraft at national level, with a distinction being drawn between two different types of operations: a) landing and take-off cycles (LTO) of aircraft in airports and b) cruising. LTO comprises the operations of landing (below 1,000 m. in altitude), the taxiing by the aircraft to the disembarking gate, the taxiing from the departure gate to the head of the runway, and take-off (once more up to 1,000 m. in altitude).

In order to effect the estimation of the fuel consumptions, the calculation has started from the data shown in the statistics of the International Energy Agency, and then the fuel consumption has been estimated for the landing and take-off operations, with the remaining consumption being attributed to cruising. The estimation of the fuel consumptions during LTO is done individually for each type of aircraft, based on the information provided by the Directorate General for Civil Aviation (DGAC) at the Ministry of Development and by the Spanish Airports Body AENA (Aeropuertos Españoles y Navegación Aérea). Table 7.1.10.a presents the fuel consumptions expressed in energy terms (TJ of lower heating power).

TABLE 7.1.10.a
FUEL CONSUMPTION

|  | FUEL CONSUMPTION (TJLHV) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Type | 1990 | 1998 | 1999 | 2000 | 2001 | 2002 |
| Aviation gasoline |  | 477 | 434 | 434 | 434 | 477 |
| Jet kerosene | 56.845 | 64.346 | 70.720 | 74.969 | 76.617 | 69.246 |
| Total | 56.845 | 64.823 | 71.154 | 75.403 | 77.051 | 69.723 |

The estimation of the emissions is done differently depending on whether it is an LTO or cruising. For LTO cycles, the emissions of CO2, CH4 and SO2 are calculated using the emission factors selected from EMEP/CORINAIR, whereas for the rest of the pollutants considered in the CRF mean emission factors have been taken each type of engine for LTO, deduced from the data supplied by the International Civil Aviation Organization (OACI). In the case of cruising, the emissions have been calculated as a function of the fuel consumption using emission factors selected from EMEP/CORINAIR.

Table 7.1.10.b shows the emissions of CO2 for each type of fuel, with this gas giving the source its nature as a key source. Table 7.1.10.c complements the previous information by including the emissions associated with combustion of CH4 and N2O, with all of the emissions of the three gases being expressed in terms of CO2 equivalent. This same table gives the index for the evolution over time (base 100 in 1990) of the CO2 equivalent emissions, and the contributions of the CO2 equivalent emissions of this category to the total of the inventory and the “Energy” sector.

TABLE 7.1.10.b
CO2 EMISSIONS

|  | CO2 EMISSIONS (Gg) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Type | 1990 | 1998 | 1999 | 2000 | 2001 | 2002 |
| Aviation gasoline |  | 35 | 32 | 32 | 32 | 35 |
| Jet kerosene | 4.135 | 4.688 | 5.156 | 5.465 | 5.587 | 5.049 |
| Total | 4.135 | 4.722 | 5.188 | 5.497 | 5.618 | 5.084 |

TABLE 7.1.10.c
CO2 EQUIVALENT EMISSIONS: INDICES AND RATIOS

|  | 1990 | 1998 | 1999 | 2000 | 2001 | 2002 |
| --- | --- | --- | --- | --- | --- | --- |
| CO2-eq (Gg) | 4.177 | 4.770 | 5.240 | 5.552 | 5.675 | 5.136 |
| Index CO2-eq | 100,0 | 114,2 | 125,4 | 132,9 | 135,9 | 123,0 |
| % CO2-eq of total | 1,47 | 1,40 | 1,41 | 1,44 | 1,48 | 1,28 |
| % CO2-eq of energy | 1,96 | 1,86 | 1,85 | 1,89 | 1,92 | 1,64 |

1A3b Road Transport

In this heading, the pollution emission source is the consumption of fuels: leaded or unleaded gasoline, diesel and liquefied petroleum gases. In terms of CO2 equivalent emissions in 1990 and 2002, the greenhouse gases are ranked as follows in table 7.1.11.

TABLE 7.1.11
EMISSIONS OF GREENHOUSE GASES

| Year |  | CO2 |  |  | N2O |  |  | CH4 |  |  | CO2 Equivalent |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1990 | 51,390 kt |  | 98.28% | 673 kt |  | 1.29% | 228 kt |  | 0.44% | 52,291 kt |  |
| 2002 | 83,418 kt |  | 97.20% | 2,197 kt |  | 2.56% | 204 kt |  | 0.24% | 85,819 kt |  |

Activity variables

Three main activity variables are used to calculate the emissions of road traffic:

- The fuel consumption figures drawn up by the inventory is of the data from the following sources:

- “Energy Statistics of OECD Countries” published by the International Energy Agency

- “Energy Balance Sheets” from EUROSTAT, and

- “Estadísticas de Consumo de Productos Petrolíferos” (Statistics on the Consumption of Oil-related Products) published by the Sub-Directorate General for Hydrocarbons at the Ministry of Economy.

- The official figures on the operational fleet of vehicles, distributed by categories, age, cylinder capacity and useful load, as published in the Anuario Estadístico (Statistical Yearbook) of the Directorate General for Traffic at the Ministry of Internal Affairs.

- The distance figures broken down by category of vehicle on the networks of roads on which vehicles travel in interurban and rural modes, namely: the general interest state network (RIGE in its Spanish acronym), the networks run by the regional governments and those of the provincial councils; these figures are provided by the Directorate General for Roads at the Ministry of Development.

Fuel consumption

The most outstanding aspect of the trend in fuel consumption seen over the years for which the Inventory has been conducted has been the sustained growth of diesel consumption, which has by itself covered the needs derived from the increased distance travelled by the vehicles. Thus, while the consumption of the different grades of gasoline has remained practically constant (the consumption of liquefied petroleum gases is marginal) at the level of 8,150 tonnes, diesel fuel has gone from 8,090 tonnes in 1990 to over 18,200 in 2002. This has led to an alteration in the relative distribution of the consumptions between petrol and diesel fuel, shifting from a practically 50:50 split in 1990 to a situation in 2002 where diesel fuel represents more than two thirds of the total consumption, as can be seen in the figure 7.1.5.

FIGURE 7.1.5
CONSUMPTION OF GASOLINE AND DIESEL
(figures in tonnes)

Fleet of vehicles

The fleet of operational vehicles has undergone considerable growth between 1990 and 2002. As can be seen in the figure 7.1.6, the total of operational vehicles on the road has grown by between 42% and 86%, depending on the category of vehicle under consideration: motorcycles and mopeds 49%, lightweight cargo vehicles 86%, heavyweight vehicles 42% and cars 56%.

FIGURE 7.1.6
FLEET OF VEHICLES

If we regard the distribution by the vehicles’ fuel type, cars can be seen to have a highly disparate evolution. The growth in petrol-engined cars (18%), while considerable, does not bear comparison with that in diesel-powered cars (around 400%). This disparity is manifested, therefore, in the age distribution of the operational fleet, a very relevant aspect when it comes to the calculation of emissions, as this phenomenon gives rise to a lower age for diesel-powered cars when compared to gasoline models (the sample median of the age distribution of diesel cars is between 3 and 4 years of age, whereas it is over 9 years for gasoline-engined cars) which is clearly consistent with the tendency towards the gradual replacement of gasoline by diesel experienced in the last few years. (The evolution of the total number of cars as well as the age and fuel distributions in 2002 can be seen in the figures 7.1.7 and 7.1.8).

FIGURE 7.1.7

FIGURE 7.1.8

Distances travelled

Between 1990 and 2002, there has been a sustained growth in the distances travelled in any of the three driving patterns considered: interurban, rural and urban, with the total distance going from 191,665 million kilometres in 1990 to 323,547 in 2002, in other words an increase of approximately 69% in thirteen years. By vehicle category, the greatest increase corresponds to lightweight vehicles for cargo transport (92%), followed by cars with 73%, and then heavyweight cargo vehicles (58%), as can be seen in the figure 7.1.9.

FIGURE 7.1.9

As for the distribution of distances travelled according to the driving pattern, it can be said that this shows great stability over the years, with interurban driving representing around 50% of the total mileage, rural driving 20% and urban mode the remaining 30%. The same could be said for the distribution of mileage according to vehicle category, where the main categories show very little variation: cars range between 73% of the total in 1990 and 75% in 2002, whereas heavyweight vehicles, whether for freight or passenger transport, go from 12.83% in 1990 to 11.78% in 2002. The figure 7.1.10 show the representation of both distributions (the distances are expressed in millions of kilometres).

FIGURE 7.1.10
distribution of distances travelled according to the driving pattern

Emission factors

The calculation of the emission factors and emissions for CO2, N2O and CH4 has been based on the publications and documents of the CORINE AIRE project working group, more specifically on the methodology set out in the technical report by the European Environment Agency: “COPERT III Computer Programme to Calculate Emissions from Road Transport”.

The CO2 emission factors have been calculated from the characteristics of the fuels (their carbon content) so as to obtain the final emissions, that is to say assuming that all of the carbon contained in the fuel will in the final reckoning end up combining with oxygen to form CO2. The calculation formula used was:

Ef,CO2F=44,011Qf12,011+1,008rHC

where:

Ef,CO2F

represents the final emissions of CO2 produced by the consumption of fuel f

Qf

is the total consumption of fuel f

rHC

is the ratio between the number of hydrogen and carbon atoms in fuel f(1.8 for petrol, 2.0 for diesel fuel and approximately 2.53 for liquefied petroleum gases).

The application of the above formula produces the following CO2 emission factors per unit of mass for each of the three types of fuel under consideration: gasoline, 3.183, diesel fuel, 3.138 and liquefied petroleum gases, 3.023 (all three expressed in terms of kg of CO2/kg of fuel).

The emission factors for N2O and CH4 per kilometre travelled, on the other hand, have been calculated as functions of the type of vehicle and the speeds representing the driving modes and vehicle categories analyzed, namely:

ei,jT=f(v)

where:

ei,jT=f(v)

is a function of speed that gives the mass of pollutant j (N2O or CH4) emitted by vehicles in class iper unit of distance travelled.

When applying this methodology, the table 7.1.12 of speeds has been used:

TABLE 7.1.12
DRIVING STYLES

|  |  | Interurban | Rural | Urban |
| --- | --- | --- | --- | --- |
| CARS | Range | 80 - 130 | 40 – 80 | 10 - 40 |
|  | Represented by | 105 | 65 | 25 |
| LigHTWEIGHT VEHICLEs | Range | 80 - 130 | 40 – 80 | 10 - 40 |
|  | Represented by | 100 | 65 | 25 |
| HEAVYWEIGHT VEHICLEs | Range | 80 - 100 | 40 – 80 | 10 - 40 |
|  | Represented by | 95 | 60 | 25 |
| BUSES AND COACHES | Range | 80 - 105 | 40 – 80 | 10 - 40 |
|  | Represented by | 95 | 60 | 25 |
| MotoRcYcles | Range | 80 - 130 | 40 – 80 | 10 - 40 |
|  | Represented by | 105 | 65 | 25 |
| MOPEDS | Range |  | 40 – 50 | 10 - 40 |
|  | Represented by |  | 45 | 25 |

The figures 7.1.11 show the functions for the emission of nitrous oxide and methane for the main vehicle categories.

FIGURE 7.1.11

FIGURE 7.1.11 (Cont.)

FIGURE 7.1.11 (Cont.)

FIGURE 7.1.11 (Cont.)

FIGURE 7.1.11 (Cont.)

Emissions

The emissions calculated by applying the COPERT III methodology to the activity variables indicated above (distances travelled, fuel consumptions and operational fleet) are shown in the table 7.1.13 and figure 7.1.12 broken down by vehicle category.

TABLE 7.1.13
CO2-EQUIVALENT EMISSIONS BY VEHICLE CATEGORY (kt)

| YEAR | HEAVYWEIGHT VEHICLES | MOTORCYCLES | LIGHTWEIGHT VEHICLES | CARS | TOTAL |
| --- | --- | --- | --- | --- | --- |
| 1990 | 18,020 | 820 | 5,582 | 27,872 | 52,294 |
| 1991 | 18,564 | 915 | 5,961 | 29,357 | 54,797 |
| 1992 | 18,707 | 1,025 | 6,434 | 31,856 | 58,023 |
| 1993 | 18,902 | 882 | 6,510 | 31,261 | 57,554 |
| 1994 | 20,067 | 989 | 7,036 | 32,676 | 60,767 |
| 1995 | 21,429 | 831 | 7,361 | 32,032 | 61,652 |
| 1996 | 22,587 | 901 | 7,792 | 34,438 | 65,718 |
| 1997 | 21,378 | 865 | 8,302 | 35,910 | 66,455 |
| 1998 | 23,871 | 873 | 9,656 | 38,793 | 73,192 |
| 1999 | 25,602 | 803 | 10,164 | 40,716 | 77,285 |
| 2000 | 27,707 | 705 | 10,145 | 40,845 | 79,402 |
| 2001 | 28,089 | 725 | 12,179 | 42,329 | 83,322 |
| 2002 | 29,313 | 590 | 11,075 | 44,840 | 85,819 |

FIGURE 7.1.12
CO2-EQUIVALENT EMISSIONS BY VEHICLE CATEGORY (kt)

G) 1A3d. Domestic maritime traffic

This category includes the emissions from merchant marine activities on voyages with their ports of origin and destination both in Spain, regardless of the flag under which the vessel is sailing or whether the nationality of the ship-owners is Spanish or foreign. This section does not include the emissions from deep-sea fishing, which are included in activity 1A4c.

In order to estimate the fuel consumption carried out in this activity, regard has been had for the publication entitled “Energy Statistics of OECD countries” from the International Energy Agency (IEA) and the data provided by the Association of Spanish Ship-Owners (ANAVE). Table 7.1.14.a presents the fuel consumptions expressed in energy terms (TJ of lower heating power).

TABLE 7.1.14.a
FUEL CONSUMPTION

|  | FUEL CONSUMPTION (TJLHV) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Type | 1990 | 1998 | 1999 | 2000 | 2001 | 2002 |
| Residual oil | 16.072 | 11.152 | 14.438 | 15.628 | 19.094 | 20.848 |
| Gas/Diesel oil | 3.659 | 6.025 | 8.600 | 8.413 | 8.057 | 9.448 |
| Total | 19.731 | 17.177 | 23.038 | 24.041 | 27.151 | 30.296 |

For the estimation of the emissions of CO2, CH4 and N2O, selected factors have been taken from EMEP/CORINAIR and from the document entitled “COPERT III. Methodology and Emission Factors”. For the other pollutants considered in the CRF (NOX, NMVOC and CO), the study entitled “Marine Exhaust Emissions Research Programme” from Lloyds Register has been used, whereas for SO2 the emission factors, yearly variables for gas-oil, have been derived from the mass balance depending on the sulphur content in the fuels.

Table 7.1.14.b shows the emissions of CO2 for each type of fuel, with this gas giving the source its nature as a key source. Table 7.1.14.c complements the previous information by including the emissions associated with combustion of CH4 and N2O, with all of the emissions of the three gases being expressed in terms of CO2 equivalent. This same table gives the index for the evolution over time (base 100 in 1990) of the CO2 equivalent emissions, and the contributions of the CO2 equivalent emissions of this category to the total of the inventory and the “Energy” sector.

TABLE 7.1.14.b
CO2 EMISSIONS

|  | CO2 EMISSIONS (Gg) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Type | 1990 | 1998 | 1999 | 2000 | 2001 | 2002 |
| Residual oil | 1.268 | 880 | 1.139 | 1.233 | 1.506 | 1.645 |
| Gas/Diesel oil | 268 | 442 | 631 | 617 | 591 | 693 |
| Total | 1.536 | 1.322 | 1.770 | 1.850 | 2.098 | 2.338 |

TABLE 7.1.14.c
CO2 EQUIVALENT EMISSIONS: INDICES AND RATIOS

|  | 1990 | 1998 | 1999 | 2000 | 2001 | 2002 |
| --- | --- | --- | --- | --- | --- | --- |
| CO2-eq (Gg) | 1.550 | 1.334 | 1.786 | 1.867 | 2.117 | 2.359 |
| Index CO2-eq | 100,0 | 86,0 | 115,2 | 120,4 | 136,5 | 152,2 |
| % CO2-eq of total | 0,54 | 0,39 | 0,48 | 0,48 | 0,55 | 0,59 |
| % CO2-eq of energy | 0,73 | 0,52 | 0,63 | 0,63 | 0,72 | 0,76 |

H) 1A1, 1A2 y 1A4. Combustion activities (except transport)

These three categories jointly make up a key source for their emissions of N2O. Since the corresponding fuel consumptions have been indicated above, Table 7.1.15.a below only presents the N2O emissions. These data are complemented with the index for the evolution over time (base 100 in 1990) for these emissions in terms of CO2 equivalent, and their contribution to the CO2 equivalent emissions of the total inventory and the “Energy” sector.

TABLE 7.1.15.a
N2O EMISSIONS

|  | 1990 | 1998 | 1999 | 2000 | 2001 | 2002 |
| --- | --- | --- | --- | --- | --- | --- |
| N2O (kt CO2-equivalent) | 2.895 | 3.407 | 3.733 | 3.792 | 3.874 | 3.888 |
| Index CO2-equivalent | 100,0 | 117,7 | 128,9 | 131,0 | 133,8 | 134,3 |
| % CO2-eq of total | 1,02 | 1,00 | 1,01 | 0,98 | 1,01 | 0,97 |
| % CO2-eq of energy | 1,36 | 1,33 | 1,32 | 1,29 | 1,31 | 1,24 |

I) 1B1. Fugitive emissions – Solid fuels

Basic information sources: The information on coal was provided by the reports from international bodies (“Energy Statistics of OECD Countries” by the IEA-OECD) or national institutions (Ministry of Economy, “Statistics on the Manufacture of Coking Paste , Coke Oven and Blast Furnace Gas” from the Ministry of Science and Technology, the “Carbunión Report” from the coal-mining companies’ business association or the “Report on the exploitation of the electrical system” from the Spanish Electricity Network).

Description of the main emitting activities: Coal-mining is the predominant source of CH4, with a share exceeding 98% of the estimates for IPCC category 1B1. By extraction technique, and despite the gradual decline observed in recent years, reference must be made to the high contribution of underground mining both in the extraction phase itself and in the post-mining activities. See figure 7.1.13

figure 7.1.13.
main ch4 emitting activities

Activity variables and emissions: The table 7.1.16 describes, by IPCC category, activity variables and methane emission sources considered in Table1.B.1 of the CRF.

table 7.1.16
activity variables and ch4 emission sources

| IPCC Category |  | CH4 Emission Source | Activity data |
| --- | --- | --- | --- |
| 1B1a. Coal mining and handling |  |  |  |
| 1B1a.i | Underground mining activities | Coal extraction | Gross coal production (anthracite, other bituminous and sub-bituminous coal) |
|  | Underground post-mining activities | First treatment (handling, transportation and pre-treatment of coals) |  |
|  |  | Coal storage |  |
| 1B1a.ii | Surface mining activities | Coal extraction | Gross coal production (anthracite, other bituminous coal, sub-bituminous coal and lignite) |
|  | Surface post-mining activities | First treatment (handling, transportation and pre-treatment of coals) |  |
|  |  | Coal storage |  |
| 1B1b. Solid fuel transformation |  |  |  |
|  |  | Coke oven door leakage and extinction | Production of coke oven coke |

Methodological breakdown by emission sources:  Each of the emission sources described in the table above has been dealt with individually, with the cluster of methane estimates so obtained being assigned to IPCC category 1B1 on the CRF. The methodology used for each emission source is briefly documented below:

- Production, first treatment and storage of coal: The methodology used is the IPCC’s Tier 2. The emission factor per tonne of product is country-specific, with a distinction being drawn by type of coal and extraction technique. This term has been determined from the methane content in coals, differentiated by the coalfields in Spain, and collected in a sectorial study prepared by AITEMIN .

- Coke oven door leakage and extinction : A default emission factor was applied, in terms of the mass of coke produced, as proposed by EMEP/CORINAIR (345 g CH4/t).

Evolution of the activity variables and emission factors implicit in the CRF: The total gross coal produced by mining has undergone a constant and practically linear decline (a fall of 40% in 2002 with respect to 1990). The breakdown by extraction techniques retains this same trend (a drop of 33.4% in open-cast productions and 46.5% from underground coalfaces), although less smoothed. The CH4 emission factors in mining present distinctive behaviour depending on the type of activity (mining or post-mining activities ) and extraction technique (underground or open-cast). Both trends depend on the percentage participation of types of coal with a high methane content (other bituminous and sub-bituminous coal) in the activity variable used for each activity. Thus, the evolution of the extraction series,  well-smoothed (decreasing for underground mining with a reduction of 9% in 2002 over 1990 and increasing for open-cast mining, up to 32%) should be noted alongside the less homogenous increase observed in the post-mining activities.

As mentioned in the methodology section, the methane emission factor for the transformation of solid fuels has remained constant for the period 1990-2002.

Evolution of the emissions:  The CH4 emissions in this category have fallen by 43% in 2002 with respect to 1990, mainly due to a fall of the same order in underground mining (43.4%). The rest of the sources included, except the post-mining activities in open-cast mining (of scant repercussion on the total), also reveal reductions (12.1% for open-cast mining activities and 18.2% for the transformation of solid fuels).

J) 1B2. Fugitive emissions – Oil and natural gas

Basic information sources: As the main sources of basic information, in the case of oil, special mention must be made of the individual questionnaires for refineries, which collect information by processes. For the transport and distribution of natural gas, it was opted to apply a similar treatment, developing specific questionnaires that were sent to two companies with extensive national coverage, one for transport and the other for distribution.

Description of the main emitting activities: The main sources of CO2 in Oil and natural gas are the processes applied in the oil refining industry (IPCC category 1B2a.iv Refining/Storage), including fluid catalytic cracking and the processing of petroleum products, with a contribution in 2002 of 66.5 and 22.1% respectively (see table 7.1.17 and figure 7.1.14).

table 7.1.17
main co2 emission sources

| IPCC category |  | CO2 Emissions (kt) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1990 | 1998 | 1999 | 2000 | 2001 | 2002 |
| 1B2a.iv | Oil refining / storage | 1.564,17 | 1.797,00 | 1.750,63 | 1.997,30 | 1.864,29 | 1.930,32 |
| 1B2.b.ii | Gas transmisión | 0,07 | 0,08 | 0,05 | 0,20 | 0,26 | 0,32 |
|  | Gas distribution | 0,18 | 0,06 | 0,09 | 0,28 | 0,35 | 0,43 |
| 1B2c.i | Oil flaring | 168,70 | 195,59 | 192,69 | 193,88 | 193,76 | 188,47 |
| 1B2c.ii | Gas flaring | 10,29 | 20,90 | 9,90 | 5,43 | 8,09 | 29,41 |
| 1B2 | Oil and natural gas | 1.743,41 | 2.013,63 | 1.953,36 | 2.197,09 | 2.066,76 | 2.148,94 |

FIGURE 7.1.14
main co2 emission sources

Activity variables and emission sources: For a better understanding of Table 1.B.2 in the CRF, the table 7.1.18 gives a brief description of the activity variable and the sources emitting CO2 included in each sub-category considered in IPCC category 1B2.

table 7.1.18
activity variables and co2 emission sources

| IPCC Category |  | CO2 Emission Sources | Activity data |
| --- | --- | --- | --- |
| 1B2a. Oil |  |  |  |
| 1B2a.ii | Production | - | Crude oil extraction from land-based and off-shores installations |
| 1B2a.iii | Transport | - | Purchase of crude oil by refineries |
| 1B2a.iv | Refining/Storage | Processing of petroleum products | Processing of crude oil at refineries |
|  |  | Coke calcining |  |
|  |  | Hydrogen plants |  |
|  |  | Steam reforming |  |
|  |  | Fluid catalytic cracking |  |
|  |  | Sulphur recovery plants |  |
| 1B2a.v | Distribution of petroleum products | - | Domestic consumption of petroleum products (sales)+ Production of gasoline |
| 1B2b. Gas natural |  |  |  |
| 1B2b.i | Production/ Processing | - | Gas extraction from land-based and off-shores installations |
| 1B2.b.ii | Transmission | Gas pipelines | Domestic consumption of natural gas |
|  |  | Venting at compression plants |  |
|  |  | Gas leak from underground storages |  |
|  | Distribution | Distribution networks (regulation and metering stations, networks or pipelines, connection points, internal mains and community installations ) | Domestic consumption of natural gas, LPG, gas manufactured from naphtha and coal, and propanized air |
| 1B2c. Venting and flaring |  |  |  |
| 1B2c.i | Flaring of oil | Flaring of waste gas and/or refinery gas at petroleum refineries | Processing of crude oil at refineries |
| 1B2c.ii | Flaring of gas | Flaring at regasification plants at the natural gas supply network | Natural gas flaring at regasification plant |

Methodological breakdown by emission sources: The emissions of CO2 from IPCC category 1B2 have been calculated by grouping together the estimates made for each emitting source indicated in the preceding table. Their estimation methodologies are indicated below:

- Coke calcining: The country-specific methodology comprises the application of an emission factor based on the mass balance of carbon in coke (loss of carbon during calcination of green coke). The emission factor estimated in this way is 1,009.8 kg CO2 per tonne of green coke, a value derived by assuming 91.8% of C in green coke and a weight loss of 30% between green coke and calcinated coke (1,009.8 = (44/12)*0.918 *0.30* *1,000).

- Steam reforming, hydrogen plants and sulphur recovery plants: Estimated emissions provided by the refineries by means of a questionnaire.

- Fluid catalytic cracking (FCC): The CO2 emission factors used, expressed in terms of feedstock mass into the FCC, have been determined at the plant level from the coke carbon retained in the units’ catalyst. With the information available in the refining sector questionnaires (feedstock processed and coke retained in the catalyst), and assuming 91.8% C in the coke, the ratios of CO2 emitted by the plant have been reached.

- Flaring at oil refineries The limitation of questionnaires to plants with a declaration of CO2 emitted has been resolved by following, when absent, the simplified EMEP/CORINAIR methodology. The default emission factor (3.14 kg CO2/tonne of crude oil processed ) is derived from expert judgement in the CORINAIR programme, which gave a figure of 0.1% of the total C in crude oil as the CO2-carbon emitted through flares. This assumption enables us to estimate CO2 emissions, taking a 85.5% of carbon content in the crude oil as the default parameter, according to the following procedure : 3.14=0.855*0.001*(44/12)*1,000.

- Gas pipelines, venting at compression stations, underground storage and flares at regasification plants: The estimation of emissions was done by taking an emission factor per unit of natural gas, after deduction of the average annual gas composition. The basic information (emissions and molar composition) has been collected by means of a questionnaire to a transport company, requiring the use of regression methods or elevation ratios to achieve the integrity of the series reported by plant and type of installation.

- Natural gas distribution network: The estimation methodology has required a modification of the detailed methodology set out in EMEP/CORINAIR, so as to ensure the completeness of the emissions for this activity. The estimations of natural gas emitted by type of installations and/or pipelines, provided by a distribution company by means of a questionnaire, were raised to the entire network length. The emission factors for natural gas emitted were deduced by considering a mean annual speciation of natural gas (information provided by the gas transport company).

Evolution of the activity variables and emission factors implicit in the CRF: Excepting gas flares, a relatively stable behaviour is assumed for the rest of the activity variables. The consumption of natural gas, increased by 286% with respect to the base year, reveals a sustained climb, much more pronounced after 1995; on the other hand, the crude oil processed at refineries shows a relative equilibrium with slight increases with respect to the base year, close to 8% in 2002. The evolution of the gas incinerated at the regasification plant, data provided by the natural gas transport company, presents considerable inter-annual fluctuations (over 200% for certain years) that may be justified by the very nature of the flares, since their use is subject to extremely variable conditions.

In connection with the emission factors, heterogeneity can be seen in their evolution. The flares, with inter-annual variations not greater than 5%, and storage and refinery, with up to 15% for certain years, have relatively consistent emission factors throughout the series 1990-2002. In gas transport and distribution the factors vary considerably throughout the series, depending greatly on the annual variation suffered by the molar compositions declared by the transport company.

Evolution of the emissions: The emissions of CO2 in this category have increased by 23.3% in the period 1990-2002 as a result of increases experienced in refining and storage of petroleum products (23.4%) and, to a lesser extent, by flares at refineries (11.7%).

The high increases in gas (330% in transport, 133% in distribution and 186% in flares) do not significantly affect the total in view of their small contributions. The evolution of the transport and distribution series is strongly marked by the speciation of the natural gas, where the influence after 1999, with recovery of the percentage of carbon dioxide contained in natural gas, combines with the increase in natural gas emitted in both networks. Natural gas flares depend solely on the variations suffered in the natural gas incinerated at the gasification plant, given the relative stability seen in the carbon content of natural gas.

OTHER SOURCES

Following the CRF sources nomenclature, other activities might additionally be considered under the heading of industrial processes albeit not as key sources in the inventory. A few of the most important of these are indicated below for the group of “Other sources”.

Category 1A1a also includes a sub-class of “other fuels” for electricity generation carried out at municipal solid waste incinerator plants as part of the strategy for energy recovery. These plants only emit CO2 that can be calculated as part of the emissions for the fossil fuel fraction in the waste incinerated and their scant size means they do not appear as key sources for the emission of this pollutant.

Category 1A1c also includes the emissions due to the combustion of liquid fuels in coke plants and in another set of activities including non-specific combustion in coal mining and the production of oil and natural gas, although, in view of the small energy consumption of these fuels at these installations, their emissions of CO2 do not constitute a key source in the inventory, whether for level or for trend.

In stationary combustion, whether of industrial origin (category 1A2) or in other sectors grouped in category 1A4, the emissions of CO2 from the use of the “other fuels” class do not constitute a key source in the inventory, whether for level or for trend, because of the scant participation of this kind of fuel in these sectors.

Other crosstabs of activities with pollutants that, although included within the energy sector, are not identified as key sources whether for level or for trend in the period of the inventory are the following:

1A3a – Civil aviation: emissions of CH4 and N2O.

1A3b – Road transport: emissions of CH4.

1A3d – National marine traffic: emissions of CH4 and N2O.

1A3c and 1A3e – railways and other transport: emissions of CO2, CH4 and N2O.

1B1 – Fugitive emissions in mining and coal processing: CO2.

1B2 – Fugitive emissions of oil and natural gas: CH4

7.2. INDUSTRIAL PROCESSES

SECTOR OVERVIEW

In 2002, the emissions from industrial processes represent 6.86% of the total emissions in Spain, in terms of CO2 equivalent, implying a reduction over the base year in which they represented 8.72% of the total. On the other hand, the emissions of CO2 equivalent in this sector have seen an increase of 9.61% in 2002 with respect to the base year, going from 22,772 kilotonnes of CO2 equivalent in 1990 to 27,418 kilotonnes in 2002. Figure 7.2.1 shows the evolution of the emissions in terms of CO2 equivalent throughout the period 1990-2002 by component categories according to the CRF nomenclature.

FIGURE 7.2.1
EVOLUTION OF CO2 EQUIVALENT EMISSIONS

-	Basic data sources (activity variables, algorithms and emission factors).

For the estimation of emissions from industrial processes, after discounting the contribution of combustion which has already been dealt with in the preceding section 7.1, the relevant activity variables are specific for each sector or activity under consideration, and may refer to the main product, the main input or a relevant raw material in the process or activity.

Apart from the characterization of the pertinent product, input or raw material, for a proper estimation of the emissions it is of great importance to distinguish where appropriate between the production processes used, as the technology applied determines to a large extent the emissions of the gases considered in many cases.

With respect to the estimation algorithms, the basic references have been taken from the IPCC Guidelines (Revised 1996 Guidelines and the Good Practice Guidance for 2000), complemented on occasion with the EMEP/CORINAIR Guidebook. In connection with the methodologies proposed by the IPCC, whenever the data required for the case have been available, advanced methodological tiers have been applied for the estimation of emissions.

Specific mention is made below to the data sources for the key activities in this sector when each of the activities is presented.

-	Completeness, transparency and time-series consistency.

In general, it is felt that a high degree of coverage has been achieved in the estimation of emissions over the total of the activities potentially generating pollutant emissions. The foregoing notwithstanding, it should be indicated that in this edition of the inventory it has not been possible to develop the methodology for the estimation of potential emissions from fluorinated gases due to considerable shortcomings in the basic information. In another activity, such as the use of limestone and dolomite, the degree of coverage is partial, with some activities such as construction and acidifier emission-abatement processes pending estimation. Furthermore, complete coverage has been achieved in the manufacture of lime with respect to the manufacturing plants devoted to the sale of lime, as well as in other sectors (iron and steel industry, paper industry). However, the diagnosis of the production levels in other self-consumption sectors, including the food sector, is pending. Mention should also be made of the fact that the existence or otherwise of emissions of halocarbons and SF6 is still pending for the semiconductor manufacturing industry.

The data on activity variables, emission factors and algorithms used to make the estimation of the emissions can be traced and are considered sufficiently transparent to be able to diagnose the inventory’s quality. The current edition of the inventory has extended the series inventoried to 2002, now covering the period 1990-2002, and whenever there have been changes in the methodology or coverage of the activity variables and emission factors chosen as representative with respect to the previous edition, the series of estimated emissions has been back-projected consistently to 1990. In this same line, whenever evidence has been found of a need for recalculation (for instance, due to the detection of errors), the entire set of the time series has also been revised.

The remainder of this section examines in detail the key data sources for the industrial processes sector.

KEY SOURCES ANALYSIS

The analysis of key sources for the period 1990-2002 has identified, within IPCC source category 2 “Industrial Processes”, the following activities:

-	Cement production (2A1) for its CO2 emission level for the entire period 1990-2002 and for its behaviour in the period 1991-2002.

-	Lime production (2A2) for its behaviour in the emissions of CO2 in 1994.

-	Nitric acid production (2B2) for its level of N2O emissions in the period 1990-2001 and for its behaviour in the period 1991-2002.

-	Aluminium production (2C3) for its behaviour in the emissions of PFCs in the years 1991 and 2000-2002.

-	Manufacture of HCFC-22 (2E1) for its level of HFC emissions in the period 1990-2001 and for its behaviour in the years 1991-2000 and 2002.

-	Consumption of halocarbons and SF6 (2F) for the overall level of emissions of HFCs and PFCs in 2001 and 2002 and for their behaviour in period 1997-2002.

Table 7.2.1 below summarizes the foregoing and gives the contribution of the emissions level and the trend for the key categories in this sector, as well as the absolute values in terms of CO2 equivalent, all referring to 2002.

TABLE 7.2.1
KEY SOURCES: LEVEL AND TREND CONTRIBUTION

| IPCC Activity |  | Gas | CO2-e (Gg) (2002) | Level contribution (2002) |  |  | Trend contribution (2002) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Code | Description |  |  | % | Key Source | Range | % | Key Source | Range |
| 2A1 | Cement production | CO2 | 15.853 | 3,97 | YES | 6 | 1,32 | YES | 16 |
| 2A2 | Lime production | CO2 | 1.433 | 0,36 | NO | 27 | 0,11 | NO | 40 |
| 2-2A1-2A2-2C1 | Other industrial processes | CO2 | 2.877 | 0,72 | YES | 19 | 0,37 | NO | 28 |
| 2B2 | Nitric acid production | N2O | 1.945 | 0,49 | NO | 25 | 1,58 | YES | 15 |
| 2C3 | Aluminium production | PFC | 192 | 0,05 | NO | 48 | 0,73 | YES | 21 |
| 2E1 | Production of HCFC-22 | HFC | 1.029 | 0,26 | NO | 31 | 1,76 | YES | 14 |
| 2F | Consumption of halocarbons and SF6 | HFC & PFC | 2.791 | 0,7 | YES | 20 | 2,09 | YES | 12 |

A) 2A1. Cement production.

In this activity the emissions envisaged are those produced during the process of manufacture of clinker as a result of the decarbonizing of limestone (calcium carbonate), to produce calcium oxide:

CaCO3  CaCO + CO2

This emission of CO2 is inherent to the process for manufacturing clinker, since it is a practically fixed value per tonne of clinker manufactured. The emissions corresponding to combustion activities are dealt with in CRF category 1A2f.

The production of cement clinker is used as the activity variable for emissions estimation. This information has been taken from the publication “Industrias del Cemento” (Cement Industries) for the years 1990-1998, whereas for the years 1999-2002 the information has been provided by the cement manufacturing association (OFICEMEN). Table 7.2.2.a presents the clinker production expressed in tonnes.

TABLE 7.2.2.a
CLiNKER production

| CLINKER PRODUCTION (tonnes) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1990 | 1998 | 1999 | 2000 | 2001 | 2002 |
| 23.211.731 | 26.103.860 | 27.280.915 | 27.840.499 | 28.382.550 | 29.357.596 |

For estimating CO2 emissions there are several references that propose different emission factors (EMEP/CORINAIR, IPCC). However, here it has been decided to use the information supplied by the Association of Cement Manufacturers, which states the emissions of CO2 arising from decarbonising as 540 kg CO2/tonne of clinker produced.

Table 7.2.2.b shows the emissions of CO2 for this activity, with this gas giving the source its nature as a key source. Table 7.2.2.c shows these emissions in terms of CO2 equivalent. This same table gives the index for the evolution over time (base 100 in 1990) of the CO2 equivalent emissions, and the contributions of the CO2 equivalent emissions of this category to the total of the inventory and the “Industrial Processes” sector.

TABLE 7.2.2.b
CO2 EMISSIONS

| CO2 EMISSIONS (Gg) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1990 | 1998 | 1999 | 2000 | 2001 | 2002 |
| 12.534 | 14.096 | 14.732 | 15.034 | 15.327 | 15.853 |

TABLE 7.2.2.c
CO2 EQUIVALENT EMISSIONS: INDEX AND RATIOS

|  | 1990 | 1998 | 1999 | 2000 | 2001 | 2002 |
| --- | --- | --- | --- | --- | --- | --- |
| CO2-eq (Gg) | 12.534 | 14.096 | 14.732 | 15.034 | 15.327 | 15.853 |
| Index CO2-eq | 100,0 | 112,5 | 117,5 | 119,9 | 122,3 | 126,5 |
| % CO2-eq of total | 4,40 | 4,14 | 3,98 | 3,90 | 4,00 | 3,97 |
| % CO2-eq of ind. proc. | 55,04 | 51,01 | 49,33 | 48,53 | 54,35 | 57,82 |

B) 2A2. Lime production

In the analysis of the key sources, this activity is not identified for its contribution to the level in any of the years of the period under consideration, but it is identified for its trend in 1994. For this reason, the information referring to this activity is given below.

The emissions considered here are those produced in the decarbonization process during the manufacture of lime and calcinated dolomite. The commercial products on which the data has been distinguished include quicklime and slaked lime, for which it is assumed that the material is calcium carbonate, and calcinated dolomite for which it is assumed that the raw material is calcium carbonate and magnesium carbonate. The production data given in Table 7.2.3.a have been obtained through the publication entitled “Industrias del Yeso y de la Cal” (Gypsum and Lime Industries) for the period 1990-1998 and by means of the information provided by the Spanish National Association of the Lime Manufacturing Industry (ANCADE in its Spanish acronym) for the period 1999-2002. This series also includes the production of lime in other industries (integrated iron and steel) corresponding to 1990.

TABLE 7.2.3.a
LIME PRODUCTION

| LIME PRODUCTION (tonnes) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1990 | 1998 | 1999 | 2000 | 2001 | 2002 |
| 1.131.566 | 1.710.780 | 1.748.364 | 1.793.099 | 1.871.984 | 2.002.275 |

The emission factors of CO2 proposed in the IPCC Reference Manual (Table 2.2) are of 790 and 910 kg CO2/t of the calcinated material depending on whether it is calcite or dolomite. However here it has been decided to use the data supplied by the Association of the Lime Manufacturing Industry (ANCADE), in which the emission factors are distinguished depending on the type of lime manufactured (quicklime or dead-burned dolomite). For hydrated lime, a conversion has been carried out for the emission factor stated for quicklime using for this purpose the following conversion factor supplied by ANCADE:

1 t of quicklime = 1,32 t of hydrated lime

considering that the quicklime is completely pure and its hydration is produced at 100%.

For the manufacture of lime in other industries, and given that the process of manufacture differs from that used in the construction industries, it has been decided to use the emission factor proposed in the IPCC Reference Manual (790 kg CO2/t of lime).

Table 7.2.3.b shows the emissions of CO2 for this activity, with this gas giving this source its nature as a key source. Table 7.2.3.c shows these emissions in terms of CO2 equivalent. This same table gives the index for the evolution over time (base 100 in 1990) of the CO2 equivalent emissions, and the contributions of the CO2 equivalent emissions of this category to the total of the inventory and the “Industrial Processes” sector.

TABLE 7.2.3.b
CO2 EMISSIONS

| CO2 EMISSIONS (Gg) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1990 | 1998 | 1999 | 2000 | 2001 | 2002 |
| 917 | 1.231 | 1.240 | 1.277 | 1.335 | 1.433 |

TABLE 7.2.3.c
CO2 EQUIVALENT EMISSIONS: INDEX AND RATIOS

|  | 1990 | 1998 | 1999 | 2000 | 2001 | 2002 |
| --- | --- | --- | --- | --- | --- | --- |
| CO2-eq (Gg) | 917 | 1.231 | 1.240 | 1.277 | 1.335 | 1.433 |
| Index CO2-eq | 100,0 | 134,2 | 135,3 | 139,3 | 145,6 | 156,3 |
| % CO2-eq of total | 0,32 | 0,36 | 0,33 | 0,33 | 0,35 | 0,36 |
| % CO2-eq of ind. proc. | 4,03 | 4,45 | 4,15 | 4,12 | 4,73 | 5,23 |

C) 2. Industrial processes (except 2A1, 2A2 and 2C1).

This is a ragbag of activities grouping together, with respect to CO2 emissions, the activities corresponding to industrial processes, except for the manufacture of cement and lime, and the processes used in the production of iron and steel. The list of activities considered warrants special attention to be paid, for their importance in CO2 emissions, to the production and use of sodium carbonate, the manufacture of ammonia, the production of carbides, ferroalloys and aluminium.

Table 7.2.4.a shows the emissions of CO2, with this gas giving this source its nature as a key source. Table 7.2.4.b complements this information by including the associated emissions of CH4, N2O, PFCs, HFCs and SF6, with the emissions as a whole being expressed in terms of CO2 equivalent. This same table gives the index for the evolution over time (base 100 in 1990) of the CO2 equivalent emissions, and the contributions of the CO2 equivalent emissions of this category to the total of the inventory and the “Industrial Processes” sector.

TABLE 7.2.4.a
CO2 EMISSIONS

| CO2 EMISSIONS (Gg) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1990 | 1998 | 1999 | 2000 | 2001 | 2002 |
| 2.399 | 2.595 | 2.649 | 2.676 | 2.839 | 2.877 |

TABLE 7.2.4.b
CO2 EQUIVALENT EMISSIONS: INDEX AND RATIOS

|  | 1990 | 1998 | 1999 | 2000 | 2001 | 2002 |
| --- | --- | --- | --- | --- | --- | --- |
| CO2-eq (Gg) | 8.610 | 11.515 | 13.096 | 13.823 | 10.665 | 9.263 |
| Index CO2-eq | 100,0 | 133,7 | 152,1 | 160,5 | 123,9 | 107,6 |
| % CO2-eq of total | 3,03 | 3,38 | 3,54 | 3,59 | 2,78 | 2,32 |
| % CO2-eq of ind. proc. | 37,81 | 41,67 | 43,86 | 44,62 | 37,82 | 33,79 |

D) 2B2. Nitric acid production

This activity considers the emissions coming from the manufacture of nitric acid. In the case of NOX, the associated emission factors depend on the type of manufacturing process used, with a distinction being drawn between low, medium or high pressure. For this reason, the data on the activity variable used (production of nitric acid) distinguishes between the amounts manufactured for each of the processes mentioned. Table 7.2.5.a presents the nitric acid production.

TABLE 7.2.5.a
NITRIC ACID PRODUCTION

| NITRIC ACID PRODUCTION (tonnes) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1990 | 1998 | 1999 | 2000 | 2001 | 2002 |
| 1.329.107 | 998.973 | 1.081.888 | 1.063.017 | 941.992 | 896.360 |

For estimating the emissions of N2O the emission factor has been taken from the report by the Business Federation for the Chemicals Industry in Spain (FEIQUE) supplied to the Ministry of Science and Technology for the inventories of greenhouse effect gases, namely 7 kg N2O/tonne of nitric acid.

Table 7.2.5.b presents the emissions of N2O for this activity, with this gas giving this source its nature as a key source. Table 7.2.5.c gives these emissions in terms of CO2 equivalent. This same table gives the index for the evolution over time (base 100 in 1990) for the emissions of CO2 equivalent, and the contributions of the CO2 equivalent emissions of this category to the total of the inventory and the “Industrial Processes” sector.

TABLE 7.2.5.b
N2O EMISSIONS

| N2O EMISSIONS (Gg) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1990 | 1998 | 1999 | 2000 | 2001 | 2002 |
| 9,3 | 7,0 | 7,6 | 7,4 | 6,6 | 6,3 |

TABLE 7.2.5.c
CO2 EQUIVALENT EMISSIONS: INDEX AND RATIOS

|  | 1990 | 1998 | 1999 | 2000 | 2001 | 2002 |
| --- | --- | --- | --- | --- | --- | --- |
| CO2-eq (Gg) | 2.884 | 2.168 | 2.348 | 2.307 | 2.044 | 1.945 |
| Index CO2-eq | 100,0 | 75,2 | 81,4 | 80,0 | 70,9 | 67,4 |
| % CO2-eq of total | 1,01 | 0,64 | 0,63 | 0,60 | 0,53 | 0,49 |
| % CO2-eq of ind. proc. | 12,67 | 7,84 | 7,86 | 7,45 | 7,25 | 7,09 |

E) 2C3. Aluminium production

Primary aluminium production constitutes a key source in the inventory because of the associated PFC emissions. The data on activity variables and the parameters of the emissions estimation algorithm have been collated by means of an individualized questionnaire for each of the three production plants.

From the information received a distinction is drawn by plants and the series of manufacturing method used (prebaked anodes with side or central worked and the vertical studs Söderberg process). Within each series, information was obtained on the following parameters:

Efficiency of the electric current

Number of anode effects per pot and day

Duration of the anode effect in minutes

Using this information, the emissions are estimated by application of the Tier 3b method referred to in the IPCC Good Practice Guidance.

The information on activity variables and process parameters are not given here as these figures are confidential since all of the plants belong to only one company.

Table 7.2.6.a shows the PFC emissions for this activity, with these gases giving this source its nature as a key source. Table 7.2.6.b complements the foregoing information by including the associated emissions of CO2, and expressing the emissions as a whole in terms of CO2 equivalent. This same table gives the index for the evolution over time (base 100 in 1990) of the CO2 equivalent emissions, and the contributions of the CO2 equivalent emissions of this category to the total of the inventory and the “Industrial Processes” sector.

TABLE 7.2.6.a
PFC EMISSIONS

|  | PFC EMISSIONS (tonnes) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1990 | 1998 | 1999 | 2000 | 2001 | 2002 |
| CF4 | 114,3 | 99,7 | 91,2 | 50,0 | 24,7 | 26,5 |
| C2F6 | 9,3 | 8,9 | 7,9 | 4,2 | 1,7 | 2,1 |

TABLE 7.2.6.b
CO2 EQUIVALENT EMISSIONS: INDEX AND RATIOS

|  | 1990 | 1998 | 1999 | 2000 | 2001 | 2002 |
| --- | --- | --- | --- | --- | --- | --- |
| CO2-eq (Gg) | 1.379 | 1.288 | 1.230 | 930 | 759 | 781 |
| Index CO2-eq | 100,0 | 93,4 | 89,2 | 67,4 | 55,0 | 56,6 |
| % CO2-eq of total | 0,48 | 0,38 | 0,33 | 0,24 | 0,20 | 0,20 |
| % CO2-eq of ind. proc. | 6,06 | 4,66 | 4,12 | 3,00 | 2,69 | 2,85 |

F) 2E1. Production of HCFC-22

This activity considers the emissions of HFC-23 arising as a by-product in the manufacture of HCFC-22. During the period under inventory, three HCFC-22 manufacturing plants have been operational, either temporarily or throughout the interval in question. The information on the production of this compound, the activity variable, has been provided by the production centres themselves.

The information on HFC-23 emissions is based on the estimates made by the centres themselves, complemented for certain years by a default emission factor. Therefore, the estimation methodology applied in this case is a combination of Tier 1 and Tier 2 in the IPCC’s terminology.

The information on activity variables and process parameters is not given here as it is confidential since all of the plants belong to two companies.

Table 7.2.7.a shows the emissions of HFC-23 for this activity, with this gas giving this source its nature as a key source. Table 7.2.7.b gives these emissions in terms of CO2 equivalent. This same table gives the index for the evolution over time (base 100 in 1990) for the CO2 equivalent emissions, and the contributions of the CO2 equivalent emissions of this category to the total of the inventory and the “Industrial Processes” sector.

TABLE 7.2.7.a
HFC-23 EMISSIONS

| HFC-23 EMISSIONS (tonnes) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1990 | 1998 | 1999 | 2000 | 2001 | 2002 |
| 205,4 | 434,0 | 503,5 | 540,4 | 248,1 | 87,9 |

TABLE 7.2.7.b
CO2 EQUIVALENT EMISSIONS: INDEX AND RATIOS

|  | 1990 | 1998 | 1999 | 2000 | 2001 | 2002 |
| --- | --- | --- | --- | --- | --- | --- |
| CO2-eq (Gg) | 2.403 | 5.078 | 5.891 | 6.323 | 2.903 | 1.029 |
| Index CO2-eq | 100,0 | 211,3 | 245,1 | 263,1 | 120,8 | 42,8 |
| % CO2-eq of total | 0,84 | 1,49 | 1,59 | 1,64 | 0,76 | 0,26 |
| % CO2-eq of ind. proc. | 10,55 | 18,37 | 19,73 | 20,41 | 10,29 | 3,75 |

G) 2F. Consumption of halocarbons and SF6

This group includes the activities of CRF category 2F that generate emissions of HFCs or PFCs, namely those corresponding to refrigeration and air conditioning (2F1), fire extinguishers (2F3) and aerosols (2F4). This section does not include the emissions of SF6 in electrical equipment.

With respect to refrigeration and air conditioning, information has been supplied for certain years by the business associations for this sector. These data have been extrapolated for recent years by the inventory working party with the help of information on evolution proxies taken from the automobile industry, which has also provided data on plants for the main gas under consideration (HFC-134a). For the national production of motor vehicles, the emission factors are those derived from the data obtained in questionnaires from the manufacturing plants, and are taken from the IPCC guidelines for the other sub-sectors.

For fire extinguishers, the information on the amounts of fluorinated gases consumed in the maintenance and new installation of fire-fighting equipment has been obtained by means of a questionnaire addressed to the main companies in the sector. On the basis of the activity variables data, the inventory working party has performed an extrapolation for the total sector on the basis of an estimation of the coverage of the companies providing data. The emissions estimation has been effected by calculating in each year the existing stock of fluorinated gases in the equipment installed and applying the default emission factor suggested in the IPCC Guidelines.

With respect to aerosols, the information on the activity variable (gases incorporated into aerosol devices) has been provided by the main association of the manufacturers in this sector. The activity variable is the apparent consumption of each gas, as the result of adding together domestic production plus the net balance of imports minus exports. As for the time-related activity variable, it is assumed that 50% of the emissions are generated in the reference year for the apparent consumption and the other 50% in the following year. The default emission factor suggested in the IPCC Guidelines is applied.

Table 7.2.8.a shows the emissions for each type of gas for each of these activities. Since the use of these gases was marginal prior to 1995 and, therefore, their associated emissions are very small, the series is given for the period 1995-2002. For this reason too, the fluorinated gases take 1995 as the basis for computing the evolution of the emissions. Table 7.2.8.b gives these emissions in terms of CO2 equivalent. This same table gives the index for the evolution over time (base 100 in 1995) for the emissions of CO2 equivalent, and the contributions of the CO2 equivalent emissions of this category to the total of the inventory and the “Industrial Processes” sector.

TABLE 7.2.8.a
HFCs & PFCs EMISSIONS

|  |  | HFCs & PFCs EMISSIONS (tonnes) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Category | Gas | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 |
| 2F1 | HFC-125 |  | 3,66 | 10,40 | 19,15 | 29,60 | 41,58 | 53,91 | 66,30 |
|  | HFC-134a |  | 58,75 | 139,54 | 259,76 | 412,43 | 577,23 | 740,41 | 905,93 |
|  | HFC-143a |  | 4,33 | 12,16 | 22,31 | 34,43 | 48,34 | 62,65 | 77,03 |
|  | C3F8 |  | 0,50 | 1,40 | 2,58 | 3,98 | 5,59 | 7,24 | 8,91 |
| 2F3 | HFC-23 | 0,26 | 0,59 | 2,70 | 6,21 | 15,75 | 33,21 | 51,55 | 67,55 |
|  | HFC-125 |  |  |  |  | 0,07 | 0,19 | 0,33 | 0,66 |
|  | HFC-227ea | 0,83 | 1,39 | 2,83 | 5,21 | 8,36 | 12,32 | 16,85 | 21,88 |
|  | HFC-236fa |  |  |  |  | 0,15 | 0,39 | 0,67 | 0,94 |
|  | C4F10 | 0,05 | 0,20 | 0,22 | 0,25 | 0,28 | 0,33 | 0,37 | 0,42 |
| 2F4 | HFC-134a | 1,62 | 3,36 | 3,56 | 93,67 | 204,20 | 230,07 | 220,22 | 159,92 |

TABLE 7.2.8.b
CO2 EQUIVALENT EMISSIONS: INDEX AND RATIOS

|  | 1995 | 1998 | 1999 | 2000 | 2001 | 2002 |
| --- | --- | --- | --- | --- | --- | --- |
| CO2-eq (Gg) | 8 | 705 | 1.255 | 1.818 | 2.348 | 2.791 |
| Index CO2-eq | 100,0 | 8.922,1 | 15.872,8 | 23.000,5 | 29.704,5 | 35.299,8 |
| % CO2-eq of total | 0,00 | 0,21 | 0,34 | 0,47 | 0,61 | 0,70 |
| % CO2-eq of ind. proc. | 0,03 | 2,55 | 4,20 | 5,87 | 8,33 | 10,18 |

OTHER SOURCES

Following the CRF sources nomenclature, other activities might additionally be considered under the heading of industrial processes albeit not as key sources in the inventory. A few of the most important of these are indicated below for the group of “Other sources”.

Category 2C1 includes the emissions for the iron and steel manufacturing processes excluding those referring to the associated combustion processes, which are in category 1A2a.

The production of halocarbons and SF6, except for the manufacture of HCFC-22 (categories 2E2 and 2E3).

Category 2F7 includes the SF6 emissions from electrical equipment. In the case of Spain, this is the only source generating emissions of this gas.

7.3.- solvents and other product use

SECTOR OVERVIEW

Emissions due to the use of solvents and other products constitutes a minor source of greenhouse gas emissions for the inventory. Specifically, they represented in 2002 0.42% of the total emissions in Spain in CO2 equivalent terms, implying a reduction with respect to the base year when they represented 0.46% of the total. On the other hand, the CO2 equivalent emissions in this sector have undergone an increase of 27.35% during 2002 with respect to the base year, growing from 1,330 kilotonnes of CO2 equivalent in 1990 to 1,694 kilotonnes in 2002. Figure 7.3.1 shows the evolution of the emissions in CO2 equivalent terms throughout the period 1990-2002 by component categories according to the CRF nomenclature.

FIGURE 7.3.1
EVOLUTION OF CO2 EQUIVALENT EMISSIONS

As can be seen in the graph, the relative importance of this category is very small in the inventory, and it does not constitute a key source by level of emission nor by trend in any year of the period under study.

7.4.- AGRICULTURE

SECTOR OVERVIEW

The Agriculture emissions represent (in 2002) 10.67% of Spain’s total emissions of CO2 equivalent (CO2-eq), implying a backwards step in terms of its significance as a source of emissions with respect to 1990 when this sector represented 13.13% of the total. In the meantime, emissions have grown by 14.12% in the same period, rising from 37,373 Kt of CO2-eq in 1990 to 42,650 in 2002.  This phenomenon is due to the greater increase in emissions from other sectors (see figure 7.4.1).

The most relevant source for base data is the “Agrarian Statistical Yearbook” published by the Ministry of Agriculture, Food and Fisheries (MAPA in its Spanish acronym). The main methodological sources, on the other hand, are the methodological documents of the IPCC, EMEP/CORINAIR and, for specific national issues, the document entitled “Estimation of Greenhouse Gas Effects 2001” by the MAPA (in its different versions).

FIGURE 7.4.1

Completeness: Most of the sources and gases in this sector have been estimated.

Transparency:  The data management system used when compiling the Inventory  has a very fine degree of detail with data disaggregated by provinces (Spain’s third-level administrative unit, of which there are 50). This implies a large amount of data to be processed and an apparent complexity in their review.

There is a difference between the base data for animal husbandry in the Inventory, supplied by the MAPA, and the data used by the FAO. This is due to the use of a number of farming surveys in the Inventory whereas the FAO only refers to the December surveys. For those years in which information is missing, recourse is had to the data for the closest year for which information is available, except where the weather conditions indicate otherwise.

KEY SOURCES ANALYSIS

The key sources identified for the period 1990-2002, in order of importance of their emissions, are: a) Enteric Fermentation in Domestic Cattle (CH4); b) Manure Management (CH4); c) Agricultural Soils - Direct Emissions (N2O); d) Agricultural Soils - Indirect Emissions (N2O); e) Agricultural Soils - Animal Production (N2O); f) Manure Management (N2O) (see table 7.4.1).

table 7.4.1
key sources

| Code | Description | GHG | CO2-eq | Contribution Level |  |  | Contribution Trend |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (2002) | % | Key S. | Range | % | Key S. | Range |
| 4A | Enteric Fermentation in Domestic Livestock | CH4 | 14,720 | 3.68 | YES | 7 | 2.29 | YES | 11 |
| 4B | Manure Management | CH4 | 8,627 | 2.16 | YES | 11 | 0.08 | NO | 46 |
| 4D1 | Agricultural Soils - Direct Emissions | N2O | 8,357 | 2.09 | YES | 12 | 2.72 | YES | 8 |
| 4D3 | Agricultural Soils - Indirect Emissions | N2O | 5,205 | 1.30 | YES | 15 | 1.19 | YES | 17 |
| 4D2 | Agricultural Soils - Animal Production | N2O | 3,306 | 0.83 | YES | 18 | 0.46 | NO | 26 |
| 4B | Manure Management | N2O | 1,633 | 0.41 | NO | 26 | 0.49 | YES | 24 |

A) IPCC 4A. Enteric Fermentation in Domestic Livestock (CH4 Emissions):

Description of the main emissions-generating activities: The main sources of CH4 in this activity are non-dairy cattle (4A1b) with 44.98% of the emissions followed by sheep (4A3) with 29.83% and dairy cattle (4A1A) with 17.48% of the total emissions in this activity in year 2002.

FIGURE 7.4.2

Activity variables: The data for the activity variable come from the “Agrarian Statistical Yearbook”, as well as from the “Monthly Agrarian Statistical Bulletins” (both published by the MAPA). As mentioned in the summary, all of the annual surveys are used and their mean value is taken for the number of animals in each type. The base data used are more disaggregated than those of the IPCC, with each animal category being divided into sub-categories based on those included in the above-mentioned MAPA documents.

Methodological breakdown: Two methodologies have been used in this activity: Tier 2 for animals with greater significance for emissions (cattle and sheep) and Tier 1 for the emissions of other animal species.

In the case of animals with Tier 1, the default emission factor (E.F.) for developed countries of the IPCC has been used (IPCC Manual, Table 4-3) but taking a value 20% lower for young animals.

For the animals with Tier 2, the IPCC methodology has been followed but, since some of the activity data needed to apply this methodology are not available in Spain, national methodologies have been used for their calculation (usually based on disaggregation by breeds, and their characteristics, within the different animal species).

Evolution of the activity variables and emission factors implicit in the CRF: The activity variable is the number of live animals. In the period 1990-2002 there is a significant increase in non-dairy cattle (54.23%), as well as in swine (increase of 47.29%), whereas dairy cattle and goat herds fell back in head numbers (27.18 and 16.83% respectively). For equine animals (horses, mules and asses), no data are available other than the 1986 census and therefore the activity variable is the same for the entire period (see figure 7.4.3).

FIGURE 7.4.3

In the emission factors for Tier 1 animals, there is a slight variation year on year because of the differing percentage of young animals born. For sheep and non-dairy cattle (both Tier 2) hardly any variation in EE can be observed. Dairy cattle, on the other hand, suffered an increase in EF from 88.61 to 104.49 Kg CH4/head and year (17.92%). This increase is due to lactation energy. This energy increases because of the growth in milk production in Spain and the drop in the number of animals.

Evolution of the emissions: For the period in question the emissions of this activity increase by 16.35%, with the main category responsible being non-dairy cattle with an increase of 105.11 kt of CH4 (2,207 kt of CO2-eq), a 50% increase. Emissions by swine also increase considerably (48.60%), but this only represents 222 kt CO2-eq more, because of their relatively low weighting. Both increases are due to a larger number of animals, not to a large variation in the emission factor. In the meantime, both goats and dairy cattle have seen their emissions cut by 2.93 and 20.15% respectively, because of the decline in the number of animals in these species.

B) IPCC 4B. Manure Management (CH4 Emissions):

Description of the main emissions-producing activities: there is one source that stands out over all the other emissions sources in this activity, namely swine herds (4B8) with 85.51% of the emissions in 2002. Of the remaining animals, mention must be made of non-dairy cattle with 4.91%, dairy cattle with 3.97% and Poultry (hens and chickens) with 3.53% of the emissions (see figure 7.4.4).

FIGURE 7.4.4

Activity variables: As in the previous case, the activity variable is the number of animals (please refer to Enteric Fermentation).

Methodological breakdown: Similarly to the case of enteric fermentation, the IPCC methodology has mostly been followed in this activity. Tier 2 has been adopted for beef and pork herds, whereas the other animals have been treated using the Tier 1 method. This group has two methodological peculiarities: the smoothing of the MCF and EF functions provided by the IPCC, and the use of a classification system for the N excreted by the animals according to the type of manure management that is not the default IPCC system. The MCF and EF functions for Tier 1 animals presented by the IPCC are staged and in view of the problems that this posed in the administrative divisions with temperatures close to 15ºC, it was decided to smooth the functions (this modification was accepted by the IPCC). With respect to the management systems, the experts in the sector provided their own calculation methodology, in view of Spain’s uniqueness in this case.

Evolution of the activity variables and emission factors implicit in the CRF: We have already explained in the previous heading (Enteric Fermentation) the increase in the numbers of non-dairy cattle and the reduction in dairy animals. In the case of swine, there is a strong increase of 47.29% (1990-2002), going from 16,992,389 live pigs in 1990 to 25,028,885 in 2002. Poultry (hens and chickens) also experienced an increase but only by 5.85% (see figure 7.4.5).

In most cases, the emission factors increased slightly. Exceptions can be seen in dairy cattle, with an increase of 17.25%, growing from 11.87 to 13.92 Kg CH4/head and year (even though their total emissions reduced over this period) and in swine herds where the reduction was 1.34%.

FIGURE 7.4.5

Evolution of the emissions: The emissions undergo a gradual increase mainly caused by the greater size of the herds. This increase is 38.68%, rising from 6,221 to 8,627 kt of CO2-eq. Practically all of this rise is due to swine where the emissions grew by 2,301 kt because of the larger numbers of animals.

C) IPCC 4D. Agricultural Soils (N2O Emissions):

Description of the main emissions-producing activities: Three key sources can be seen in this activity: a) Direct Emissions (4D1), b) Indirect Emissions (4D3) and c) Animal Production (4D2). In addition to these activities, there is another that refers to emissions from sludge and compost but its weighting is practically nil. The main sources of emissions in this activity are 4D1 (which represents 49.14%), 4D3 (30.60%) and 4D2 (19.44%) of the emissions for this activity in 2002 (see table 7.4.6).

FIGURE 7.4.6

Activity variables: The activity variable is the N contained in each of the different emissions sources involved in these activities. For synthetic fertilization, the activity variable is the N content in fertilizers and the information is provided by the Spanish National Association of Fertilizer Manufacturers (ANFFE). The rest of the activity variables are obtained from the data in the MAPA’s “Agrarian Statistical Yearbook”. To calculate the N contained in the crop residues, a national methodology is used. The data on the use of sludge and compost come from IPCC Group 6 (Waste).

Methodological breakdown: For the calculation of the emissions, the IPCC methodology is generally used with equations 4.20, 4.30 and 4.18 from the IPCC Good Practices Handbook, as well as the other equations used for the preceding ones. The specific changes adopted are: i) to find the N contained in the manure from grazing animals (4D2), the disaggregation provided by the IPCC for Western Europe has not been followed but instead an in-house methodology has been applied (consistent with that of the emissions in activity 4B); ii) the value taken as the FracGASF parameter was that obtained in the calculation of the emissions of NOX and NH3 included in the Inventory; iii) for the FracLEACH  parameter, the value adopted was 0.15 Kg. N / Kg. N applied, a figure within the IPCC range but not the default value (0.3).

Evolution of the activity variables and emission factors implicit in the CRF: The consumption of synthetic fertilizers goes through considerable inter-annual variations but, taking the extremes of the period, there is only a variation of -6.55%. The N contained in the animal fertilizers increased by 15.49% as did the animal production, which increased by 18.32%, in both cases due to the increase in herd numbers. The A.V. for the indirect emissions increases as a result of the increase in the rest of the A.V. The considerable increase of 123.25% in the N contained in sludge must be mentioned, although its low weighting in the emissions reduces its impact to almost nothing.

The emission factors correspond to the default values given by the IPCC and do not have any annual variability.

Evolution of the emissions: The total emissions for this activity undergo a slight increase of 4.49% in the period 1990-2002, going from 16,277 to 17,008 kt of CO2-eq. By activities, direct emissions (4D1) suffer a decline of 2.08% in emissions due to the reduction of the emissions from synthetic fertilization that is not counteracted by the increase in those due to organic fertilization. Indirect emissions (4D3) increased by 7.62%, whereas those due to animal production (4D2) rose by 18.32% because of the larger number of animals.

D) IPCC 4B. Manure management (N2O Emissions):

Description of the main emission-producing activities: One source is outstanding over all the others producing emissions in this activity, namely the treatment of manure by means of solid storage and dry lot, responsible for close to 95% of the emissions. No anaerobic lagoons are used in Spain and the “other” treatments are only used on a small fraction of dairy cattle (see figure 7.4.7).

FIGURE 7.4.7

Activity variables: In this case, the activity variable is the N contained in each type of manure treatment. This is calculated using the number of animals (please refer to Enteric Fermentation), the excretion of N per animal and the percentage of that N in each treatment system.

Methodological breakdown: The IPCC methodology (equation 4.18, Good Practices) has been used. For the parameter of N excreted, the values taken were those in the table of default values in the IPCC Reference Manual (Table 4-20), using the row entitled “Near East & Mediterranean” (subsequently applying age-related correction factors as recommended by the IPCC). The percentage of N in each treatment system has already been explained in the section on “Manure Management (CH4)”.

Evolution of the activity variables and emission factors implicit in the CRF: There is a widespread increase in the numbers of animals as explained above. The excretion of N does not vary in practice (as it is only affected by the percentage of young animals). The calculation of the N contained in each management system is the same every year. The solid systems fell by 1.47% (1990-2002) with the N content going from 160,571 t in 1990 to 158,217 t in 2002. The liquid systems, on the other hand, increase considerably by 38.03%, going from 133,456 t in 1990 to 184,207 t in 2002.

The figure 7.4.8 shows the Pasture Range (IPCC 4D2) and Daily Spread as both as destinations of the N excreted by the animals.

FIGURE 7.4.8

Evolution of the emissions: The emissions of this group almost do not change during this period, going from 1,632 to 1,633 kt of CO2-eq between 1990 and 2002. This is because, despite the considerable increase in herd numbers, the distribution of manure treatment strategies has changed towards those with a lower emission factor, resulting in an almost non-existent variation.

7.5.- LAND USE CHANGES AND FORESTRY: NET UPTAKE OF CO2 BY THE FORESTRY BIOMASS

Two aspects are differentiated:

-	FOREST AREA calculation

-	MASS calculation

The forest area and its mass  in Spain have been estimated on the basis of the data used in the National Forestry Inventory (NFI) and, specifically for this work, data have been taken from the Second NFI for the provinces with data up to the middle of August 2003 from the Third NFI, as listed in the APPENDIX.

We estimate that the Second NFI takes the year 1990 as the base year for its production (2nd NFI, 1986-1995) and the Third NFI the year 2000 (3rd IFN, 1997-2006).

The calculation method accepts the relative error of less than 10% assumed in the National Forestry Inventory with a confidence factor of 95%.

The methods of calculation are given below in a simplified format (please refer to the APPENDIX for further details).

FOREST AREA CALCULATION

The data correspond to the provincial values, a consideration that must be taken into account in the calculations at the level of Region (called Autonomous Community in Spain) or at the National level, as the increase or decrease factors calculated are not the same for each province.

The FOREST AREA used for the calculation corresponds to the sum of the following uses referred to in the Second and Third National Forestry Inventories:

-	Forest Land: Land covered with trees with a Tree Crown Cover (Fracción de Cabida Cubierta or FCC) greater than or equal to 20%.

-	Thinly and sparse populated forests: Land covered with trees with a Tree Crown Cover (Fracción de Cabida Cubierta or FCC) between 5 and 20%.

PROVINCIAL FOREST AREAS

Taking the FOREST AREA in hectares as the starting point, a provincial level list is obtained with the following distribution:

1	List of the years following 1990

2	Increase in the forest area (ha): this is the difference between the forest areas for two consecutive years.

3	Forest Area (ha): from the figures given in the Second and Third National Forest Inventories, the annual increase or decrease factor (CIAS in its Spanish acronym) is calculated by comparing these figures, assuming a potential extension of the forest area over time. In those province where there is no value for the Third NFI, this variation factor has been estimated from the values of the provinces for which such data are available (please refer to the APPENDIX). Through this variation factor and starting from the forest area for 1990 given in the Second NFI, it is possible to obtain the surface areas for the remaining years.

The table 7.5.1 shows an example of these surfaces for the province of Cáceres.

TABLE 7.5.1
forest area for the province of cáceres

| YEAR | CÁCERES |  |
| --- | --- | --- |
|  | FOREST AREA INCREMENT(ha) | FOREST AREA (ha) |
| 1990 | 23.688 | 756.289 |
| 1991 | 24.430 | 779.977 |
| 1992 | 25.195 | 804.407 |
| 1993 | 25.985 | 829.603 |
| 1994 | 26.798 | 855.588 |
| 1995 | 27.638 | 882.386 |
| 1996 | 28.504 | 910.024 |
| 1997 | 29.396 | 938.527 |
| 1998 | 30.317 | 967.924 |
| 1999 | 31.267 | 998.241 |
| 2000 | 32.246 | 1.029.507 |
| 2001 | 33.256 | 1.061.753 |
| 2002 | 34.298 | 1.095.009 |

FOREST AREAS BY REGIONS AND AT NATIONAL SCALE

By Regions and at National level, we also obtain the table 7.5.2 with the following columns.

1	List of the years following 1990

2	Increase in the forest area (ha): this is the difference between the forest areas for two consecutive years.

3	Forest Area (ha): obtained from adding together the forest areas for the different provinces and regions respectively

table 7.5.2
forest area in extremadura region

| YEAR | EXTREMADURA |  |
| --- | --- | --- |
|  | FOERST AREA INCREMENT (ha) | FOREST AREA (ha) |
| 1990 | 40.740 | 1.457.591 |
| 1991 | 41.897 | 1.498.331 |
| 1992 | 43.087 | 1.540.228 |
| 1993 | 44.311 | 1.583.315 |
| 1994 | 45.570 | 1.627.626 |
| 1995 | 46.866 | 1.673.196 |
| 1996 | 48.199 | 1.720.063 |
| 1997 | 49.571 | 1.768.262 |
| 1998 | 50.982 | 1.817.833 |
| 1999 | 52.435 | 1.868.816 |
| 2000 | 53.929 | 1.921.250 |
| 2001 | 55.466 | 1.975.179 |
| 2002 | 57.047 | 2.030.645 |

FOREST AREA BY SPECIES AT NATIONAL SCALE

In order to complete the first column of table 7.5.3, the forest  area is obtained by species or group of species, calculating the percentage of forest area represented by each species or group of species vis-à-vis the total forest area for each year and assuming that this percentage is maintained for the other years.

The species or groups of species used have been taken from table 106 in the Second National Forestry Inventory, grouped together into the main species or groups (please refer to the Appendix).

table 7.5.3
forest area by species at national scale

| SPECIES | FOREST AREA 2ND NFI 1990 | % | FOREST AREA 1991 | .... | FOREST AREA 1998 | FOREST AREA 1999 | FOREST AREA 2000 | FOREST AREA 2001 | FOREST AREA 2002 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abies alba | 60.216,14 | 0,43 | 61.791 |  | 74.141 | 76.112 | 78.140 | 80.227 | 82.374 |
| Pinus halepensis | 1.544.689,52 | 11,11 | 1.585.091 |  | 1.901.885 | 1.952.459 | 2.004.489 | 2.058.019 | 2.113.097 |
| Pinus nigra | 565.589,79 | 4,07 | 580.383 |  | 696.377 | 714.895 | 733.946 | 753.546 | 773.713 |
| Pinus pinaster | 1.354.196,48 | 9,74 | 1.389.616 |  | 1.667.342 | 1.711.680 | 1.757.293 | 1.804.221 | 1.852.507 |
| Pinus pinea | 473.801,15 | 3,41 | 486.193 |  | 583.363 | 598.876 | 614.835 | 631.254 | 648.148 |
| Pinus radiata | 314.927,76 | 2,26 | 323.165 |  | 387.752 | 398.063 | 408.671 | 419.584 | 430.813 |
| Pinus sylvestris | 1.617.752,24 | 11,63 | 1.660.065 |  | 1.991.843 | 2.044.809 | 2.099.299 | 2.155.362 | 2.213.045 |
| Pinus uncinata | 52.273,80 | 0,38 | 53.641 |  | 64.362 | 66.073 | 67.834 | 69.645 | 71.509 |
| Other Conifers | 928.560,69 | 6,68 | 952.847 |  | 1.143.282 | 1.173.684 | 1.204.960 | 1.237.139 | 1.270.248 |
| Other Conifers | 6.912.007,57 | 49,71 | 7.092.792 |  | 8.510.348 | 8.736.651 | 8.969.466 | 9.208.997 | 9.455.453 |
| Castanea sativa | 83.497,18 | 0,60 | 85.681 |  | 102.805 | 105.539 | 108.351 | 111.245 | 114.222 |
| Fagus sylvatica | 311.115,22 | 2,24 | 319.252 |  | 383.058 | 393.244 | 403.723 | 414.505 | 425.598 |
| Quercus faginea | 247.653,29 | 1,78 | 254.131 |  | 304.921 | 313.029 | 321.371 | 329.953 | 338.784 |
| Quercus ilex | 1.384.912,60 | 9,96 | 1.421.135 |  | 1.705.161 | 1.750.504 | 1.797.152 | 1.845.145 | 1.894.526 |
| Quercus pyrenaica | 340.454,17 | 2,45 | 349.359 |  | 419.181 | 430.328 | 441.795 | 453.593 | 465.733 |
| Quercus robur | 188.101,67 | 1,35 | 193.021 |  | 231.599 | 237.757 | 244.093 | 250.611 | 257.318 |
| Quercus suber | 152.329,43 | 1,10 | 156.314 |  | 187.554 | 192.542 | 197.672 | 202.951 | 208.383 |
| Populus spp. | 53.767,98 | 0,39 | 55.174 |  | 66.201 | 67.962 | 69.773 | 71.636 | 73.553 |
| Eucalyptus spp. | 379.852,69 | 2,73 | 389.788 |  | 467.690 | 480.127 | 492.921 | 506.085 | 519.629 |
| Other Broadleaves | 3.850.967,81 | 27,70 | 3.951.690 |  | 4.741.470 | 4.867.553 | 4.997.264 | 5.130.716 | 5.268.028 |
| Other Broadleaves | 6.992.652,04 | 50,29 | 7.175.546 |  | 8.609.641 | 8.838.584 | 9.074.116 | 9.316.441 | 9.565.773 |
| TOTAL | 13.904.659,61 | 100,00 | 14.268.338 |  | 17.119.988 | 17.575.235 | 18.043.582 | 18.525.438 | 19.021.226 |

MASS CALCULATION

Just like the forest area calculation, the data correspond to provincial species values, a consideration that must be taken into account in the calculations at the level of Regions or at the National scale, as the calculated increase or decrease factors are not the same in each province.

The main variable used is the Volume Over Bark of the National Forestry Inventory, the volume convertible to lumber with bark in m3 or the trunk of the tree measured from the base at the breast heigh diameter of 1.3 m.

Taking the Volume Over Bark (m3) as the starting point, we obtain several tables by species and provinces, by species and regions and by species at the national scale; these tables are discussed below.

VOLUMES BY SPECIES AND PROVINCIES

Annual mass increase factor (CIAM): calculated by means of a comparison of the Second and Third National Forest Inventories (where no provincial data are available in the Third NFI, these are estimated from the other provinces calculated so far), given as m3/year (please refer to the APPENDIX).

Volume Over Bark of the years after 1990: calculated by multiplying the VCC and the annual increase factor (CIAM), verifying that it coincides with the figure from the Third NFI or 2000 (whenever this information is available).

table 7.5.4
volumen by species and provincies

| CÁCERES | Specie | ANNUAL MASS INCREMENT FACTOR (CIAM) | Volume Over Bark 2ND NFI (m3) | Volume Over Bark 1991 (m3) | ...... | Volume Over Bark 1999 (m3) | Volume Over Bark 2000 (m3) | Volume Over Bark 2001 (m3) | Volume Over Bark 2002 (m3) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Quercus pyrenaica | 1,069 | 1.531.763 | 1.638.857 |  | 2.814.083 | 3.010.831 | 3.221.335 | 3.446.556 |
|  | Quercus ilex | 1,069 | 3.066.081 | 3.278.414 |  | 5.601.505 | 5.989.422 | 6.404.202 | 6.847.706 |
|  | Quercus suber | 1,056 | 1.193.149 | 1.260.643 |  | 1.957.819 | 2.068.570 | 2.185.585 | 2.309.220 |
|  | Árboles fuera de monte (ribera arbolada) | 1,157 | 123.802 | 143.331 |  | 462.630 | 535.606 | 620.094 | 717.909 |
|  | Castanea sativa | 1,017 | 582.814 | 592.856 |  | 679.678 | 691.389 | 703.302 | 715.420 |
|  | Pinus pinaster | 1,055 | 3.287.936 | 3.471.033 |  | 5.354.778 | 5.652.972 | 5.967.772 | 6.300.101 |
|  | Pinus pinea | 1,097 | 116.636 | 128.048 |  | 270.206 | 296.644 | 325.669 | 357.533 |
|  | Eucalyptus camaldulensis | 1,025 | 530.990 | 544.632 |  | 667.173 | 684.314 | 701.895 | 719.928 |
|  | Other broadleaves | 1,100 | 83.751 | 92.195 |  | 198.809 | 218.853 | 240.918 | 265.208 |
|  | Pinus sylvestris/Pinus nigra | 0,10 | 29.321 | 26.389 |  | 2.932 | 0 | 0 | 0 |
|  | Juniperus communis/J. oxycedrus | 0,10 | 9.193 | 8.274 |  | 919 | 0 | 0 | 0 |
|  | TOTAL CACERES |  | 10.555.436 | 11.184.671 |  | 18.010.535 | 19.148.602 | 20.370.772 | 21.679.582 |

VOLUMES BY SPECIES AND REGIONS

Volume Over Bark for the years after 1990: where there are several provinces in a Region, the values of the Volume Over Bark for the species of the different provinces are added together and a table is obtained with the Volume Over Bark for the Region.

BIOMASS BY SPECIES AND REGIONS

Biomass: calculated as the product of the Volume Over Bark (m3) and the BEF*, measured in tonnes of dry matter per cubic metre (t ms/m3), obtaining tonnes of dry matter for the corresponding species in the community in question.

table 7.5.5
biomass by species in extremadura region

| BIOMASS EXTREMADURA | Specie | BEF (t ms/m3) | BIOMASS 1990 (t ms) | BIOMASS 1991 (t ms) | ..... | BIOMASS 1999 (t ms) | BIOMASS 2000 (t ms) | BIOMASS 2001 (t ms) |  | BIOMASS 2002 (t ms) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Quercus pyrenaica | 1,11 | 1.700.257 | 1.819.131 |  | 3.123.632 | 3.342.023 |  | 3.575.682 |  | 3.825.678 |  |
|  | Quercus ilex | 1,28 | 10.310.195 | 10.952.799 |  | 17.782.855 | 18.895.690 |  | 20.078.699 |  | 21.336.343 |  |
|  | Quercus suber | 1,28 | 2.825.345 | 2.951.621 |  | 4.210.498 | 4.404.627 |  | 4.608.385 |  | 4.822.273 |  |
|  | Wooded bank | 0,62 | 76.757 | 88.865 |  | 286.831 | 332.076 |  | 384.458 |  | 445.103 |  |
|  | Castanea sativa | 0,75 | 437.111 | 444.642 |  | 509.759 | 518.542 |  | 527.477 |  | 536.565 |  |
|  | Pinus pinaster | 0,55 | 2.267.235 | 2.390.412 |  | 3.650.756 | 3.849.321 |  | 4.058.711 |  | 4.279.518 |  |
|  | Pinus pinea | 0,73 | 516.433 | 553.262 |  | 964.369 | 1.034.357 |  | 1.109.582 |  | 1.190.451 |  |
|  | Eucalyptus camaldulensis | 0,81 | 1.184.472 | 1.205.203 |  | 1.386.523 | 1.411.266 |  | 1.436.505 |  | 1.462.251 |  |
|  | Other broadleaves | 0,80 | 183.841 | 204.143 |  | 472.650 | 525.046 |  | 583.275 |  | 647.988 |  |
|  | Pinus sylvestris/Pinus nigra | 0,62 | 18.179 | 16.361 |  | 1.818 | 0 |  | 0 |  | 0 |  |
|  | Juniperus communis/J. oxycedrus | 0,80 | 7.354 | 6.619 |  | 735 | 0 |  | 0 |  | 0 |  |
|  | TOTAL |  | 19.527.180 | 20.633.059 |  | 32.390.426 | 34.312.947 |  | 36.362.773 |  | 38.546.171 |  |

*: BEF: Biomass expansion factor  (from VCC to total biomass for the tree); Source: CREAF, Centro de Investigación Ecológica y Aplicaciones Forestales (Environmental and Forestry Applications Research Centre). For the species which do not appear in the CREAF list, BEF values have been assigned by similarity between species, or the value of 0.8 given by the IPCC in “Revised 1996 IPCC Guidelines for National Greenhouse Gas Inventories”.

CARBON BY SPECIES AND REGIONS

Tonnes of Carbon assimilated by the biomass of a species in a given community, obtained by using the factor of 0.45 for converting dry matter to carbon, as given by the IPCC in “Revised 1996 IPCC Guidelines for National Greenhouse Gas Inventories”.

CO2 BY SPECIES AND REGIONS

The value of 44/12* (Revised 1996 IPCC Guidelines for National Greenhouse Gas Inventories) is multiplied by the figure for tonnes of carbon to give the tonnes of CO2 estimated as being in the mass of woodlands for a particular specie in a given Region.

7.6.- WASTES

SECTOR OVERVIEW

The emissions corresponding to the treatment and disposal of waste in 2002 have been estimated to be 15,666 kilotonnes of CO2 equivalent (CO2-eq), a figure representing 3.9% of the CO2-eq emissions of the entire inventory that year; the relative contribution has increased with respect to the equivalent figure for 1990, when this percentage was 3.3%. The main pollutant emitted in this sector is methane, which represented, in 2002, 35% of the total emissions of this gas according to the inventory, and secondly, a long way behind, nitrous oxide, for which the waste sector’s contribution has been 4.2% of the inventory total in 2002. In terms of the distribution among the different sources of emissions (disposal of municipal solid waste (MSW) in landfills, waste water treatment, incineration of waste, and others), the pie charts below, referring to 1990 and 2002, clearly show the dominance of the disposal of MSW in landfills, followed by waste water treatment.

FIGURe 7.6.1
EVOLUtion of CO2 EQUIVALENT emissions
(years 1990 and 2002)

The most important source of basic information is the annual publication entitled “Environment in Spain” from the Ministry of the Environment. The general methodological sources used have essentially been the IPCC Guidelines (Reference Manual for the 1996 Revised Guidelines, and the 2000 Good Practice Guide) and the EMEP/CORINAIR Handbook.

In the waste sector, the coverage of the activities is almost complete, except for the incineration of industrial waste which, because of the difficulty in obtaining information on this activity, has not been included in this edition of the inventory.

KEY SOURCES ANALYSIS

The analysis of the emissions for the period 1990-2002 has allowed the identification of the following key sources: i) Disposal of MSW in landfills, ii) Waste water treatment. As shown in the table below, the first of these activities contributes as a key source to the level and to the trend, while the second contributes only to the level.

TABLE 7.6.1
KEY SOURCES: LEVEL AND TREND CONTRIBUTION

| Code | Description | Gas | CO2-e (kt) | Contribution Level (2002) |  |  | Contribution Trend |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (2002) | % | Key S. | Range | % | Key S. | Range |
| 6A | Disposal in landfills | CH4 | 10.892 | 2.7 | YES | 10 | 2.5 | YES | 10 |
| 6B | Waste water treatment | CH4 | 3.519 | 0.6 | YES | 23 | 0.2 | NO | 34 |

- 6A. Disposal of MSW in landfills. CH4 Emissions.

The disposal of MSW in landfills is the main system for treatment municipal solid waste in Spain, with a percentage, in 2002, of 61% of the total of MSW generated. Other treatment methods used include: dumping in unmanaged sites (4%), composting (22%), selective collection (8%), and incineration (5%).

In terms of management, a distinction may be drawn between controlled and uncontrolled landfill sites. In the case of controlled sites, there may also be systems for the capture and energy recovery of the biogas produced, implying in turn lower methane emission rates. In the case of uncontrolled sites, part of the mass is burnt, in order to reduce the volume, and in this case, apart from the biogas emissions from the unburnt MSW fraction, there are also emissions corresponding to the combustion of the fraction burnt.

The methodology applied for the estimation of emissions at controlled landfills and the unburnt fraction of uncontrolled ones has been the first order kinetic methodology proposed in the Reference Manual and the IPCC Good Practices Guidelines. The parameters of the algorithm for emissions estimation are those in the Reference Manual except for the methane conversion factor parameter (MCF) for uncontrolled landfill sites, because of an ignorance of the depth of the MSW deposited there (for the inventory, it has been assumed that 50% of the uncontrolled disposals are deep, i.e. more than 5 metres, using an MCF value of 0.8 and that the other 50% is shallow, i.e. less than 5 metres, associated with an MCF value of 0.4. The data source for characterization and quantification of the waste has been the annual publication entitled “Environment in Spain” from the Ministry of the Environment.

In the case of uncontrolled dumping, the estimation of the emissions from the burnt fraction has been effected by multiplying the activity variable by the corresponding emission factors. Of the total waste burnt in uncontrolled dumpsites, it has been assumed that 85% are of renewable organic origin and 15% are of fossil origin. The value of the CO2 emission factor for the fossil fraction of the burnt waste is estimated at 2,933 g CO2/tonne of fossil fraction of burnt waste. This value is reached by assuming a combustion performance percentage of 80% (thus 2,933 = 0.8 * 1,000 * 44/12). For SO2, NOX, N2O, heavy metals and persistent organic pollutants, the same factors have been taken as for the incineration of MSW. For NMVOC, CH4 and CO, the basic information comes from the CORINAIR Manual (1992).

- 6B. Treatment of industrial and residential-commercial waste water: CH4 Emissions.

This source category includes the treatment of waste water from industrial origins as well as residential-commercial origin.

For the treatment of industrial waste water, the methodology followed has been derived from the IPCC Reference Manual for the area sources (general statistic information) and the EMEP/CORINAIR Guidebook for the point sources (sectorial questionnaires). The activity variable taken for point sources has been the volume of waste water purified in the oil refineries and paper pulp works, where the information has been obtained through questionnaires, and for area sources it has been the organic load in terms of chemical oxygen demand in water and sludge, with the basic variables coming from the discharge regulation studies carried out by the Directorate General for Hydraulic Works and Water Quality at the Ministry of the Environment for the food sectors in 1994 and chemistry sector in 1996. In order to extend the time series homogeneously for production (food) or consumption (chemistry), the industrial production index (IPI) produced by the Spanish National Statistics Institute (INE in its Spanish acronym) was used. The emissions on the water and sludge lines are obtained as the product of the degradable organic load (water and sludge) from the methane emission factors, discounting the amount of methane recovered from this product. The values of the parameters needed by the algorithm referred to in the IPCC Guidelines have been as follows: the maximum capacity for methane production B0 = 0.25 (kg CH4/kg COD), methane conversion factors (MCF) from the water line MCFW = 0.15 and from the sludge line MCFS = 0.3, and the fraction of waste (water and sludge) for each type of treatment WS = 0.67 and SS = 0.5.

For the treatment of waste water in the residential and commercial sectors, the methodology used has been derived from the IPCC Reference Manual and the EMEP/CORINAIR Guidelines. The activity variable taken has been the organic load in terms of tonnes of BOD5. To calculate this variable, the datum used has been the population effectively served by the residential waste water treatment plants, as shown in the publication entitled “Environment in Spain” from the Ministry of the Environment. For the degradable organic load, a burden of 325 mg BOD5/litre of waste water and a flow of 250 litres/inhabitant equivalent per day has been assumed, and 365 days of operation a year (29.66 kg BOD5/inhabitant equivalent per year) and 0.75 as the fraction for the degradable organic load. The emissions on the water and sludge lines are obtained as the product of the activity variable by the methane emission factors, discounting from this result the amount of methane recovered. The values assumed for the parameters needed by the algorithm referred to in the IPCC Guidelines have been as follows: the maximum capacity for methane production B0 = 0.6 (kg CH4/kg BOD5), methane conversion factors (MCF) from the water line MCFW = 0.005 and from the sludge line MCFS = 0.3 and the fraction of waste (water and sludge) for each type of treatment WS = 1 and SS = 0.85.

OTHER SOURCES ANALYSIS

It is of interest to highlight the following non-key sources generating greenhouse gases estimated in this sector, among others: the consumption of proteins (as emissions of nitrous oxide), and the incineration of MSW.

The methodology followed for the estimation of nitrous oxide emissions is that of the IPCC Reference Manual. Protein consumption has been obtained from the publication entitled “Food in Spain (La alimentación en España)” from the Spanish Ministry of Agriculture, Fisheries and Food (MAPA). The values of the paramaters required by the algorithm for the emissions estimation are those proposed by Reference Manual IPCC: the nitrogen fraction in the protein is 0.16 kg N/kg protein and the emission factor is 0.01 kg N2O-N/kg N in the sewage waters.

The amount of municipal solid waste entering the incineration process in all of the incinerators in operation without energy recovery was obtained from the publication “The Environment in Spain”. The information of the emission factors has been taken assuming that the control technique used is the one for “control of particles”. The reference sources for the emission factors are the following: For SO2, NOX, VOC, CO, N2O, NH3  the EMEP/CORINAIR Guide Book, in the case of the VOC, 95% of NMVOC and 5% of CH4 have been assumed. For the CO2 a factor of 324 kg/ton has been assumed, calculated assuming 36% of fossil origin and 64% of biogenic origin in the waste and considering that the overall factor for CO2 per ton of waste is 900 kg (fossil + biogenic)/ton.

The values stated for the CO2 have been deduced by the working team for the Inventory based on the data for the composition of the waste. For the heavy metals, PCB and HCB, the information has been taken from the PARCOM-ATMOS Manual; nevertheless, PCB and HCB have been corrected since it is understood that they are mistaken and that the units in which they were expressed were also incorrect, all of which is reached after a comparison with the reference “Compilation of Emission Factors for POPs, a case study of Czech and Slovak Republics”. The EMEP/CORINAIR Guide Book has also been used for the DIOX and for the PAH as the sum of the corresponding factors of benzo[bk]fluoranthene, value 6.3 mg/t, and of the benzo[a]pyrene, value 0.7 mg/t.

##### APPENDIX
methodology for carbon dioxide estimation in the forestry sector

This Appendix comprises the following sections:

1)	List of the provinces with data in the Second and Third National  Forest Inventories until the date of calculation (August, 2003).

2)	Method for calculating the forest area:

-	Calculation of the annual forest area increase factors (CIAS)

3)	Method for calculating the mass of forests:

-	Calculation of the annual mass increase factors (CIAM)

-	Calculation of the volume over bark values when there are no species data in the 2nd NFI or in the 3rd NFI

4)	Biomass Expansion Factors (BEFs).

5)	Groupings of species or species groups in table 106 of the Second National Forest Inventory into the main species or groups of species used.

6)	Methodology for calculating the carbon emissions.

1)	List of the provinces with data in the Second and Third NATIONAL Forest Inventories until the date of calculation (August, 2003).

| REGIONS | PROVINCE | 2ND NFI DATA | 3RD NFI |
| --- | --- | --- | --- |
| PAÍS VASCO | ALAVA | YES | ASSESSED |
|  | GUIPUZCOA | YES | ASSESSED |
|  | VIZCAYA | YES | ASSESSED |
| CATALUÑA | BARCELONA | YES | YES |
|  | GIRONA | YES | YES |
|  | LLEIDA | YES | YES |
|  | TARRAGONA | YES | YES |
| GALICIA | CORUÑA | YES | YES |
|  | LUGO | YES | YES |
|  | ORENSE | YES | YES |
|  | PONTEVEDRA | YES | YES |
| ANDALUCIA | ALMERIA | YES | ASSESSED |
|  | CADIZ | YES | ASSESSED |
|  | CORDOBA | YES | ASSESSED |
|  | GRANADA | YES | ASSESSED |
|  | HUELVA | YES | ASSESSED |
|  | JAEN | YES | ASSESSED |
|  | MALAGA | YES | ASSESSED |
|  | SEVILLA | YES | ASSESSED |
| PRINCIPADO DE ASTURIAS | PRINCIPADO DE ASTURIAS | YES | YES |
| CANTABRIA | CANTABRIA | YES | YES |
| LA RIOJA | LA RIOJA | YES | YES |
| REGION DE MURCIA | MURCIA | YES | YES |
| COMUNIDAD VALENCIANA | ALICANTE | YES | ASSESSED |
|  | CASTELLON | YES | ASSESSED |
|  | VALENCIA | YES | ASSESSED |
| ARAGON | HUESCA | YES | ASSESSED |
|  | TERUEL | YES | ASSESSED |
|  | ZARAGOZA | YES | ASSESSED |
| CASTILLA LA MANCHA | ALBACETE | YES | ASSESSED |
|  | CIUDAD REAL | YES | ASSESSED |
|  | CUENCA | YES | ASSESSED |
|  | GUADALAJARA | YES | ASSESSED |
|  | TOLEDO | YES | ASSESSED |
| CANARIAS | LAS PALMAS | YES | ASSESSED |
|  | STA CRUZ DE TENERIFE | YES | ASSESSED |
| COMUNIDAD FORAL DE NAVARRA | NAVARRA | YES | YES |
| EXTREMADURA | CACERES | YES | YES |
|  | BADAJOZ | YES | YES |
| ISLAS BALEARES | BALEARES | YES | YES |
| COMUNIDAD DE MADRID | MADRID | YES | YES |
| CASTILLA Y LEON | AVILA | YES | YES |
|  | BURGOS | YES | ASSESSED |
|  | LEON | YES | ASSESSED |
|  | PALENCIA | YES | ASSESSED |
|  | SALAMANCA | YES | YES |
|  | SEGOVIA | YES | ASSESSED |
|  | SORIA | YES | ASSESSED |
|  | VALLADOLID | YES | ASSESSED |
|  | ZAMORA | YES | ASSESSED |

2)	METHOD FOR CALCULATING THE FOREST AREA

CALCULATION OF THE ANNUAL FOREST AREA INCREASE FACTORS (CIAS)

The calculations have been made at the provincial level (very important; the values for each province are different, so it must be remembered that obtaining the same factors at the Region or National scale will result in values that will not coincide with those obtained at the provincial scale).

Two different calculation situations may arise:

1	When data are available in the Second and Third NFI:

Assuming a potential increase in the forest area (SUP), the formula applied is:

CIAS = (SUP2000/SUP1990)1/10

| PROVINCE | FOREST AREA 2ND NFI (ha) | FOREST AREA 3RD NFI (ha) | CIAS |
| --- | --- | --- | --- |
| BARCELONA | 424.187 | 470.665 | 1,010 |
| GIRONA | 326.507 | 365.930 | 1,011 |
| LLEIDA | 450.294 | 537.883 | 1,017 |
| TARRAGONA | 193.086 | 252.587 | 1,027 |
| CORUÑA | 344.163 | 401.189 | 1,015 |
| LUGO | 368.576 | 463.819 | 1,023 |
| ORENSE | 198.249 | 319.627 | 1,048 |
| PONTEVEDRA | 134.388 | 220.817 | 1,050 |
| PRINCIPADO DE ASTURIAS | 368.129 | 451.116 | 1,020 |
| CANTABRIA | 165.543 | 214.257 | 1,026 |
| LA RIOJA | 128.917 | 169.552 | 1,027 |
| MURCIA | 269.278 | 316.292 | 1,016 |
| NAVARRA | 372.467 | 462.664 | 1,021 |
| CACERES | 756.289 | 1.029.507 | 1,031 |
| BADAJOZ | 701.302 | 891.743 | 1,024 |
| BALEARES | 122.475 | 186.377 | 1,042 |
| MADRID | 195.465 | 270.086 | 1,032 |
| AVILA | 158.131 | 239.307 | 1,042 |
| SALAMANCA | 330.888 | 533.384 | 1,048 |

The way this formula is obtained is similar to that for the mass increase factor (please refer to Calculation of the Annual Mass Increase Factors).

2	When there are data in the Second NFI, but not in the Third:

Starting from the table of surface areas for the Provinces and Regions taken in both inventories, the sum total of these is obtained and the formula in the section above is applied to the result:

CIAS = ( Σ SUP2000/ Σ SUP1990)1/10

| PROVINCES WITH 3RD NFI | FOREST AREA 2ND NFI (ha) | FOERST AREA 3RD NFI (ha) |  |
| --- | --- | --- | --- |
| BARCELONA | 424.187 | 470.665 |  |
| GIRONA | 326.507 | 365.930 |  |
| LLEIDA | 450.294 | 537.883 |  |
| TARRAGONA | 193.086 | 252.587 |  |
| CORUÑA | 344.163 | 401.189 |  |
| LUGO | 368.576 | 463.819 |  |
| ORENSE | 198.249 | 319.627 |  |
| PONTEVEDRA | 134.388 | 220.817 |  |
| PRINCIPADO DE ASTURIAS | 368.129 | 451.116 |  |
| CANTABRIA | 165.543 | 214.257 |  |
| LA RIOJA | 128.917 | 169.552 |  |
| MURCIA | 269.278 | 316.292 |  |
| NAVARRA | 372.467 | 462.664 |  |
| CACERES | 756.289 | 1.029.507 |  |
| BADAJOZ | 701.302 | 891.743 |  |
| BALEARES | 122.475 | 186.377 |  |
| MADRID | 195.465 | 270.086 |  |
| AVILA | 158.131 | 239.307 |  |
| SALAMANCA | 330.888 | 533.384 | CIAS |
| TOTAL | 6.008.334 | 7.796.803 | 1,026 |

In this case, the factor so calculated and the value of 1.026 will be applied to all of the provinces not yet completed in the Third National Forest Inventory.

| PROVINCES | FOREST AREA 2ND NFI (ha) | FOERST AREA 3RD NFI (ha) | INCREMENT FACTOR |
| --- | --- | --- | --- |
| ALAVA | 143.506 | 186.223 | 1,026 |
| GUIPUZCOA | 118.255 | 153.455 | 1,026 |
| VIZCAYA | 128.244 | 166.418 | 1,026 |
| BARCELONA | 424.187 | 470.665 | 1,010 |
| GIRONA | 326.507 | 365.930 | 1,011 |
| LLEIDA | 450.294 | 537.883 | 1,017 |
| TARRAGONA | 193.086 | 252.587 | 1,027 |
| CORUÑA | 344.163 | 401.189 | 1,015 |
| LUGO | 368.576 | 463.819 | 1,023 |
| ORENSE | 198.249 | 319.627 | 1,048 |
| PONTEVEDRA | 134.388 | 220.817 | 1,050 |
| ALMERIA | 101.849 | 132.166 | 1,026 |
| CADIZ | 167.675 | 217.586 | 1,026 |
| CORDOBA | 371.634 | 482.256 | 1,026 |
| GRANADA | 221.724 | 287.723 | 1,026 |
| HUELVA | 539.922 | 700.638 | 1,026 |
| JAEN | 311.128 | 403.740 | 1,026 |
| MALAGA | 115.728 | 150.176 | 1,026 |
| SEVILLA | 276.592 | 358.923 | 1,026 |
| PRINCIPADO DE ASTURIAS | 368.129 | 451.116 | 1,020 |
| CANTABRIA | 165.543 | 214.257 | 1,026 |
| LA RIOJA | 128.917 | 169.552 | 1,027 |
| MURCIA | 269.278 | 316.292 | 1,016 |
| ALICANTE | 88.407 | 114.723 | 1,026 |
| CASTELLON | 197.991 | 256.925 | 1,026 |
| VALENCIA | 341.882 | 443.648 | 1,026 |
| HUESCA | 458.657 | 595.182 | 1,026 |
| TERUEL | 458.518 | 595.002 | 1,026 |
| ZARAGOZA | 268.357 | 348.238 | 1,026 |
| ALBACETE | 345.239 | 448.004 | 1,026 |
| CIUDAD REAL | 338.818 | 439.672 | 1,026 |
| CUENCA | 564.190 | 732.130 | 1,026 |
| GUADALAJARA | 431.360 | 559.760 | 1,026 |
| TOLEDO | 171.615 | 222.698 | 1,026 |
| LAS PALMAS | 17.578 | 22.811 | 1,026 |
| STA CRUZ DE TENERIFE | 87.336 | 113.333 | 1,026 |
| NAVARRA | 372.467 | 462.664 | 1,021 |
| CACERES | 756.289 | 1.029.507 | 1,031 |
| BADAJOZ | 701.302 | 891.743 | 1,024 |
| BALEARES | 122.475 | 186.377 | 1,042 |
| MADRID | 195.465 | 270.086 | 1,032 |
| AVILA | 158.131 | 239.307 | 1,042 |
| BURGOS | 394.213 | 511.557 | 1,026 |
| LEON | 269.209 | 349.343 | 1,026 |
| PALENCIA | 128.238 | 166.409 | 1,026 |
| SALAMANCA | 330.888 | 533.384 | 1,048 |
| SEGOVIA | 195.984 | 254.321 | 1,026 |
| SORIA | 353.582 | 458.830 | 1,026 |
| VALLADOLID | 111.600 | 144.819 | 1,026 |

Obtaining the forest area for the year 2000 using the formula:

SUP2000 = SUP1990 x (CIAS) 10

3)	METHOD FOR CALCULATING THE MASS OF WOODLAND

CALCULATION OF THE ANNUAL MASS INCREASE FACTORS (CIAM)

These calculations are effected at the provincial level by species (very important; the values for each province and species are different, so it must be remembered that obtaining the same factors at the Region or National scales will result in values that will not coincide with those obtained at the provincial scale).

Two different calculation situations may arise:

1	When data are available in the Second and Third NFI:

The formula applied is:

CIAM = (VCC2000 / VCC1990)1/10

This formula is obtained as follows:

First, the 10-year increase factor is obtained as the quotient of the volume convertible to lumber with bark in 2000 over the same concept in 1990.

VCC2000 / VCC1990

In order to obtain the annual increase, as we assume that this growth has not been constant, we apply the following equations:

VCC1990 x CIAM = VCC1991;

VCC1991 x CIAM = VCC1992;		VCC1991 = VCC1992 / CIAM

…

VCC1999 x CIAM = VCC2000;

As:

VCC1990 x CIAM = VCC1991 = VCC1992 / CIAM;

VCC1990 x (CIAM)2 = VCC1992

Therefore:

VCC1990 x (CIAM)10 = VCC2000;	(CIAM)10 = (VCC2000/VCC1990);

and we obtain:

CIAM = (VCC2000/VCC1990) 1/10

2	When there are data in the Second NFI, but not in the Third:

| Regions and porvincies 3rd NFI | Volume Over Bark 1990 | Volume Over Bark 2000 | INCREMENT FACTOR (VCC/año) |
| --- | --- | --- | --- |
| ASTURIAS | 32.577.252 | 47.300.541 | 1,037 |
| BALEARES | 5.451.407 | 7.525.457 | 1,032 |
| CANTABRIA | 19.309.317 | 25.206.929 | 1,027 |
| CATALUÑA | 80.040.753 | 118.288.352 | 1,039 |
| CL-AVILA | 7.389.753 | 12.057.318 | 1,050 |
| CL-SALAMANCA | 6.433.118 | 11.202.914 | 1,057 |
| EXTEMADURA | 19.060.835 | 33.255.502 | 1,057 |
| GALICIA | 90.397.509 | 133.092.753 | 1,039 |
| LA RIOJA | 9.569.894 | 15.516.950 | 1,049 |
| MADRID | 6.798.915 | 10.895.345 | 1,048 |
| MURCIA | 3.142.054 | 6.919.544 | 1,082 |
| NAVARRA | 45.349.066 | 54.651.039 | 1,018 |
| TOTAL | 325.519.873 | 475.912.643 | 1,038 |

From the table with the Volumes Over Bark for the provinces and Regions taken in both inventories, the total sum of these is obtained and the formula in the section above is applied to the result:

CIAM = (Σ VCC2000 / Σ VCC1990)1/10

In this case, the factor so calculated and the value of 1.038 will be applied to all of the species in the provinces not yet completed in the Third NFI.

| Regions and porvincies 3rd NFI | Volume Over Bark 1990 | Volume Over Bark 2000 |
| --- | --- | --- |
| ANDALUCÍA | 40.794.494 | 59.641.875 |
| ARAGÓN | 44.622.769 | 65.238.843 |
| ASTURIAS | 32.577.252 | 47.300.541 |
| BALEARES | 5.451.407 | 7.525.457 |
| CANARIAS | 9.452.863 | 13.820.161 |
| CANTABRIA | 19.309.317 | 25.206.929 |
| CASTILLA LA MANCHA | 49.524.458 | 72.405.151 |
| CASTILLA LEÓN incompleto | 72.854.262 | 106.513.510 |
| CATALUÑA | 80.040.753 | 118.288.352 |
| CL-AVILA | 7.389.753 | 12.057.318 |
| CL-SALAMANCA | 6.433.118 | 11.202.914 |
| EXTEMADURA | 19.060.835 | 33.255.502 |
| GALICIA | 90.397.509 | 133.092.753 |
| LA RIOJA | 9.569.894 | 15.516.950 |
| MADRID | 6.798.915 | 10.895.345 |
| MURCIA | 3.142.054 | 6.919.544 |
| NAVARRA | 45.349.066 | 54.651.039 |
| PAÍS VASCO | 41.589.090 | 60.803.580 |
| VALENCIA | 10.946.126 | 16.003.323 |
| TOTAL ESPAÑA | 595.303.935 | 870.339.087 |

Obtained from the formula:

VCC2000 = VCC1990 x (CIAM) 10

CALCULATION OF THE VCC VALUES WHEN THERE ARE NO SPECIES DATA IN THE IFN2 OR IN THE IFN3

There are SItuations in certain species or groups of species for which the volume was calculated in one of the inventories but not in the other, so it is not possible to obtain a potential annual mass increase factor by comparison of the inventories. In these cases a constant increase or decrease in volume over time is assumed and the calculation is as follows:

-	where the species were not included in 3rd NFI but it was in 2nd NFI values will be given by the following equations:

VCC(YEAR) = VCC(PREVIOUS YEAR) – (VCC1990/10)

Example: in the province of Cáceres in 2nd NFI the following groups of species were included but are not referred to in 3rd NFI: Pinus sylvestris/Pinus nigra and Juniperus communis/J. oxycedrus, therefore a constant reduction is assumed for these species.

| CÁCERES | Speciee | COEF. OF THE ANUAL INCREASE | VCC IFN2 (m3) | VCC 1991 (m3) | .... | VCC 1999 (m3) | VCC 2000 (m3) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pinus sylvestris/ Pinus nigra | 0,10 | 29.321 | 26.389 |  | 2.932 | 0 |
|  | Juniperus communis/ J. oxycedrus | 0,10 | 9.193 | 8.274 |  | 919 | 0 |
|  | TOTAL CACERES |  | 10.555.436 | 11.184.671 |  | 18.010.535 | 19.148.602 |

-	and where the species in question have not been counted in IFN2 and appears in IFN3.

VCC(YEAR) = VCC(PREVIOUS YEAR) – (VCC2000/10)

As an example of this contrary case, we can cite the Acacia spp. in the province of Pontevedra.

| PONTEVE-DRA | Especie | COEF. OF THE ANUAL INCREMENT | VCC IFN2 (m3) | VCC 1991 (m3) | .... | VCC 1999 (m3) | VCC 2000 (m3) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Acacia spp. | 0,10 | 0 | 24.997 |  | 224.975 | 249.972 |
|  | TOTAL PONTEVEDRA |  | 14.576.056 | 15.250.626 |  | 22.294.825 | 23.435.637 |

4)	BIOMASS EXPANSION FACTORS (BEFs)

|  |  |  |  |  |  | TREE SPECIES |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CREAF values |  |  |  |  | Ref. Flora europaea |  |  |
|  | values by comparison with other species |  |  |  |  |  | BROADLEAVESS |  |
|  | Good practice guideline values (1.6*0.5=0.8) |  |  |  |  |  | CONIFERS |  |
| SPECIE | BEF | COMPARACION SPECIE |  |  | SPECIE | BEF | COMPARACION SPECIE |  |
| Acacia spp. | 0,80 |  |  |  | Abies alba | 0,61 |  |  |
| Acer spp. | 0,90 | Ulmus |  |  | Abies pinsapo | 0,61 | Abies alba |  |
| Alnus glutinosa | 0,62 |  |  |  | Cedrus sp. | 0,55 | P pinaster |  |
| Árboles fuera de monte (ribera arb. ) | 0,62 | Alnus |  |  | Chamaecyparis lawsoniana | 0,44 | P radiata |  |
| Arboles ripícolas | 0,62 | Alnus |  |  | Coníferas alóctonas | 0,44 | P radiata |  |
| Arbutus unedo | 0,80 |  |  |  | Coníferas, excepto pinos y abetos | 0,64 | P nigra |  |
| Betula spp. | 0,73 |  |  |  | Cupressus sp. | 0,55 | P pinaster |  |
| Castanea sativa | 0,75 |  |  |  | Juniperus spp. | 0,80 |  |  |
| Ceratonia SIliqua | 1,28 | Q ilex |  |  | Larix spp. | 0,64 | P nigra |  |
| Corylus avellana | 0,80 |  |  |  | Otras coníferas | 0,64 | P nigra |  |
| Crataegus spp. | 0,80 |  |  |  | Otros pinos | 0,64 | P nigra |  |
| Erica spp. | 0,80 |  |  |  | Picea abies | 0,44 | P radiata |  |
| Eucalyptus spp. | 0,81 |  |  |  | Pinus canarienSIs | 0,55 | P pinaster |  |
| Fagus sylvatica | 0,81 |  |  |  | Pinus halepenSIs | 0,74 |  |  |
| Fraxinus spp. | 0,83 |  |  |  | Pinus nigra | 0,64 |  |  |
| Ilex spp. | 0,80 |  |  |  | Pinus pinaster | 0,55 |  |  |
| Laurus azorica | 0,80 |  |  |  | Pinus pinea | 0,73 |  |  |
| Mezcla de árboles de ribera | 0,62 | Alnus |  |  | Pinus radiata | 0,44 |  |  |
| Mezcla de pequeñas frondosas | 0,80 |  |  |  | Pinus sylvestris | 0,62 |  |  |
| Myrica faya | 0,80 |  |  |  | Pinus uncinata | 0,61 |  |  |
| Olea europaea | 1,28 | Q ilex |  |  | Pseudotsuga menziesSIi | 0,44 | P radiata |  |
| Otras especies | 0,80 |  |  |  | Sabinares/enebrales | 0,80 |  |  |
| Otras frondosas | 0,80 |  |  |  |  |  |  |  |
| Otras lauriSIlvas | 0,80 |  |  |  |  |  |  |  |
| Persea indica | 0,80 |  |  |  |  |  |  |  |
| Phillyrea latifolia | 0,80 |  |  |  |  |  |  |  |
| Phoenix canarienSIs | 0,80 |  |  |  |  |  |  |  |
| Platanus spp. | 0,90 | Ulmus |  |  |  |  |  |  |
| Populus sp. | 0,62 | Alnus |  |  |  |  |  |  |
| Prunus spp. | 0,80 |  |  |  |  |  |  |  |
| Quercus canarienSIs | 1,00 |  |  |  |  |  |  |  |
| Quercus faginea | 1,11 |  |  |  |  |  |  |  |
| Quercus ilex | 1,28 |  |  |  |  |  |  |  |
| Quercus petraea | 0,84 |  |  |  |  |  |  |  |
| Quercus pubescens (Q. humilis) | 0,89 |  |  |  |  |  |  |  |
| Quercus pyrenaica | 1,11 | Q faginea |  |  |  |  |  |  |
| Quercus robur | 0,84 | Q petraea |  |  |  |  |  |  |
| Quercus rubra | 0,80 |  |  |  |  |  |  |  |
| Quercus suber | 1,28 | Q ilex |  |  |  |  |  |  |
| Robinia pseudacacia. Sophora japonica | 0,80 |  |  |  |  |  |  |  |
| Robinia pseudacacia. Sophora japonica. GleditSIa triacanthos. | 0,80 |  |  |  | SOURCES: |  |  |  |
| Salix spp. | 0,80 |  |  |  | Revised 1996 IPCC Guidelines for National Greenhouse Gas Inventories |  |  |  |
| Sorbus spp. | 0,80 |  |  |  | National Forestry Inventory (DGCN; Ministry of the Environment) |  |  |  |
| Tilia spp. | 0,80 | Ulmus |  |  | Environmental and Forestry Applications Research Centre (CREAF) |  |  |  |
| Ulmus spp. | 0,90 |  |  |  | Preparation of the SPCAN according to IFN data |  |  |  |

5)	GROUPINGS OF SPECIES OR SPECIES GROUPS IN TABLE 106 OF THE SECOND FOREST INVENTORY INTO THE MAIN SPECIES OR GROUPS OF SPECIES USED.

| ESPECIES O GRUPOS DE ESPECIES TABLA 106 IFN2 | SUPERFICIE (ha) IFN2 (1990) | ESPECIES O GRUPOS DE ESPECIES EMPLEADAS |
| --- | --- | --- |
| Coníferas exóticas alóctonas | 35.159,44 | Resto Coniferas |
| Pinus sylvestris | 678.685,53 | Pinus sylvestris |
| Pinus uncinata | 52.273,80 | Pinus uncinata |
| Pinus pinea | 172.858,58 | Pinus pinea |
| Pinus halepenSIs | 1.046.978,45 | Pinus halepenSIs |
| Pinus nigra | 405.982,54 | Pinus nigra |
| Pinus pinaster cantábrico | 285.046,52 | Pinus pinaster |
| Pinus pinaster meditteráneo | 723.819,86 | Pinus pinaster |
| Pinus canarienSIs | 69.644,11 | Resto Coniferas |
| Pinus radiata | 174.229,16 | Pinus radiata |
| Abies alba | 60.216,14 | Abies alba |
| Abies pinsapo | 2.307,57 | Resto Coniferas |
| Juniperus thurifera | 75.203,25 | Resto Coniferas |
| Quercus robur, Q petraea | 99.550,70 | Quercus robur |
| Quercus pyrenaica | 263.472,66 | Quercus pyrenaica |
| Quercus faginea | 51.925,24 | Quercus faginea |
| Quercus ilex | 823.447,26 | Quercus ilex |
| Quercus suber | 104.254,92 | Quercus suber |
| Árboles de ribera | 36.087,58 | Resto Frondosas |
| Populus nigra, P.* canadenSIs | 53.767,98 | Populus spp. |
| Eucalyptus spp, mediterráneo | 293.988,41 | Eucalyptus spp. |
| Eucalyptus spp, cantábrico | 85.864,28 | Eucalyptus spp. |
| Olea europaea | 16.997,65 | Resto Frondosas |
| Fagus sylvatica | 311.115,22 | Fagus sylvatica |
| Castanea sativa | 83.497,18 | Castanea sativa |
| Pinus sylvestris y Pinus uncinata | 68.406,31 | Pinus sylvestris |
| Pinus sylvestris y Pinus nigra | 616.245,73 | Pinus sylvestris |
| Pinus sylvestris y Pinus pinaster | 85.675,19 | Pinus sylvestris |
| Pinus sylvestris y frondosas | 168.739,48 | Pinus sylvestris |
| Pinus pinea y Pinus pinaster | 170.712,83 | Pinus pinea |
| Pinus pinea y frondosas | 130.229,74 | Pinus pinea |
| Pinus halepenSIs y Pinus nigra | 229.678,56 | Pinus halepenSIs |
| Pinus halepenSIs y Pinus pinaster | 183.184,66 | Pinus halepenSIs |
| Pinus halepenSIs y Quercus ilex | 84.847,85 | Pinus halepenSIs |
| Pinus nigra y Pinus pinaster | 159.607,25 | Pinus nigra |
| Pinus radiata y Pinus pinaster cantabrico | 140.698,60 | Pinus radiata |
| Pinus pinaster y Eucalyptus spp, cantabricos | 345.330,10 | Pinus pinaster |
| Juniperus thurifera y Quercus ilex | 124.156,41 | Resto Coniferas |
| Quercus robur, Quercus petraea y Fagus sylvatica | 88.550,97 | Quercus robur |
| Quercus pyrenaica y Quercus ilex | 76.981,51 | Quercus pyrenaica |
| Quercus faginea y Quercus ilex | 195.728,05 | Quercus faginea |
| Quercus ilex y Quercus suber | 561.465,34 | Quercus ilex |
| Quercus suber y quercus canarienSIs | 48.074,51 | Quercus suber |
| Myrica faya y Erica arborea (fayal-brezal) | 16.318,52 | Resto Frondosas |
| Frondosas-Latifoliadas | 502.602,52 | Resto Frondosas |
| Coníferas y frondosas/Resinosas y latifoliadas | 622.089,91 | Resto Coniferas |
| Matorral, pastizal o cultivo con arbolado ralo | 3.278.961,54 | Resto Frondosas |
| TREE COVERED FOREST AREA 1900 (HA) | 13.904.659,61 |  |

With this assimilation of species the following table is obtained:

| SPECIES | FOREST AREA 2ND NFI (ha) | % | FOREST AREA 1991 (ha) | ......... | FOREST AREA 1999 (ha) | FOREST AREA 2000 (ha) | FOREST AREA 2001 (ha) | FOREST AREA 2002 (ha) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abies alba | 60.216,14 | 0,43 | 61.791 |  | 76.112 | 78.140 | 80.227 | 82.374 |
| Pinus halepenSIs | 1.544.689,52 | 11,11 | 1.585.091 |  | 1.952.459 | 2.004.489 | 2.058.019 | 2.113.097 |
| Pinus nigra | 565.589,79 | 4,07 | 580.383 |  | 714.895 | 733.946 | 753.546 | 773.713 |
| Pinus pinaster | 1.354.196,48 | 9,74 | 1.389.616 |  | 1.711.680 | 1.757.293 | 1.804.221 | 1.852.507 |
| Pinus pinea | 473.801,15 | 3,41 | 486.193 |  | 598.876 | 614.835 | 631.254 | 648.148 |
| Pinus radiata | 314.927,76 | 2,26 | 323.165 |  | 398.063 | 408.671 | 419.584 | 430.813 |
| Pinus sylvestris | 1.617.752,24 | 11,63 | 1.660.065 |  | 2.044.809 | 2.099.299 | 2.155.362 | 2.213.045 |
| Pinus uncinata | 52.273,80 | 0,38 | 53.641 |  | 66.073 | 67.834 | 69.645 | 71.509 |
| OthersConifers | 928.560,69 | 6,68 | 952.847 |  | 1.173.684 | 1.204.960 | 1.237.139 | 1.270.248 |
| Total Conifers | 6.912.007,57 | 49,71 | 7.092.792 |  | 8.736.651 | 8.969.466 | 9.208.997 | 9.455.453 |
| Castanea sativa | 83.497,18 | 0,60 | 85.681 |  | 105.539 | 108.351 | 111.245 | 114.222 |
| Fagus sylvatica | 311.115,22 | 2,24 | 319.252 |  | 393.244 | 403.723 | 414.505 | 425.598 |
| Quercus faginea | 247.653,29 | 1,78 | 254.131 |  | 313.029 | 321.371 | 329.953 | 338.784 |
| Quercus ilex | 1.384.912,60 | 9,96 | 1.421.135 |  | 1.750.504 | 1.797.152 | 1.845.145 | 1.894.526 |
| Quercus pyrenaica | 340.454,17 | 2,45 | 349.359 |  | 430.328 | 441.795 | 453.593 | 465.733 |
| Quercus robar | 188.101,67 | 1,35 | 193.021 |  | 237.757 | 244.093 | 250.611 | 257.318 |
| Quercus suber | 152.329,43 | 1,10 | 156.314 |  | 192.542 | 197.672 | 202.951 | 208.383 |
| Populus spp. | 53.767,98 | 0,39 | 55.174 |  | 67.962 | 69.773 | 71.636 | 73.553 |
| Eucalyptus spp. | 379.852,69 | 2,73 | 389.788 |  | 480.127 | 492.921 | 506.085 | 519.629 |
| Other broadleaves | 3.850.967,81 | 27,70 | 3.951.690 |  | 4.867.553 | 4.997.264 | 5.130.716 | 5.268.028 |
| Total Broadleaves | 6.992.652,04 | 50,29 | 7.175.546 |  | 8.838.584 | 9.074.116 | 9.316.441 | 9.565.773 |
| TOTAL | 13.904.659,61 | 100,00 | 14.268.338 |  | 17.575.235 | 18.043.582 | 18.525.438 | 19.021.226 |

6)	METHODOLOGY FOR CALCULATING THE CARBON EMISSIONS

The Amount of biomass removed is obtained by multiplying the figure given in the Agrarian Statistical Yearbook corresponding to the years 1990-2000 for lumber and firewood extracted by the BEF to obtain the complete biomass (1.6), by the dry matter conversion factor (0.5 t ms/m3) and by 0.001 in this last case to convert the figure to kilotonnes.

By multiplying by the carbon conversion factor of 0.45 t C/t dry matter we obtain the Gigagrammes of carbon corresponding to the biomass extracted.

The years 2001-2003 are obtained through the mean values for the previous years.

The comparison of the five-year average 1998-2002 with the reference year is similar to what must be done for the quinquennium 2008-2012 with 2010 as the representative value for comparison with the reference year.

In the period 1990-2002, its contribution to the total weighted emissions varies between 20.4%and 17.5%.

The range is extended to 1.1%-2.3% when considering the period 1990-2002.

The respective metrics for the level and trend are calculated by the following equations:

(1)	Lx,t=Ex,tEt

(2)	Tx,t=Lx,t∗(Ex,t−Ex,0)Ex,t−(Et−E0)Et

where:

Lx,t is the level assessment for source x in year t

Tx,t is the trend assessment for source x in year t

Ex,t y Ex,0  are the emission estimates of source category x in years t and 0, respectively

0  is the base year (i.e. 1990)

IPCC suggests that proxy variables be applied for both groups (IPCC Reference Manual for 1996)

The absolute figures are used with or without the year-on-year variation rates listed in the provisional statistics and sectorial reports such as the Statistical Bulletin on Hydrocarbons from CORES (the Spanish Corporation for Strategic Reserves of Oil-Based Products) or the annual  Statistical Update from SEDIGAS.

Except for crude oil where a standard country-specific factor has been used

Contrast aimed at detecting partial coverage in both the allocation of fuel and the identification of combustion-related source activities in the Inventory.

The main headings correspond to tar and pitch, products which are assumed to to have a fraction larger than that proposed by the IPCC.

AITEMIN (Association for Technological Investigation of Mining Equipment), “Measurement of the layer concentration of firedamp in several Spanish coal-mining areas”, 1989.

The rest of the terms involved in the emission factor (please refer to the equations in the IPCC Reference Manual) have been established by obtaining expert opinion.

EMEP/CORINAIR  Guidebook (1996), Table 4, chapter B146.

The CO2 emissions come from the processing of petroleum products, fluid catalytic cracking and flaring.

Reference source for the parameters: “Oil-refining, natural gas and petrochemistry”.

Emissions in the catalyst regenerator

Reference source for the parameters: “Oil-refining, natural gas and petrochemistry”.

Activity variable chosen on the grounds of the defective data coverage available for incinerated gases.

The company sent additional documentation reporting on the estimation techniques used, in line with the detailed methodology of EMEP/CORINAIR.

Discriminating by type of material and pipeline pressure.

CO2 fugitive emissions have not been estimated for this activity as it is explaned below when commenting on the inventory exhaustivity performance. So it is pending the diagnostic of this activity as a eventual key source when its emission estimates be computed.

Flaring and venting emissions in petroleum and natural gas production activities have not been estimated as the necessary primary information was not available. So it is pending the diagnostic of these activities as a eventual key source when their emission estimates be computed, although taking into account the comparativily minor activity rates of these petroleum and natural gas production activities in Spain, it is envisaged that they will not be in the list of the inventory key sources.

Key Source by level in 1990-2002 and by trend in 1991-1995, 1997-2002.

Key Source by level in 1990-2002 and by trend in 1991, 1993-1994, 1996, 1998.

Key Source by level in 1990-2002 and by trend in 1991-2002.

Key Source by level in 1990-2002 and by trend in 1991-1995, 1997-2002.

Key Source by level in 1990-2002 and by trend in 1992, 1995, 1998-1999.

Key Source by level in 1990-1994, 1996-1998 and by trend in 1994-1995, 1997-2002.

This activity includes the emissions due to “synthetic fertilizers”, “animal wastes applied to soils”, “N-fixing crops” and “crop residue”.

This activity includes the emissions for “atmospheric deposition” and “leaching and run-off”.

Due animal manure deposited while grazing.

ICONA: Second National Forest Inventory (1986-1995). National Nature Conservation Institute Ministry of Agriculture, Food and Fisheries.

DGCN: Third National Forest Inventory (1997-2006). Directorate-General for Nature Conservation. Ministry of the Environment.