# Sweden

## Sources of Information

- Report to the Monitoring Mechanism, December 2001

- Report to the Monitoring Mechanism, December 2002

- Report to the Monitoring Mechanism, December 2003

- Sweden’s Third National Communication to the UNFCCC, 2001

## QuAlity and Transparency of Reporting

The information on policies and measures is taken from the monitoring paper and the third national communication.  A clear discussion is presented of the aims of the various policies and measures and the policies and measures are summarised in a Table. Good details are provided of the costs and implementation of the policies. For a few measures, quantification of their effect is given but only until 2005.  The reports to the Monitoring Mechanisms provide updates on the status of policies and measures and in the 2003 report new projections for agriculture.

- Table 1: Information Provided on Policies and Measures

| - Information provided | - Level provided | - Comments |
| --- | --- | --- |
| - Policy names | - +++ |  |
| - Objectives of policies | - +++ |  |
| - Which GHGs? | - CO2, CH4 and halogenated gases |  |
| - Status of Implementation | - +++ |  |
| - Implementation body specified | - +++ |  |
| - Quantitative assessment of implementation | - + | - Provided for some but not all measures and only up to 2005 |
| - Interaction with other P&Ms discussed | - + | - Discussed for some measures |

- The latest projections contain a with measures scenario covering all six greenhouse gases with a base year of 1990 for carbon dioxide, methane and nitrous oxide and 1995 for hydrofluorocarbons, perfluorocarbons and sulphur hexafloride.  It is not clear which of the individual policies and measures is included in the projections.  The totals are disaggregated by gas and by sector (energy, transport, process, agriculture and waste), and individual gases are disaggregated further.  For 2010, one projection is provided but for 2020 there are 2 scenarios dependent on assumptions made about the nuclear power plants.  The modelling and parameter assumptions made are described with a good level of detail.

- Table 2: Information provided on Projections

| - Category of Information | - Level of Information Provided | - Comments |
| --- | --- | --- |
| - Scenarios considered | - With measures |  |
| - Expressed relative to inventory for previous years | - Yes 1990-1999 all gases |  |
| - Starting year | - 1990, 1995 for halogenated gases |  |
| - Split of projections | - ++ | - Projections split by gas and sector but base year emissions not given for all splits |
| - Presentation of results | - ++ | - Results presented in both tabular and graphical form. Base year emissions difficult to identify. |
| - Description of model (level of detail, approach and assumptions) | - +++ | - Details of the models and parameter assumptions good |
| - Discussion of uncertainty | - + | - Some discussion of uncertainty |
| - Details of parameters and assumptions | - +++ | - Good level of detail on energy use in different sectors and on underlying assumptions |

## Assessment of Policies and Measures

Table 3 shows an overview of the areas in which policies and measures are defined.  Many of the policies are not quantified.  Table 4 gives more details of the measures.

- Table 3: Overview of National Policies and Measures (MtCO2)

|  | With measuresa | With additional measures |
| --- | --- | --- |
| Energy supply | 1.5b |  |
| Business | Nq |  |
| Industrial processes | Nq |  |
| Transport | 0.1 |  |
| Residential | Nq | Nq |
| Public | Nq |  |
| Agriculture | Nq |  |
| Land use change | Nq |  |
| Waste management | 0.7 |  |
| International | 0.2 |  |
| Total | Nq |  |

- a It is not clear whether the policies are included in the with measures scenario, these figures represent all implemented policies.  Not all measures are quantified.

- b This figure is for policies in the energy field and will include energy savings in other sectors

- Nq not quantified but policies in that area

- Table 4: Detailed Information on Polices and Measures

| Sector | Name | Objective | GHG affected | Type of instrument | Status | Implementing entity | Estimate of savings | CCPM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 2005 |  |
| Energy | Certificate system for renewable energy | Quota bases electricity certificate system - quotas based on a target to increase renewable energy generation by 10 TWh between 2002 and 2010 | CO2 | Regulatory | Implemented | Government | 5 | n |
| Energy | Development of technologies and market launch of wind power | To develop large-scale wind power applications | CO2 | R&D | Implemented | Government |  | n |
| Energy | Transitional support for wind power | To provide additional support to wind when the electricity certificate system is in place | CO2 | Fiscal | Implemented | Government |  | n |
| Energy | Energy Tax | Reduce energy use in all sectors | CO2 | Fiscal | Implemented | National tax board | 8 | n |
| Energy | Reform of electricity market | Liberalises the electricity market | CO2 | Fiscal | Implemented | STEM | - | n |
| Energy | Special pilot project for biomass fuels | Makes biomass fuel more competitive | CO2 | Fiscal | Implemented | STEM | - | n |
| Energy | Investment grants for biomass fuelled CHP | Increase renewable electricity generation | CO2 | Fiscal | Implemented | STEM | 655 | n |
| Energy | Investment grants and subsidies for renewable power | Increase renewable electricity generation | CO2 | Economic | Implemented | STEM | 292 | n |
| Energy | Conversion from electric to district heating | Reduce electricity consumption | CO2 | Economic | Implemented | STEM | 162 | n |
| Energy | Conversion from electric to other individual heating | Reduce electricity consumption | CO2 | Economic | Implemented | Local Government | 57.5 | n |
| Energy | Information, procurement and labelling of new energy technologies | Reduce energy consumption | CO2 | Economic/information | Implemented | STEM | 300 | n |
| Energy | Measures to develop electric heating supply in Southern Sweden | Need to compensate for closure of Barseback | CO2 | Economic | Implemented | STEM | 70 | n |
| Energy | Municipal energy planning | Reduce energy consumption | CO2 | Regulatory | Implemented | STEM | - | n |
| Energy | Planning, building and housing act | Reduce energy consumption | CO2 | Regulatory | Implemented | National board of housing | - | n |
| Energy | Environmental code for infrastructure products | Assess impact of projects on climate | CO2 | Regulatory | Implemented | Swedish Environmental Protection Agency | - | n |
| Energy | Standards for energy use in residential and commercial properties | Reduced energy consumption | CO2 | Regulatory | Implemented | National board of housing | - | n |
| Energy | Comprehensive municipal planning | Reduce energy consumption | CO2 | Regulatory | Implemented | National board of housing | - | n |
| Energy | Measures for R&D | R&D to develop new fuels and technologies | CO2 | R&D | Implemented | STEM | - | n |
| Energy | Local investment programmes for ecological adjustment | Municipal adjustment to achieve ecological sustainability | CO2 | Economic | Implemented | Local Government | - | n |
| Energy | General environmental factors to be considered under the Environmental Code | Establish certain fundamental principles | CO2 | Regulatory | Implemented | Swedish Environmental Protection Agency | - | n |
| Energy | Investment programmes for ecological buildings | Reduce energy consumption | CO2 | Economic | Implemented | National board of housing | - | n |
| Waste | Collect landfill gas | Reduce methane emissions | CH4 | Regulatory | Implemented | Swedish Environmental Protection Agency | - | n |
| Waste | Ban of landfilling of organic waste | More stable landfills and use of waste as a resource | CH4 | Regulatory | Implemented | Municipalities | - | n |
| Waste | Ban on landfilling of sorted burnable waste | Improvement in disposal of all burnable waste | CH4/CO2 | Regulatory | Implemented | Swedish Environmental Protection Agency | 781 | n |
| Waste | Waste tax | Reduce the quantity of landfilled waste | CH4 | Fiscal | Implemented | Swedish Environmental Protection Agency | - | n |
| Waste | Landfill directive | Improve waste management | CH4 | Regulatory | Implemented | Swedish Environmental Protection Agency | - | y |
| Residential | Grants for home solar heating systems | Increase the use of solar energy | CO2 | Regulatory | Implemented | Local Government | 4 | n |
| Residential | Heat insulation for buildings | Tougher requirements for thermal bridges | CO2 | Regulatory | Implemented | National board of housing | - | n |
| Residential | Swan criteria for oil burners | Nordic eco-labelling has produced criteria for oil burners with an output up to 120kW | CO2 | Regulatory | Implemented | - | - | n |
| Residential | Investment grants for ecological building | Reduce energy consumption | CO2 | Economic | Implemented | Ministry of Finance/Board of housing | - | n |
| Residential | Energy declarations for apartment buildings | Better overview of the energy status of Swedish buildings | CO2 | Regulatory | Proposed | National board of housing | - | n |
| Residential | Individual metering of heating and hot water | Increasing awareness of energy use | CO2 | Voluntary | Proposed | National board of housing | - | n |
| Residential | EC directive on efficient energy use in buildings | To increase energy efficiency of buildings | CO2 | Regulatory | Proposed | Government |  | y |
| Residential | Tax reductions for small self-contained houses to improve environment | To encourage installation of energy-efficient windows and bio-fuel heating systems | CO2 | Fiscal | Proposed | Government |  | n |
| Forestry | Start-up grants for energy forests | Cultivation of energy forest | CO2 | Economic | Implemented | National Board of agriculture | - | n |
| Forestry | Environmental factors to be considered under the Silviculture Act | Sustainable forestry | CO2 | Regulatory | Implemented | National board of forestry | - | n |
| Forestry | National Board of Forestry recommendations on forest fuel extraction | Optimise biomass fuel abstraction | CO2 | Regulatory | Implemented | National board of forestry | - | n |
| Forestry | Protected areas | Protect certain forest ecosystems | CO2 | Regulatory | Implemented | National board of forestry | - | n |
| Forestry | Environmentally related forestry certification | Sustainable forestry | CO2 | Misc | Implemented | National board of forestry | - | n |
| Forestry | Tighter restrictions on the use of nitrogen fertilisers on forest soils | Reduce N leaching from forest soils | N2O | Regulatory | Implemented | National board of forestry | - | n |
| Transport | Procurement of ethanol/petrol hybrid cars | Increase the scope for use of biomass motor fuels | CO2 | Voluntary | Implemented | NUTEK | - | n |
| Transport | Promoting development and use of IT and traffic information methods | Reduce fuel consumption | CO2 | R&D | Implemented | VV | - | n |
| Transport | Green car | Reduce fuel consumption | CO2 | R&D | Implemented | Ministry of Industry and Trade | - | n |
| Transport | Transport quality assurance and Economical Driving transport projects | Reduce fuel consumption | CO2 | Misc | Implemented | VV | 100 | n |
| Transport | Joint programmes for development of more environmentally compatible vehicles | Reduce fuel consumption | CO2 | Misc | Implemented | NUTEK | - | n |
| Transport | Greater investment in tramway infrastructure | Make trams more competitive | CO2 | Misc | Implemented | National rail administration | - | n |
| Public | The Challenger Municipalities project | Begin phasing out use of fossil fuels inn five municipalities | CO2 | Misc | Implemented | Municipalities | - | n |
| Public | Climate objectives set by municipalities | Reduce impact on climate | CO2 | Misc | Implemented | Municipalities | - | n |
| Industry | Encouraging the introduction of environmental management systems in small and medium sized business | Reduce impact on climate | CO2 | Information | Implemented | NUTEK | - | n |
| Halogenated gases | The Refrigerants Order | Governs the use of refrigerants including halocarbons | F-gases | Regulatory | Implemented | Swedish Environmental Protection Agency | - | n |
| International | Joint implementation | Improve efficiency of the Baltic energy systems | CO2 | Economic | Implemented | STEM | 220 | n |
| International | Participation in world bank Carbon Fund | Develop flexible mechanism | CO2 | Economic | Implemented | Ministry of Industry and Trade | - | n |

Update on policies and measures 2003.

The Act concerning electricity certificates came into force on 1st May 2003 and replaced a former subsidy system for renewable electricity production. Companies producing renewable electricity will be allotted electricity certificates by the Government. An obligation will be placed on electricity users to acquire a certain number of electricity certificates in relation to their consumption during each calendar year. This compulsory quota obligation will be gradually increased. The aim is to increase the consumption of renewable electricity by 10 TWh by 2010.

From 1st January 2004, the carbon dioxide tax on fuels will increase by 18% for households and the service sector.  After adding the annual index adjustment, the carbon dioxide tax will increase from 0.76 to 0.91 per kg of carbon dioxide. An increase in the carbon dioxide tax on fossil fuels will limit their use, but also means that the relative price of electricity falls.  The energy tax on electricity will therefore increase by SEK 0.01 per kWh for households and the service sector. The current zero tax scheme for electricity used in industry will be ended and a tax of SEK 0.005 /kWh, equivalent to the minimum level in the forthcoming Energy Tax Directive, will be introduced. This new tax will apply to manufacturing industry, the greenhouse sector and agriculture. The increases come into effect on 1st January 2004.

## Evaluation of Projections

The projections presented here are those contained in the 3rd National Communication and on the 1990 inventory reported in that document.  The breakdown by greenhouse gas is presented in Table 5.  The with measures projections meets the Kyoto commitment so a with additional measures projection is not presented.

- Table 5: Summary of projections by gas in 2010 (MtCO2)

|  | Base year | with measures | with additional measures |
| --- | --- | --- | --- |
| CO2 | 55.9 | 57.7 |  |
| CH4 | 6.7 | 4.5 |  |
| N2O | 8.8 | 8.5 |  |
| F-gases | 0.5 | 1.1 |  |
| Total | 71.9 | 71.8 |  |
| % change rel 1990 |  | 0% |  |
| CO2 removals, land use and forestry | -20.3 | -24.3 |  |

- Table 6 summarises the projections by sector.  All sectors except transport and residential are projected to decrease in the with measures projections.

- Table 6: Summary of projections by sector in 2010 (MtCO2)

|  | Base year | With measures | % change rel 1990 | with additional measures | % change rel 1990 (additional measures) |
| --- | --- | --- | --- | --- | --- |
| Energya | 34.6 | 33.2 | -4% |  |  |
| Transport | 19.7 | 22.4 | +14% |  |  |
| Industrial processes and F-gases | 5.6 | 7.0 | +25% |  |  |
| Agriculture | 9.5 | 8.3 | -13% |  |  |
| Waste management | 2.6 | 1.0 | -62% |  |  |
| Total | 71.9 | 71.8 | 0% |  |  |

a This is the total for all energy use except for transport

An assessment of the target is presented in Table 7.  The commitment is for +4% compared to 1990 and the with measures projection is +1%.

Table 7: Assessment of Target

|  | - MtCO2 equiv. | - % (six gas basket) |
| --- | --- | --- |
| - Base year from projection | - 71.9 |  |
| - Commitment | - 74.8 | - +4% |
| - With existing P&Ms | - 71.8 | - 0% |
| - Gap | - -3.0 | - 4% |
| - Additional P&Ms | - - | - - |

## Description of Modelling Approach

Three main model types have been used for projections and scenarios in the Third National Communication.

- Economic-technical models for energy use (National Energy Administration) and energy supply (MARKAL) combined with analytical models for future transport demand (SIKA's passenger and goods transport model).

- Spreadsheet models in which expert assessments are made of future changes in premises (activity data and emission factors). Emissions are quantified using IPCC/UNFCC methodology.

- Statistical analyses and supplementary expert assessments.

The scenarios for carbon dioxide emissions from the energy sector are based on calculations and assessments of developments in the energy system. The energy system comprises energy use as well as supply. Scenarios have been produced for various energy sub-sectors. These are then combined to form an "energy balance". Energy consumption is balanced by energy supply. In addition to end use in industry, housing and services, and transport, the user side also includes conversion and distribution losses, as well as international shipping. The supply side comprises total supply of fuel and hydropower generation, nuclear power generation, wind power and net import of electricity. The methods and analyses used are based on a socio-economic perspective.

Method for expert assessments and use of spreadsheet models

The basic approach is the methodology used to determine emissions of greenhouse gases developed by the IPCC and used in the common reporting format under the Climate Convention. In some cases, UN methodology has been supplemented with national emission factors or has been expanded to better describe national conditions. These calculations principally require input data in the form of activity data, emission factors, correction factors and their changes over time. Assessments have been made of the way this input data changes over time as a consequence of the objectives and measures decided up to 1999. This has been done in consultation with experts at various agencies and trade organisations. The advantage of these models is that the same type of input data is used for historical emissions and the time series is thus congruent. The drawback is that input data for the calculations must be determined independently – often in the form of output data from another model or with the help of experts.

Spreadsheet models of this kind are used for projections for halocarbons, methane and nitrous oxide from agriculture, forest sinks of carbon dioxide and methane from waste (landfill).

The drawback of the simple analysis is that the models do not specifically take into account any overlaps or synergies between sectors or policy areas. The estimates of future emissions must therefore be supplemented with expert assessments of the potential impact of current political objectives, measures and instruments on emissions.

| Parameter | 1997 | 2010 | Unit |
| --- | --- | --- | --- |
| Population |  | 9039000 | 0.14% growth per annum (1997-2010) |
| GDP | 1.9 |  | % change per annum (1997-2010) |
| Renewables Share | 47% | 46% | Includes large-scale hydro |
| Crude Oil (International price) | 19.12 | 17 | USD / barrel |
| Coal (International price) | 44.1 | 42 | USD / tonne |
| Natural Gas (International price) | 2.3 | 2.6 | USD / Mbtu |
| Transport Passenger growth | 118 | 148 | Passenger/km |
| Freight growth | 82 | 103 | Tonnes/km |

Projection parameters for STEM Model.  Source: Questionnaire

## Country Conclusions

In the documents provided under the EU Monitoring Mechanism, the description of the policies in terms of costs and implementation is good.  However, the greenhouse gas savings arising from the policies are stated only for 2005.  No additional policies and measures are described as the with measures projection already meets the commitment under the burden sharing agreement.  New policies introduced in 2002, strengthen action in renewable energy and in the energy efficiency field.  There has also been an increase in Green taxation.

Details of the methodology and parameters used are good.