# Estonia

## Sources of Information

Estonia’s Third National Communication under the United Nations Framework Convention on Climate Change, 2001

## Reporting.

Two chapters in The Third National Communication deal with projections and measures.   Chapter four provides tables, with policies and measures to reduce greenhouse gas emissions in the Energy, Transport, Industry, Agriculture, Forestry and Waste management sectors.  The impact of single measures has been partly quantified for years 2000 and 2005.

- Table 1: Information Provided on Policies and Measures

| - Information provided | - Level provided | - Comments |
| --- | --- | --- |
| - Name of policy/measure | - +++ |  |
| - Type of instrument | - +++ |  |
| - Which GHGs? | - CO2, CH4, all GHG | - Some NMVOC, SO2 |
| - Status of Implementation | - +++ |  |
| - Implementation body specified | - ++ | - In most cases |
| - Quantitative assessment of implementation | - + | - Estimated mitigation effect for 2005; only some measures are quantified in non energy sectors |
| - Interaction with other P&Ms discussed | - ++ | - In some cases |

- +, ++, +++ level of information available increases as the number of + signs increases

- Table 2: Information provided on Projections

| - Category of Information | - Level of Information Provided | - Comments |
| --- | --- | --- |
| - Scenarios considered | With measures - With additional measures | - Scenarios are given for sectors corresponding with IPCC sectors and gases. |
| - Expressed relative to inventory for previous years | - no |  |
| - Starting year | - 1999 ? | - Not clear from the text |
| - Projections | - 2000,2005,2010,2015,2020 |  |
| - Split of projections | - yes | - Projections split by IPCC main sectors and gases. |
| - Presentation of results | - +++ | - Results presented in both tabular and graphical form |
| - Description of model (level of detail, approach and assumptions) | - ++ | - Basic description of the models and further references provided |
| - Discussion of uncertainty | - No |  |
| - Details of parameters and assumptions | - +++ | - Information on type of indicators used in scenarios provided |

- +, ++, +++ level of information available increases as the number of + signs increases

## Policies and Measures

The Estonian National Environmental Strategy (RT I 1997, 26, 390), approved by the Parliament in 1997, is the major basis document for the policy-making process in the field of environment. In the National Environmental Action Plan (NEAP) there are defined concrete conceptual, legislation, organisational, educational, training and also investment measures for reaching the objectives set in the National Environmental Strategy. The implementation process of the new NEAP, accepted by the Government on 05.06.2001 is in progress.

The implementation process of the new NEAP, accepted by the Government on 05.06.2001 is in progress. Overall coordination in the implementation phase lies with the Ministry of Environment.  Several other ministries jointly responsible for implementation

of some actions have incorporated those actions into their sectoral plans and budgets. County and local governments develop regional and local environmental action plans based on the NEAP experience. Several NGOs promote and raise the NEAP awareness among general public. They are also expected to watchdog actively the NEAP implementation process. Finally, the industry sector is expected to use the NEAP as a reference for their future plans related to environmental management.

- Table 3: Summary of the Effect of Policies and Measures by 2010 included in the Projections (MtCO2  eq.)

|  |  | With measuresa | With additional measuresb |
| --- | --- | --- | --- |
| CO2 |  |  |  |
| CH4 |  |  |  |
| N2O |  |  |  |
| F-gases |  |  |  |
|  | Energy (IPCC Sector 1) | -22.7 | -0.8 |
|  | Industry (IPCC Sector 2) | -0.3 | 0.0 |
|  | Agriculture (IPCC Sector 4) | -1.0 | -0.4 |
|  | Forestry (IPCC Sector 5) | -0.9 | -1.1 |
|  | Waste (IPCC Sector 6) | -0.6 | -0.4 |
|  | Total | -25.5 | -2.6 |

a The effect of policies implemented or adopted is derived from the sum of the potentials of the individual scenarios comparing without ant with measures,

Table 4 gives details of the policies and measures by sector.

#### Table 4: Policies and measures Detailed Information on Polices and Measures (estimated mitigation effect in 2010, in million tons CO2 equivalent)

### Energy Sector

Transport sector

#### Industrial sector

#### Forestry sector

#### Waste management

## Projections

The data in Tables 5-7 are based on information from the Third National Communication. The projections of the total GHG are based on the concept of Global Warming Potential (GWP) within a 100 year time horizon.   The WOM scenario baseline, “without measures”) are calculated assuming that the net emission of GHG (on CO2 equivalent) per every unit of GDP should be constant over time i.e. that the total increase of GDP is closely connected with the increase of GHG. This assumption is the based on close correlation between the decrease of GHG emission and GDP from 1990 to 1994.

- The two projections elaborated were:

- WM-projection (“with measures” projection), which reflects the impact of planned measures and the policies and measures implemented in period 1995-2000;

- WAM-projection (“with additional measures” projection), which encompasses additional policies and measures that may be taken in future.

Table 5: Summary of projections by gas in 2010 (MtCO2 equivalent)

|  | Base year | with measures | with additional measures |
| --- | --- | --- | --- |
| CO2 with LUCF | 31.8 | 8.64 | 6.91 |
| CH4 | 4.4 | 2.54 | 1.83 |
| N2O | 1.0 | 0.48 | 0.39 |
| HFC |  |  |  |
| PFC |  |  |  |
| SF6 |  |  |  |
| Total | 37.2 | 11.66 | 9.13 |
| % change relative to base year |  | -68.6% | -75.4% |

- The with measures projection shows that the currently implemented or adopted measures in Estonia  could  reduce greenhouse gas emissions by 2010 by 68.6 %% and with additional measures by 75.4%.

| Table 6: Summary of projections by sector in 2010 (MtCO2 eq.) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Base year | With measures | % change relative to 1990 | with additional measures | % change relative 1990 (additional measures) |
| Energy (IPCC Sector 1) | 38.8 | 16.2 | -58.4% | 15.4 | -60.3% |
| - of which transport |  |  |  |  |  |
| Industry (IPCC Sector 2) | 0.6 | 0.3 | -44.6% | 0.3 | -46.3% |
| Agriculture (IPCC Sector 4) | 2.4 | 1.4 | -42.9% | 1.0 | -57.9% |
| Forestry (IPCC Sector 5) | -6.3 | -7.2 | 13.9% | -8.3 | 31.2% |
| Waste (IPCC Sector 6) | 1.6 | 1.0 | -36.4% | 0.7 | -58.4% |
| Total without LUCF | 43.5 | 18.9 | -56.5% | 17.4 | -59.9% |
| Total with LUCF | 37.2 | 11.7 | -68.5% | 9.1 | -75.4% |

| Table 7: Assessment of the Target |  |  |  |
| --- | --- | --- | --- |
| Without LUCF | MtCO2 equiv. | % of 1990 level (six gas basket) |  |
|  | Ref. scenario |  |  |
| Base year emissions (from projections) | 43.5 |  |  |
| Commitment ( base year emissions) | 40.0 | -8.0% |  |
| 2010 emissions with measures | 18.9 | -56.5% |  |
| 2010 emissions with additional measures | 17.4 | -59.9% |  |
| Gap between with measures and commitment (-ve means no gap) | -21.1 | -48.5% |  |
| Effect of additional P&Ms | 1.5 | 3.4% |  |

## Description of Modelling ApproacH

Emissions from the energy sector are projected with the modelling framework of the MARKAL model.  Scenarios in agricultural sector assumed Estonia will reach the level of EU countries.  Projections of CO2 sinks in Forestry and land use are primarily focused on protection of forest, increase area of reforestation and regulation of annual harvest. The WAM scenario grounds mainly on forestry activities.

Key parameters used in projections

## Conclusions

The main document used for this summary was the Third National Communication.  The level of detail in policies and measures chapter is appropriate and is line with UNFCCC guidelines.  The details of the methodology for the projections are partly described.  The scenarios are provided for sectors and for total GHGs , but not separately for sectors-all gases.  The key policies and measures are summarised in a Table, which makes clear the potential greenhouse gas savings.

The projected decrease from the base year with measures implemented (excluding LUCF) and adopted is 56.5% in the basket of key GHGs by 2010.  Additional policies and measures are applicable to reduce GHG emissions to 59.9% below base year levels by 2010. This shows that Estonia has reserves to fulfil its commitments.