# Slovakia

## Sources of Information

Slovakia’s Third National Communication under the United Nations Framework Convention on Climate Change, 2001

## Reporting

Two chapters in The Third National Communication of Slovakia deal with projections and measures.    Policies and measures to reduce greenhouse gas emissions are provided for the Energy, Transport, Industry, Agriculture, Forestry and Waste management sectors.  The impact of the measures has been quantified in most cases. The table on policies and measures follows the UNFCCC guidelines, but not all discussed policies and measures are presented in the table.

- Table 1: Information Provided on Policies and Measures

| - Information provided | - Level provided | - Comments |
| --- | --- | --- |
| - Name of policy/measure | - +++ |  |
| - Type of instrument | - +++ |  |
| - Which GHGs? | - CO2, CH4, N2O, PFCs,HFCs,SF6 |  |
| - Status of Implementation | - ++ |  |
| - Implementation body specified | - ++ | - In most cases |
| - Quantitative assessment of implementation | - ++ | - Estimated mitigation effect for 2010; a few measures are not quantified |
| - Interaction with other P&Ms discussed | - ++ | - In some cases |

- +, ++, +++ level of information available increases as the number of + signs increases

- Two approaches are taken to develop projections of greenhouse gas emissions and to calculate the total effect of policies and measures. Energy and Forestry projections are based on model calculations; the other sectors projections are based on expert judgement. Both approaches include without, with measures and with additional measures scenarios.

- Table 2: Information provided on Projections

| - Category of Information | - Level of Information Provided | - Comments |
| --- | --- | --- |
| - Scenarios considered | Without measures With measures - With additional measures | - Scenarios are given for sectors corresponding with IPCC sectors and gases. |
| - Expressed relative to inventory for previous years | - no |  |
| - Starting year | - 1998 |  |
| - Projections | - 2000,2005,2010,2015 |  |
| - Split of projections | - yes | - Projections split by IPCC main sectors and gases. |
| - Presentation of results | - +++ | - Results presented in both tabular and graphical form |
| - Description of model (level of detail, approach and assumptions) | - ++ | - Basic description of the models and further references provided |
| - Discussion of uncertainty | - No |  |
| - Details of parameters and assumptions | - + | - Limited information on type of indicators used in scenarios provided |

- +, ++, +++ level of information available increases as the number of + signs increases

## Assessment of Policies and Measures

- Table 3 gives an overview of effects of policies and measures.  The without measures scenario represents extrapolation of the actual situation from the aspect of source structure and fuel consumption.  The figures given for the with measures projection are for policies and measures implemented or adopted by 2001.

- The with additional measures scenario includes the effects of policies applicable and/or planned. The effect of policies planned is derived from the difference between the with measures and the with additional measures scenarios of the model calculations.

- Table 3: Summary of the Effect of Policies and Measures by 2010 included in the Projections (MtCO2  eq.)

|  | With measuresa | With additional measuresb |
| --- | --- | --- |
| CO2 | NA |  |
| CH4 |  |  |
| N2O |  |  |
| F-gases |  |  |
| Energy (IPCC Sector 1) | -1.5 | -3.9 |
| - of which transport | 0.0 | -0.1 |
| Industry (IPCC Sector 2) | 0.0 | 0.0 |
| Agriculture (IPCC Sector 4) | 0.3 | -1.0 |
| Forestry (IPCC Sector 5) | -0.7 | -0.4 |
| Waste (IPCC Sector 6) | -0.4 | -0.1 |
| Total | -2.3 | -5.5 |

a The effect of policies implemented or adopted is derived from the sum of the potentials of the individual scenarios comparing without ant with measures,

- b The effect of policies planned is derived from the difference between the with measures and the with additional measures scenarios of the calculations. NA- information is not provided in 3NC.

Table 4 gives details of individual policies and measures.

Table 4 Detailed Information on Polices and Measures (estimated mitigation effect in 2010, in million tons CO2 equivalent)

#### Characteristics and reduction potential of some mitigating measures in Energy sector

#### Characteristics and reduction potential of some mitigating measures in Agriculture

#### Characteristics and reduction potential of some mitigating measures in Forestry

#### Characteristics and reduction potential of some mitigating measures in Waste management

I – policy and measure have been already implemented (using criteria of updated IPCC Guidelines 1999/7)

S – adopted, approved policy or measure

P – planned, prepared policy/measure

## Evaluation of Projections

The data in Tables 5-7 are based on information from the Third National Communication.

Table 5: Summary of projections by gas in 2010 (MtCO2 equivalent)

|  | Base year | without measures | with measures | with additional measures |
| --- | --- | --- | --- | --- |
| CO2 | 57.2 |  |  |  |
| CH4 | 6.8 |  |  |  |
| N2O | 5.9 |  |  |  |
| HFC | 0.0 |  |  |  |
| PFC | 0.3 |  |  |  |
| SF6 | 0.03 |  |  |  |
| Total | 70.1 |  |  |  |
| % change relative to base year |  |  |  |  |

- Table 6 summarises the projections by sector.  Transport is projected to increase emissions in the with measures projections, Forestry is decreasing sinks, whereas energy as a whole, industry, agriculture, and waste sector reduce emissions by 2010.

- The with measures projection shows that the currently implemented or adopted measures of Slovakia could reduce greenhouse gas emissions by 2010 by 27% and with additional measures by 34%.

| Table 6: Summary of projections by sector in 2010 (MtCO2 eq.) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Base year | Without measures | With measures | % change relative to 1990 | with additional measures | % change relative 1990 (additional measures) |
| Energy (IPCC Sector 1) | 57.8 | 43.5 | 42.0 | -27.3% | 38.1 | -34.1% |
| - of which transport | 5.2 | 5.4 | 5.4 | 3.8% | 5.3 | 1.9% |
| Industry (IPCC Sector 2) | 4.7 | 4.2 | 4.2 | -10.6% | 4.2 | -10.6% |
| Agriculture (IPCC Sector 4) | 7.9 | 5.5 | 5.8 | -26.6% | 4.8 | -39.2% |
| Forestry (IPCC Sector 5) | -2.4 | -1.1 | -1.8 | -25.0% | -2.2 | -8.3% |
| Waste (IPCC Sector 6) | 2.1 | 1.6 | 1.2 | -42.9% | 1.1 | -47.6% |
| Total without LUCF | 72.5 | 54.8 | 53.2 | -26.6% | 48.2 | -33.5% |
| Total with LUCF | 70.1 | 53.7 | 51.4 | -26.7% | 46.0 | -34.4% |

| Table 7: Assessment of the Target |  |  |  |
| --- | --- | --- | --- |
| Without LUCF | MtCO2 equiv. | % of 1990 level (six gas basket) |  |
|  | Ref. scenario |  |  |
| Base year emissions (from projections) | 72.5 |  |  |
| Commitment ( base year emissions) | 66.7 | -8.0% |  |
| 2010 emissions with measures | 53.2 | -26.6% |  |
| 2010 emissions with additional measures | 48.2 | -33.5% |  |
| Gap between with measures and commitment (-ve means no gap) | -13.5 | -18.6% |  |
| Effect of additional P&Ms | -5.0 | -6.9% |  |

## Description of Modelling ApproacH

Emissions from the energy sector are projected with the modelling framework of the ENPEP model.  Scenarios in agricultural sector assumed the reduction of total number of livestock and/or change of representation of number of livestock in individual categories, treatment of animal waste-biogas utilisation, increase fertiliser consumption and increase of per hectare production.  Projections of CO2 sinks in Forestry and land use were modelled using measures that are primarily focused on protection of forests and agricultural soil stock, regulation of timber extraction and afforestation of non-forest areas.  Slovakian greenhouse gas emission inventory and emissions are calculated in accordance with IPCC inventory methodology.

### Some parameters used in scenarios

| Parameter | 1998 | 2000 |  |  | 2005 |  | 2010 |  | 2015 | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GDP total | 612.7 | 634.3 |  |  | 724.7 |  | 864.9 |  | 1022.2 | Bill.SKK.Constant prices of 1995) |
| ### Crude oil processed | 5.3 | 5.3 |  |  | 5.4 |  | 5.5 |  | 5.6 | mil tones |
| Annual growth rate heat consumption | 0.31 |  | 1.49 |  |  | -1.84 |  | -1.84 |  | % |
| Annual growth rate of electricity consumption | 0.76-2.26 |  |  |  |  | 1.4-2.5 |  | 2.67-4.87 |  | % |
| Share of industry on GDP formation | 23.2 | 22 |  | 18.2 |  |  | 15.9 |  | 15.9 | % |
| Coal mining |  | 2.86 |  | 3.0-1.71 |  |  | 3.1-1.66 |  | 3.04-1.51 | Mt |
| Max. share of biogas combustion | 0 | 0 |  | 10 |  |  | 25 |  | 40 | % |

## Country Conclusions

The main document used for this summary was the Third National Communication.  The level of detail in policies and measures chapter is appropriate and is line with UNFCCC guidelines.  The details of the methodology for the projections are partly described.  The scenarios are provided for sectors and for total GHGs , but not separately for CO2 methane and nitrous oxide.   The key policies and measures are summarised in a Table, which makes clear the potential greenhouse gas savings.

The projected decrease from the base year with measures implemented and adopted is -26.6% in the basket of six greenhouse gases by 2010, excluding land use change and forestry.  Additional policies and measures are applicable to reduce GHG emissions to 33.5% below base year levels by 2010 (excluding LUCF). This shows that Slovakia has potential to go beyond Kyoto targets.