# Slovenia

## Sources of Information

Slovenia’s Second and Third National Communication to the Conference of the Parties of the United Nations Framework Convention on Climate Change, Republic of Slovenia, Ministry of the Environment, Spatial Planning and Energy, July 2004

## Reporting

In Slovenia’s Second and Third National Communication, Slovenia describes an approach to reducing its GHG emissions by 8 % in the 2008 - 2012 period, relative to the base year, in order to meet Slovenia's Kyoto Protocol target. The base year for CO2, CH4 and N2O is 1986 and the base year for F-gases is 1995 for Slovenia.

In the base year, greenhouse gas emissions in Slovenia were approximately 20.7 million tons of CO2 equivalent, and an 8 % reduction would entail annual average emissions of no more than 19.0 million tons of CO2 eq. during the first commitment period (2008-2012).  Based on the additional domestic measures projection and the inclusion of allowed sinks, Slovenia plans to meet the required 8 % reduction in the first commitment period.

Beside domestic measures, the Kyoto Mechanisms that could be considered are Emissions Trading, Joint Implementation and the Clean Development Mechanism.  Slovenia sees Emission Allowance Trading as the most important flexible mechanism, which will enable polluters to reduce emissions in the most cost effective way. Joint Implementation is interesting to Slovenia as a performer of projects in other countries in Annex 1. Projects based on JI from which Slovenia would obtain emission rights would also improve the technological penetration of the Slovenian economy.

Chapter 4 in Slovenia’s National Communication deals with policies and measures and Chapter 5 addresses projections and the total effect of policies and measures.  Policies and measures to reduce greenhouse gas emissions and projected emissions are discussed for the following sectors: energy, transport, industry, agriculture and forestry, waste, F-gases and cross-sectoral measures.  Summary tables on policies and measures are clearly presented. Projections for the with domestic measures and with additional domestic measures scenarios are provided, with a breakdown by gas and by sector.  Appendix C gives more detail on the assumptions and parameters that the projections are based on.

- Table 1: Information Provided on Policies and Measures

| - Information provided | - Level provided | - Comments |
| --- | --- | --- |
| - Name of policy/measure | - +++ | - PaMs discussed in individual sectors (energy, transport, industry, agriculture and forestry, waste, F-gases and cross-sectoral measures); clear summary table for PaMs |
| - Type of instrument | - +++ | - Type of instrument provided for every policy and measure |
| - Which GHGs? | - CO2, CH4, N2O, F-gases |  |
| - Status of Implementation | - ++ | - Status of implementation given for policies and measures as implemented, planned or adopted. Table did not split out PaMs into those for the two different projection scenarios. |
| - Implementation body specified | - +++ | - Implementing entity given in all cases |
| - Quantitative assessment of implementation | - ++ | - Quantified for most PaMs |
| - Interaction with other P&Ms discussed | - ++ | - Some discussion on interactions and impacts of other PaMs |

- +, ++, +++ level of information available increases as the number of + signs increases

- Slovenia’s Second and Third National Communication provided a high standard of reporting on policies and measures.  All categories required by the directive, such as status and quantification of implementation, were covered for most or all policies and measures.  A policies and measures summary table clearly listed the details of each policy and measure in a straightforward layout, with policies grouped by sector.

- Table 2: Information provided on Projections

| - Category of Information | - Level of Information Provided | - Comments |
| --- | --- | --- |
| - Scenarios considered | - With measures and with additional measures | - With additional measures projection is based on a higher implementation intensity of adopted and implemented measures and the realisation of all planned measures. |
| - Expressed relative to inventory for previous years | - +++ | - Overall emission estimates in CO2 eq. given for 2005, 2010, 2015, 2020 and 2008-2012 average compared with revised base year (1986) and also 1990, 1995, 2000 and 2002 |
| - Starting year | - 1999 |  |
| - Projections | - 2005, 2010, 2015, 2020 average 2008-2012 | - Comprehensive split by GHG and sector |
| - Split of projections | - Yes | - Projections split by IPCC main sectors and by gas |
| - Presentation of results | - +++ | - All results presented in a summary table with more detailed tables given for each sector projection |
| - Description of model (level of detail, approach and assumptions) | - +++ | - Good description of model approach, detail and key assumptions in model provided |
| - Discussion of uncertainty | - +++ | - Key uncertainty issues explained and quantified where relevant |
| - Details of parameters and assumptions | - +++ | - Key assumptions and parameters presented in a table for the relevant years (past and projected) |

- +, ++, +++ level of information available increases as the number of + signs increases

## Assessment of Policies and Measures

- Table 3 gives an overview of effects of policies and measures. The ‘with domestic measures’ scenario shows total potentials of implemented and adopted measures by sector and gas.  The ‘with additional domestic measures’ scenario shows the potentials of all planned measures.

- Table 3: Summary of the Effect of Policies and Measures by 2010 (MtCO2  eq.)

|  | with domestic measures scenario* | with additional domestic measures scenario* |
| --- | --- | --- |
| CO2 | 1193 | 1089 |
| CH4 | 226 | 101 |
| N2O | 30 | 51 |
| F-gases | 0 | 274 |
| Total by gas | 1449 | 1515 |
| Energy | 1234 | 1098 |
| Industrial processes | 0 | 274 |
| Agriculture | 39 | 50 |
| Waste | 176 | 93 |
| Total by Sector | 1449 | 1515 |

*Assessments of the effect of measures were made to meet the requirements of the 3NC and differ in terms of the IPCC energy sector from the quantified effects of measures presented in the policies and measures table and from the savings presented in the Action Plan for Reducing GHG emissions.

Table 4:	Detailed Information on Polices and Measures

| Sector | Name | Objective | GHG affected | Type of instrument | Status | Implementing entity | Estimate of savings (MtCO2) 2010 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Energy | Promotion of electricity production from Renewable Sources and CHP Generation | Increased share of RES and CHP generation | CO2 | Economic, regulatory | Implemented | MOPE | 0.5 |
| Energy | Natural gas market opening | Switching from coal to natural gas | CO2 | Regulatory | Implemented | MOPE |  |
| Energy | Electricity market opening | Change in structure of electricity production | CO2 | Regulatory | Implemented | MOPE | 1.35 |
| Energy | Construction of large hydro-power plants | Increased share of RES in electricity production | CO2 |  | Implemented | HSE, MOPE | 0.4 |
| Energy | incentives for carrying out the EEU measures for investment in RES | Higher energy efficiency and increased share of RES | CO2 | Economic, promotion | Implemented | MOPE, Government | - |
| Energy | Education, training and public awareness | Higher level of public awareness and information release | CO2 | Promotion, education, information | Implemented | MOPE | 0.1 |
| Energy | energy labelling of household devices | Higher energy efficiency | CO2 | Information, regulatory | Implemented | MOPE |  |
| Energy | Regular checks on small boilers and air conditioning units | Higher energy efficiency | CO2 | Regulatory | Implemented | MOPE, municipalities | - |
| Energy | Thermal protection and energy labelling of buildings | Higher energy efficiency | CO2 | Information, regulatory | Implemented | MOPE | 0.31 |
| Energy | Cost accounting for heat according to actual consumption | Stimulation of inhabitants for efficient energy use. | CO2 | Information | Implemented | MOPE, Government | 0.019 |
| Energy | Third party financing | Incentives for investment in efficient energy use | CO2 | Economic/voluntary | Implemented | Public sector, municipalities | 0.018 |
| Energy | Certification of energy source | Electricity production from RES | CO2 | Regulatory | Planned | MOPE | - |
| Energy | Demand side management | Stimulation of energy efficiency use for consumers | CO2 | Promotion | Planned | MOPE | - |
| Energy | Introduction to excise tax on fossil fuels and electricity | Increase of energy efficiency in domestic and public sector | CO2 | Fiscal | Planned | MF | - |
| Transport | Excise tax on fuels | Reduce fuel consumption | CO2 | Fiscal | Implemented | MF | 0.15 |
| Transport | Control of exhaust fume | Emission reduction due to improved vehicles | CO2 | Regulatory | Implemented | MOPE | 0.185 |
| Transport | Informing consumers on fuel consumption and CO2 emissions and ACEA agreement | Emission reduction due to more energy efficient vehicles | CO2 | Promotion, information | Adopted | MOPE | 0.25 |
| Transport | Promotion of biofuel use | GHG emission reduction | CO2 | Fiscal | Adopted | MF, MOPE | 0.1 |
| Transport | Promotion of public transport use | Reduce pollution in towns | CO2 | Promotion | Planned | MOPE, MP, Municipalities | 0.1 |
| Transport | Increase in the share of railway transport of freight and passengers | GHG emission reduction | CO2 | Promotion, regulatory | Planned | Government, MP | 0.05 |
| Transport | Strategy of the renewable and spatial development | Reduce journey lengths | CO2 | Regulatory | Adopted / planned | MOPE, municipalities | 0.065 |
| Industry | EEU in industry |  | CO2 | Promotion , economic | Implemented | MOPE | 0.4 |
| Industry | Promote environment systems ISO 14001 and EMAS | Higher industrial energy efficiency | CO2 | Promotion, economic | Implemented | MOPE, MG |  |
| Industry | Eco-labelling | Higher energy efficiency | CO2 | Voluntary | Implemented | Government | - |
| Industry | IPPC Directive | Higher industrial energy efficiency | CO2 | Regulatory | Adopted | MOPE, MG, GZS | 0.06 |
| Agriculture and Forestry | Slovenian agriculture-environmental programme and programme of countryside development | Sustainable practice in agriculture | N20, CH4, CO2 | Economic | Implemented | MKGP | 0.009 |
| Agriculture and Forestry | quality agricultural practices on fertilisation | Nitrous oxide emissions reduction | N2O | Regulatory | Implemented | MKGP, MOPE | 0.011 |
| Agriculture and Forestry | Promotion of biogas for electricity and heat production | GHG emission reduction by using animal and rural waste to generate energy | CH4 | Economic, regulatory | Implemented / planned | MOPE | 0.02 |
| Agriculture and Forestry | Incentives for agricultural plants for biodiesel | Fossil fuel reduction in transport | CO2, N2O | Regulatory, economic | Planned | MKGP | - |
| Waste | Regulation on waste disposal and waste management | Improved waste handling | CH4 | Regulatory | Implemented | MOPE | - |
| Waste | Waste disposal tax | Reduced quantity of waste | CH4 | Fiscal | Implemented | MOPE, MF | - |
| Waste | Separate waste collection and packaging waste management | Reduced quantity of waste by source | CH4, CO2 | Regulatory | Adopted | Municipalities, MOPE, GZS | 0.042 |
| Waste | Landfill gas extraction and combustion, energy exploitation or use of landfill gas | Combustion of methane | CH4, CO2 | Economic, regulatory | Adopted | MOPE | 0.126 |
| Waste | Thermal waste processing | Reduction of waste and energy generation | CH4, CO2 | Regulatory | Planned | MOPE | 0.088 |
| F-gases | Directive on F-gases | F-gases emission reduction | HFC, PFC, SF6 | Regulatory | Planned | Government | 0.2 |
| Cross Sectoral | CO2 Tax | More efficient energy use | CO2 | Fiscal | Implemented | MOPE, MF | - |
| Kyoto Mechanisms | Emission Trading Scheme | Cost effective emission reduction | CO2 | Regulatory, economic | Adopted | MOPE | - |
| Kyoto Mechanisms | Clean Development Mechanism (CDM), Joint Implementation (JI) | GHG emission reduction | All GHG | Economic | Planned | MOPE | - |
| Sinks* | Co-natural forest management | Conservation of biodiversity, productivity, regeneration and volume of forest | CO2 | Regulatory | Implemented | MKGP | 0.84 |

*Allowed sinks are estimated at 1.68 Mt CO2 annually for 2008-2012.  A conservative assessment of sinks exploitation has been made at 0.84 Mt CO2 since they need to be presented as the consequence of direct human activities in order to be used as a fulfilment of country's obligations.

## Evaluation of Projections

The data in Tables 5-7 are based on information in Slovenia’s Second and Third National Communication, July 2004.

Table 5: Summary of projections by gas in 2010 (MtCO2 equivalent)

|  | Base year 1986* | with measures scenario (2008 - 2012 average) | with additional measures (2008-2012 average) |
| --- | --- | --- | --- |
| CO2 | 16.00 | 17.30 | 16.06 |
| CH4 | 2.53 | 2.22 | 2.13 |
| N2O | 1.79 | 1.53 | 1.48 |
| HFCs* | 0.03 | 0.28 | 0.10 |
| PFCs* | 0.29 | 0.05 | 0.05 |
| SF6* | 0.03 | 0.09 | 0.02 |
| LUCF | 0.00 | -0.84 | -0.84 |
| Total without LUCF | 20.66 | 21.48 | 19.85 |
| Total with LUCF | 20.66 | 20.64 | 19.01 |
| % change relative to base year (without LUCF) |  | 4.0% | -3.9% |
| % change relative to base year (with LUCF) |  | -0.1% | -8.0% |

- *The base year for F-gases is 1995

- Table 5 presents the emission projection scenarios for the commitment period, the average over 2008-2012, split by greenhouse gas.  The base year for all gases is 1986, except for F-gases which have a base year of 1995.  Under the ‘with domestic measures’ projection excluding sinks, Slovenia’s emissions increase to 4% above the base year, but if sinks are included the emissions projections are 0.1% below the base year in the commitment period.  Based on the ‘with additional domestic measures’ scenario, emission projections are 3.9% below the base year without sinks and 8% below the base year with sinks.  The Kyoto commitment of an 8% cut is met if savings from sinks and additional measures are accounted for.

- Table 6: Summary of projections by sector in 2010 (MtCO2e)

|  | Base year 1986 (1995 F-gases) | with domestic measures scenario | with additional domestic measures scenario | % change relative to base year 'with measures' | % change relative to base year 'with additional measures' |
| --- | --- | --- | --- | --- | --- |
| Energy (IPCC Sector 1) | 15.60 | 16.94 | 15.69 | 8.6% | 0.6% |
| Industrial proc. (IPPC Sector 2) | 1.37 | 1.42 | 1.16 | 3.5% | -14.9% |
| Solvent use (IPCC Sector 3) | 0.13 | 0.04 | 0.04 | -72.7% | -71.9% |
| Agriculture (IPCC Sector 4) | 2.56 | 2.19 | 2.15 | -14.6% | -16.3% |
| Waste (IPCC Sector 6) | 1.00 | 0.91 | 0.82 | -9.2% | -18.3% |
| Sinks (LUCF) | 0.00 | -0.84 | -0.84 | - | - |
| Total without LUCF | 20.66 | 21.48 | 19.85 | 4.0% | -3.9% |
| Total with allowed sinks | 20.66 | 20.64 | 19.01 | -0.1% | -8.0% |

Table 7: Assessment of the Target (2010)

|  | Excluding sinks |  | Including allowable sinks |  |
| --- | --- | --- | --- | --- |
|  | MtCO2 eq. | % of base year (six gas basket) | MtCO2 eq. | % of base year (six gas basket) |
| Base year emissions (from projections) | 20.66 | 100% | 20.66 | 100% |
| Commitment | 19.01 | 92% | 19.01 | 92% |
| 2008-12 average emissions 'with domestic measures' | 21.48 | 104% | 20.64 | 100% |
| Gap (-ve means no gap) | 2.47 | 12% | 1.63 | 8% |
| Effect of additional P&Ms | 1.64 | 8% | 1.64 | 8% |
| Remaining gap 'with additional measures' | 0.84 | 4% | 0.00 | 0% |

## Description of Modelling ApproacH

Slovenia’s 2nd and 3rd National Communication gives a comprehensive breakdown of assumptions and parameters for the projections model and a detailed explanation of the methodology behind the projections.

To prepare the IPPC Energy sector projections, Slovenia used a set of models whose main tool is a reference energy ecological model called REES-SLO, based in the MESAP (Modular Energy Systems Analysis and Planning) environment.  In addition to the REES-SLO the following models were used:

- PETSLO – market penetration of energy-saving final use technologies assessment model

- ELAM-SLO – simulation of an electrical load, curve model

- ELBIVIM – model calculating electricity production balance on a free market

The REES-SLO model was developed as a linear network model of processes and connections (reference energy system).  This enables consistent modelling of energy use based on needs of energy services and energy supply.

There are certain assumptions in the models that are subject to greater uncertainty; these are outlined in the National Communication.  The dynamics of transition to natural gas in electricity production depends on future market trends and social problems with reducing the domestic coal production.  This uncertainty in the future of coal mining means the fugitive emission projections are uncertain.  Transport emission projections are uncertain due to the potential increase in transit transport and possible influences of neighbouring countries.  Projections in the energy sector for the higher growth scenario shows the span of GHG emissions as between 16.0 and 17.2 Mt CO2 eq in 2010, which reflects the uncertainty of the performed projections.

Some of the main assumptions behind the projections, presented in the National Communication, are shown below.  Assumptions for the energy sector projections are at three levels: general data, scenario data and a system of strategic instruments for GHG reduction.  A key assumption for the agricultural projection is the number of livestock.

Main assumptions for projections (Slovenia’s 2nd and 3 National Communication)

|  | 1990 | 1995 | 1997 | 2000 | 2005 | 2010 | 2015 | 2020 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Population (M) | 2.00 | 1.99 | 1.99 | 1.99 | 1.99 | 1.98 | 1.98 | 1.96 |
| GDP growth (%) | no data | 4.1 | 4.6 | 3.4 | 3.40 | 3.70 | 2.30 | 2.10 |
| Oil (USD 2000/barrel) |  |  | 20 | 28 | 23.00 | 21.00 | 24.00 | 28.00 |
| Natural gas (USD 2000/GJ) |  |  | 2.8 | 2.6 | 2.80 | 2.70 | 3.10 | 4.00 |
| Coal (USD 2000/t) |  |  | 47 | 34 | 47.00 | 47 | 47 | 47 |
| Passenger transport (M p km/a) |  | 16578 | 24295 | 25772 | 28435 | 31088 | 33491 | 35374 |
| Goods transport (M t km/a) |  | 6385 | 7600 | 8593 | 10967 | 13344 | 15847 | 17930 |
| Cattle |  | 546,048 | 495,535 | 445,724 | 493,670 | 491,566 | 520,269 | 527,700 |
| Pigs |  | 557,878 | 592,034 | 578,193 | 603,594 | 657,819 | 661,410 | 665,000 |
| Energy intensity (toe/M Euro 1995) |  |  |  |  |  |  |  |  |
| With measures projection |  | 402.5 | 397.5 | 348.7 | 331 | 285.6 | 261.3 | 232.3 |
| With additional measures |  | 402.5 | 397.5 | 348.7 | 327 | 278.3 | 247.3 | 225.2 |
| Total energy consumption (Mtoe) |  |  |  |  |  |  |  |  |
| With measures projection |  |  |  |  | 7.09 | 7.32 | 7.5 | 7.39 |
| With additional measures | 6.18 | 6.1 | 6.49 | 6.33 | 7 | 7.13 | 7.1 | 7.17 |

## Country Conclusions

The main document used for this summary was Slovenia’s Second and Third National Communication, July 2004.   The projections and the base year emissions (1986 and 1995 for F-gases) in the tables above are based on this Nation Communication.  The presentation in chapters dealing with policies and measures is in line with UNFCCC guidelines. The measures for the with domestic measures scenario and with additional domestic measures scenario are discussed for the following sectors: energy, transport, agriculture and forestry, waste, F-gases and cross sectoral.

The key policies and measures are clearly presented in tables, potential greenhouse gas savings are summarised by sector and for some individual policies and measures.  Projections are provided for the with domestic measures and with additional domestic measures scenarios.  Projections are broken down into 6 greenhouse gases and by IPPC sector (energy, industrial processes, solvent use, agriculture, waste and sinks). Key assumptions behind the projections are provided and a more detailed breakdown of parameters and assumptions behind both projection scenarios is given in Appendix C of the National Communication.

Slovenia’s Kyoto commitment is an 8% reduction in greenhouse gas emissions relative to the base year.  The with additional domestic measures projection shows that with implemented, adopted and planned measures, Slovenia will decrease its greenhouse gas emissions by 2010 by approximately 4% without LUCF.   When allowable sinks are included in the with additional domestic measures projection, Slovenia’s emissions decrease by 8% relative to the base year, and Slovenia is therefore projected to meet its Kyoto target in the first commitment period.

Although allowed sinks are estimated at 1.68 Mt CO2 annually for 2008-2012, a conservative assessment of sinks exploitation has been made at 0.84 Mt CO2 since they need to be presented as the consequence of direct human activities in order to be used as a fulfilment of country's obligations.  It is this 0.84 Mt CO2 saving from sinks that gives the emission reduction of 8% relative to the base year for the additional domestic measures projection.